Module 11

Appendix C

Structural Collapse

Operational Checklist

STRUCTURAL COLLAPSE OPERATIONAL CHECKLIST

I.	ESTABLISH COMMAND

_____	Communicate the exact type of incident.

Type:

_____	Communicate the exact location of the incident.

Location:

_____	Communicate the staging areas for incoming apparatus.

Staging Area:

_____	Communicate the name of the incident.

Name of Incident:

_____	Communicate the location of the command post.

Location of Command Post:

II.	DETERMINE THE EXTENT OF THE INCIDENT

_____	Establish hazard and safe zone and evacuate the area.

_____	Remove surface casualties and walking wounded if safe.

- Number Injured (Triage)  _____	Minor Injuries  _____

- Deceased  _____			Delayed 	 _____

Immediate  	 _____

_____	Number of people trapped:  _____.

- Location(s):

1.

2.

3.

4.

5.

_____	Number of people missing:  _____.

- Last known location(s):

1.

2.

3.

4.

5.

II.	DETERMINE THE EXTENT OF THE INCIDENT (cont'd)

_____ 	Type of structure	___	Residential.

Description:	___	Apartment House.

___	School.

___	Hospital or Care Facility.

___	Multi-Story/# Floors: _____.

___	Mercantile.

___	Commercial.

___	Industrial.

_____	Type of construction	___	Wood Frame.

Description:	___	Unreinforced Masonry.

___	Reinforced Masonry.

___	Concrete Tilt-Up.

___	Concrete.

___	Steel Frame.

_____	Is further collapse possible?	___	Yes	___	No

_____	Are utilities secured?

- Gas	___	Yes	___	No

- Electric___	Yes	___	No

- Water	___	Yes	___	No

_____  Do atmospheric hazards

exist? 	___      Yes      ___     No

III.	DEVELOP INCIDENT ACTION PLAN AND MANAGEMENT ORGANIZATION

A.	Requesting Additional Resources

___	First Alarm--Apparatus

___	Second Alarm--Apparatus

___	Third Alarm--Apparatus

___	Fourth Alarm--Apparatus

___	Fire Strike Team(s)

(Basic Rescue)

___	Light Rescue Strike Team(s)

(Wood Frame)

A.	Requesting Additional Resources (cont'd)

___	Medium Rescue Strike Team(s)

(Cinderblock, two-story URM,

heavy timber, tilt-up)

___	Heavy Rescue Strike Team(s)

Type 1 & 2 construction, steel

structures, confined space)

___	Medical Strike Team(s)

Level of response ____

___	Urban Search and Rescue Task Force	 TF-

62-Member/Self-sustaining 72 Hours      TF-

Search/Rescue/Medical/Technical Team TF-

Heavy Concrete/Technical Rescue	 TF-

___	Specialized Resources:		
	(See directory)

___	Staging Area/Base Considerations:

- Is the area large enough?	___ Yes	___ No

- Is security needed?	___ Yes	___ No

- Is a med./rehab area needed?	___ Yes	___ No

- Are feeding operations needed?	___ Yes	___ No

- Are toilet facilities needed?	___ Yes	___ No

- Are rest accommodations

needed?	___ Yes	___ No

- Is logistical support for equipment

repair and rehabilitation needed?	___ Yes	___ No

___	Communications Network:

- Channel  _____  Designator

- Channel  _____  Designator

- Channel  _____  Designator

- Channel  _____  Designator

- Channel  _____  Designator

- Channel  _____  Designator

- Channel  _____  Designator

B.	Structure Information

_____	Utilize structural engineers for assistance.

_____	Type of collapse pattern(s).

_____	V-collapse.

_____	Lean-to-collapse.

_____	Unsupported lean-to-collapse.

_____	Cantilever.

_____	Pancake collapse.

____	Locate building owner or occupant(s) to obtain additional information concerning structure layout.

_____	Utilize building blueprints or drawings if available.

_____	Today's Date:  _____	Time: _____   Day of Week: _______

_____	Structure size:

- Number of stories:  ___________.

- Size/length: ____ (ft); Width: ____ (ft); Height: ____  (ft).

_____	Construction type and materials.

_____	Attempt to establish location of voids.

C.	Structure Entry

_____	Establish search plan.

_____	Have personnel report to check-in officer (note time).

_____	Brief personnel on search plan and possible location of victims and void spaces.

_____	Have all personnel check safety equipment.

_____	Designate communications channel and call sign.

_____	Access best entry route.

_____	Utilize search markings as needed.

_____	Establish a Rapid Intervention Team and stage within the inner perimeter.

D.	Scene Safety

_____	Remove unnecessary civilians and personnel from area.

_____	Establish a perimeter:

Inner area: ____________________________________________

Outer area: ____________________________________________

_____	Designate:

- Safe travel zones.

- Danger zones.

- Overhead dangers.

- Hazardous areas.

- Restricted areas.

- Areas of retreat.

- Rehabilitation areas.

- Triage area.

_____	Designate:

- A restricted air space over the incident.

- Heavy equipment staging area--(beware of noise and vibrations).

- Equipment staging, work area, cutting station.

_____	Secure utilities:

____ Gas.

____ Electric.

____ Water.

_____	Provide exterior shoring and cribbing where needed to prevent further collapse and stabilize the structure.

_____	Locate points of entry and designate same with building markings which should include date, time, company, and specialized hazards.

_____	Assign an entry officer to the inner perimeter "Hot Zone" and restrict access to authorized personnel only.  Establish an entry and exit time log.

_____	Utilize an emergency evacuation signaling system.  All personnel who enter the inner perimeter should be familiar with these signals:

- Cease operations/all quiet--1 long blast (3 seconds).

- Evacuate area--1 short blast (1 second each).

- Resume operations--1 long and 1 short blast.

E.	Exploration of voids

___  	Shore and crib as needed for safety.

___  	Establish emergency evacuation routes (keep clear).

___  	Utilize selective debris removal, cutting and breaching.

___  	Test for toxic and explosive gases as needed.

___  	Establish oxygen percentage: ___________.

___  	Identify hazards and mark same.

___  	Follow OSHA confined space guidelines as applicable.

___  	Provide positive pressure ventilation or utilize SCBAs where needed.

___  	Be aware of fatigue of rescue personnel.

- Check entry times.

- Rest, rotate and rehabilitate as needed.

F.	Victim Priorities

Once the victim(s) has/have been located:

___	Attempt to make contact. Establish their:

- Mental status.

- Medical condition.

- Environment.

___	Establish extrication procedures.

- Consider what is trapping the victim(s).

- Consider possible load shift, secondary collapse or aftershock.

- Consider the victim's status and environment.

___	If multiple victims, evaluate the following:

- Survivability.

- Extrication difficulty.

- Estimated extrication time.

- Prioritize victim extrication based upon these categories.

____	Once patient contact has been made:

- Consider crush syndrome and treat for same.

- Treat and package the victim for removal.

G.	Situation Status

_____	Conduct situation briefings.

_____	Examine problems.

_____	Develop strategies.

_____	Re-establish priorities as needed.

_____	Request additional resources as needed.

_____	Medical control.

_____	Utilize and control volunteers.

H.	Incident Wrap-up

___	Establish that areas have been searched and all victims have been removed from the structure.

___ 	Develop plan to retrieve victims' personal items.

___      Stabilize structure and provide security.

___	Rest and rehabilitate all personnel.

___	Debrief all personnel.

___	Demobilize all equipment and apparatus.

___	Maintain accurate records regarding actions taken, times and personnel, equipment, apparatus, and agencies involved with the incident.

___	Conduct critical incident stress debriefing as needed.

___	Critique the incident.

___	Follow up with both victim (s) and rescuer status checks.

___	Document response activities.