Module 11:

Structural Collapse Simulation Exercise

Objective

Upon completing the module, you will be able to effectively manage an incident involving a structural collapse.

Methodology

This module uses lecture, discussion, and a simulation exercise.

NFPA 1021 Standards Cross-Reference Matrix

3-6, 3-7, 4-6, 4-7

Overview of Structural Collapse

Causes and Complexities of Structural Collapse

Collapse can occur while we, the fire department, are already operating on the scene, while we are en route to a scene, or may occur and be the reason we are sent to the scene.

Structures collapse from a variety of causes. Over 500 major collapses occur in the United States each year. Collapse can be caused by various factors, including explosion, construction accidents, structural deterioration, transportation accidents, and natural disasters.

### Fire or Explosion

Fire commonly causes structural collapse by destroying building materials and weakening support elements. Explosions cause rapid collapse due to blast forces on the structure.

Collapses that occur because of fire damage can vary from those that are localized and minimal to those that are large and catastrophic. A collapse that occurs during firefighting operations not only creates an additional problem for civilians that are still in the building, but also for the firefighting forces that are working inside and out. In addition to dealing with the collapse, firefighting operations will have to continue as well.

Incident Commanders (ICs) may have to make a change in the overall strategy, deal with collateral damage to rigs and other buildings, and be concerned with the additional weight that continued firefighting operations is adding to the already weakened building.

A localized collapse can involve something as simple as a section of drywall falling from a wall or ceiling to a roof, ceiling, or floor, assembly suffering partial failure.

Fire can destroy or damage structural members to the point of failure that can bring down walls, roofs, floors, or the entire building. Heating of structural elements can cause deformity that results in major collapse of roofs, floors, or the entire building.

### Construction Accidents

These may result from design problems, overloading, or poor construction. They may occur during or after construction has been completed and when the structure is occupied or in use.

### Structural Deterioration

As structures age, structural weaknesses develop from building material failure, settling, or other factors. A minor causative event may result in collapse.

### Transportation Accidents

Vehicles, trains, ships, and aircrafts may crash into structures. When structures collapse in these incidents, the complexity of the response increases.

### Natural Disasters (Earthquakes, Hurricanes, Tornadoes, Floods, and Landslides)

Structures react differently to stresses caused by earth movement, water, or wind forces, thus creating different rescue problems and strategies. These events may cause widespread damage with multiple-site rescues and large numbers of victims.

Specific Causes and Factors

### Natural Disasters

Earthquakes, hurricanes, tornadoes, floods, mudslides, high winds, snow, heavy rainfall, tsunami, ocean waves, ground subsidence, and landslides may cause major damage to structures, numerous victims and hazards, and the infrastructure over a large geographic area.

### Wall Failure

Supporting walls can collapse from foundation failure, deterioration, and vertical and horizontal stresses. These collapses most commonly create lean-to void spaces.

### Overloaded Floors

The most common causes of failure from heavy loading are heavy machinery and equipment. Additional loads tremendously stress the beams, and even slight impact loads applied to the floor may cause beam failure. Victims may be found in V-shaped or lean-to void spaces, or pinned under stock and debris.

### Overloaded Roofs

Possible causes of collapse are excessive weight, such as air handling units, large signs, heavy snow or rain, and improper or blocked drainage ducts. Roof failure may cause partial or entire roof pancake collapse and wall collapse.

### Column or Arch Failure

If a column supporting a beam joint fails, or an arch fails, collapse will occur, sometimes with little warning. These collapses generally create V-shaped voided spaces.

### Structural Weakness

This problem is one of the most difficult to detect because the majority of the structural elements vital to the building’s stability may be hidden by interior or exterior coverings such as plaster and siding materials. Exposure to weather, shaking from earthquakes, constant vibration, and neglect cause structures to deteriorate rapidly. Collapses in already weakened structures may result in a variety of conditions and voids.

### Improper Alterations

This is a common cause of structural collapse in the urban environment. These alterations may result in the removal of vital supporting materials or the addition of and installation of inadequate and nonengineered structural materials, thereby changing and weakening structural strength and stability.

### Fire-Weakened Structural Members

This is a common cause of structural collapse during or after a fire. Buildings that have suffered previous fire damage and have not been repaired face collapse hazards from other factors such as high winds, shaking forces, and heavy rain.

### Explosions

Leaking natural or propane gas may accidentally cause explosions, as can explosive devices. Explosions may cause the collapse and demolition of an entire building and damage surrounding structures. The force of the blast subjects the structural members to extreme stresses and may cause extensive amounts of debris and demolished construction materials to be blown throughout the inside of the structure and into the surrounding outside area. The number of survivable void spaces may be limited, and victims may be found anywhere in the debris. Injuries related to explosions generally are severe.

### Collision Impact

Structural collapse may be caused by collision from various types of transportation vehicles or heavy equipment. Generally, the collapse area is localized but may be complex due to the victim locations in the vehicle, victims in the structure, and the potential for spillage of fuel and cargo.

### Progressive Collapse

Progressive collapses are chain reactions caused by the collapse of one structure or part of a structure onto another structure. Walls, floors, or entire buildings may collapse progressively in domino fashion.

Examples of Structural Collapse

The following examples of major incidents occurred during the 1980’s and 1990’s. Many of these incidents focused national attention on our capability to manage and perform search and rescue operations at structural collapse incidents. That focus has resulted in improvements in construction techniques and response capability.

### Harbor Cay Condominium Collapse (Cocoa Beach, Florida, 1981)

This building was under construction at the time of collapse. Heavy floor and wall construction consisted of pre-cast reinforced concrete slabs and cast-in place concrete components. All five floors and the roof of the condominium collapsed in a pancake configuration, trapping a large number of construction workers. Eleven were killed and 23 injured. The incident involved more than 60 hours of continuous rescue operations and resources from 5 county fire districts, 16 municipal fire departments, and a response of Civil Defense, military, and private sector technical specialists.

### Hyatt Regency Skywalk Collapse (Kansas City, Missouri, 1981)

During a large social event at the hotel, two suspended walkways, which were overloaded with people, collapsed from 50 feet above the atrium, leaving 113 people dead and 186 injured. The suspended walkways, constructed of structural steel and lightweight concrete, spanned 120 feet across the atrium space above hundreds of people on the floor below. The high number of dead and injured, the location of the collapse, the size of the collapsed material, and the ineffectiveness of the typical emergency service tools created severe rescue limitations. The incident required a large number of medical personnel working alongside the rescuers. Twenty-nine live victims were removed from under the debris. Heavy rigging and construction specialists and heavy equipment were needed to remove the debris during rescue operations.

### Mexico City Earthquake (Mexico, September, 1985)

The devastating Mexico City earthquake caused the collapse of more than 264 major structures (many were 10 to 18 stories tall) and widespread damage and partial collapse of more than 7,000 smaller structures. The quake also had a major impact on the city’s infrastructure, causing many problems for the responders as well as the victims. It is estimated that more than 20,000 of the 18 million residents of the city were killed, 30,000 injured, and more than 300,000 left homeless. This incident clearly focused the attention of the world on the problem of collapsed structure search and rescue and major incident management. It also brought to light the high degree of risk and danger associated with collapse rescue operations when more than 100 rescue personnel died during rescue operations (the majority were killed in a major aftershock that caused additional collapses).

Numerous other incidents involving structural collapse occurred during the 80’s. They challenged and expanded the concepts of Urban Search and Rescue (US&R) operations and incident management. Among these incidents are the following:

- Hurricane Alicia, Texas, 1982;

- Propane explosion and collapse, Buffalo, New York, 1983;

- Coalinga earthquake, Coalinga, California, 1983;

- L’Ambiance Plaza collapse, Bridgeport, Connecticut, 1987;

- Whittier Narrows earthquake, Los Angeles area, 1987;

- Department store collapse, Brownsville, Texas, 1988;

- Armenia earthquake, Soviet Armenia, USSR, 1988;

- Building collapse, West 31st Street, New York City, 1988;

- Loma Prieta earthquake, San Francisco Bay area, 1989;

- Hurricane Hugo, east coast, 1989;

- San Bernardino train derailment and structural collapse, California, 1989.

The 1990’s were a decade of continuous challenges in structural collapse rescue emergency management. This also has been the decade of the most advances in these operations, with improved tools and equipment, search and rescue techniques, safety requirements, training, additional resource capabilities and coordinated response, and the expanded use of the Incident Command System (ICS). Some examples of significant incidents are:

- Explosion and collapse of the Crested Butte State Bank, Colorado, 1990;

- Hurricane Andrew, southeast Florida, 1992;

- Explosion, World Trade Center, New York City, 1993;

- Northridge earthquake, Los Angeles area, 1994;

- Explosion, Murrah Building, Oklahoma City Oklahoma, 1995;

- Kobe earthquake, Kobe, Japan, 1995;

- Explosion, Humberto Vidal Building, Puerto Rico, 1996;

- Tornadoes, southeast Michigan, 1997;

- Tornadoes, Atlanta, Georgia, 1998.

Structural collapse incidents may be considered low probability but high consequence events. The rescue of trapped victims may be both complex and dangerous, involving the response of various levels of capability in a time-critical situation to locate and remove trapped or injured victims safely.

Hazards

Structural collapse creates many hazardous conditions for the rescuers who may suffer injury, illness, psychological problems, or even death. A hazard is anything presenting a risk or danger to the rescue effort. The best methods to reduce the risks of injuries or illnesses during the rescue operation are prevention and avoidance. Rescue personnel can reduce the threats inherent in rescue operations through knowledge and awareness of potential hazards. Types of potential threats that may be encountered by the responder are discussed below.

Physical

Two common threats that can be encountered are atmospheric contamination and the changes in temperature that affect bodily functions.

Atmospheric contamination may involve a toxic or flammable condition or a reduced level of oxygen, sometimes found in confined spaces. The proper safety equipment, air monitoring devices, and operational procedures are necessary in these environments.

The human body functions efficiently within a narrow temperature range. During rescue operations responders may be exposed to cold or heat over a prolonged time, or may experience a rapid increase or decrease in temperature that affects normal bodily functions.

In many rescue operations, dehydration is a problem that affects both victims and rescuers. An environment does not have to be dry for dehydration to occur. Adequate fluid intake prevents dehydration.

Adequate nutrition may be a problem in long-term rescues. Food is fuel for body functioning, and inadequate nutritional intake can impair performance. Physical exertion and the stress of rescue operations require high-energy output. A high-energy output with a lack of readily available energy may result in weakened physical status, reduced coordination, irritability, or increased susceptibility to hypothermia.

Overeating, which stresses the digestive process, can reduce both physical and mental performance. Adequate nutrition can be provided through a supply of high-energy foods and the consumption of small meals on a periodic basis.

Good physical conditioning, in both strength and stamina, is important for the rescuer because it allows prolonged exertion and the peaks of power needed to perform rescue activities. Emergency responders should participate in regular exercise programs that promote conditioning.

Medical

Medical threats may include pre-existing conditions such as heart disease, lung disease, or diabetes. These conditions certainly pose a serious threat to the rescuer and to the rescue operation. Responders with known pre-existing medical problems should not participate in rescue operations.

Short-term medical problems such as headaches and stomach disorders or minor cuts and scrapes may be more of an annoyance than an emergency, but could become debilitating. If not treated, the problems may take the responder out of action. Short-term medical problems should be taken care of before they become debilitating. In either case, we do not want the rescuers to become part of the problem. They are part of the solution, and must be able to perform at their highest capacities.

Environmental

Extreme working environments caused by cold, wet, or hot weather affect the human body and can cause hypothermia and heat exhaustion. Confined spaces or enclosed areas also may cause similar problems. Body core temperature differences of only a few degrees cause bodily malfunctions in such areas as thinking, judgment, and coordination. As the temperature moves away from the normal range, dysfunction increases, and unconsciousness or even death may result.

Hyperthermia is caused by exposure to heat. Increased body temperature may result in heat exhaustion or stroke, which may be fatal if not treated.

Hypothermia is caused by exposure to cold and results in the lowering of the body core temperature. The body’s neurological and psychological systems are affected, causing impaired mental functions, loss of coordination, unconsciousness, and possibly, death.

Exposure to chilling winds or water may cause frostbite, which, if prolonged, freezes and destroys body tissue. Susceptible areas are feet, hands, ears, and nose.

The danger of hazardous materials released in a structural collapse should be considered a factor at nearly every incident. The type of occupancy is a factor on assessing that risk, as is the availability of monitoring equipment. Product identification is critical in determining the methods used to respond and to mitigate the problem.

Biohazards from body fluids are a hazard to rescuers working around injured or deceased victims. Suitable precautions should be taken to protect the rescuer.

External

External threats include those caused by terrain, unstable surfaces, electrical shock, and falling objects. The risk of falling is also an external threat.

Terrain may present major obstacles to search and rescue operations, such as distance to site, access, obstacles, and hazards. These factors may increase the time to reach the rescue site and result in extreme physical exertion for rescuers.

Collapsed structures may present the risk of falling, which could result in injury or death. Working in elevated situations may require the use of barrier lines to prevent access to dangerous edges, the use of safety lines to belay persons at risk of falling, wearing safety equipment, and the designation of an Incident Safety Officer (ISO).

Unstable surfaces that must be negotiated may present a potential for a fall or loss of control. Unstable surfaces may not have the strength to support weight (that is, the pounds per square inch (psi) of the supporting surface is less than the psi of the weight of the rescuers). Any questionable or untested surface must be considered hazardous. These areas must be identified and either avoided or made safe. Slippery materials such as water or oil on a concrete or metal surface also may cause unstable surfaces. These problems may be mitigated by identification, using safety equipment, avoidance of the area, reducing exposure in the area, or removal of the hazard.

Electrical current in wires or equipment may cause electrical shock. Collapsed structures or stand-alone electrical hazards should be de-energized or avoided by establishing a danger zone around the hazard.

Lightning may be a threat to rescuers in certain areas. Develop a safety plan to include criteria for suspension of rescue operations, avoiding metal surfaces, projections, and conductive surfaces.

Falling Objects

Areas where rescuers are most likely to encounter falling objects, such as around an unstable collapsed structure, should be designated a collapse hazard or fall zone. Personnel should be observant and wear safety equipment. A safety officer and assistants may be used to warn of danger or to assist in the mitigation of potential falling objects by securing or removing them. All persons working in the hazard area should understand signals and escape routes.

Avalanches and Landslides

Avalanches or landslides may cause or threaten to cause structures to collapse, thereby placing rescuers in potentially dangerous situations on the lower side of the slope. Information, communication, and emergency plans are critical for this type of rescue.

Psychological

Psychological impacts from the rescue operations may include stress or anxiety from the fear of heights (acrophobia) or of enclosed spaces (claustrophobia). Rescuers need to recognize the signs and symptoms, communicate concerns, and obtain professional guidance.

Stress during and after (delayed) rescue operations may have many causes, and those with stress may exhibit many symptoms such as irritability, chronic fatigue, difficulty sleeping, and changes in social behavior. It is important for agencies to have a Critical Incident Stress Management (CISM) program in place before an incident.

Safety Considerations

Structural collapse results in many unsafe conditions for the rescuer and victim. Possible safety issues that may be encountered consist of:

- Unstable rescue areas (creating the possibility of secondary collapse);

- Confined spaces;

- Flammable or toxic hazards;

- Oxygen deficient atmospheres;

- Ignition sources; and

- Sharp, irregular, or unstable surfaces.

Safety considerations are a high priority in the response and management of a structural collapse incident. The following five issues should always be considered when you are involved in such an incident.

Safety Starts with Preplanning and Training

Understanding the causes and hazards of collapsed structures provides the knowledge needed to develop Standard Operating Guidelines (SOGs) and training programs for the rescuers. An integral part of the preparedness phase is safety. Identify equipment requirements that provide the level of safety needed for the rescuers in various types of potentially hazardous environments.

### Use the ICS

The ICS provides an effective, all-hazard incident management tool that incorporates safety into the organization, from the responsibility of the IC to the ISO, to all personnel in the incident organization. Responsibility and accountability are major components of the ICS.

### Provide an ISO, a Safety Plan, and a Rapid Intervention Crew (RIC)

Risks must be reduced and managed through a variety of methods, including the establishment of safety plans which may be a written part of the Incident Action Plan (IAP), the designation of a safety officer and assistants, where needed, who have direct responsibility and authority for scene safety, and the deployment of an RIC to be on immediate response standby at the incident for contingencies involving the rescue of response personnel.

### Use a Personnel Accountability System

The location of response personnel during search and rescue operations is critical to their safety. It becomes the individual’s responsibility to make sure supervisors are aware of his or her location, and the responsibility to know where subordinates are at any given time. Good discipline, training, and communications essential to the safety of the rescuer in these hazardous environments. An adequate accountability system must also be in place.

### Require Protective Clothing and Equipment

The level of protection must be determined for the hazard before rescuers enter the hazard zone.

### Rescue Risk/Benefit Ratio

Incident information must be analyzed to determine the best rescue risk-to-benefit ratio.

Response Versus Recovery Operations

Many times, structural collapse incidents result in survivors as well as deceased victims. The priority of efforts should be directed toward the safe location and removal of the live victims. Some incidents result in the death of all those in or around the structure. Survivability factors change our priorities and the margin of the risks we take versus the benefits of the resultant rescues.

Response Operations

Response operations involve the search for, and rescue of, live victims. Many times the rescue operation involves “the delicate application of force” to extricate victims safely and quickly.

Time is a critical factor in the survivability of the trapped or injured victim. The initial strategy should include a time factor for completing various rescue objectives commensurate with deployment of adequate resources. Victim condition, viability, and location are all factors in determining response priorities.

Risk/benefit and safety are major considerations in the response phase. Hazard identification and the development of a risk management plan are essential parts of developing strategy used in the response operation. Risks to the rescuer must be minimized or eliminated if possible. A decision not to send rescuers in may have to be made in order to ensure the safety of rescue personnel. A thorough size-up is needed, and intelligence is vital to the development of the response plan.

The general strategy in response is:

“Do the greatest good for the greatest number in the shortest period of time.”

Recovery Operations

Recovery operations involve the removal of the deceased victims, as well as personal items and equipment, from the structure.

Time is not a critical factor. Taking additional safety precautions, such as adding more shoring or using heavy equipment for debris removal, reduces the risk level to the rescuer. These tactics help protect the rescuers from potential hazards by making the rescue site safer to work in.

The decision to move to the recovery phase from the response phase of operations may be difficult to make without very accurate information about the victims in the structure. Victims trapped in structural collapses have survived in void spaces for up to two weeks.

Recovery of personal items, important materials, and equipment from a structure after the removal of both live and deceased victims should be organized using a specialized task force or group, including the fire department for access and safety, the police department for identification of ownership and security, and public works or private contractors to assist with structural stabilization. The IC should prioritize sites or areas for recovery.

A CISM program should be in place. A major factor causing stress for the rescuer is the body recovery operation. A pre-briefing and a defusing of rescue personnel should be strongly considered along with limited exposure in the area and adequate rehabilitation.

Law enforcement and the coroner are involved in the recovery and investigation. These incidents require effective coordination, the preservation of evidence, and a logistics system that may be required to support a long-term operation (more than two weeks).

The final stages of the recovery operation involve the stabilization of the structure and providing security for the site. Stabilization may involve the fire department, structural engineers, and private contractors with heavy equipment and materials to complete the task. The local police or private security services, depending on the type of occupancy and ownership, should handle site security. A crime scene would have different and stricter requirements.

Introduction to Response Resource Capability

The IC at a structural collapse incident must be able to identify the type of collapse that occurred and the hazards to rescuers and victims. He or she must be able to match the appropriate level of rescue capability with the problem.

There are many levels of capability, including spontaneous volunteers, standard fire companies, heavy rescue squads, and technical rescue teams, (for example, confined space, the Federal Emergency Management Agency [FEMA], and National Urban Search and Rescue [US&R] Task Forces). There are many agencies that may respond to a structural collapse. This type of incident is normally a multi-agency response because of many tasks that must be accomplished (for example, medical, law enforcement, and heavy equipment resources).

Structural collapse resources may be available from:

- Local jurisdictions,

- State and Federal governments,

- Members of the private sector, and

- Trained civilian volunteers.

Government Levels of Response

Local Level

The local level includes trained volunteers, standard fire companies, fire companies with specialized equipment, technical rescue, and heavy rescue teams or companies.

Regional Level

The regional level provides access to resources from surrounding jurisdictions through a mutual-aid or contract.

State Level

Resources may include specialized teams, equipment, task forces, and the National Guard.

Federal Level

The Federal level includes FEMA, Task Forces, the military, and many other supporting agencies. (There are 28 FEMA Task Forces.)

International Level

Response may involve search and rescue teams of varying capabilities from other countries.

The IC must know what resources are needed and how to manage and coordinate those resources.

Risk/Hazard Analysis

The fire service has a key role in collapsed structure response and should have the capability to respond effectively to the various types of incidents encountered. The question is, “What type of capability is needed in each community?” This question may be answered by performing a risk/hazard analysis.

After a specific incident, many departments from technical rescue teams have shown a deficiency or inability to handle the rescue safely and effectively. In some cases, a team is developed before a major rescue incident occurs due to the expectation of emergencies created by risks in the community.

In determining whether a team is needed in your community, you must first do some research to evaluate the risks in your area. A risk analysis helps you to determine the level of risk and potential hazards so that you can decide whether a team is really needed. This is particularly important for two reasons. First, political leaders want to know what risks exist to justify funding a team. Second, you want to know what risks confront your department, what type of hazardous scenarios to train for, and what rescue equipment is needed to address the risks. A thorough risk analysis should define your objectives and justify the effort of forming a team.

Risk/Hazard Analysis Elements

A risk/hazard analysis involves the following elements.

### Performing a Risk Assessment

A risk assessment is based on historical data plus analysis of newly introduced hazards and potentially high-risk problems. Begin by assessing past rescue needs in your response area. You may look at incident reports to determine frequency, incident type, and location. Other potential sources of data include your state worker’s compensation office, state and national Occupational Safety and Health Administration (OSHA) offices, construction and contractor’s associations, building officials and inspectors, and safety managers at local businesses. Experience may indicate the likelihood of technical rescue type incidents during major construction projects.

Regardless of the size or economic makeup of the community, almost every jurisdiction is subject to some kind of risk.

### Considering Target Hazards

A department faces specific risks each day. You also must consider target hazards in your response area, or those you anticipate in the future. Target hazards are specific risk areas that confront your department in a rescue emergency that have a high potential for life loss or injury. Make a list of target hazards that present special rescue challenges requiring special training and equipment.

A hazard analysis may be used to assist in the development of findings and conclusions using a scoring system based on four criteria:

- History,

- Vulnerability,

- Maximum threat, and

- Probability.

### History

The history or the record of previous emergencies is important in hazard analysis. A past record of incidents indicates a predisposition for the same kinds of problems in the future. Unless specific conditions no longer exist or they have been substantially reduced or mitigated, similar emergencies may happen again.

### Vulnerability

Vulnerability includes all persons who may be killed, injured, or contaminated and all property that may be destroyed, damaged, or contaminated by an incident. Determining the number of people and the value of property may be identified in vulnerability descriptions, for example, power plants, hospitals, the aged, the handicapped, and children.

### Maximum Threat

Maximum threat is the worst-case scenario of a hazard. In determining this factor, assume both the greatest impact (for example, a maximum credible earthquake in an urban center on a weekday during business hours). A maximum threat may also involve secondary threats such as haz mat problems.

### Probability

Probability is the likelihood that an event will occur. It can be expressed as the odds that an incident may occur during a given period, such as the odds are 1 in 100 that it will occur in any given year. There is a correlation between historic data and probability; however, recent development of new hazardous conditions in a community may increase probability over that indicated by history alone.

### Rating System

A numbered weight or percentage may be given to each criterion, and a rating of low, medium, or high may be assigned to each criterion for each hazard.

The list of hazards can be ranked by the product of the criterion weights and ratings. The highest-ranking hazards should receive priority consideration.

### Analyzing Data

Developing a frequency rate projects the likelihood of a technical rescue emergency. To demonstrate the likelihood of a technical rescue incident, the frequency and incident type must be shown over a given period in the community involved or in adjacent communities with similar problems. A potential collapse incident and associated hazards must be weighted heavily in the analysis.

For example, confined space incidents may be projected in the future by estimating the number of confined spaces in future years, and then multiplying this by the rate of current incidents per confined space.

### Establishing a Risk Threshold for the Hazard

Establishing a risk threshold reflects response capability for equipment and trained personnel. It is the final determination in weighing the potential risk to the community and the potential risk to emergency responders. Each community must decide what an acceptable level of risk is and the threshold that necessitates the formation of a special rescue team. The community and city administrators should know exactly what the fire department’s rescue capabilities and limitations are, what risks confront the community, and the dangers that rescuers face in performing rescues.

### Determining the Type of Team (or Capability) Needed to Respond

The risk analysis should help in determining whether a team is needed. The next step is determining the kind of team for the particular type of hazard. Will the team handle only basic rescue or will it be expected to perform complex rescues? The level of required capability must be developed. The response capability may come from within the jurisdiction, from a single agency or multiple agencies, or from mutual aid or contract with other jurisdictions or agencies in the region. Response time is a critical consideration in this assessment.

Levels of Operational Capability

Operational capability refers to different types of operations and resource deployment. They are defined in the National Fire Protection Association (NFPA) Standard 1470, Standard on Search and Rescue Training for Structural Collapse Incidents. This standard establishes levels of operational capability based on the degree of hazard and training level of personnel and the availability of resources. Although this standard has been superseded by NFPA Standard 1670, Standard on Operations and Training for Technical Rescue Incidents, 1999, the levels may be used as a guide in developing levels of capability. Most of the information in NFPA 1470 has been incorporated into NFPA 1670.

Today, most fire departments would be able to perform at the basic and light operational levels and many at the medium level of capability. Departments need to determine their levels of capability using this standard as a guide.

Levels of operational capability should be established within each jurisdiction to conduct search and rescue operations safely and effectively.

Established levels are based on hazard and risk assessment, training level of personnel, and availability of internal and external resources.

Basic Operational Level

The basic operational level represents the minimum capability to conduct safe and effective search and rescue operations at noncollapse incidents. Personnel at this level shall be competent at surface rescue that involves minimal removal of debris and building contents to extricate easily accessible victims from noncollapsed structures.

Light Operational Level

The light operational level represents the minimum capability to conduct safe and effective search and rescue operations at structure collapse incidents involving the collapse or failure of light-frame construction, and basic rope rescue operations.

Medium Operational Level

The medium operational level represents the minimum capability to conduct safe and effective search and rescue operations at structure collapse incidents, involving the collapse or failure of reinforced and unreinforced masonry, concrete tilt-up, and heavy timber construction.

Heavy Operational Level

The heavy operational level represents the minimum capability to conduct safe and effective search and rescue operations at structural collapse incidents involving the collapse or failure of reinforced concrete or steel-frame construction, and confined space rescue operations.

Four Levels of Operational Capability—Minimum Training

### Basic Operational Level

The basic operational level represents minimum capability to operate safely and effectively at noncollapse incidents. Personnel at this level shall be competent at surface rescue and rescue involving minimal removal of debris and building contents to extricate easily accessible victims from noncollapsed structures. Rescue operations would include removal of victims from under furniture and appliances and from the surface of a debris pile.

### Light Operational Level

Personnel shall meet all basic level training requirements. In addition, personnel shall be trained in hazard recognition, equipment use, and techniques required to operate safely and effectively at structural collapse incidents involving the collapse or failure of light-frame construction, and basic rope rescue as specified below.

Personnel shall be trained to recognize the unique hazards associated with the collapse or failure of light-frame construction. Training should include, but not be limited to the following:

- Recognition of the building materials and structural components associated with light-frame construction;

- Recognition of unstable collapse and failure zones of light-frame, ordinary construction; and

- Recognition of collapse patterns and probable victim locations associated with light-frame construction.

Personnel shall have a working knowledge of the resources and procedures for performing search operations intended to locate victims who are not readily visible and who are trapped inside and beneath debris of light-frame construction. Training should include, but not be limited to the following:

- Types of search resources. US&R dogs, optical instruments (search cameras), seismic/acoustic instruments (listening devices);

- Capabilities of search resources; and

- Acquisition of search resources.

Personnel shall be trained in the procedures for performing access operations intended to reach victims trapped inside and beneath debris associated with light-frame construction. Training should include, but not be limited to the following:

- Lifting techniques to lift structural components (walls, floors, or roofs) safely and effectively;

- Shoring techniques to construct safe and efficient temporary structures needed to stabilize and support structural components to prevent movement from walls, floors, or roofs;

- Breaching techniques to create openings in structural components of walls, floors, or roofs safely and effectively; and

- Operating appropriate tools and equipment to accomplish the above tasks safely and efficiently.

Personnel shall be trained in the procedures for performing extrication operations involving packaging, treating, and removing victims trapped inside and beneath debris associated with light-frame construction. Training should include, but not be limited to the following:

- Packaging victims within confined areas;

- Removing victims from elevated or below-grade areas;

- Providing initial medical treatment to victims at the Basic Life Support (BLS) level at a minimum; and

- Operating appropriate tools and equipment to accomplish the above tasks safely and efficiently.

### Medium Operational Level

Personnel shall meet all light level training requirements. In addition, personnel shall be trained in hazard recognition, equipment use, and techniques required to operate safely and effectively at structural collapse incidents involving the collapse or failure of reinforced and unreinforced masonry concrete tilt-up, and heavy timber construction.

### Heavy Operational Level

Personnel shall meet all medium level training requirements. In addition, personnel shall be trained in hazard recognition, equipment to use, and techniques required to operate safely and effectively at structural collapse incidents involving the collapse or failure of reinforced concrete or steel-frame construction and confined space rescue.

Rescue Skills

Generally, rescue skills needed or performed in structural collapse incidents correspond to the type of victim entrapment.

About 50 percent of injured victims (not trapped) are rescued by spontaneous volunteer rescuers.

About 30 percent of victims in nonstructural and light entrapment may be rescued by trained community teams (Community Emergency Response Teams [CERTS]).

About 15 percent of victims entrapped in structural collapse void spaces are rescued by emergency service providers.

About 5 percent of victims entombed in a structural collapse are rescued by specialized heavy rescue teams.

Emergency responders must be able to control spontaneous volunteers who may be working at the scene upon responder arrival.

Supervising volunteers involves ensuring that certain basic procedures are followed:

- The incident or each situation is isolated;

- All information is evaluated;

- Tasks are delegated; and

- Needs are communicated.

The safety of volunteer workers is of primary importance and should not be compromised.

NFPA Standard 1670

NFPA 1670, Standard on Operations Training for Technical Rescue Incidents, pertains to levels of functional capability for conducting operations safely and effectively at technical rescue incidents including structural collapse. Standards for levels of proficiency have been developed, ranging from awareness to operations to technician. Each level has increasing operational requirements and functions. NFPA 1670 supercedes NFPA 1470

Rescue Resources

Fire departments across the United States have assumed a major role as primary responders to rescue incidents that involve, among other things, structural collapse, trench cave-ins, confined spaces, industrial and agricultural machinery, water emergencies, and people trapped above or below grade level. These emergencies are grouped into a category called technical rescue. Technical rescue incidents often are complex, requiring specially trained personnel and special equipment to complete the mission. Natural forces such as earthquakes, rain, temperature extremes, and swift water currents often complicate technical rescue incidents. The presence of flammable vapors and toxic chemicals also can increase the level of risk. The safety of crews conducting technical rescue operations is of special concern.

Fire and rescue departments throughout the country perform technical rescues on a daily basis. Some complex technical rescue incidents last many hours or even days, as rescue personnel carefully assess the situation, obtain and set up the appropriate rescue equipment, monitor scene safety, and remove hazards before they can finally reach, stabilize and extricate the victims. The presence of hazards such as flammable vapors or dust often forces rescuers to take additional precautions and time to ensure that operations are conducted safely. Experience has shown that hasty rescue operations can endanger the lives of both rescuers and victims. At the same time, rescuers know that a victim’s survival chances often are dependent on quick extrication and transportation to a hospital.

Some departments are prepared better than others to perform technical rescue operations. To deal with these complicated rescue operations, many fire departments have created special technical rescue teams. A technical rescue team is a specialized group of personnel having advanced training and special equipment to conduct specialized rescue operations safely and efficiently. The specialists and capabilities of individual teams vary greatly, depending on their members’ levels of training, number of trained personnel, and availability of specialized rescue tools and equipment. For example, some departments have the training and equipment to perform rescues at collapsed structures by cutting through concrete and removing heavy debris, while other departments are limited to working with picks and shovels to remove debris.

Many departments have single-discipline rescue teams such as a trench rescue team. These teams are trained and equipped to handle one type of rescue. Other departments have multidiscipline teams that are prepared to perform more than one type of rescue.

The formation of a technical rescue team, whether single or multidiscipline, requires careful planning, a long-term commitment from the team members, equipment research and acquisition, risk analysis, training and funding.

Technical Rescue Capabilities

### Confined Space Rescue

A confined space is an enclosed area with limited entry or egress, which has an internal configuration not designated for human occupancy, such that an entrant could become trapped or asphyxiated. It may have inwardly converging walls or a floor that slopes downward and tapers to a smaller cross section. These spaces include sewers, vats, caves, tanks, and other areas. Rescues from such spaces are dangerous, especially if the interior environment is toxic or oxygen deficient. OSHA terms these dangerous areas “permit required confined spaces.” OSHA estimates that there are over 240,000 such permit-required spaces across the United States.

### Collapse Rescue

This involves building collapse or other structural collapse, such as the collapse of various buildings in the 1994 Northridge, California, earthquake or the collapse of the elevated highway in Oakland, California, during the 1989 Loma Prieta earthquake. Many collapse rescue teams have been established in earthquake-prone areas. A collapse rescue capability is a necessity in most parts of the country, with both natural and human-caused collapses occurring with unusual regularity. Recent disasters and acts of terrorism have increased the need for heavy rescue capability.

### Trench/Cave-In Rescue

Trench or cave-in rescue could occur in almost any jurisdiction across the country. Trenches often are found in areas of new construction where pipes or cables are being buried or excavated for new construction footing and foundation work. The most common trench rescue scenario involves rescuing a construction worker trapped when the trench walls collapse.

### Rope Rescue

High-angle or low-angle rescues occur around cliffs, ravines, caves, mountainous areas, or high-rise buildings, communication towers, water towers, or silos. These rescues may require complex rope and hauling systems to secure personnel and extricate victims safely.

### Industrial Rescue

Industrial machinery presents many challenges to rescuers. Many industrial rescues involve confined spaces or heavy extrication to free victims trapped by machinery.

Structural Collapse Resources

Many different kinds of resources from both the government and the private sector are available for response to structural collapses:

- Local jurisdictions,

- State and Federal governments,

- Members of the private sector,

- Trained civilian volunteers, and

- FEMA US&R Task Forces.

Each resource may have varying capabilities and functions for the broad scope of search and rescue requirements. The key to effective resource use is to match the right resource to the job. This becomes problematic when dealing with difficult rescue situations where a very high level of capability is needed because of the scarcity of these resources throughout the country.

Government Resources

Government response can occur at the following levels:

- Local,

- Regional,

- State,

- Federal, and

- International.

Government resources may consist of fire department engine, truck, and heavy rescue companies; specialized US&R companies; and squads, confined space teams, high-angle rescue teams, technical rescue teams, canine search teams, technical search teams, state and FEMA task forces, and specialized medical teams. Most of these resources come from the fire service, but other agencies (such as public works departments and law enforcement) also may have such specialized resources to offer.

Support resources may include haz mat teams, Emergency Medical Services (EMS) responders, public works departments, law enforcement, the military, the Army Corps of Engineers, and technical specialists such as structural engineers.

### The National US&R System

The National US&R Response System, established under the authority of FEMA in 1989, is a framework for structuring local emergency services personnel into integrated disaster response task forces. These task forces, complete with the necessary tools and equipment and requisite skills and techniques, can be deployed by FEMA to rescue victims of structural collapse.

When the Federal government mobilizes resources and conducts activities to support sate and local response efforts to disasters, it does so under 12 Emergency Support Functions (ESFs). Each ESF is led by a primary agency that has been selected based on its authorities, resources, and capabilities in a particular functional area. FEMA is the primary agency for ESF #9, US&R.

After a request for Federal assistance from a governor is received and approved by the U.S. President, task forces may be activated or placed on alert to respond to a major disaster. The alerted task forces start locating personnel and organizing their mobilization. Each task force can be airborne and heading to its destination in a matter of hours.

Currently, there are 28 FEMA US&R Task Forces spread throughout the continental United States. These task forces are trained and equipped by FEMA to handle structural collapse, and encompass local emergency service personnel from 18 States. Any operational task force can be deployed by FEMA to a major disaster to provide assistance with structural rescue. Two task forces also have responded to several international disasters under the auspices of the U.S. Agency for International Development, Office of Foreign Disaster Assistance.

A FEMA US&R Task Force comprises 62 specialists and is divided into four major functional elements:

- Search,

- Rescue,

- Technical, and

- Medical.

Task force members include structural engineers and specialists in the areas of hazardous materials, heavy rigging, search (including highly trained search dogs), logistics, rescue, and medicine. By design, there are two task force members assigned to each position to allow rotation and relief of personnel during around the clock task force operations.

A comprehensive equipment cache totaling 58,000 pounds supports each task force. The cache elements sent to the disaster scene include communications, locating devices, rope, rigging, hauling, lifting, and pulling equipment. Shoring, structural movement sensing, victim extrication, cutting, and drilling devices are included to perform the often difficult assignments encountered by a FEMA US&R Task Force.

The medical team comprises four medical specialists and two physicians. Many of the medical specialists on US&R teams are both paramedics and firefighters, and thus have both rescue experience and extensive experience in pre-hospital medical care. Most of the physicians involved in US&R are emergency medicine specialists, and also have taken special courses in confined space medicine and crush syndrome.

The medical team is designed to bring the emergency department out to the field. It carries all of the Advanced Life Support (ALS) equipment available in any ALS ambulance. In addition to providing advanced emergency medical care in the field, it has training in hazardous materials, public health issues relevant to disaster management, confined space medicine, and other issues important to the function of the US&R team.

### Task Force Capabilities

Task force capabilities include the following:

- Physical search and rescue operations in damaged or collapsed structures;

- Emergency medical care for entrapped victims, task force personnel, and search canines;

- Reconnaissance to assess damages and needs, and to provide feedback to local, state, and Federal officials;

- Assessment and shutoff of utilities to houses and other buildings;

- Hazardous materials survey and evaluations;

- Structural and hazard evaluations of buildings needed for immediate occupancy to support disaster relief operations; and

- Stabilizing damages structures, including shoring and cribbing operations on damaged buildings.

Figure 11-1 illustrates the organization of a typical 24-hour, 62-member state/national US&R task force.

Figure 11-1: US&R State/National Task Force

### Private Sector Resources

Private sectors can provide resources such as construction and demolition contractors (with heavy equipment and various technical specialists), search dogs and handlers, structural engineers, trained volunteer and industry teams, (for example, confined space, high-angle rope rescue, and technical search), EMS providers, equipment rental and supply companies, utility companies, and trained Community Emergency Response Teams (CERTs).

It is important to know what resources are available from the private sector in each jurisdiction. A survey of these resources needs to be conducted as part of the response planning process in every agency with responsibility for structural collapse or technical rescue.

### US&R Resource Types

US&R resources have been typed according to the capability of the resource. This typing is based on the equipment carried on the apparatus and the training of the personnel.

- US&R resources are used by many fire departments for EMS and haz mat response. This same system is used for collapsed structure response.

- US&R Task Force: Heavy rescue capability, including search, medical, technical, and support components (multi-disciplined Technician Level).

- Type I (Heavy)— Heavy rescue company with heavy rescue capability (Technician Level).

- Type II (Medium)— Truck or rescue company with medium rescue capability (Operational Level).

- Type III (Light)— Engine or truck company with light rescue capability (Awareness Level).

- Type IV (Basic)—surface rescue.

### Tiered Response System

A concept called tiered response is used by many rescue agencies across the country. The concept is to train and equip personnel or units throughout a department to different response levels, or tiers, from a basic rescue level to an advanced rescue capability.

The basic premise of a tiered response system begins with training all personnel to a basic rescue awareness level that familiarizes them with rescue hazards, dangers, and some basic, practical rescue skills. In the event of a complicated rescue, they will request the response of an advanced team and initiate measures within their capabilities until the advanced team arrives. The tiered response system for technical rescue is similar to a tiered EMS response system that uses a basic emergency medical technician to initiate care until a paramedic arrives on the scene, or the similar system used for the haz mat responders.

At the top of the tier in US&R capability are the FEMA and State US&R Task Forces with mobile, multifunctional, 62-person teams equipped for the most complex and diversified rescue operations, yet reliant on support for transportation and sustained operations (beyond 72 hours) in the field. For many reasons, including cost, there are limitations on how many task forces can be fully developed. Therefore, a regional response capability is needed to provide the initial and more rapid response to various types of technical resources, no matter how complex they may be.

The first level of response is the spontaneous rescuer. Civilian volunteers, who are normally the first responders, can be trained to perform basic search and rescue operations such as the CERTs trained in Los Angeles and many other cities and countries throughout the country. The second level, light search and rescue, and third level, medium search and rescue, are made up of trained and equipped fire and rescue agencies.

The regionalization of US&R resources usually allows the formation and response of specialized teams that can perform at the medium or heavy level of response. The FEMA and State Task Forces and properly equipped and staffed search and rescue companies fall into the category using NFPA Standard 1470 as a guide to levels of capability.

There are several advantages to a tiered response system:

- It provides basic rescue training for all personnel.

- All potential rescuers become more aware of the dangers of different situations and recognize situations that are beyond their capabilities.

- A smaller number of personnel can develop a high level of expertise in a particular area.

- It fits well with a regional rescue response system where the personnel with basic training can handle a basic incident on their own and have the option of calling an advanced regional rescue team for assistance.

- It eliminates the expense and time required to equip and train all personnel to an advanced rescue level.

An example of a tiered response system for a structural collapse incident may be organized as follows:

Tier Level 1:	Engine company with light rescue capability. (Awareness Level) (Type III ICS)

Tier Level 2:	Truck company with medium rescue capability. (Operational Level) (Type II ICS)

Tier Level 3:	Heavy rescue company with heavy rescue capability. (Technician Level) (Type I ICS)

Tier Level 4:	US&R task force with heavy rescue capability, including search, medical, technical, and support components. (Technician Level)

Coordinating With Other Agencies

A regional multi-agency coordination system is needed to provide rapid information processing, decision-making, resource deployment, and incident support.

Earthquakes and other natural disasters occur quite often across local jurisdictional boundaries, and, in some cases, cross state lines. When this happens, multijurisdictional authority and responsibility can become issues.

Depending on the type and scope of the disaster, a large number of emergency response or disaster agencies may be involved in response and recovery efforts. For example, fire service, law enforcement, medical, public works, private contractors, military, and many other agencies all may be represented during incidents of this type. Generally, the greater the number of agencies involved, the more complex the problem can become in terms of who is in charge and who is responsible for specific functions.

In most cases, the authority for overall command is defined in statutes or laws governing specific jurisdictional areas. This designation may be different in adjoining jurisdictions affected by the emergency. For example, the fire department may be charged with a responsibility in one community or area, while in the next the same responsibility is placed with a law enforcement agency, and in still another, it belongs to a different government entity.

Unless authority and responsibility for overall command have been defined clearly in formal disaster plans and are understood clearly by all agencies before the incident, confusion and conflict can arise.

A structural collapse incident may involve many agencies and organizations, as did the Murrah Building bombing in Oklahoma City. A system to coordinate the response and on-scene operations of resources is needed for effective incident management.

Levels of coordination may occur:

- At the incident with the IC and agency representatives, or at an incident coordination center;

- At the department dispatch center or department operating center—The department operating center is the location where the top staff of the department, for example, fire or police, gathers to oversee incident operations. The department operating center is remote from the incident and usually is near the dispatch center and local Emergency Operations Center (EOC);

- At the local EOC—The EOC for the local jurisdiction may be activated to provide information and resource coordination for the IC. Representatives from agencies needed to coordinate and support the incident from the local government and outside agencies work at the EOC;

- At the county EOC—The county or regional EOC coordinates and supports the incident within the region;

- At the state multi-agency coordination system and the state EOC—The state provides a level of coordination needed to access other regions, agencies, organizations, the National Guard, and the coordination link with the Federal government;

- At the Federal coordinating system—The Federal Response Plan (FRP) is implemented and Federal agencies are coordinated at this level; and

- At the city level (mayor or city manager), and county level (county executive).

Scene Management Issues

The initial actions taken by the Incident Commander (IC) sets the tone for the incident. The initial size-up and structural triage provide information needed to:

- Develop the action plan:

- Size-up provides the information needed to develop the Incident Action Plan (IAP).

- Structural triage helps identify and prioritize the rescue areas with the highest probability of success.

- Many factors regarding the collapsed structure incident must be considered to develop a rescue operational plan, objectives, priorities, command organization, and resource requirements.

- Provide for the safety of both rescuers and victims:

- The IC should initiate the risk management process to determine the safe commitment of resources.

- A personnel accountability system should be used to ensure rescuer safety.

- Hazards and dangerous working conditions may be reduced or eliminated through effective incident management.

- Increase operational effectiveness:

- Scene control must be initiated early to establish a safe and functional worksite.

- A structural collapse incident may have major effects on a community that includes:

- Media attention;

- Psychological and emotional impact on victims, survivors, and rescuers;

- Political consequences; and

- Economic consequences.

Incident Emergency Management Planning Phases

An IC may be faced with something as simple as a single-site incident (that is, one building or a single rescue within a building) or a multisite devastation.

Depending upon the size and extent of the devastation, the IC may be faced with situations that require immediate decisions regarding the implementation of the operational plan. This initial plan is developed from the size-up, and the assessment of the incident is continuous throughout the incident.

Once the initial assessment is underway, the IC must begin to identify the overall mission objectives which should include:

- Assessing the general situation at the designated rescue site(s),

- Planning strategy and priorities,

- Assigning resources,

- Managing ongoing operations, and

- Following up on the progress and making adjustments to the plan.

IAPs follow phases of response from small incidents through large complex incidents to demobilization:

- Initial response plan:

- Size up the incident,

- Develop initial plan (objectives and priorities),

- Request and/or assign resources,

- Develop organization, and

- Evaluate operations.

- Expanded response (use of ICS Form 201):

- Continue size-up,

- Develop objectives and priorities,

- Request and assign resources,

- Provide logistical support,

- Expand the organization, and

- Evaluate operations.

- Extended response:

- Continue size-up;

- Develop objectives and priorities;

- Request and assign resources;

- Provide logistical support;

- Expand the organization;

- Add to IAP as needed (for example, safety, medical, and transportation plans); and

- Evaluate operations.

- Demobilization (continue IAP):

- Continue size-up,

- Develop objectives and priorities,

- Assign and release resources,

- Provide logistical support,

- Reduce the organization,

- Evaluate operations,

- Provide Critical Incident Stress Management (CISM),

- Collect all records, and

- Secure site and release all resources.

Size-up

Size-up involves obtaining information about the incident so that a plan can be developed.

The size-up should include:

- The problem’s cause (how the structure collapsed);

- Hazards involved (for example, additional collapse, fire, Haz mat, utilities, flooding, and dust, toxic or flammable atmosphere);

- Incident conditions (that is, structural stability, time, and weather, access);

- Victims (how many exist as well as their locations, number per loction, and degree of rescue difficulty); and

- Internal or external exposures.

The size-up of the collapsed structure and victim potential is much like that of a structure fire size-up. Consideration must be given to rescuer risk versus the benefit of rescuing a victim.

Size-up Considerations

The following size-up considerations must be addressed when assessing a rescue problem.

### Time

The time of day is an important aspect when attempting to locate possible victims. In a residential structure during night hours, victims may be found where bedrooms are located. At other times of the day, residential structures may have less of an occupant load. Commercial buildings may be virtually empty at night, but their occupancy grows exponentially during business hours.

The time of day also may influence the amount and type of other agency resources that may be available to assist in rescue operations.

### Occupancy

The occupancy type (residential, commercial, and industrial) plays an important role in the size-up of a collapsed building. The occupancy type, coupled with the time factor, yields valuable information about occupant load at the time of collapse.

### Age/Era of Structure Construction

The age of a building and the era in which it was constructed are important factors. The construction era of a building may reveal if retrofit ordinances were adopted to make up for original building flaws. Understanding the age of a building and possible retrofit programs that it may have been subject to greatly aids rescue workers in making a safe rescue. It cannot be overstressed that unreinforced masonry buildings having been through a retrofit program, are still unreinforced buildings. Particular attention must be given to ensure rescuer safety before attempting rescues in these types of buildings.

### Load Shift (Previous Versus Current Load)

When a building has partially collapsed, rescuers may be lulled into a false sense of security that the building will not collapse further. The previous structural load was along an axial plane as designed and built before collapse, but partial collapse of the building now may spread the load as an eccentric load. Other parts of the structure may be applying a torsional load, making additional collapse of the structure likely in an aftershock, during debris removal, or possibly during normal rescue operations.

During shoring operations, care must be taken to shore the structure in the location that it is resting in. Shoring should not be used to try to push the building back to its original position. Shoring only attempts to distribute the unequal load of the building equally back to the earth.

### Construction

Different construction types have inherent strengths and weaknesses. Ordinary construction, wood frame, and stucco offer the highest rescue potential because of the materials involved. In many cases, wood from a collapsed portion of the structure may be used for shoring before attempting rescue. This type of construction also is easily penetrated using normal truck company tools. Because of its mass and strength, heavy reinforced concrete provides the biggest challenge for the rescuer attempting to cut, break, breach, and shore.

Each type of building construction and building technique produces characteristic collapse patterns. Knowledge of construction type, construction techniques, and collapse patterns assists responders in assessing the existence of void spaces and victim survival.

### Risk Calculations

Collapse patterns, secondary collapse potential, utilities, and potential hazardous material situations influence the risk versus gain calculation that must be made before rescue attempts. Rescue safety is paramount and cannot be overstressed.

As with any size-up, consider the following items when making initial risk calculations:

- Placement of apparatus,

- Placement of equipment,

- Placement of personnel (keep only essential personnel in hazard areas), and

- Placement of support functions for safety and ease of work.

Plan on secondary collapse. Have escape routes planned and make them known. Use all available safety equipment. Appoint a person to act as a safety officer and give that person the authority to stop actions deemed unsafe. Appoint a person or persons to constantly recon the area for additional signs of a potential secondary collapse and additional hazards such as utilities or hazardous materials that may not have been recognized at the start of rescue operations. Consider surface rescues of partially trapped victims before attempting more time-consuming rescue operations. A collapsed structure with victims trapped and their location known may be an extremely emotional situation. Do not allow your emotions or the emotions of someone on your team to go unchecked. Use your head!

Remember for every action there may be an equal and opposite reaction. So slow down . . . think things out. Rescuer safety is your number one priority.

Each type of structure has unique characteristics that present different problems and advantages for rescuers in a collapse. In the event of failure, entire walls may fall, creating large piles of bricks or building debris, possibly trapping people on sidewalks and in automobiles. Roofs and floors may collapse completely, forming voids that may enclose trapped victims.

The problems of identifying hazards after structural collapse are extremely difficult. Buildings are often complicated, and there are many different types and configurations. After the event triggering the collapse ends, the danger of further collapse often is still present. Brittle conditions pose one of the greatest threats due to the probability of sudden failure. As many hazards as possible should be identified, and risk factors should be assigned to them. Measures to avoid and mitigate the danger then can be factored into the overall search and rescue effort.

How you proceed will depend on the amount of preplanning that has been done, how well your personnel are trained, and the resources available to effect the rescue. Search and rescue in a collapsed building is dangerous. Take your time, shore and support the structure as you proceed, and keep your wits about you. Your survival depends on it, and it all starts within the size-up.

### Building Construction Type

There are four general types of building construction. It is important for responders to understand these types in order for them to make accurate assessment of the hazards, the rescue possibilities, and the types of resources needed for the operation. The construction types and occupancy of structures determine the use of a variety of different techniques and materials. The four general construction categories rescuers will most likely encounter in collapse situations are light-frame, heavy wall, heavy floor, and pre-cast concrete construction.

### Light-Frame Construction

Materials generally are lightweight and provide a high degree of flexibility to applied forces such as earthquakes, hurricanes, and tornadoes. These structures typically are built with a skeletal structural frame system of wood or light-gauge steel components that provide support to the floor of roof assemblies. Examples of this construction type are wood frame structures used for private residences; low-rise, multiple occupancies; and light commercial occupancies up to four-stories high. Light-gauge, steel-frame buildings include commercial business and light manufacturing occupancies and facilities.

### Heavy-Wall Construction

Generally, materials are heavy and are employed in an interdependent structural or monolithic system. These types of materials and their assemblies tend to make the structural system inherently rigid. This construction type usually does not use a skeletal structural frame. It uses a heavy wall support and assembly system to provide support for the floors and roof assemblies. Occupancies using tilt-up concrete construction are typically one to three stories high and consist of multiple monolithic concrete wall panel assemblies. They also use an interdependent girder, column, and beam system to provide lateral wall support of floor and roof assemblies. Occupancies are typically commercial, mercantile, and industrial. Other examples of this type of construction include reinforced and unreinforced masonry buildings one to six stories high for any type of occupancy.

### Heavy-Floor Construction

Structures of this type are built using cast-in-place concrete construction consisting of flat slab panel, waffle, or two-way concrete slab assemblies. Pre-tensioned or post-tensioned reinforcing steel rebar or cable systems are common components for structural integrity. The vertical structural supports include integrated concrete columns, concrete enclosed, or steel frame that carry the load of all floor and roof assemblies. This type includes heavy timber construction that may use steel rods for reinforcing. Examples of this type of construction include offices, schools, apartments, hospitals, parking structures, and multipurpose facilities. Heights vary from single story to high-rise.

### Pre-Cast Construction

Structures of this type are built using modular pre-cast concrete components that include floors, walls, columns, and other subcomponents that are field connected upon placement on site. Individual concrete components use imbedded steel reinforcing rods and welded wire mesh for structural integrity. Steel beam, column, or concrete framing systems may be used for the overall structural assembly and building enclosure. These structures rely on single or multipoint connections for the floor and wall enclosure assembly and are a safety and operational concern during collapse operations. Examples of this type of construction include commercial, mercantile, office, and multi-use or multifunction structures, including parking structures and large occupancy facilities.

Construction Techniques

### Light Frame—Wood and Stucco

Connections can be bolted. Plywood sheathing of wood structures makes them very tough and resistant to earthquakes, as long as the sheathing is nailed properly. Wood-frame buildings are seen in single-family dwellings and up to four-story buildings. They also include older balloon-frame structures and nonuniform buildings, which are corner- or odd-shaped buildings (E-, H-, L-, T-, or U-shaped).

Problems with these buildings include slipping of foundations and failure of chimneys, air conditioners, and facades. In odd-shaped buildings, cracks or separations occur at the overstressed inside corners. The more plywood shear panels used in construction, the less damage seen.

### Masonry Construction

Unreinforced masonry construction exists throughout the world. Even in earthquake-prone California, there are approximately 40 to 50 thousand unreinforced masonry buildings and 7 to 8 thousand unreinforced masonry buildings in Los Angeles alone. Walls are made from three or more bricks laid lengthwise, side-by-side, for 4 to 7 courses, then a course with bricks at 90 degrees. The 90-degree brick course is called a king’s row or header row. Other recognition factors include arched windows, steel plates or concrete lintels over door and window openings, and thick walls and door openings. Lime and sand mortar is placed between the bricks. The strength and seismic resistance of unreinforced masonry is highly dependent on the mortar strength. The shear strength of mortar can vary from 15 pounds per square inch (psi) to over 150 psi and is determined by the proportion of lime to Portland cement, as well as by the workmanship. Decorative brick veneers are a special seismic problem. A veneer often is laid up with building paper between it and the unreinforced masonry wall and is anchored with iron or galvanized ties. The ties normally corrode within 20 years, leaving a heavy brick face just waiting to peel off when subjected to a lateral load.

These buildings are usually one to six stories high. There is no steel reinforcement on ledges formed for the floors and roof to sit on. Even with the tie plates that anchor the joists and rafters to the exterior walls and plywood sheathing on the roof, these buildings present the greatest risk of collapse during an earthquake. The risk is greatest at the corners, which tend to blow out.

Reinforced masonry construction consists of cinder block or clay brick. The mortar is made of sand and cement. Grout can be used to fill vertical cavities in the block. Horizontal and vertical steel is used for additional strength. Floors and roofs are connected directly to these walls. Things to look for are cracks, out-of-plumb walls, and connection points pulling away from each other.

### Reinforced Concrete

Older concrete-frame buildings may or may not have steel reinforcement. Floors are thick and heavy and can pancake collapse as columns punch through each floor.

Pre-cast concrete structures are one to six stories high with concrete floors supported by pre-cast columns and girders. Collapse is initiated by the failure of the joint between the slab and wall or the girder and column.

Tensioned concrete can have rebar for longitudinal tension stress and enclosed-type steel ties that can be tensioned as the structure is assembled. Wall-like structures of cast, pre-cast, and tensioned concrete have out-performed frame construction in most earthquakes.

Tilt-up concrete wall buildings are usually one to two stories high with a lightweight roof. These buildings are made of concrete slabs that have been tilted up to form exterior walls. Before 1972, the walls were not placed in footings; after 1972, they were placed in footings. Failures occur from separation of walls and roof.

### Steel Construction

Steel construction is found in high-rise buildings. Steel is tough and strong, but it needs to be fireproofed. It starts to lose strength around 700 F (371 C). It is ductile; that means it can be overstressed and severely bent, but it will retain enough strength to resist failure, giving ample warning of collapse. The weakest points are the connections that are welded, bolted, or riveted in older buildings. Beams must be laterally braced so as not to buckle about their weak axis from foundation shear.

### Collapse Voids

Void spaces are created when furniture, machinery, and other strong bulky objects support sections of collapsed floors and walls. Larger spaces and voids are created by collapsed wooden floors, which tend to remain intact in large sections. Collapsed void spaces are divided into four major categories:

- Lean-to collapse,

- V-type collapse,

- Cantilever collapse, and

- Pancake collapse.

All four of these collapse patterns create potential void spaces where victims may be found.

- Lean-To Collapse

The lean-to collapse creates the greatest chance of victim survival. This type of void space is created when a floor or roof section becomes dislodged, and one end falls to rest on the floor below. The other end of the dislodged section remains attached to the wall member. Care must be taken to shore the wall properly for a torsional load.

- V-Type Collapse

This type of void space is created when a floor or roof section breaks into two pieces and collapses to the floor. It creates two void spaces, one on either side of the break. In this situation, both exterior walls are loaded with a torsional load and require shoring.

- Cantilever Collapse

This type of collapse is common in unreinforced masonry buildings where the exterior wall has been destroyed completely on one side. This may create many void spaces that have a high victim survival rate. The actual cantilevered portion creates an extreme hazard for rescue personnel and must be shored properly to prevent additional collapse.

- Pancake Collapse

This type of collapse is a total collapse of many floors of a structure, creating many smaller void spaces where victims may be located. This type of collapse requires tunneling to access the void spaces. Proper shoring techniques must be employed to allow responders to access the void spaces safely.

Operational Considerations

The IC needs to develop an IAP that includes appropriate priorities, objectives, command structure, and resource requirements. The development of this plan should include consideration of the following 24 rational factors:

- Time—The time of day provides information on the occupancy load and location of people in the structure.

- Location—Access is important to an effective operation.

- Occupancy—Knowledge of the occupancy yields information on hazards, occupant use, and types and number of businesses.

- Height and area—Consider all six sides and the area involved.

- Size of collapse area and structural hazards—This assessment will dictate resource requirements and safe methods of rescue.

- Fire problems and hazardous materials—Fire or hazardous materials problems may impede a collapsed structure rescue operation.

- Exposures—Interior and exterior exposures should be considered to prevent additional damage or injury.

- Utilities—Control of gas, water, and electricity is a major safety factor to both rescuers and victims.

- Weather—Temperature variations affect rescuers and victims. Wind and rain certainly may create additional problems inside and outside the structure.

- Victims—Victim location is a priority in the initial rescue plan and may be determined by a variety of methods.

- Traffic—Speed of response and access to the collapse site is critical. Alternate routes and traffic control should be planned.

- Rail—Surface and underground rail systems may be part of the collapse problem or may affect it because of vibration.

- Personnel—Rescue operations require a multidisciplined response from fire, EMS, police, public works, building department, transportation department, volunteers, and many others.

- Incident command—The complexities involved in rescue require an effective Incident Command System (ICS) to manage and coordinate operations, planning, and support.

- Communications—Intra-agency and interagency communication capabilities are essential to effective and safe operations.

- Medical—Rescue medical operations need to provide for victims as well as have a component to handle the needs of responders.

- Safety—Safety is the top priority in rescue planning and operations and must be considered throughout the incident.

- Special equipment—Collapsed structure rescue operations may require the use of specialized search equipment, and portable cutting, breaking, and breaching equipment.

- Construction equipment—Large, mechanized construction equipment may be needed to remove debris so that rescue operations can be expedited.

- Shoring materials—A large amount of shoring materials may be required for safe access to victims and for structural stabilization. Pre-incident planning of supply sources is important.

- Information updates—Continuous information updates are needed during every stage of the rescue operation.

- Staging areas—Staging areas should be established for incoming resources so that the response into the rescue site can be managed effectively.

- Responder rest, recovery, and relief—Long-term rescue operations necessitate periodic rest periods for rehabilitation of rescue workers, including provisions for relief so that operations may continue without pause.

- Secondary collapse—The hazard of secondary collapse must be considered, whether from an earthquake aftershock or from failure of an already weakened support structure.

Life Safety Personnel Considerations

Hazards at a collapse site may be numerous, involving structural failure, nonstructural damage, and environmental conditions requiring specific mitigation and protection for responders and victims. Structural collapse generally is classified as a low occurrence, high-risk incident, and unfortunately, is where most responders are killed and/or injured.

Collapse Hazards

Hazards associated with secondary collapse originate in damage caused by the primary event.

Nonstructural damage does not carry the risk of secondary collapse. Indications may be the obvious broken window, cracks in plaster and drywall, and doors that may not open easily.

Structural damage indicates that the stability of the building has been compromised. This can be anything from doors ajar, exterior and interior walls that are racked or tilted, or floors and ceilings staffing or buckled. Hazards to be aware of are the possible instability of the building materials in the areas of the structure that are being worked in. Things to look at are the main connection points of the structure.

Nonstructural failures such as those listed below may result in secondary collapse:

- Chimneys—Failure may be indicated by cracks and partial collapse, especially in renovated structures. Chimneys may need to be taken down before any rescue work is initiated.

- Mechanical equipment—Air conditioners, heaters, and coolers on the roof or in the attic, or signs and billboards on the roof pose a hazard.

- Parapets, dormers, and facades can be hazards, especially with newer construction. The connection points are weaker than in older buildings, and facades can collapse totally without warning.

- Glass can always be a hazard, especially the larger and thicker pieces. If there is glass around an entry point or where responders are working, break it out to eliminate any chance of it falling during aftershocks.

Utilities and Adjunct Hazards

### Natural Gas or Propane

Natural gas and propane, when free in an open-air situation, are not very serious hazards. However, when either is leaking in a closed environment, it can be deadly. If you suspect a gas leak (and you should), it is important to shut off the gas at the meter. If the meter is at the location of the leak, ventilate the area before entering, if possible, and keep all civilians from entering.

When dealing with propane, remember that it is heavier than air, settles in low areas, and may not dissipate like natural gas. When it reaches a source of ignition, it can explode! Natural gas distribution lines found in the street carry from 10 to 55 psi, and the lines that feed a single-family structure are pressurized to 1/3 psi. In commercial applications, pressures can vary depending on the needs of the business; however, it will not exceed the maximum distribution pressure of 55 psi.

### Electrical Hazards

Just as in firefighting operations, it is very important to be aware of electrical hazards that could exist after a collapse. Before entering a building, it is important that all utilities are shut off and the area is secured with fire line tape so that no unauthorized persons can enter the area.

Even when no obvious hazard exists, there still can be live wires that pose a hazard. Anything over 750 volts usually is marked on a pole by a “high voltage” sign. Anything that is located on the pole higher than this sign is considered high voltage. Transmission lines can carry up to 500,000 volts, and smaller lines can carry 34,500 volts. Transformers drop the high voltage to service currents of 240, 480, 4,800, and even 34,500 volts in some industrial and commercial applications. If these lines are down, it is very important to treat them as though they are hot.

### Hazards Associated with Water

After a collapse, it is not uncommon to find that the utility lines have been severed. Water pooling from broken pipes on upper floors may cause a secondary collapse. Water also may flow into the basement area causing problems for victims trapped in those locations. Water also increases the possibility of electrical shock if the electricity has not been disconnected or severed.

### Hazardous Materials Situations

After any event large enough to cause a significant amount of damage, structures displaying a 704 placard should be approached with extreme caution. Chemicals in their normal state have certain properties that are predictable. When they are mixed or are involved in a fire, they become very unpredictable. Always treat any situation involving haz mat as though it were the worst-case scenario. Even buildings that do not have the 704 placard can be potentially dangerous to first responders. Keep your eyes open to what is going on around you and continue to gather information pertinent to the incident.

Fire-Related Incidents

Incidents involving fire should be handled in the same manner as a normal firefighting operation, with the following exception: an aggressive interior attack on the fire should be reconsidered. Consider what is burning, what types of life hazards exist (if any), and the condition of the structure, that is, is a secondary collapse imminent? If any of the above raise concerns in your mind, then discretion is the best way to approach these fires. Make sure that the exposures are covered, handle any life hazards first, and then extinguish the fire in the safest manner.

First responders are capable of doing only so much with the resources that are available at the time. It might be prudent to keep civilians away, keep responders out of these badly damaged buildings, and keep the fire from spreading to adjacent structures.

### Dust/Asbestos Atmospheres

In any collapse situation, whether from an earthquake, explosion, or collision, there probably will be a tremendous amount of airborne particulate matter. First responders need to protect themselves from whatever is in the air. There could be asbestos or other harmful things in the air, which may not affect responders right away.

Some fire departments provide their members with dust masks. They should be worn at all times in contaminated atmospheres. If you suspect something is in the air, you should at least have a dust mask on.

### Flooding (Not Caused by Broken Water Pipes)

Water, from something other than broken water pipes, can pose a problem. A dam letting go or a swimming pool failing above a rescue site would put a crew in a dangerous position.

Risk Management

Risk management is the process of evaluating and mitigating hazards in the environment. The IC must perform this activity to ensure the safety of the rescuers. Although other people within the ICS provide the IC with advice, ultimately it is the IC who is responsible for making the “go”/“no go” decision.

The risk management process involves five steps:

Situation awareness involves observing and obtaining accurate information. It is the product of combining long-held attitudes and knowledge with new information as it is gathered to build a new perception. The more accurate and timely the new information, the closer one’s awareness is as to the reality of the situation. This is the foundation on which decisions are based. Good information is the key to good decision-making.

Hazard assessment is identifying and evaluating the hazards and their potential. Exercising judgment on the probability of a hazard and its potential severity is inherent in the hazard assessment step. Assess the potential collapse hazards, environmental hazards, and the hazards that result from the tactics selected.

Risk control is applying measures to reduce or eliminate the hazard. Risk controls can vary from a simple briefing that provides awareness of a hazard to shoring an unstable structure before entry. Providing appropriate protective gear and a communication system are also risk controls.

Reaching a decision point is accomplished by:

- Evaluating hazards and risk controls and

- Deciding to commit (or not to commit) resources to the assignment. There are three key questions to ask before starting operations. They should all be answered yes!

- Can personnel work safely in the collapse zone?

- Does everyone understand the strategy and tactics?

- Has a briefing been given with a feedback opportunity?

Evaluation is accomplished by:

- Ensuring that the plan is working and

- Planning to evaluate continuously.

Leaders earn their pay in this step of the risk management process. You must coordinate the resources working for you so that they can accomplish the objectives of the plan. This means you should follow up during an operation to ensure the plan is working, do continuous reevaluation of the operation to make adjustments as the situation changes, and incorporate lessons learned for future use. Whenever an adjustment is needed, it should be a cue to update the situation awareness. This puts you back at the first step of the risk management process. The structural collapse environment is dynamic, and this means that you must be able to adapt to continuous change.

### Personnel Accountability

The IC must ensure that a personnel accountability system is established early during initial response, that the accountability level goes down through the incident organization, and that it is maintained throughout the incident. This system must include accurate information identifying company, crews, and personnel assignments and locations. Good communications and a safety plan are essential elements of this system. Reporting procedures and signaling systems should be understood well by all.

Continuous documentation is important in resource tracking.

### Incident Commander Responsibility for Scene Safety

Hazards and dangerous working conditions may be reduced or eliminated through effective incident command that:

- Provides leadership and organization;

- Obtains accurate information and develops a plan;

- Makes safety a top priority;

- Assigns a safety officer and Rapid Intervention Crew (RIC);

- Provides for appropriate protective measures and safety equipment;

- Rotates crews and provides rehabilitation;

- Plans for contingencies;

- Monitors, isolates, confines, contains, and mitigates hazards;

- Communicates and uses the chain of command;

- Has periodic briefings; and

- Plans for injuries and stress management.

Establishing Scene Control

Establishing scene control should coincide with the initial size-up and IAP.

Some of the actions the IC should take when managing the scene of a structural collapse include:

- Isolating the area;

- Establishing zones :

- Collapse-hazard zone (hot zone),

- Rescue work zone, and

- Operational work area;

- Marking and identifying areas;

- Evacuating bystanders and nonessential people;

- Controlling perimeters is accomplished by:

- Controlling and managing spontaneous responders,

- Providing access/entry routes into worksite,

- Providing for outer perimeter access control,

- Developing a traffic plan, and

- Requesting (and using) law enforcement for this function;

- Establishing site security by:

- Requiring authorization to enter site,

- Media control, and

- Use of law enforcement;

- Establishing incident facilities such as:

- Command Post (CP),

- Staging,

- Medical triage and treatment area,

- Mase,

- Cribbing and shoring work station,

- Evacuation area, and

- Rehabilitation area;

- Investigating the incident by:

- Seeking cause determination (criminal or accidental),

- Preserving evidence, and

- Documenting; and

- Requesting assistance.

Operational Phase

A major structural collapse incident can progress through a total of five phases. This first phase—the initial response—involves initiation of the following actions by the first IC:

- Establishing command,

- Conducting a size-up,

- Developing an action plan, and

- Deploying resources.

The incident can then proceed to the second phase—the reinforced response. This type of response requires a larger incident organization and additional support requirements.

The third phase is referred to as an extended response (involving 24-hour operations). This phase requires:

- A detailed Incident Action Plan (IAP),

- Augmented organization, and

- An effective coordination system.

Five Operational Phases Overview

The response and buildup to a major incident as well as downgrading, closure, and return to normal activities can be defined by the following five phases:

Phase 1:  Initial response,

Phase 2:  Expanded (reinforced) response,

Phase 3:  Extended response (24-hour operation),

Phase 4:  Demobilization, and

Phase 5:  Return to a state of readiness.

Phase 1: Initial Response

This initial phase involves the following six actions:

- Establishing command—This is accomplished by:

- Announcing command;

- Assuming all command and general staff responsibilities;

- Providing a brief radio report (that is, location, type of structure, and situation) to the dispatch center. (Responding companies should be monitoring this report); and

- Establishing a command post.

- Performing a size-up—Size-up involves the following tasks:

- Surveying the site,

- Determining the type of problem,

- Identifying hazards,

- Assessing conditions,

- Determining victim locations and viability,

- Identifying exposures, and

- Assessing potential for escalation.

The Structural Collapse Operational Checklist (provided in Appendix C) is a good administrative tool to use during size-up.

- Developing an IAP—Essential steps in action planning are:

- Understanding the situation,

- Establishing objectives and strategies,

- Developing tactical direction and assignments,

- Preparing the plan (at this point it may not be written),

- Implementing the plan, and

- Evaluating the plan.

An IAP helps establish priorities, points out hazards, and reviews items such as risk/benefit and safety.

The initial IAP may involve the use of a form that begins the written plan. The ICS Form 201 (Incident Briefing) may be used for this purpose and as a briefing form for the transfer of command to another officer. This form includes a map or diagram of the area involved, incident objectives and actions taken, and the incident organization and resource summary.

- Requesting resources

Once the IAP is developed and objectives have been established, request the resources required to achieve the objectives. For example, to achieve the objective “search the first and second floor for victims,” request an engine company and a truck company.

- Deploying and organizing resources

The initial response may include from one to five single resources. The ICS organization at an initial response level consists of the IC and the single resources assigned to specific tasks (as illustrated in Figure 11-1).

Figure 11-1: Initial Response

The IC must deploy appropriate resources to the incident in order to accomplish priority objectives.

Scene management must be initiated early by isolating the area and establishing zones, for example, the collapse zone.

- Evaluating response

Evaluation requires accurate information and good communication (status/progress reports).

Determine resource effectiveness from the evaluation by comparing progress to objectives.

Phase 2: Expanded (Reinforced Response)

An expanded (or reinforced) response is initiated when the IC determines that initial resources are insufficient to handle the incident. The expanded (reinforced) response involves

- Size-up—A size-up must:

- Be continuous,

- Anticipate the need for an extended operation, and

- Document information.

- Transfer of Command—Transfer of command may take place during the expanded response phase.

During transfer of command, the officer assuming command communicates with the officer being relieved and a briefing takes place (ICS Form 201 should be used). This briefing covers:

- Incident conditions,

- IAP,

- Progress toward completing objectives,

- Safety considerations,

- Resource assignments,

- Need for additional resources, and

- Critical issues.

The officer being relieved then is reassigned and a change in command is communicated.

- Evaluating the current situation—The following factors should be considered during the evaluation of incident operations:

- What has happened?

- What progress has been made?

- How good is the current plan?

- What is the incident growth potential?

- What is the present and future resource and organizational capability?

- Developing a risk management plan

Hazards need to be assessed and controlled or avoided. Risks need to be managed.

- Establishing scene control—Establishing scene control includes accomplishing the following:

- Establishing zones,

- Evacuating bystanders,

- Controlling perimeters,

- Establishing site security, and

- Establishing incident facilities:

- Suitable CP,

- Staging area(s), and

- Triage and treatment area.

- Developing an IAP—A more detailed IAP may be needed for an expanded response. This IAP should include a:

- Clear statement of objectives and actions,

- Basis for measuring work effectiveness and progress, and

- Basis for providing accountability.

Essential plan elements are:

- Statement of objectives,

- Description/review of incident organization,

- Tactics and resource assignments, and

- Support plans (traffic, medical, communications, safety, demobilization, and others as needed).

IAP objectives always should be:

- Achievable—Make sure that you can accomplish the objectives with the resources you have;

- Measurable—Place a time required for the completion of each objective; and

- Flexible.

Dynamic incidents change, so the plan needs to be flexible and broad enough to accommodate change.

Written IAPs should be used when:

- Two or more jurisdictions are involved,

- Operational periods are required,

- Many organizational elements exist,

- Required by an agency, or

- The IC decides to use a written plan.

- Expanding the ICS organization—The ICS organization may be expanded based on the size and magnitude of the incident. Expansion is based on resource and management needs.

Figure 11-2 illustrates an ICS for expanded response, showing how the system builds, and subsequent resource assignments.

Figure 11-2: Expanded Response

In the example of an expanded response organization displayed in the figure, the initial response organization has been reinforced with the arrival of additional fire department resources and resources from law enforcement and public works agencies:

- The IC has assigned a safety officer to ensure personnel safety and an information officer to work with media.

- A staging area has been established to check in arriving resources.

- Public works has been assigned to debris removal.

- The incident has been divided into two divisions to manage resources better:

- Original engine and truck companies are grouped together to form one task force.

- Second-to-arrive local engine and truck companies are grouped together to form another task force.

Phase 3:  Extended Response (24-Hour Operation)

An extended response involves operations over a 24-hour period.

### Detailed IAP

A detailed IAP is necessary for incidents with an extended response involving operational periods and multi-agency or multijurisdiction involvement. The plan documents the activities developed for a given period of time and, when all attachments are included, specifies:

- Control objectives,

- Tactics (to meet those objectives),

- Resource assignments,

- Incident organization,

- Required maps and information specific to the incident,

- A communications plan,

- A medical plan,

- A safety plan, and

- A traffic plan.

The IAP serves as a guide so that operational effectiveness can be evaluated based on the objectives set for the operational period. As work progresses, priorities change based upon a continual evaluation of the incident. Flexibility is important, and contingency plans are a necessity in dynamic and hazardous incidents.

### Operational Periods

Operational periods are planned periods needed to achieve objectives. Rapidly changing incidents require shorter operational periods.

The planning process for an operational period involves the following 10 steps:

1. State incident objectives.

2. Give situation and resource briefing.

3. State primary and alternative strategies to meet objectives.

4. Designate branch, division, group boundaries, and functions.

5. Describe tactical operations and tactics.

6. Make tactical resource assignments.

7. Specify reporting locations and additional facilities needed.

8. Develop the resources, support, and overhead personnel requirements.

9. Consider additional support requirements needed because of  communications, traffic, safety, medical, etc.

10.	Finalize, approve, and implement the plan.

### Logistic Support

Logistic support is critical to maintaining extended rescue operations. An incident of major complexity and size involving possibly hundreds of response personnel requires a logistics section capable of meeting the needs of the incident through demobilization.

For example, long-term operations may require:

- Lighting for night operations or reduced visibility;

- Large food and water supplies;

- Major equipment repair and supply functions;

- Special equipment acquisitions;

- Other support functions specific to a structural collapse incident (for example, heavy equipment, structural stabilization resources, security measures including barricades, and fencing);

- Coordinated communications and technical equipment;

- Expanded facilities; and

- Additional facilities for rehab and CISM and air operations, for example.

### Incident Facilities

Incident facilities such as the CP and a staging area should be set up early in the best possible locations. Incident facilities used for an extended operation at a major structural collapse may require:

- Several staging areas for specific resources (for example, one for heavy equipment);

- A large base to accommodate personnel through rehab, rest, and feeding cycles, and vehicle and specialized equipment fueling and repair functions;

- A supply and equipment distribution system;

- A CP with the capability to sustain a large command staff and effective communications;

- A larger triage and treatment area;

- A morgue; and

- A decontamination area.

### Extended ICS Organization

The extended ICS organization for a structural collapse incident of major magnitude may require:

- An incident organization involving resources from many agencies and

- A unified command organization.

Figure 11-3 shows a multibranch organization chart illustrating how a major ICS may be organized for an extended operation.

Figure 11-3: Multibranch Response

The multibranch response organization displayed in this figure has provided for a larger expansion to accommodate additional resources and functions needed for extended operations.

The IC has assigned a logistics section chief and a finance/administration section chief.

The operations section has established five branches with similar functions to better coordinate and manage resources and maintain an effective span-of-control.

The planning, logistics, and finance/administration sections have several units operational to support the large amount of resources at the incident.

### Incident Command System/Emergency Operations Center Interface

An Incident Command/Emergency Operations Center (EOC) interface is needed to provide for effective information flow and coordination between the incident site and the EOC.

An EOC is a location from which centralized emergency management can be performed. The activation and level of activity and staffing depends on the size and complexity of the emergency.

The local government EOC is activated when field response agencies need support and to facilitate overall coordination of multi-agency operations.

Local government EOCs serve as the central point for coordination within and outside the jurisdiction. These EOCs gather and disseminate information and assist in supporting field operational incident commands.

Field-level command post coordination with the local EOC may go through the dispatch center to the EOC, or in some jurisdictions, the ICS field command may have direct communications with or receive policy direction from the EOC. Coordination may be with other agencies, organizations, the media, and citizens within and outside of the local government involved.

EOCs may be managed using the five primary ICS functions of command, operations, planning, logistics, and finance/ administration.

The IC would most likely interact with the EOC/operations section chief. In some jurisdictions, local policy may provide for direct IC to EOC management interaction. This may occur when there is a large single incident that has major impact on the community.

It may be useful in some situations to have direct coordination between incident sections and their counterpart EOC section (that is, planning section to planning section).

Under unified command, the EOC interactions are similar to those described above using their department’s contact in the operations section.

Phase 4: Demobilization

Demobilization planning must start early. Each section of the ICS must participate by providing information needed to ensure a smooth demobilization process. Important information elements include:

- Resource information (location, operation, designation, type, and kind);

- Agency agreements (use conditions or time used requirements);

- Physical condition of personnel (determines if personnel need rehabilitation before re-release);

- Transportation (requirements for transportation from incident);

- Costs (cost accounting for time and use of resources); and

- Priority (or critical) needs (priority of need for resource to return to home agency or another incident).

Development of the demobilization plan must begin early in the incident. The plan should include:

- Information on the demobilization procedure:

- Responsibilities for incident personnel:

- Planning section provides information on resources;

- Liaison officer knows terms of agreements on release;

- Safety officer considers physical condition of personnel and transportation;

- Logistics section handles transportation, communications, and maintenance;

- Operations section knows continuing needs for tactical resources; and

- Finance section processes claims, time records, and costs.

- Release procedures (may include rehabilitation for personnel, maintenance and re-supply)—Agency dispatch centers give priorities for the timely release of resources. Release priorities should include priority factors as follows:

- Type of resource,

- Critical need elsewhere,

- Cost, and

- Travel distance;

- Debriefing or defusing procedures as necessary in accordance with the incident CISM plan; and

- Collecting all forms and records.

Phase 5: Return to a State of Readiness

Emergency response organizations must be able to return to a state of readiness as soon as possible to provide protection to the community they serve. Returning to a state of readiness includes:

- Rehabilitating personnel should be the first step. Provisions should be in place for a CISM program (so members may receive debriefings, defusings, or follow-up care);

- Repairing or replacing equipment;

- Forwarding records, reports, and other information for processing;

- Completing an after-action analysis as necessary depending on size, complexity, or other circumstances such as agency requirements. (The purpose of this analysis is to evaluate the response and to capture lessons learned for application to future operations.); and

- Adjusting operating procedures to incorporate lessons learned.

Activity 11.1:

Structural Collapse: Tree Down into an Apartment Building

Student Activity Worksheet

Time: 150 minutes (simulation and debrief) Purpose: To provide practice in using the ICS organization to manage a structural collapse incident. Directions: - As a group, you will staff positions within the ICS of the Central City Fire Department (CCFD) for the exercise: - Incident Commander - Incident Commander Aide (not used unless nine students in a group) - Operations Chief - Operations Aide (a second Aide designated for groups with more than 10) - Planning Chief - Logistics Chief - Safety Officer - Information Officer - Liaison Officer - You will record all command decisions and actions during the exercise on the Simulation Action Chart. Entries on the ICS chart are made in response to written or visual messages or in anticipation of problems that could surface during the incident. Simulation outcomes: - Incident Commander: Team report - Operations Chief: Initial action plan and organizational chart - Operations Aide: Command worksheet, which includes objectives, strategies and tactics and an incident map - Planning Chief: Alternate plan - Logistics Chief: Resource list - Safety Officer: Safety message - Information Officer: Press release - At the conclusion of the exercise, all groups will participate in a debriefing facilitated by the instructors. Your group will display both the ICS charts and Simulation Action Charts that you developed during the exercise. Be prepared to explain your reasons for your decisions.

Incident overview: Engines 1, 2, and 3; Truck1; Ambulance 1; and Battalion 1 respond to the structure collapse in Central City. Callers report a large tree down into an apartment building. The time is 0230 hours. Incident priorities: - Life safety - Incident stabilization - Property conservation Strategies: - Rescue  Ventilation - Exposures  Salvage - Confinement - Extinguishment - Overhaul - Using the incident priorities and strategies, develop the control objectives. - Develop an incident organization. - Assign and manage resources.

Central City Fire and Emergency Medical Services Fire Resources Overhead Positions—one Fire Chief, one Assistant Chief, one Chief Fire Marshal, seven Fire Inspectors, one Training Captain, two Training Lts., three Deputy Chiefs (1 per shift), six Battalion Chiefs (2 Battalions x 3 shifts). Eleven engine companies—Engines 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, and 11. Each engine company is staffed with one officer and three firefighters. Four truck companies—Trucks 1, 3, 5, and 8. Each truck company is staffed with one officer and three firefighters. Squad Company 1. The squad company is staffed with one officer and two firefighters fully trained in rescue practices. Crash/Fire/Rescue Engine. Airport-staffed with one officer and two firefighters. Engine 103, Engine 104, Engine 105, and Engine 109. Reserve Engines—fully equipped. Truck 103. Reserve Truck—fully equipped. Engine 104 (1,500-gpm foam pumper). Apparatus contains one 500-gallon foam tank; 400 five-gallon cans of foam are stored at Engine 4. All members of Engine 5 and Truck 5 have been fully trained as a Hazardous Materials Response Team. Mask Service Unit 106. Engine 206 (light truck). Boat 2 - 16’ Boston Whaler. EMS Resources (third service operated by the city and stationed in the fire stations): Three EMS supervisors – (one per shift x 3 shifts) operating out of fire station 1. Five ALS ambulances – ALS 2, 4, 7, 9, and 11 Five BLS ambulances – BLS 3, 5, 6, 8, and 10

Central City Fire/EMS Response Criteria 1st-Alarm Resource Card Response time: 5 min. Engine 1 Truck 1 Battalion 1 Engine 2 Ambulance 2 MSU 6 Engine 3 Squad 1 2nd-Alarm Resource Card Response time: 10 min. Engine 4 Truck 3 Battalion 2 Engine 5 Ambulance 3 Deputy Chief Engine 6 EMS Supervisor 3rd-Alarm Resource Card Response time: 15 min. Engine 7 Truck 5 Fire Marshal (Bn. Chief) Engine 8 Trng. Battalion Chief Engine 9 Adm. Battalion Chief 4th-Alarm Resource Card Response time: 15 min. Engine 10 Truck 8 Engine 11 5th-Alarm Resource Card Response time: 20 min. Fisherville Engine 1 Harvest Junct. Engine 2 Kingston Engine 1 6th-Alarm Resource Card Response time: 20 min. Apple Valley Engine 1 Bayport Engine 2 Jasper Engine 2 7th-Alarm Response Card Response time: 25 min. Deep Water Engine 2 Kingston Engine 3

Simulation Action Chart

MSSG.		TIME			ACTION TAKEN

Health and Safety Message

| Incident Name: |  | Date Prepared: |  |  | Time Prepared: |
| --- | --- | --- | --- | --- | --- |
| Operational Period Date: From: To: |  |  | Operational Period Time: From: To: |  |  |
| Major Hazards and Risks: |  |  |  |  |  |
| Narrative: |  |  |  |  |  |
| Prepared By; | Company Name: |  |  | ICS Position: Safety Officer |  |
| Approved By: | Company Name: |  |  | ICS Position: |  |

Activity 11.1:

Structural Collapse: Tree Down into an Apartment Building

Student Assessment sheet

Debriefing: Debriefings are discussions centered on the cues contained in the written and visual messages, the strategies you developed as responses to the cues, and your organization and management of the incident staff. - What critical cues did you observe? - What ICS organization did you develop? - Did your group effectively prioritize responsibilities and actions? - Did you pass information to all who needed to be aware of it? - Did you promptly deal with messages? - Did you identify alternate methods of performing tasks? - Did you recognize the need for additional/outside resources? - Did you coordinate the allocation of resources? - Did you periodically re-evaluate resources? - How did your actions differ from the actions in the matrix? Why did they differ? - What are your strengths and weaknesses in working on an IMT for this incident? - How will you apply what you learned in this exercise to preplanning for a target hazard of this type in your community? - How will you apply what you learned in this exercise to responding to an incident at a target hazard of this type in your community?

Notes:

Module Summary

A risk/hazard analysis is needed to determine resource requirements for potential structural collapse incidents.

Initial scene assessment and building triage provide for the development of the response plan and set the tone for operations at the incident. A major structural collapse incident response grows from the initial response involving establishing command, performing the size-up, developing an action plan, and deploying resources to an expanded response requiring reinforced resources, a larger incident organization, and additional support requirements.