Module 13:

Enclosed Mall Simulation Exercise

Objective

Upon completing the module, you will be able to effectively manage an incident at an enclosed mall.

Methodology

This module uses lecture, discussion, and a simulation exercise.

NFPA 1021 Standards Cross-Reference Matrix

3-6, 3-7, 4-6, 4-7

Structure

Shopping malls have been referred to as “horizontal high-rises.” Mall constructions vary. There are open and enclosed malls, but all contain combustible materials in ceiling supports, store partitions, and display equipment. Mall roofs consist of metal decking (most common), concrete, wood, or glass (Plexiglas™ or Lexan™). There are large areas under the roof. Most major building codes require building components (columns, beams, and floor/ceiling systems) to be protected. Protection can range from 1 to 3 hours, depending on the code in force. Store partitions may not penetrate the ceiling or reach the underside of the roof. Fire service personnel must determine and note fire walls or fire separations on preplan. Display equipment may include extremely combustible materials and/or possibly block sprinkler cone spray coverage. Enclosed malls with high-rise structures attached may include hotels, office buildings, or a combination of both. High-rise structures may open directly into malls, creating significant exposure problems, especially smoke propagation. Numerous ventilators, skylights, and HVAC units are located on the roofs of malls. Roof surveys and preplans should indicate what occupancies these units serve.

Anchor stores are major stores, usually part of a chain, such as Sears. There may be as many as six anchors or more in a single mall. These buildings are large mercantile (possibly multilevel) occupancies that may have their own fire alarm, suppression, and/or detection systems. They also contain large storage areas that may confuse and trap firefighters. Storage areas for these occupancies may be remotely located in large maze-like storage rooms in lower levels or to the rear of the mall. Specialty stores or linking stores face an open mall or concourse area, which connects the anchor stores. There are often small booths usually located throughout the mall in the concourse or pedestrian traffic area, making it difficult to navigate in blackout conditions.

A mall is made up of many different types of occupancies under one common roof. Frequently malls are remodeled, resulting in breached firewalls and second or third dropped ceilings. Open ceilings and common lofts may allow for rapid undetected fire spread and require effective search for the fire. Fire load in most malls is heavy due to a large variety of Class A-type combustibles. This problem is compounded by restaurant cooking facilities. Automobiles may be on display in a mall concourse.

Mall entrances and store entrances often are locked or limited. It is important that communications regarding entrances are specific. In some cases, multiple entrances provide for good access and evacuation. It is strongly recommended that local fire departments devise a unique marking system to identify each of the many mall entrances with numbers or letters.

Malls frequently are located on the fringes of a city or between cities. Sufficient resources should be dispatched on the initial alarm to support sprinkler and standpipe systems, and augment water supply. Water supply may be inadequate, and response time may be increased due to distance and traffic.

Internal Fire Protection Systems

Internal fire protections systems include:

- Zoned sprinkler and standpipe systems (with shut offs located significant distances from grid served);

- Hydrants on property; and

- Smoke removal systems.

Pre-Incident Planning

The best type of preplanning for this type of structure is participation in the planning and construction phases of the project. Ensure that adequate water supply, fire department access, and life safety issues have been considered. Long stretches of hose line may be necessary, even when using standpipes. Making use of old/used rescue rope marked at 50-foot intervals may be used to estimate the length of hose line necessary from standpipe outlets or from engines to potential fire locations.

A thorough knowledge of structure and fire protection systems and frequent drills and incident preplanning sessions are essential. Include establishing good working relationships with management and security in the pre-incident preparation. Consider assigning a single inspector to monitor the site during initial construction, and any renovations to ensure that fire suppression, fire detection, and fire alarm systems remain in service. The fire department must control access, fire loading, and use of hazardous materials (for example, flammable or caustic substances). Keeping up with store names is difficult. Many mall management companies notify the water department, power company, realtors, and others of changes in rental occupants via e-mail. One possible answer is to have your preplan officer added to the mall notification tree.

Incident Considerations

- Divide the building into geographic areas inside, outside, and on the roof for all operations.

- Assist with evacuation. First alarm companies must be pre-assigned to proper standpipe and sprinkler connections.

- Protect lanes of egress.

- Provide search and rescue. The large areas and configuration of a mall require larger search teams than for other occupancies.

- Provide for medical assistance.

Forms of Notification

The fire department may receive calls for assistance in numerous manners. Alarm systems, cell phone calls, and landline request for assistance are possible mediums. Types of fire alarm and detection systems and how they may be transmitted to the fire department are shown below:

- Water flow—reported by sprinkler zones;

- Smoke detector—reported in separate smoke detector zones, and may be tied to HVAC systems;

- Pull-box fire alarm—may be reported by pull-box station or in separate zone;

- Heat detectors in restaurant kitchens; and

- Small, dry chemical, CO2, or other suppression system activation may trip separate alarm.

Other types of calls for assistance and how they may be characterized are listed below:

- Calls for assistance at an address, for example, “12,811 Jones Branch Road.”

- Calls for assistance at a store name, for example, “At  Smith’s Shoe Store a woman has been injured.”

- Calls for assistance at mall entrances, for example, “At the number 2 entrance, an automobile out front is on fire.”

- Calls for assistance by parking area, for example, “At the south-parking garage, a pedestrian has been struck.”

All of the above possibilities must be accounted for in pre-incident planning.

Fire Extinguishment

Firefighting in malls involves large personnel needs for search and rescue, water management, fire attack, and ventilation. A ventilation group or roof division may be an entire single-alarm assignment. Fire can spread in two or more directions after reaching the mall area. Key areas include front, rear, flanks, and roof. Large merchandise value necessitates that effective salvage efforts be started as soon as it is practical. Preplan locations of floor drains to remove potentially large amounts of water resulting from sprinkler activation. Consider using mall water vacuums, from mall maintenance that may be available.

- Support private fire suppression systems;

- Confirm location of fire in mall by the read out on the alarm panel;

- Assign resources to strategic locations and establish ICS;

- Cover flanks and monitor cockloft for possible fire spread;

- Determine fire flow requirements; and

- Provide sufficient initial personnel, forcible entry resources, ventilation, and relief personnel.

Support internal fire protection systems. First, water should go to sprinklers. Know sprinkler/standpipe locations and zones. Mall fires require long attack line stretches (300 to 500 feet). A single attack line probably will require personnel from more than one engine company. It may be more advantageous to extend attack lines through the closest entrance to the fire, rather than make use of horizontal standpipes in some cases. This should be noted on preplan. It is important to extend attack lines through the closest entrance to the fire. This may require a search within the structure before committing the initial hand line. Firefighters must be instructed to make use of tether lines or hose lines when entering large open areas, or confusing, maze-like hallways found in malls. Thermal imagers may help. However, a marking system must be used to follow when leaving the building.

Consider master streams inside structures but avoid opposing streams. To avoid opposing streams, delegate fire attack responsibilities out to branches and divisions with specific direction included in the incident action plan.

Some roofs may be difficult to breach for ventilation. Use mechanical roof openings such as skylights and ventilators. Be aware of the dangers of rooftop work such as false fronts, hidden drop-offs, heavy equipment (for example, HVAC, generators, and signs), and wires. Take advantage of HVAC system. It may be possible to regulate air supply to evacuate smoke from involved areas of the mall and reduce smoke advancement into uninvolved areas of the mall. Preplan with the building engineer. Consider making use of HVAC to control fire spread. If this is not possible, shut down HVAC as soon as possible.

Activity 13.1:

Enclosed Mall Simulation Exercise

Student Activity Worksheet

Time: 150 minutes (simulation and debrief) Purpose: To provide practice in using the ICS organization to manage an enclosed mall incident. Directions: - As a group, you will staff positions within the ICS of the Central City Fire Department (CCFD) for the exercise: - Incident Commander - Incident Commander Aide (not used unless nine students in a group) - Operations Chief - Operations Aide (a second Aide designated for groups with more than 10) - Planning Chief - Logistics Chief - Safety Officer - Information Officer - Liaison Officer - You will record all command decisions and actions during the exercise on the Simulation Action Chart. Entries on the ICS chart are made in response to written or visual messages or in anticipation of problems that could surface during the incident. Simulation outcomes: - Incident Commander: Team report - Operations Chief: Initial action plan and organizational chart - Operations Aide: Command worksheet, which includes objectives, strategies and tactics and an incident map - Planning Chief: Alternate plan - Logistics Chief: Resource list - Safety Officer: Safety message - Information Officer: Press release - At the conclusion of the exercise, all groups will participate in a debriefing facilitated by the instructors. Your group will display both the ICS charts and Simulation Action Charts that you developed during the exercise. Be prepared to explain your reasons for your decisions.

Incident overview: The incident occurs in Columbia Mall, a large structure with more than 2.1 million square feet of space under roof. Built in 1980’s, the building includes 5 major department stores and over 196 assorted specialty shops and restaurants. Construction is concrete block covered by a bar joist, flat pan roof. The entire mall is sprinkled, with the heads mounted below the drop ceiling. Access to the facility can be made from two roads (south and north) into the 7,500-car parking lot, which surrounds the building. There are three parking garages, one on the north and two on the south sides of the complex. Fire apparatus cannot enter the raised portions of the parking garages. The alarm sounds just before 1400 hours on the Friday after Thanksgiving. The mall has been heavily patronized since it opened at 0900. Virtually all parking places are occupied with some cars parked on grassy areas at the perimeters of the parking lot. Outside temperature is 76°F (24.4°C) with an east wind of 10 mph. Mall maintenance and the Central City Gas Company have been working on a reported minor gas odor at Entrance 4 all morning. An explosion occurs in the egress hallway of Entrance 4 on the south side of the complex, on the lower level. The fire that is triggered extends into the Miaro Bistro and down the hallway towards the main east-west mall concourse where it spreads to stores on the south side of that corridor. The sprinkler system in the egress corridor area has been damaged, making it inoperable in that section. All stores have been overstocked for the holiday shopping season, so the fuel load is high. Incident priorities: - Life safety - Incident stabilization - Property conservation Strategies: - Rescue  Ventilation - Exposures  Salvage - Confinement - Extinguishment - Overhaul - Using the incident priorities and strategies, develop the control objectives. - Develop an incident organization. - Assign and manage resources. Notes:

Enclosed Mall

Quick Access Fire Plan

| Building Address: 1000 Columbia Avenue (Route 50) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Building Description: Shopping Mall 2.1 million square feet Two and three stories, portions built in 1980’s CBS--5 major dept. stores, 196 shops and restaurants Roof Construction: Bar joist flat pan--Lexan™ skylight length of center mall Floor Construction: Concrete, covered by ceramic tile/carpet |  |  |  |  |  |  |  |  |
| Occupancy Type: Shopping center |  |  |  | Initial Resources Required: One commercial bldg. alarm |  |  |  |  |
| Hazards to Personnel: Ingress, egress, horizontal interior hose lays |  |  |  |  |  |  |  |  |
| Location of Water Supply: Hydrants on site (10) 12-inch main |  |  |  | Available Flow: 1,600-gpm per hydrant |  |  |  |  |
|  |  | Estimated Fire Flow |  |  |  |  |  |  |
|  | Level of Involvement | 1 store |  | 3-5 | Dept. Store |  | Wing |  |
|  | Estimated Fire Flow | 500 gpm |  | 1,000 gpm | 3 - 5,000 gpm |  | 5,000 gpm |  |
| Fire Behavior Prediction: Horizontal spread, common ceiling between stores. |  |  |  |  |  |  |  |  |
| Predicted Strategies: Rescue (primary search), confinement, ventilation. |  |  |  |  |  |  |  |  |
| Problems Anticipated: Access to fire, occupants, zoned sprinklers, and long hose lays. |  |  |  |  |  |  |  |  |
|  Standpipe: Horizontal |  |  |  Sprinklers: Zoned |  |  |  Fire Detection: Smoke, water flow, heat detection |  |  |

Enclosed Mall Plot Plan

Enclosed Mall Plot Plan

Side D

Side B

Enclosed Mall Plot Plan

Enclosed Mall Plot Plan

Central City

Incident Locator Preplan—Central City Mall

| SPRINKLER ZONE 1 | SPRINKLER ZONE 2 | SPRINKLER ZONE 3 |
| --- | --- | --- |
| - Jones Shoes - Johnson’s Fashions - Office Providers - Multi-Screen Theaters - Bert’s Drug Store - Phil’s Steak and Cheese - Doctor John Offices - Central Women’s Store | - Tri-City Toys - Jeffries Department Store - U.S. Cards - Personal Things - O’Neil’s Gifts - Bob’s Dental - Murgallis Fine Dining - Phelps Dollar Store | - Gaines Electronics - MacDonald’s Running Store - Mall Conference Rooms - Maxine’s Pets - Central Specialty Foods - Tomorrow’s Fashions - Cooking Things |
|  | SPRINKLER ZONE 4 |  |
|  | - Women Today - Shoe Family - Harrison’s Restaurant - China Specialty Store - Dancing Dan’s Furniture - Josephine’s Furnishings - The Arcade - Goode Hunting Equipment - Boyd’s School Supplies |  |

Occupancies at Central City Mall

| Alphabetical List | Sprinkler Zone | Smoke Detector Zone | Entrance | Heat Detector |
| --- | --- | --- | --- | --- |
| Bert’s Drug Store | 1 | A | Clock | AA |
| Bob’s Dental | 2 | A | Fashion | AA |
| Boyd’s School Supplies | 4 | B | Grand | N/A |
| Central Specialty Foods | 3 | C | Stable | CC |
| Central Women’s Store | 1 | A | Clock | AA |
| China Specialty Store | 4 | B | Grand | N/A |
| Cooking Things | 3 | C | Stable | CC |
| Dancing Dan’s Furniture | 4 | B | Grand | N/A |
| Doctor John Offices | 1 | A | Fashion | AA |
| Gaine’s Electronics | 3 | C | Stable | CC |
| Goode Hunting Equipment | 4 | B | Grand | N/A |
| Harrison’s Restaurant | 4 | B | Grand | BB |
| Jeffries Department Store | 2 | A | Clock | AA |
| Johnson’s Fashions | 1 | A | Clock | AA |
| Jones Shoes | 1 | A | Clock | AA |
| Josephine’s Furnishings | 4 | B | Grand | N/A |
| MacDonald’s Running | 3 | C | Stable | CC |
| Mall Conference Rooms | 3 | C | Stable | CC |
| Maxine’s Pets | 3 | C | Stable | CC |
| Multi-Screen Theaters | 1 | A | Clock | AA |
| Murgallis Fine Dining | 2 | A | Clock | AA |
| Office Providers | 1 | A | Clock | AA |
| O’Neil’s Gifts | 2 | A | Clock | AA |
| Personal Gifts | 2 | A | Clock | AA |
| Phelps Dollar Store | 2 | A | Clock | AA |
| Phil’s Steak and Cheese | 1 | A | Clock | AA |
| Shoe Family | 4 | B | Grand | N/A |
| The Arcade | 4 | B | Grand | N/A |
| Tomorrows Fashions | 3 | C | Stable | CC |
| Tri-City Toys | 2 | A | Clock | AA |
| U. S. Cards | 2 | A | Clock | AA |
| Women Today | 4 | B | Grand | N/A |

Central City Fire and Emergency Medical Services Fire Resources Overhead Positions—one Fire Chief, one Assistant Chief, one Chief Fire Marshal, seven Fire Inspectors, one Training Captain, two Training Lts., three Deputy Chiefs (1 per shift), six Battalion Chiefs (2 Battalions x 3 shifts). Eleven engine companies—Engines 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, and 11. Each engine company is staffed with one officer and three firefighters. Four truck companies—Trucks 1, 3, 5, and 8. Each truck company is staffed with one officer and three firefighters. Squad Company 1. The squad company is staffed with one officer and two firefighters fully trained in rescue practices. Crash/Fire/Rescue Engine. Airport-staffed with one officer and two firefighters. Engine 103, Engine 104, Engine 105, and Engine 109. Reserve Engines—fully equipped. Truck 103. Reserve Truck—fully equipped. Engine 104 (1,500-gpm foam pumper). Apparatus contains one 500-gallon foam tank; 400 five-gallon cans of foam are stored at Engine 4. All members of Engine 5 and Truck 5 have been fully trained as a Hazardous Materials Response Team. Mask Service Unit 106. Engine 206 (light truck). Boat 2 - 16’ Boston Whaler. EMS Resources (third service operated by the city and stationed in the fire stations): Three EMS supervisors – (one per shift x 3 shifts) operating out of fire station 1. Five ALS ambulances – ALS 2, 4, 7, 9, and 11 Five BLS ambulances – BLS 3, 5, 6, 8, and 10

Central City Fire/EMS Response Criteria 1st-Alarm Resource Card Response time: 5 min. Engine 1 Truck 1 Battalion 1 Engine 2 Ambulance 2 MSU 6 Engine 3 Squad 1 2nd-Alarm Resource Card Response time: 10 min. Engine 4 Truck 3 Battalion 2 Engine 5 Ambulance 3 Deputy Chief Engine 6 EMS Supervisor 3rd-Alarm Resource Card Response time: 15 min. Engine 7 Truck 5 Fire Marshal (Bn. Chief) Engine 8 Trng. Battalion Chief Engine 9 Adm. Battalion Chief 4th-Alarm Resource Card Response time: 15 min. Engine 10 Truck 8 Engine 11 5th-Alarm Resource Card Response time: 20 min. Fisherville Engine 1 Harvest Junct. Engine 2 Kingston Engine 1 6th-Alarm Resource Card Response time: 20 min. Apple Valley Engine 1 Bayport Engine 2 Jasper Engine 2 7th-Alarm Response Card Response time: 25 min. Deep Water Engine 2 Kingston Engine 3

Simulation Action Chart

MSSG.		TIME			ACTION TAKEN

Health and Safety Message

| Incident Name: |  | Date Prepared: |  |  | Time Prepared: |
| --- | --- | --- | --- | --- | --- |
| Operational Period Date: From: To: |  |  | Operational Period Time: From: To: |  |  |
| Major Hazards and Risks: |  |  |  |  |  |
| Narrative: |  |  |  |  |  |
| Prepared By; | Company Name: |  |  | ICS Position: Safety Officer |  |
| Approved By: | Company Name: |  |  | ICS Position: |  |

Press Questions

What is the current situation?

How many occupants have been injured?

Have any deaths been reported?

How did the fire start? Where?

Did the smoke detectors operate properly?

Activity 13.1:

Enclosed Mall Simulation Exercise

Student Assessment sheet

Debriefing: Debriefings are discussions centered on the cues contained in the written and visual messages, the strategies you developed as responses to the cues, and your organization and management of the incident staff. - What critical cues did you observe? - What ICS organization did you develop? - Did your group effectively prioritize responsibilities and actions? - Did you pass information to all who needed to be aware of it? - Did you promptly deal with messages? - Did you identify alternate methods of performing tasks? - Did you recognize the need for additional/outside resources? - Did you coordinate the allocation of resources? - Did you periodically re-evaluate resources? - How did your actions differ from the actions in the matrix? Why did they differ? - What are your strengths and weaknesses in working on an IMT for this incident? - How will you apply what you learned in this exercise to preplanning for a target hazard of this type in your community? - How will you apply what you learned in this exercise to responding to an incident at a target hazard of this type in your community?

Notes:

Module Summary

Managing an incident at an enclosed mall requires knowledge of the complex structure of enclosed malls and extensive pre-incident planning.

Incident considerations for an incident with heavy life risk include internal fire protection systems, large water supply needs, ventilation, and high personnel requirements.