Module 14:

Public Assembly/Mass Casualty Simulation Exercise

Objective

Upon completing the module, you will be able to effectively manage a public assembly/mass casualty incident.

Methodology

This module uses lecture, discussion, and a simulation exercise.

NFPA 1021 Standards Cross-Reference Matrix

3-6, 3-7, 4-6, 4-7

Assembly facilities

Definition

Assembly occupancies include, but are not limited to, all buildings or portions of buildings used for gathering together 50 or more persons for such purposes as deliberation, worship, entertainment, eating, drinking, amusement, or awaiting transportation.

The size and use are both essential pieces of information in classifying buildings as assemblies. A restaurant with an occupant load of less than 50 has the same use as one with the capacity for 110 persons, but the smaller one is not an assembly. The code requirements for the nonassembly will vary greatly from those adopted to keep assembly occupants safe during times of fire, panic, or other emergencies.

Types of Assembly Occupancies:

- Armories,

- Assembly halls,

- Auditoriums,

- Bowling lanes,

- Churches,

- College/university classrooms (50 persons or more),

- Conference rooms,

- Courtrooms,

- Dance halls,

- Drinking establishments,

- Exhibition halls,

- Gymnasiums,

- Libraries,

- Mortuary chapels,

- Motion picture theaters,

- Museums,

- Passenger stations and terminals of air/surface, underground, and marine public transportation facilities,

- Pool rooms,

- Recreation piers,

- Restaurants,

- Skating rinks, and

- Theaters.

Critical Factors Regarding Assemblies

Construction of the assembly will be a critical factor in developing strategy for a fire or mass casualty incident:

- Ordinary construction,

- Bowstring construction,

- Truss construction,

- Steel bar joists,

- Concrete/steel construction with an atrium (mid-rise/high-rise style),

- Wood-frame construction,

- Prefabricated structures, and

- Tents.

Characteristics of Assembly Design

- Large, unbroken area;

- Multiple levels of occupancy;

- Open stairwells;

- Below-grade assembly areas;

- Doors often locked for security reasons;

- Exit doors often hidden by furnishings;

- Often narrow passageways leading to assembly area;

- Occupancy flow designed for entering and exiting from the same point (People evacuating will seek the same door to exit from which they entered.); and

- Apparatus placement limited by single access roads.

Fire Protection

- Sprinklers may or may not be present;

- Large kitchens are common;

- Knowledge of Heating, Ventilating, and Air Conditioning (HVAC) systems;

- Various types of alerting systems are installed;

- Employee training in fire safety usually is limited; and

- Water supply for firefighting may be limited because of assembly location, such as a remote area.

Mass Casualty Incidents (MCIs)

Preparedness

### Target Hazards

Are you aware of the mass casualty potential in your area?

For example: If airports are in your jurisdiction, you should have an airport procedure. Network with an urban department to identify what that procedure entails.

Prisons are potential sites of Mass Casualty Incidents (MCIs) that would present special problems (for example, Camp Hill, Pennsylvania, October 1989). Safety of your people is a paramount consideration in writing a procedure, holding a drill, or working an actual incident. Do not be misled by a uniform. Prisoners have exchanged their attire with guards by force in the past. Security must be a major part of your prison plan.

Manufacturing plants can vary greatly (for example, Chicken Mfg., North Carolina 1991). Familiarization is important. If you are at least cognizant of what is manufactured and how it is manufactured, it will assist in your capability to handle an emergency of any kind at the facility.

Chemical plants are another type of facility where familiarization is important (for example, Bhopal, India, 1985). You should be provided a list of hazardous materials on site. You must be aware of the potential problems, not only on site, but also to the surrounding area. Evacuation plans should be established. Identify key plant personnel.

Public transportation systems recently have been the source of several MCIs (for example, Philadelphia, Pennsylvania, March 1990). They can present unique problems. Be aware of the problems and solutions that those responders faced so that you can learn from their experiences.

Interstate highway systems can present problems as varied as major accidents or haz mat releases (for example, Calhoun, California, February 1991). Knowledge of accesses and alternative means of reaching the scene are important. Removing victims from the scene also can be difficult. You will need cooperation.

Geographical concerns—If your potential for MCIs is especially acute during certain times of the year (for example, floods, tornadoes in the spring, hurricanes in the late summer), do you prepare for these seasonal problems? (An example is the earthquake in San Francisco, California, October 1989.)

### Teambuilding

Identify how you can build a stronger team. Identify, via training and drills, potential problems. Do not wait for an emergency situation to identify problems.

Be diplomatic enough to address problems in a way that strengthens the team rather than causes resentment or hard feelings. Your goal is to build a team, not paralyze or destroy it.

You should have a sound professional relationship with neighboring departments. You need each other. Is a mutual-aid plan in effect or needed? If it is, you must be familiar with it. The chain of command and lines of authority should be clear to all. Jurisdictional disputes are unwarranted and unprofessional. Mass casualties will expose your shortcomings if you do not work on them.

What has been said about interdepartmental considerations also applies to interagency. You must communicate with potential interagency responders before an emergency. Review your relationships and/or procedures regarding other agencies, for example, police, medical, helicopter, and utilities.

### Training

Identify what relative material or courses you can provide for your people. Have a positive program. You are teambuilding.

Instructors must be motivated properly and care about the people they are instructing and the quality of their work. Instructors must have credibility.

If you have a training facility, use it. Your people will be more able to learn if you provide an environment that lends itself to that goal.

If you do not have a training facility, the area you use should be made as conducive to learning as possible. Control the environment. Remove distractions such as newspapers; turn off televisions and radios.

Conduct mass casualty drills. Be as realistic as possible. Students must realize that a drill is serious and act accordingly.

Use what you learn in this course to set up your drill. Make drill as full scale as possible. It will pay off. It will improve morale and confidence.

Evaluate your own performance and how, as an individual, you can improve. Share it with others. This is leadership. Reinforce the idea that training is a learning experience.

Conduct a critique of your drill: What did you learn? What worked? What went wrong? Why did it go wrong? Identify and rectify areas of concern.

Start planning for your next drill.

Access

### Above-grade

Elevated trains—If there are elevated trains in your area, you should be prepared for mass casualty potential.

Mountaintops/sides—You must be able to reach the incident to be of value. You must be able to get there with the needed equipment.

Highway spans/bridges are potential areas of difficult access if a MCI should occur there. It may be necessary to move injured persons to the opposite side of your jurisdiction. All bridges that have separate jurisdictions on opposite ends should have a bridge procedure. If your bridge has standpipes, you must be knowledgeable of connections.

### Below-grade

Subways—Tunnels are potential mass casualty locations. You must have plans. Is there access between stations? You may need water, and lighting is essential. Long carries up several levels are personnel intensive. Power must be turned off immediately for a safer atmosphere for workers. Remember, you may have victims coming out at different locations.

Mines—Explosions or collapses in mines may trap workers. Balancing speed and safety is agonizing but necessary. Secondary collapse or explosions are a threat.

Highway spans/tunnels—Highways that are underground or underwater are difficult to access. Traffic may be gridlocked.

### Remote Locations

Is the incident difficult to access?

Evaluate response times and nearness of medical facilities. It may be necessary to request medical and surgical teams to come to the scene. You should know beforehand how victims will be transported. If your area has access to a medical helicopter, a policy for its use should be in place.

### Separate but Related

It is possible for an incident that is spread over a large area to appear to be separate incidents, for example, air, land, or water collisions. Keeping control of all aspects of the incident is vital. You may have units responding from diverse locations. All units and personnel involved in the incident must be integrated into the Incident Command System (ICS).

Resources

### Vehicles/Apparatus

What vehicles and apparatus do you have available?

### Medic Units

Do you have the necessary transportation vehicles, for example, medic units and ambulances?

### Police Units

Police will be on the scene. You must identify how they will be of assistance. Police can be of assistance for crowd control, traffic control, security, and, if necessary, drive vehicles if medic unit drivers are committed. They also can be of assistance in transporting medical supplies and personnel. It is necessary to have police be a part of your ICS.

### Helicopters

Helicopters can be used. Your helicopter policy will identify when they may be requested and who is in charge. Policy on establishing landing zones should be in place.

### Buses

If a large number of victims have sustained minor injuries, a bus may be a feasible way of rapidly removing victims. EMTs or paramedics should be on the bus. Do not take large numbers of minor injuries to the closest facility if you anticipate more seriously injured shortly thereafter.

### Tow Truck

Tow trucks can be used to separate wreckage.

### Fire Apparatus and Equipment

You may need pumpers, ladders, and special equipment, for example, jaws of life and heavy rescue equipment. Call for help early and maintain scene discipline. Mass casualty is manpower intensive.

### Personnel

People are the most vital resource. Without people, nothing would move or operate. Personnel fall into two classifications: direct and peripheral.

Direct personnel refers to those involved with the emergency scene directly. These people must be considered when thinking of assistance, fatigue, weather conditions, and emotional/stress conditions. A staging area should provide people at-the-ready for additional help or relief to members directly involved. This can be on a rotating basis or permanent relief. (If the emergency involves great loss of life and/or injury, the emotional/ stress load on the emergency personnel must be considered.) A resource pool for direct people could be any of the following:

- Further alarms (on-duty members);

- Fire and/or EMS training schools dismissed to the emergency;

- Call back (off-duty members);

- Volunteers (only qualified personnel);

- Mutual aid (neighboring townships/counties); or

- National Guard or Reserves.

Peripheral personnel refers to those indirectly involved with the emergency scene, yet very important in the totality of management of the overall scene. The needs would vary from laborious tasks to professional help. A resource pool for peripheral people could be any of the following:

- Police department,

- Police training schools,

- College/university,

- High school seniors, and

- Large corporations.

All of these suggested personnel pools require preplanning and development of interagency partnerships. A crisis, by its nature, causes people to pull together to accomplish feats beyond comprehension. All of the agencies listed would be very interested in the preservation of life and/or property within their community. Look around your community to determine your resources. Develop a sound emergency plan.

### Hospitals

Trauma centers—Victims in need of trauma center care must be identified and taken to that facility. Contact with the facility is necessary to know capacity limits.

### Pediatric Care Center

If you have a pediatric care center nearby, pediatric cases should be transported there. Again, contact with the facility is necessary to see how many can be handled.

### Special Care—Burns, Spinal Injuries, Eye Injuries

Victims needing a special-care facility should, of course, go to that facility if one is available.

### Equipment

Medical—Do you have appropriate and adequate medical equipment?

You must know where adequate supplies can be obtained on short notice, for example, spine boards, collars, splints, and drugs.

### Lighting

Extensive lighting may be necessary at many mass casualty operations. Are you capable of supplying the need? Ascertain who may be able to provide assistance.

### Construction Equipment

Contractors may have specialized equipment that can be helpful, for example, cranes and backhoes. You should have some agreement with such people as to what they could provide in an emergency. Naturally, this agreement cannot be abused.

Communications

### In-House

Do you have enough radios? Can hard-line telephones be obtained, and will they help? In addition, cellular telephones are a valuable tool, if you can obtain them.

### Interagency

Communications must flow smoothly through various agencies on scene. One aspect of the ICS is the span-of-control and how communications should flow. ICS should be used on small incidents so that terminology and theory of incident command is familiar to everyone in the organization.

### Microphone

Can you provide communication equipment to trapped victims if they are not within voice distance?

### Medical

Keep medical communication on separate band or frequency so as not to tie up one band.

### Media

The media should be accommodated but controlled. The Information Officer (IO) will keep the media apprised of the situation at the proper time and place. If information is not available or cannot be released immediately, inform the media and assure them that their needs are being considered.

Figure 14-1: Fire/Medical ICS Organization

Figure 14-2: Medical ICS Organization

Safety/Security

### Rescuers

Do all that can be done to establish safety for the rescuers.  Assign a safety officer.

### Casualties

Use the principle of “No Further Harm” for casualties.

### Police Assistance

Secure the area from onlookers and the press. Anyone not involved in the incident or not needed at the scene should be kept away.

Precautions must be taken to prevent the possibility of looting.

### Technical Assistance

Is there a higher level of expertise that can be contacted, either through phone or on site, for example, a chemical or structural engineer? Have the engineer work with the planning section unless the IC requests his or her presence at the Command Post (CP).

### Media/Bystanders

Secure areas from the media/press and bystanders. Anyone not involved in bringing the event to a conclusion should not be in close proximity to the incident. If necessary, enlist police assistance.

### Monitor the Scene

Constantly monitor. Do not let secure areas be compromised. Assign responsibility for this chore.

### Haz Mat Considerations

Make arrangements to handle hazardous and medical waste.

Do it right. Do not leave yourself open to criticism at this point. By this time, you may be fatigued or emotionally spent. Don’t let these emotions deter you from sound decisions.

Conclusion/Recovery

### Withdrawal

It is necessary at any emergency scene to have an orderly, effective, and safe conclusion.

Check the area of the incident and make sure all responders and casualties are withdrawn.

### Security

Secure the scene for investigators. Use the proper personnel for this function. Make sure the roles of security personnel are known.

### Transfer

Have an effective transition with the agency that will remain on scene after withdrawal.

### Debriefing

Determine if critical incident debriefing is appropriate and necessary. Are you prepared to have a critical incident debriefing team interface with your people?

### Critique

Have a critique as soon as possible after the event. Remember, the critical incident debriefing and the critique are never related. A critique should be a learning experience. If you feel or know that an individual made mistakes, be tactful and diplomatic. If members see critiques as a fault-finding operation, they will lose their effectiveness.

Activity 14.1:

Airport Pre-Incident Planning

Time: 60 minutes Purpose: To anticipate strategy development and resource management for a target hazard incident. Materials: - Easel pads - Markers - Airport Terminal Quick Access Prefire Plan (QAP) Directions: - Your instructor will divide the class into small groups of four or five students. - Your group will brainstorm and identify the most important pre-incident planning concerns for the Central City Airport. The airport, located in Liberty County, is under the responsibility of the Central City Fire Department (CCFD) for emergency fire protection. - Record your pre-incident planning concerns on an easel pad. You will have 30 minutes to identify and record your concerns. - Select a spokesperson to report your group’s concerns to the class. Airport Information: - Location: Liberty County, ½ mile outside of Central City. - Type: Index “B”—includes aircraft at least 90’, but less than 126’ in length. Accommodating small jets and prop aircraft. Often 60 “souls on board” can be anticipated on an aircraft. - Access: Two-lane road in and out of airport. Five-degree upward slope entering airport. - Construction: One and two-story noncombustible structure. Second floor houses control tower at junction of A/B sides. - Occupancy: Airport terminal building. Limited number of consumer shops and car rental agencies. Restaurant/bar located on first floor, Sides B/C. Propane heating and cooking system. Garage located on Side C. - Fire Protection: Garage houses maintenance vehicles and quick-response foam pumper carrying 1,500 gallons of water, 100 gallons of Aqueous Film-Forming Foam (AFFF) concentrate, and 500 pounds of sodium-based dry chemical. Three firefighters/EMTs staff airport from 0600 to 2200 hours daily during period airport is open for incoming and outgoing aircraft. - First-due Central City engine company has approximately a 3-minute response time. - EMS: First aid equipment on pumper/semi-automatic external defibrillator. - Runways: Two available for aircraft, parallel, running east and west. - Fuel Depot: Two 40’ high x 30’ diameter atmospheric tanks (external floating roofs) located ¼ mile from terminal building of airport. - Miscellaneous: Often during peak travel periods 100 commuters may be at the airport. - Staffing: One officer, two firefighters/EMTs on duty between 0600 hours and 2200 hours. Refer to the Quick Access Prefire Plan (QAP) for additional information.

Airport Terminal

Quick Access Prefire Plan

| Building Address: 110 Sky View Drive |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Building Description: One- and two-story—Noncombustible—75’ x 175’ Roof Construction: Steel bar joist with concrete asphalt deck and covering Floor Construction: First floor—concrete slab Second floor—Steel bar joist, concrete |  |  |  |  |  |  |  |  |
| Occupancy Type: Airport terminal building |  |  |  | Initial Resources Required: First-alarm assignment |  |  |  |  |
| Hazards to Personnel: Restaurant facilities—fuel tank storage |  |  |  |  |  |  |  |  |
| Location of Water Supply: Hydrants—8-inch county system—hydrants 100’ apart |  |  |  | Available Flow: 1,000-gpm per hydrant |  |  |  |  |
|  |  | Estimated Fire Flow |  |  |  |  |  |  |
|  | Level of Involvement | 10% | 25% |  | 50% |  | 100% |  |
|  | Estimated Fire Flow | 550 | 1,400 |  | 2,750 |  | 5,500 |  |
| *Fire flow based on first floor or terminal building with one exposure story. |  |  |  |  |  |  |  |  |
| Fire Behavior Prediction: Internal exposures, open areas, terminal Flammable liquid fire, storage areas, aircraft |  |  |  |  |  |  |  |  |
| Predicted Strategies: Rescue, confinement, extinguishment, ventilation |  |  |  |  |  |  |  |  |
| Problems Anticipated: Access, water supply, aircraft |  |  |  |  |  |  |  |  |
|  Standpipe: N/A |  |  Sprinklers: N/A |  |  |  |  Fire Detection: |  |  |

Notes:

Activity 14.2:

Airport Incident Simulation Exercise

Student Activity Worksheet

Time: 150 minutes (simulation and debrief) Purpose: To provide practice in using the ICS organization to manage a public assembly/mass casualty incident. Directions: - As a group, you will staff positions within the ICS of the Central City Fire Department (CCFD) for the exercise: - Incident Commander - Incident Commander Aide (not used unless nine students in a group) - Operations Chief - Operations Aide (a second Aide designated for groups with more than 10) - Planning Chief - Logistics Chief - Safety Officer - Information Officer - Liaison Officer - You will record all command decisions and actions during the exercise on the Simulation Action Chart. Entries on the ICS chart are made in response to written or visual messages or in anticipation of problems that could surface during the incident. Simulation outcomes: - Incident Commander: Team report - Operations Chief: Initial action plan and organizational chart - Operations Aide: Command worksheet, which includes objectives, strategies and tactics and an incident map - Planning Chief: Alternate plan - Logistics Chief: Resource list - Safety Officer: Safety message - Information Officer: Press release - At the conclusion of the exercise, all groups will participate in a debriefing facilitated by the instructors. Your group will display both the ICS charts and Simulation Action Charts that you developed during the exercise. Be prepared to explain your reasons for your decisions.

Incident overview: The incident occurs at 1400 hours on December 23, one of the busiest travel days of the year. The airport is a regional facility that handles commuter and medium jet aircraft. It had been snowing since 0430, but stopped at 0800. At the time of the incident, accumulation is 15 inches. This snow was added to previous snow accumulation of 66 inches from 3 previous storms. Winds were blowing from the northeast at 18 to 22 mph. No planes were flying due to blizzard conditions. The Liberty County Airport is located near the town of Kingston. The airport is governed by the Liberty County Airport Authority, which has contracted with Central City for fire, EMS, and security services. The agreement with the Authority places the airport property within Central City jurisdictional control. Flights were restarted at 1300 hours, so approximately 100 persons are still at the airport. Most are in the gate areas, the terminal, and the restaurant and adjacent bar. At 1403 hours, the roof covering the B/C corner of the terminal collapses into the restaurant/bar and the lobby/check-in area on the first floor. Approximately 75 people are trapped. Incident priorities: - Life safety - Incident stabilization - Property conservation Strategies: - Rescue  Ventilation - Exposures  Salvage - Confinement - Extinguishment - Overhaul - Using the incident priorities and strategies, develop the control objectives. - Develop an incident organization. - Assign and manage resources.

Airport Terminal

Quick Access Prefire Plan

| Building Address: 110 Sky View Drive |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Building Description: One- and two-story—Noncombustible—75’ x 175’ Roof Construction: Steel bar joist with concrete asphalt deck and covering Floor Construction: First floor—concrete slab Second floor—Steel bar joist, concrete |  |  |  |  |  |  |  |  |
| Occupancy Type: Airport terminal building |  |  |  | Initial Resources Required: First-alarm assignment |  |  |  |  |
| Hazards to Personnel: Restaurant facilities—fuel tank storage |  |  |  |  |  |  |  |  |
| Location of Water Supply: Hydrants—8-inch county system—hydrants 100’ apart |  |  |  | Available Flow: 1,000-gpm per hydrant |  |  |  |  |
|  |  | Estimated Fire Flow |  |  |  |  |  |  |
|  | Level of Involvement | 10% | 25% |  | 50% |  | 100% |  |
|  | Estimated Fire Flow | 550 | 1,400 |  | 2,750 |  | 5,500 |  |
| *Fire flow based on first floor or terminal building with one exposure story. |  |  |  |  |  |  |  |  |
| Fire Behavior Prediction: Internal exposures, open areas, terminal Flammable liquid fire, storage areas, aircraft |  |  |  |  |  |  |  |  |
| Predicted Strategies: Rescue, confinement, extinguishment, ventilation |  |  |  |  |  |  |  |  |
| Problems Anticipated: Access, water supply, aircraft |  |  |  |  |  |  |  |  |
|  Standpipe: N/A |  |  Sprinklers: N/A |  |  |  |  Fire Detection: |  |  |

Central City Fire and Emergency Medical Services Fire Resources Overhead Positions—one Fire Chief, one Assistant Chief, one Chief Fire Marshal, seven Fire Inspectors, one Training Captain, two Training Lts., three Deputy Chiefs (1 per shift), six Battalion Chiefs (2 Battalions x 3 shifts). Eleven engine companies—Engines 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, and 11. Each engine company is staffed with one officer and three firefighters. Four truck companies—Trucks 1, 3, 5, and 8. Each truck company is staffed with one officer and three firefighters. Squad Company 1. The squad company is staffed with one officer and two firefighters fully trained in rescue practices. Crash/Fire/Rescue Engine. Airport-staffed with one officer and two firefighters. Engine 103, Engine 104, Engine 105, and Engine 109. Reserve Engines—fully equipped. Truck 103. Reserve Truck—fully equipped. Engine 104 (1,500-gpm foam pumper). Apparatus contains one 500-gallon foam tank; 400 five-gallon cans of foam are stored at Engine 4. All members of Engine 5 and Truck 5 have been fully trained as a Hazardous Materials Response Team. Mask Service Unit 106. Engine 206 (light truck). Boat 2 - 16’ Boston Whaler. EMS Resources (third service operated by the city and stationed in the fire stations): Three EMS supervisors – (one per shift x 3 shifts) operating out of fire station 1. Five ALS ambulances – ALS 2, 4, 7, 9, and 11 Five BLS ambulances – BLS 3, 5, 6, 8, and 10

Central City Fire/EMS Response Criteria 1st-Alarm Resource Card Response time: 5 min. Engine 1 Truck 1 Battalion 1 Engine 2 Ambulance 2 MSU 6 Engine 3 Squad 1 2nd-Alarm Resource Card Response time: 10 min. Engine 4 Truck 3 Battalion 2 Engine 5 Ambulance 3 Deputy Chief Engine 6 EMS Supervisor 3rd-Alarm Resource Card Response time: 15 min. Engine 7 Truck 5 Fire Marshal (Bn. Chief) Engine 8 Trng. Battalion Chief Engine 9 Adm. Battalion Chief 4th-Alarm Resource Card Response time: 15 min. Engine 10 Truck 8 Engine 11 5th-Alarm Resource Card Response time: 20 min. Fisherville Engine 1 Harvest Junct. Engine 2 Kingston Engine 1 6th-Alarm Resource Card Response time: 20 min. Apple Valley Engine 1 Bayport Engine 2 Jasper Engine 2 7th-Alarm Response Card Response time: 25 min. Deep Water Engine 2 Kingston Engine 3

Simulation Action Chart

MSSG.		TIME			ACTION TAKEN

Health and Safety Message

| Incident Name: |  | Date Prepared: |  |  | Time Prepared: |
| --- | --- | --- | --- | --- | --- |
| Operational Period Date: From: To: |  |  | Operational Period Time: From: To: |  |  |
| Major Hazards and Risks: |  |  |  |  |  |
| Narrative: |  |  |  |  |  |
| Prepared By; | Company Name: |  |  | ICS Position: Safety Officer |  |
| Approved By: | Company Name: |  |  | ICS Position: |  |

Press Questions

What has happened here?

How many people have been injured? Are there any fatalities?

We have a report that a bomb exploded? Is this a terrorist incident?

How long will the airport be shut down?

Can the area hospitals treat all the injured?

Is snow causing problems in transporting the injured?

How many resources are at the scene?

How long will it be before the incident is controlled?

Activity 14.2:

Airport Incident Simulation Exercise

Student Assessment sheet

Debriefing: Debriefings are discussions centered on the cues contained in the written and visual messages, the strategies you developed as responses to the cues, and your organization and management of the incident staff. - What critical cues did you observe? - What ICS organization did you develop? - Did your group effectively prioritize responsibilities and actions? - Did you pass information to all who needed to be aware of it? - Did you promptly deal with messages? - Did you identify alternate methods of performing tasks? - Did you recognize the need for additional/outside resources? - Did you coordinate the allocation of resources? - Did you periodically re-evaluate resources? - How did your actions differ from the actions in the matrix? Why did they differ? - What are your strengths and weaknesses in working on an IMT for this incident? - How will you apply what you learned in this exercise to preplanning for a target hazard of this type in your community? - How will you apply what you learned in this exercise to responding to an incident at a target hazard of this type in your community?

Notes:

Module Summary

Critical factors that affect strategy in managing an incident at a public assembly:

- Construction of the assembly;

- Design;

- Access and egress points; and

- Fire protection.

The seven critical considerations for mass casualty incidents are:

- Preparedness,

- Access,

- Resources,

- Communications,

- Coordination,

- Safety/security, and

- Conclusion/recovery.