Module 15:

High-Rise Simulation Exercise

Objective

Upon completing the module, you will be able to effectively manage a high-rise incident.

Methodology

This module uses lecture, discussion, and a simulation exercise.

NFPA 1021 Standards Cross-Reference Matrix

3-6, 3-7, 4-6, 4-7

High-Rise Definition

A high-rise building is a structure that has a height of 75 feet or more. However, some buildings that are less than 75 feet present the same problems as ones over that height. In addition, it is likely that buildings, constructed just short of code requirements for a high-rise, do not have all the fire protection and life safety features of a building that meets a high-rise code.

Pre-Incident Planning

An important point to remember is that not all high-rise buildings are of the same design or construction, nor do they all have the same types of fire protection features. Construction methods and code requirements have changed through the years and can be a factor in fire behavior within a building. These changes can also affect the built-in protection features that help to mitigate or control fires within the building. Because high-rise buildings can vary greatly, it is critical that firefighting personnel be familiar with each building and the operation of the life safety and fire protection systems that it contains.

Strategy and tactics for high-rise fires are greatly affected by the fire personnel’s knowledge of the building’s construction and design features. Some of the questions that fire personnel need to have answered in the preplan include:

- Are there built-in life safety or fire protection features? What are they? Where are they? How do they operate?

- What is the current occupancy and layout of those parts of the building affected by the fire?

- Is the fire floor compartmented?

- Is the fire floor an open space area that will allow the fire to spread quickly?

This critical information must be gathered ahead of time through effective pre-incident planning. The degree of firefighting success may be measured not only by the presence of fire safety features, but also by the ability of firefighting personnel to take advantage of these features.

Old-Style High-Rise Construction

Old-style high-rise buildings were constructed with bearing walls made of masonry. Most of these buildings were constructed of reinforced concrete. Due to the weight of the construction materials, the walls at the bottom were many feet thick. As the building rose and the upper levels supported less weight, the walls were tapered. These buildings are really the epitome of what fire-resistive construction means.

Old-style high-rises have more mass because of their concrete construction. Old-style high-rises generally have operable windows, and they are least likely to collapse under fire conditions because of the mass of the bearing materials and the supporting nature of the exterior walls.

The symbol of old style construction is New York City’s Empire State Building. When a B-25 bomber crashed into the building, it only suffered some minor structural damage and content loss (due to running gasoline fires).

There are many high-rise buildings that have unreinforced masonry wall construction. The contents of these buildings are prone to destruction during fires as well as earthquakes.

New-Style High-Rise Construction

The modern method of constructing high-rise buildings is commonly called core construction. This method involves erecting a steel skeleton by using a column, girder, and beam system. The elevators, stair shafts, and utility shafts are placed in a core area. Most commonly, this core is found in the center of the building and is referred to as center-core construction. It should be noted that in some buildings this core would be found on the side. When this occurs, the construction is referred to as side core. In the case of side-core construction, stairways and elevators are located on exterior walls.

Core construction has less mass than old-style construction and is more vulnerable to heat from a fire. Floors have been known to sag nearly 2 feet under intense fire conditions and have caused, in at least one case, several portions of the wall assembly to fall. Some of these buildings have a sprayed coating on support members that assists in maintaining strength when subjected to heat or flames.

Table 15-1 lists high-rise structural features and components. This table provides a comparison of features found in old- and new- style high-rise buildings.

Table 15-1: High-Rise Structural Features and Components

| Feature/Component | Found in Old Style | Found in New Style | Found in Both Styles |
| --- | --- | --- | --- |
| Structural Framing Systems |  |  |  |
| Reinforced concrete | X |  |  |
| Structural steel |  | X |  |
| Exterior Walls |  |  |  |
| Part of bearing member | X |  |  |
| Prefabricated/curtain |  | X |  |
| Roofs |  |  |  |
| Concrete |  |  | X |
| Shingled/other |  |  | X |
| Shafts |  |  |  |
| Stair |  |  | X |
| Conventional |  |  | X |
| Pressurized |  | X |  |
| Scissor |  | X |  |
| Elevator |  |  | X |
| HVAC |  |  |  |
| Electrical |  |  |  |
| Mail |  |  |  |
| Dumbwaiter |  |  |  |
| Floors |  |  |  |
| Concrete poured over metal deck |  | X |  |
| Poured concrete | X |  |  |
| Interior Walls |  |  |  |
| Poured concrete | X |  |  |
| Concrete block |  |  | ü |
| Drywall on wood or metal stud |  | X |  |
| Compartmentation |  |  |  |
| Concrete with walls running floor to floor | X |  |  |
| Large open areas |  | X |  |
| Drywall with walls running floor to floor |  | X |  |
| Partition walls |  | X |  |
| Ceiling Assemblies |  |  |  |
| Steel wire attached to grid of metal channels (with plenum area) |  |  | X |
| Electrical Systems |  |  |  |
| Standard system |  |  | X |
| Emergency generator(s) |  |  | X |
| Elevators |  |  |  |
| Multiple |  |  | X |
| Low, medium, high-rise banks |  |  | X |
| Firefighter Service |  |  | X |
| Active Smoke Control Systems |  |  |  |
| Stairway pressurization |  | X |  |
| Building smoke control |  | X |  |
| Zone smoke control |  | X |  |
| Corridor smoke control |  | X |  |
| Elevator smoke control |  | X |  |
| Atrium smoke control |  | X |  |
| HVAC Systems |  |  |  |
| Standard system |  | X |  |
| Water Supply |  |  |  |
| 1 ½ inch wet standpipe | X |  |  |
| 2 ½ inch dry standpipe |  |  | X |
| 2 ½ inch wet standpipe |  |  | X |
| Sprinkler system |  | X |  |
| Combination system |  |  |  |
| Standpipe sprinkler |  |  |  |
| Emergency Communication Systems |  |  |  |
| Hardwired, jacks at every floor |  | X |  |
| Fire Control Room/Station |  |  |  |
| Sometimes on basement level |  |  | X |

Structural Features

### Structural Framing Systems

The structural frame of a high-rise building is the skeleton of the structure that supports not only the dead load of the building, but also live loads (such as occupants and building contents). Most commonly, high-rise buildings use one of the following framing systems as the basis for forming the building skeleton:

- Reinforced concrete (old style) or

- Structural steel (new style).

Both types use vertical interior and exterior columns to which lateral girders are attached. The girders span the horizontal distance between the columns and structural support beams. Although there is a difference between elements used for concrete and steel frames, they both serve the same function.

The construction design for high-rise buildings is based on the concept that the structural integrity of the building must be maintained sufficiently in any potential fire. Principal structural components have a high degree of heat resistivity. However, a number of structural stability concerns that fire personnel must be concerned with exist during fire conditions:

- Component failure (possible under prolonged exposure to sufficient heat);

- Floor beam failure (somewhat serious, but is also a localized occurrence);

- Girder failure (far more critical than floor beam failure as it would affect a significantly larger area);

- The failure of one or two girders causing column instability (potentially leading to a progressive collapse of the framing system); and

- Column failure resulting in serious structural instability (Depending on the location of the column, it could conceivably trigger extensive collapse damage to the structure.).

To achieve fire protection required by building codes for Type I (fire-resistive) construction, steel frame members in high-rise buildings are fireproofed. Fireproofing is accomplished by encasing the steel frames in concrete, sheet rock, or by spraying them with a protective coating. Concrete has the advantage of being the most permanent type of fireproofing, but its use is limited because it increases the dead-weight of a building. Improperly applied, sprayed-on, protective coatings can spall during a fire, which may leave the steel structural member exposed and subject to failure from excessive heat.

Concrete-frame structures tend to resist the effects of fire better than steel-frame structures, but they are less resistant to the effects of earthquakes. The ability of properly designed and constructed steel-frame high-rise buildings to withstand moderate earthquakes has been proven in many parts of the world in recent years.

### Exterior Walls

As stated earlier, the exterior walls of an old-style high-rise are part of the bearing members. Wall construction is reinforced concrete. These walls are very thick at the bottom and become smaller as they rise.

The exterior walls of modern high-rise buildings are commonly prefabricated and are typically lighter in weight than those in older buildings. In many cases, these walls are non-load-bearing and may be referred to as curtain walls. A complete curtain wall consists of a panel with finished surfaces and a means for attaching it to the building frame.

The most common method for attaching curtain walls to the building is by bolting them to clips that are attached to the structural frame or floor slab. This method of attaching walls often leaves a space of several inches between the end of the floor and the exterior wall. Unless this space is sealed with an effective fire and smoke barrier, it can provide a ready path for fire and smoke to spread to floors above, and allow water to penetrate to floors below.

The outside finish of a modern high-rise building is often referred to as the skin and usually consists of decorative materials such as aluminum, stainless steel, or lightweight concrete. Large window areas are also often present. These windows may be made of plain, tempered, or decorative glass. Metal alloy frames, backed up with conventional construction, hold the glass in place.

### Roofs

Roofs on high-rise buildings are required to have at least a 2-hour fire-resistive rating. In most cases, concrete construction exceeds this requirement. Careful consideration must be given to roof configuration during pre-incident planning. Pay particular attention to:

- Stair shaft exits and

- Other obstructions that would limit certain types of ventilation activities on the roof.

In many cases, not all stair shafts in the building will exit to the roof. Knowing which stair shafts exit to the roof can be critical when moving occupants to the roof for safe refuge or evacuation and when using stair shafts to exhaust smoke. It is also important to know if it is possible to land a helicopter on the roof for a top-down approach to firefighting, interior rescue, or to lift building occupants from the roof. Evacuation stairs should be vented (if possible) at roof level to make stairs tenable for occupants.

In most cases (unless the code under which the building was built required the provision of a helipad), landing a helicopter on the roof will probably be impossible. Various obstructions, such as machinery rooms, antennas, or lack of adequate landing space, will usually exist.

### Shaft Enclosures

Shaft enclosures in high-rise buildings are required to have a minimum of a 2-hour fire-resistive rating. Examples of shaft enclosures are stair, elevator, utility, HVAC, supply/return, and electrical shafts. Under fire conditions, any vertical shaft in a high-rise building can transfer heat and smoke to other parts of the structure. Therefore, it is critical to maintain shaft integrity. These shafts may be used as escape routes for building occupants or as access routes for firefighters. Failure to maintain the integrity of vertical shafts can:

- Transfer combustion products to remote parts of the building;

- Impede safe exit of building occupants; and

- Greatly restrict the ability of fire personnel to perform tactical operations.

### Stair Shaft Systems

Stair shaft systems are construction components of high-rise buildings about which firefighting personnel lack adequate knowledge.

As a rule, high-rise building stair shafts are built into the center core. Additional stair shafts may be on the structure’s outer perimeter (depending on the height and occupancy type of the building).

Shafts other than conventional, return-type shafts can be found in high-rise buildings. The building may contain pressurized or smoke-proof stair shafts. Activation of special equipment can provide a smoke-free atmosphere within these stair shafts. In many cases, however, even though the building has multiple stair shafts, only one stair shaft may be designed to provide this smoke-free environment. Some high-rise buildings feature stair shafts that are often referred to as smoke towers. These shafts are either fully or partially open to the outside atmosphere to prevent smoke from filling the shaft.

Many newly constructed high-rise buildings have “scissor” stair shafts that feature two sets of stairs in one common shaft. In some cases, each set of stairs may serve every floor, but entry points at alternate floors are on different sides of the center core. Others are designed so that one set of stairs serves odd-numbered floors while the other set serves even-numbered floors. While these subtle differences may not seem important, under fire conditions, they can be responsible for firefighters approaching the fire from a less than desirable location, or they can result in fire personnel going to the wrong floor.

Doors that provide access to the stair shaft from individual floors are often locked from the stair-shaft side. If they are locked, firefighting personnel must have a key to provide immediate access to the floors from the stair shaft. A substantial amount of time can be wasted forcing entry to the floor if a key is unavailable. Forcing the door is often difficult because of the metal construction of the door and jamb. Sometimes it is faster to poke a hole through the wall and reach inside to open the door.

As a rule, stair shafts in high-rise buildings are not designed to handle the total occupant load of the building simultaneously. This design is further complicated by the fact that the number of usable stair shafts may be reduced by heat, smoke, or fire department operations. This lack of usability is one of the main reasons why total evacuation of building occupants during a high-rise fire is often impractical.

During early fire stages, when there is a rapid spread of fire products to floors above the fire area, it may be best to relocate occupants from upper floors to safe refuge areas below the fire rather than attempting to evacuate them from the building. When relocating occupants, it is critical that occupants be placed in areas that will not be subject to smoke or heat. An escape route from the area must be maintained, and responsible personnel (such as police, floor wardens, or building security) must remain along the route to prevent panic.

Stair shafts in high-rise buildings should be marked at each landing with signs that provide specific information about the shaft. Signs should identify the stair shaft by name or number (for example: Stair #1 and Stair #2). They should specify what floor you are currently on and list the lower and upper terminal points of that stair shaft (for example: B-3 to 18). Signs should also indicate whether the stair shaft provides access to the roof of the building. This seemingly simple information can be critical to occupants who are using the stair shaft under fire conditions, and it can provide critical information for firefighters.

### Floors

Floors in high-rise buildings are also required to have a minimum of a 2-hour fire-resistive rating. Floors are normally concrete poured over a metal deck that remains in place after the concrete has set. After the concrete has set, holes are bored in the concrete to allow for passage of various utility lines or equipment between floors. This procedure is called poke-through construction. Poke-throughs, if not properly sealed around the bored holes, can seriously diminish the floor’s 2-hour fire resistivity.

While most recent codes require that poke-through openings be sealed with a material that re-establishes 2-hour fire resistivity, in many cases, it is not done properly or is completely overlooked. In older buildings, poke-throughs may not be sealed due to a lack of code requirements when the building was built. Not sealing these poke-through spaces can allow fire and smoke to travel to upper floors and provide a path for water to travel to floors below. During a fire, a check must be made of the floors above and below the fire floor to ensure that poke-throughs are not causing a problem.

### Interior Partition Construction and Compartmentation

In old-style construction, interior walls are usually poured concrete or concrete block. There is generally a high level of compartmentation. Walls usually run from floor to floor and provide a high degree of horizontal fire-spread protection.

In new-style construction, the interior partitions and walls are usually constructed of drywall on a wood or metal stud. There may or may not be a high level of compartmentation. Office-occupancy buildings may have large, open areas containing new modular furniture and cubicles. This design may be combined with certain portions of the floors also having conventional offices. Another configuration is to have the entire floor consist of full-size offices.

Residential-occupancy buildings usually have floor-to-floor drywall on wood or metal studs enclosing each living unit or apartment. There may only be floor-to-ceiling partitions on the interior of the living unit or apartment. This design allows a common area above the entire living unit.

### Floor Configuration

There are two general design concepts for horizontal floor separations in high-rise buildings. They are referred to as compartmentation and open space. Compartmentation in high-rise buildings is based on the concept that small, protected areas that are separated from others will allow the fuel within them to burn out. These protected areas keep the fire from spreading beyond the area. An example of compartmentation is a typical high-rise apartment building.

Compartmentation can be an essential design consideration in limiting the size of a high-rise fire. Compartment separations must offer adequate fire resistivity and must divide plenum areas above dropped ceilings. They must also prevent vertical fire travel through protective construction features around vertical shafts and above windows. Proper compartmentation also requires all poke-through openings between floors to be properly fire stopped.

Examples of the open-space concept are high-rise office buildings where floors are virtually wide open. This openness is designed to allow unrestricted movement of employees throughout the floor. In a fire, however, the lack of physical barriers will allow the fire to spread quickly throughout the floor. High-rise floors often are divided by partitions that extend from the floor to the dropped ceiling. These conditions do not represent true compartmentation. Should the fire reach the open plenum area above the dropped ceiling, it will move unrestricted through the plenum and extend into other areas of the floor.

### Ceiling Assemblies

Ceiling assemblies in high-rise buildings are usually suspended from the floor assembly by steel wires attached to a grid of metal channels. These channels hold acoustical tile (or other ceiling material), and in most cases, they also hold lighting fixtures. The open space between the suspended ceiling and the floor above is normally used for horizontal distribution of utility services (for example, air conditioning ducts, electrical conduits, and plumbing lines). It often serves as a common exhaust plenum for the Heating, Ventilation, and Air Conditioning system (HVAC).

WARNING: Under prolonged exposure to sufficient heat, suspension wires will weaken, often causing ceiling assemblies to fall. When ceiling assemblies fall, they can greatly impede firefighter progress or cause firefighters to become entangled in the wire or trapped.

Building Systems

Electrical Systems

Electrical systems in high-rise buildings can be extremely complex and very hazardous under fire conditions. The amount of electrical power required for building operation and the complex equipment used to distribute it must be considered when fires occur. Electrical chases are one cause of vertical fire spread.

Much of the electrical equipment is likely to be located in the basement of the building. This location makes it susceptible to flooding due to broken pipes or water used to control the fire. The danger of working near electrical equipment when water is present is well known and must be remembered. Sending fire personnel into electrical vaults to terminate building power is usually not recommended for the following reasons:

- The shutdown procedure is usually complicated and requires specific knowledge to perform it safely.

- Randomly throwing switches in these situations can be extremely dangerous. (You could terminate power to equipment that should continue to operate.)

- If power must be terminated on the floor or floors involved in the fire, it can usually be done through subpanels that control the electrical supply to specific floors.

- Because of high voltage and power present in electrical vaults, a sudden shutdown by inexperienced personnel can cause dangerous surges that can harm personnel. A utility company or the building engineer should perform any shutdown in electrical vaults.

An emergency power supply, usually provided by an engine-driven generator, may be found in many high-rise buildings. The building systems that receive power from the emergency system will vary and are usually dependent on code requirements in effect when the building was constructed. In older buildings, the emergency power may supply only exit lighting in the stair shafts. In newer buildings, it may serve a large number of fire protection or life safety features such as fire pumps, elevators, and smoke removal systems. Emergency power activation may be automatic when normal power is interrupted, or it may require manual activation (by throwing switches). During pre-incident planning inspections, be sure to determine if the building has emergency power, what it supplies, and how it is activated.

Elevators

Under normal conditions, elevators are the only practical method of moving between floors in a high-rise building. Under fire conditions, elevator operation can become erratic and extremely dangerous. Many elevator system control components can be affected by smoke, moisture, and heat—all of which are present during a fire. Control components that can be affected include floor call-buttons, electrical contacts located in shafts, and electrical elements located at the bottom of the hoistway.

Safe use of elevators under fire conditions requires:

- Knowledge of how elevators work;

- An understanding of what malfunctions may occur (for example, erratic movement, traveling to unselected floors, traveling to the fire floor, ceasing to operate, and doors opening without use of open-door button); and

- Familiarity with Standard Operating Guidelines (SOGs) and their use under emergency conditions. A department-wide policy regarding the use of elevators during fire conditions should be developed and adhered to by all department personnel.

Hoistways are the vertical shafts in which elevator cars travel. In buildings with multiple elevators, all elevator cars in a bank are usually in a common hoistway. Some high-rise buildings are equipped with low, medium, and high-rise bank elevators—this design is known as split bank. These elevators are configured so that some elevators serve only lower floors of the building while others serve the upper floors. It is important to know whether the building has split-bank elevators and, if so, which floors the different banks serve. This information can be critical to deciding whether it is safe to use the elevator system.

The hoistway is separated from each floor by a hoistway door. This door is opened by movement of the elevator car door (once the car is level with the floor landing). Smoke and heat under pressure at the fire floor can enter the hoistway (even though the hoistway doors are closed) and travel up or down the shaft. If a large volume of fire enters the hoistway shaft, the shaft acts like a chimney and draws the fire upward where the heat may be sufficient to ignite materials on upper floors next to the hoistway. As heat and smoke rise within the hoistway, pressure will force it out the hoistway doors onto the upper floors.

Elevator cars will burn to the point where hoisting cables can fail and cause the car to fall down the shaft. If fire has penetrated an elevator car or the hoistway, it is important that personnel be assigned to floors above and below the fire floor (including the floor where the shaft terminates) to check for spread of fire or smoke.

Almost every high-rise building is required to be equipped with elevator emergency service features that automatically move the elevator cars to specific locations under fire conditions. The feature also allows firefighting personnel to place the elevator cars in a firefighter service mode that provides specific safety features. Automatic recall may be initiated whenever an alarm device is activated. Manual recall can be done through recall switches located in a lobby control panel or in a fire control room. Automatic or manual recall of elevators (available through firefighter service mode) is important for these reasons:

- It reduces the possibility of occupants being trapped in an elevator car.

- It provides fire department access to elevator cars (if the decision is made to use them).

### Firefighter Service During Fire Operations

Whenever possible, elevators equipped with a firefighter service mode should be used for fire operations. The following guidelines should be followed when using these elevators:

- Assure that the elevators have been placed in the firefighter service mode.

- First arriving units should (if possible) initially avoid a firefighter service equipped elevator that is capable of stopping at all floors.

- Many of the converted service or freight elevators are so arranged and are therefore capable of being affected by fire on any floor.

- Only after the Incident Commander (IC) has determined that the fire is not adjacent to the shaft should these elevators be utilized. Experience indicates that many fires in high-rise office buildings have been found in the service elevator lobby (in piles of collected rubbish). Heat and flames have affected the doors and control wiring of nearby service elevators.

- Personnel should never take an elevator that services all floors in order to travel to a floor above the fire.

- When assigned to go above the fire by elevator, choose an elevator that has a blind shaft on the fire floor.

- Remember, an elevator equipped with firefighter service is not necessarily a safe elevator. It can still be affected by heat, smoke, or water entering the shaft.

- If there is no blind shaft elevator to go above the fire, use stair shafts. Note: When available, use a fire tower to ascend.

The decision to use elevators during a fire in a high-rise building is one that must be tempered with good judgment. While it is true that using elevators will speed up initial investigation and fire control efforts, malfunction that causes response to a nonselected floor can result in loss of firefighter lives. Therefore, using stair shafts is the safest method of ascending to the fire floor.

The decision to use elevators should be based on assurances that the elevator lobby on any involved floor is safe and that the cars that are used are not physically capable of reaching the fire floor (they belong to a split bank). Fire personnel already on fire floors can confirm that the elevator lobbies on those floors are tenable.

Even when assurances are in place that elevators can be safely used, any additional safety features or procedures should be employed. These include the use of split-bank elevators that terminate at least five floors below the lowest reported fire floor. Only use cars that allow firefighter service. In addition, all personnel riding in elevator cars should wear full protective equipment and have: forcible entry tools, a means of communication, an extinguisher, and a knowledgeable firefighter assigned to operate the car.

All firefighting personnel should be well trained in the operation of firefighter service controls on elevator cars. The time to conduct this training or to develop department policies regarding emergency use of elevators is not on the day of the fire.

Do not use an elevator in a bank that services the fire floor unless it is determined to be safe using local SOGs. The one exception to this rule is that early consideration of elevator use is acceptable when split-bank elevators exist in which the top of the shaft and machinery room is a minimum of five floors below the reported fire floor. In this case, take the low- or medium-bank elevator to the highest floor, and then take the most desirable access stair shaft to the fire floor. Progress up the stair shaft and check the floor two floors below the reported fire floor for use by staging.

Transmit to the IC the information on the staging floor and stair shaft number used by fire attack personnel. A good rule of thumb is to give the IC an update every two to three floors during the ascent. On arrival at the reported fire floor, update the IC on the conditions on the floor as well as for the floor above. In addition, fire attack personnel should give a periodic update on conditions and fire location to the IC.

The first thing that should be done when assessing the safety of elevators that service the fire floor is to account for all cars serving the floor and check them for victims. Upon verification by fire department personnel that the elevators are safe to use during emergency operations, a fire department member should be designated to control the operation of each elevator car. The operator, in addition to required safety equipment, must have a portable radio to maintain communication with the systems unit.

Even though the elevator may be capable of traveling directly to the fire floor, it is always recommended that all elevator travel cease five floors below the lowest fire floor. Initial fire attack team personnel traveling in elevator cars, even though the maximum travel is five floors below the reported fire floor, must be equipped with donned breathing apparatus and face piece, portable radio, dry chemical extinguisher, forcible entry tools, and portable spotlights. Breathing apparatus face pieces should be connected to regulators and ready for immediate use. The firefighter must be assigned to keep the dry chemical extinguisher in readiness with the lock pin removed and the nozzle pointed at the elevator door. The portable spotlight is to be used in case of lighting-power failure. It can also be used to check the hoistway for smoke before use. If even the slightest amount of smoke is in the hoistway, the elevator should not be used.

At all times that the elevator is in motion, firefighters should be prepared to take immediate action that will cause the doors to close if the car comes to a floor where smoke or fire conditions are present. The action will be dictated by the elevator control equipment and the current operating mode. Precautionary stops should be made to confirm elevator operation and to check for smoke in the hoistway.

If sound powered phone jacks are available in the elevator car, they should be placed in service with a sound powered phone/headset. In addition, consider the weight capacity for the elevator car to prevent overloading with personnel and equipment.

Departments that allow personnel to use elevators at emergency incidents must consider the following items as a minimum when developing SOGs:

- Only use an elevator car with the firefighter service feature that allows for emergency control of the elevator car. (Note: In older high-rise buildings, firefighter service is sometimes identified as firemen service.)

- Consult with the company that installed the elevator. Ask about the machine’s features and use.

- Follow all previous guidelines for split-bank elevators.

All personnel who respond to high-rise fires in your community must be trained on elevators and procedures. SOGs are not enough!

### Procedures: Firefighters Trapped in Stalled Elevator Cars During Fire Operations

If the elevator car door opens on the fire floor and exposes the firefighter to severe heat and/or smoke, discharge the dry chemical extinguisher to knock down a flame front and attempt to close the door immediately. Close the door by either pushing the DOOR CLOSE button or forcing the doors closed. Push the button for a lower floor and exit the elevator when it reaches that floor.

If the car fails to move, check the emergency stop button. It may have been accidentally activated. Deactivate it by pulling it out, or if it is a switch type, move the switch to the OFF position. If necessary, open the top emergency exit (if available) to relieve smoke in the car. Try to keep low in the car, and don your SCBA face piece if necessary. Remember, it is important to conserve air. Communicate the situation to your supervisor.

Another option is to open the elevator roof access and take control of the car by using controls located on the car roof. When two or more elevator cars exist in the same shaft, it may also be possible to gain access to the adjoining car. (Access may be gained through the roof or side panels.)

If necessary and available, use the side emergency exit for a rope slide to get to the safety of a lower floor. If this is to be attempted, have power removed to the adjacent car. In an extreme emergency, a fire department hose can be used to slide down to the floor below. If more than one length of hose is used, first tie the lengths together; then couple them. Personnel can be lowered to the hoistway door interlock and exit at the floor landing below.

Hand lines on the floor below can be used to spray a fog stream between the car and the hoistway door. A 30-degree fog pattern should be used to cool and protect trapped people during the rescue operation.

### Pertinent General Information

- Take time to become familiar with specific elevators before leaving the lobby. Early staffing by one or more personnel who have become familiar with the elevators is important.

- Use stairs whenever possible, and limit elevator use to those in banks that cannot be affected by the fire.

- Consider calling in an elevator repair/service company that provides personnel on emergency duty. Many high-rise buildings have these personnel on 24-hour call. The telephone numbers must be posted in the elevator machinery room and are often posted near the elevator lobby.

- Conduct fire department operations using elevators that have been placed in manual mode if firefighter service equipped elevators have not been installed in the building.

- Use all applicable sections of this procedures guide when using elevators in the manual mode.

### Hoistway Doors—Operational Considerations

For security reasons, some occupants lock hoistway doors on their floors when the building is closed. If your elevator arrives at the selected floor, but the car door does not open, do not attempt to force it. In this instance, the locked hoistway door (attached through the vane to the elevator car door) is keeping both doors closed. Any attempt at forcing them open may damage the interlock, putting the car out of service. The following procedures may be used to deal with a locked hoistway door.

- If the hoistway door security lock can be removed or opened with no damage to the door, do so.

- If removal of the locking device threatens to bend or warp the door or cause the door to buck, make no attempt at removal. Drop down to a floor where exit is possible. Find the stair shaft and walk up to the original floor.

Caution:  Warping or springing of the door assembly may interfere with the car’s electrical circuits and put the car out of service.

Ventilation

Ventilation complexities usually depend on the type of windows installed in the building. Inoperable windows complicate ventilation procedures. Operable windows, used in conjunction with normal smoke removal equipment, simplifies ventilation.

High-rise residential buildings normally have operable windows made from regular plate glass. In addition, many high-rise apartment buildings have large sliding glass doors that open onto balcony areas. Windows in high-rise office buildings are often inoperable and are typically made of plate glass. When broken, plate glass often produces large shards that can cause serious injury to those below. To reduce this risk, special tempered glass windows may be required at certain locations. When a tempered glass window is broken, it will shatter into very small pieces, providing a degree of safety not offered by plate glass under the same circumstances. Depending on applicable building codes, tempered glass or operable windows on every floor may be required in sealed buildings. Usually, they are located in each corner of the exterior wall and at specific horizontal intervals. These special windows are normally required to be aligned vertically throughout the building. Tempered glass windows are normally marked as such in one of the lower corners. A decal may be affixed in a visible place near the window.

Instead of tempered glass for emergency ventilation, some buildings may be equipped with special operable window panels that are secured from the inside by a tool-operated locking device. This tool is required to be kept on the premises.

It is important to note that removing window glass during a fire situation, whether caused by the fire or done intentionally for ventilation purposes, can create a situation where fire can extend up the building exterior to the floors above. Anytime glass is removed or fails, consideration must be given to the possibility of exterior lapping.

Smoke Control Systems

Smoke and its toxic products account for more than 80 percent of fire deaths in the United States. Plastics greatly increase the volume and toxicity of smoke. For example, polyvinyl chloride (commonly known as PVC) produces 500 times as much smoke as red oak. High-rise buildings have contents that, like most occupancies, are petrochemical products that produce large amounts of smoke and toxic gas.

The forces that affect smoke movement in a high-rise building include stack effect, expansion, wind, and HVAC systems. Smoke control can be either passive or active in nature. Passive smoke control measures have been in use for many years. They consist of:

- Barriers,

- Curtains,

- Gravity venting,

- Smoke proof towers, and

- Smoke removal shafts.

Active smoke control systems are relatively new and normally automatic. In addition to methods of passive smoke control that might be in use, active smoke control employs mechanical assistance to route smoke in a planned manner. Active smoke control systems can be used to control the movement in many different ways.

A small high-rise may have a single HVAC system that controls the atmosphere on all floors. This simple system may have a single control to exhaust the entire building. This control is referred to as the building smoke control system.

Zoned HVAC systems exist in more complex high-rise buildings. It may be possible to control single floors or an entire zone through these types of systems. (See the HVAC section of this manual for a discussion of zoned systems.) HVAC zones could cover:

- A given number of floors;

- A stairway pressurization system (often accomplished by having fans in all or certain stair shafts);

- A corridor smoke control system;

- An elevator shaft control system; and

- An atrium smoke control system.

It is important to have this information on the prefire plan and to work together with the building engineer to control operation of various HVAC zones.

These smoke control systems are prone to fail under fire conditions. The areas that they are designed to clear smoke from should be closely monitored. In the event of adverse effects, the system should be immediately shut down.

HVAC Systems

HVAC systems are designed to provide conditioned air throughout the structure by means of a ductwork system. For reasons of economy and efficiency, these systems operate on the concept of recirculating most of the air within the building. Under fire conditions, smoke or heat can enter the system at fire floors and quickly fill other parts of the building with contaminated air. When smoke and heat are pumped through the building in this manner, many occupants can be exposed to highly toxic gases that place them in serious jeopardy (even though they may be on floors remote from the fire). In a high-rise building with a recirculating air handling system, the fire may be small, but smoke spread can be a major problem.

The air handling system in most new high-rise buildings is required by code to have dampers in the system. These dampers are smoke-activated and control the spread of fire products from the area of origin to other parts of the building. Don’t rely totally on these dampers. In many cases, however, the building may not have dampers installed or these dampers may not function properly.

Since the spread of fire products throughout the building is critical in life safety terms, the best approach is to shut down the system when there is any doubt or concern that it may be contributing to the spread. Once the fire is controlled and the safety of building occupants has been ensured, the system can be reactivated (if it has the capability of exhausting smoke from the building).

Methods of shutting down the HVAC system vary, depending on the particular building. In some buildings (especially older ones), it may be necessary to close switches that control system air-intake fans. Many times these switches are located in machinery rooms on upper floors of the building, or they may be located on the roof of the building. In many newer buildings, and in some older buildings that have been modified, the air handling system will shut down automatically under fire conditions. In some cases, they will provide exhaust capability on the fire floor and pressurization of the floors above and below the fire floor.

Information about the air handling system in a high-rise building should be a critical part of prefire planning inspections. Planning should include how the system operates under fire conditions and where and how the system can be deactivated if necessary. When the building engineer is available, it is imperative to use his or her assistance with the HVAC system.

Water Supply

A variety of different water supply systems can be found in high-rise buildings. They may include:

1 ½ -inch wet standpipe systems,

2 ½ -inch dry standpipe systems,

2 ½ -inch wet standpipe systems, and

### Sprinkler Systems

Note: The sizes 1 ½ -inch and 2 ½ -inch refer to the diameter of the fire hose discharge connections on the standpipe system. Piping within the standpipe system may be considerably larger than this.

The importance of the water supply systems built into high-rise buildings demands that we have knowledge of how these systems work and what problems may be expected in emergencies. The specific types of water supply systems found in high-rise buildings will vary with the age of the particular building and code requirements that were in effect at the time it was constructed. Pre-incident planning information should include specifics on the water supply system, its capacity, and functional components.

### 1 ½ -Inch Wet Standpipe Systems

For many years, 1½ -inch wet standpipe systems have been used in high-rise buildings. These systems are often supplied by the domestic water system and are intended as a first aid device for building occupants. They have limited water volume and pressure, and their hose lines and nozzles are inferior and nozzles. The 1½ -inch wet standpipe system should not be considered as adequate for primary fire department attack. If the 1½ -inch system is used for initial attack, any attached hose or nozzles should be replaced with standard fire department equipment.

### 2½ -Inch Dry Standpipe Systems

The 2½ -inch dry standpipe systems are used in many older high-rise buildings and, in some cases, in newer buildings that do not exceed specific heights. These systems are relatively simple in design compared with wet standpipe systems, but they have some important differences that must be considered.

Since they do not have a constant water supply, it is important that they be charged by an engine company who hooks to the fire department connection as quickly as possible. This will give firefighters an available water supply for fire attack. Fire department connections on 2 ½ -inch dry standpipe systems typically serve only one standpipe riser. It is critical that the riser being supplied is the same one that is used for fire attack lines.

### 2½ -Inch Wet Standpipe Systems

The 2½ -inch wet standpipe systems are required by code in all new high-rise buildings over certain heights. These systems provide a constant supply of water under pressure adequate to produce effective hose streams on each floor of the building. The primary water supply source for these systems may be the domestic supply that can be supplemented by an auxiliary supply (kept in a holding tank in the building). The 2½ -inch wet standpipe systems differ in design. They may serve both 1½ -inch and 2½ -inch outlets as well as the sprinkler system, if the building is so equipped.

The necessary pressure and flow for a 2½ -inch wet standpipe system is usually provided by one or more fire pumps that serve as the primary supply. Fire pumps for high-rise buildings are usually multistage centrifugal pumps. They may be powered electrically or with diesel engines. These pumps are designed to produce the required flow at a pressure that is sufficient for working streams at the highest point in the building. If an emergency or backup pump is required by code, there will be a backup system that activates automatically should power to the electric pump(s) fail. Backup pumps are usually diesel driven. In many older high-rise buildings, the water flow capacity in gallons per minute (liters per minute) is inadequate for the fire potential within the building.

Note: It is important to know what outlet pressure your system produces in order to determine the type of nozzle to use (smooth bore or fog). Typical outlet pressure found is approximately 65 pounds per square inch (psi). This pressure would require a smooth bore tip. Fog nozzles that require 100 psi at the nozzle will produce ineffective streams.

Since wet standpipe systems must contain sufficient pressure to produce effective hose streams at the top most floor of the building, the pressure within the standpipes at lower floors must be reduced. This is normally accomplished by pressure-reducing devices installed at each outlet. These valves are preset to provide the proper outlet pressure for that location. Pressure Reducing Valves (PRVs) have the advantage of being able to supply multiple hose lines (within reason) while maintaining the proper pressure and flow rate. These valves control the pressure but can automatically adjust to varying flows depending on the size of the hose and nozzle (or the number of hose lines). Pressure-reducing valves on each floor should be checked for proper operating pressure and flow before the floor is occupied.

In place of a valve, there may be orifice plates in the outlet valve barrel. Orifice plates are stainless steel or brass washers with calibrated holes and are designed to handle one line. These holes control the outlet pressure by restricting the flow from the outlet. The plates are often tack welded into the standpipe valve outlet barrel. The outlet pressure from these devices is not reduced until water is flowing.

Pressure-restricting devices are yet another method of providing proper pressure to standpipe hose lines. They reduce outlet pressure in much the same manner as orifice plates. The pressure-restricting device allows the valve to be opened only a predetermined distance. Firefighters who remove orifice plates or alter the setting of pressure-restricting devices need to be aware that the outlet will then deliver increased pressure from the system.

Two drawbacks to the orifice plate and other pressure-restricting devices are that they:

- Have no effect on static pressure and

- Do not allow for multiple hose lines (because of the limited flow that comes through the orifice opening).

If orifice plates are removed to provide for multiple hose lines from an outlet, the pressure to the lines must be controlled at the standpipe valve, and care must be taken when opening or closing nozzles.

Fire Loading

The quantity of fuel that is available to a fire on any given floor directly affects the firefighters’ ability to gain fire control. Where fuel is limited (such as on a vacant floor), it may be possible to mount a greater effort to keep the fire from getting by that floor.

Heat Buildup

Fire in high-rise buildings generates large quantities of heat. Unfortunately for firefighters, this heat cannot be easily dissipated from the building. Usually, there is no means to effectively ventilate the building. This high heat also takes its toll on the firefighter. The higher the heat, the more one perspires. Perspiration dehydrates the firefighter and removes energy. Rehydration at staging is critical.

Sprinkler Systems

Sprinkler systems in high-rise buildings are now required by code in virtually every area of the country. However, many older high-rise buildings (maybe some in your jurisdiction) do not contain sprinkler systems. There is no doubt that sprinkler systems provide the added degree of life safety for newer buildings that is sadly lacking in older high-rise buildings that do not have sprinkling systems. In some cases, retroactive legislation, enacted as the result of tragic high-rise fires, has mandated that older high-rise buildings be fully equipped with sprinklers. However, these cases are the exception rather than the rule. Pre-incident planning inspections should take particular note of sprinkler systems when present, what areas they serve, and how they can be supplemented.

SOGs require initial response units to supplement any built-in water supply system in a high-rise building during a fire. To do this effectively, firefighting personnel must be acquainted with the building, the water supply system, and the location of fire department water supply inlets.

A fire department must have SOGs for connecting to and supplying the high-rise sprinkler system. Officers and pump operators must understand the pressure and flow required to be supplied from the engine(s) supplying the sprinkler system. Current national standards for supplying sprinkler systems (such as NFPA 13E) should be referenced.

Communications

When discussing problems that occur at emergency incidents, communication always seems to be at the top of the list. High-rise fires are no exception. Communications problems can be magnified and their results much more severe than those seen at ground level incidents. In any type of an emergency, good communications are vital to effective operations—maybe even more so at a high-rise emergency.

It is a known fact that portable fire department communications equipment can be ineffective or even completely unusable in a high-rise. There are locations inside high-rise buildings where it is virtually impossible to transmit or receive messages using portable radios. In some cases, satisfactory communications will cease with the movement of the radio location by only a few feet.

There is a definite correlation between portable radio effectiveness and the frequencies on which they operate. As a rule, radio frequencies in the VHF band are very ineffective. Those in the UHF band are fairly effective in most situations. Those in the 800-megahertz band produce the most consistent results although they are not perfect. It is important to note that any frequency (in any building) may have inherent transmission/reception problems. Evaluate the system during preplanning to avoid future trouble.

Many new high-rise buildings (and a number of older ones that have been retrofitted) have built-in emergency communications systems. These hardwired systems have jacks at specific locations on every floor (and in some cases even in elevator cars) that allow fire personnel at different building locations to communicate. Using the system requires plugging into it with a handset or headset. A number of handsets are normally kept on site. A built-in emergency communications system can be used as a primary communications channel if portable equipment is not functioning properly. It can also be used as a secondary channel to avoid overloading fire department frequencies.

Built-in emergency communications systems are not the same in every high-rise building. Effective use of these systems requires pre-incident planning by fire department personnel on how the particular system works and how it would be used during an emergency.

Detection And Alarm Systems

There are three basic types of detection and alarm systems:  1) smoke/heat detectors, 2) enunciator panels, and 3) manual fire alarm stations.

Smoke/heat detectors in a high-rise building may or may not be connected to an enunciator panel. It is possible for smoke detectors be incorporated into the HVAC system and be located in building air shafts. These detectors may then activate fire dampers within the HVAC system. Effective management of a high-rise fire through preplanning (obtaining knowledge of these types of building characteristics) is necessary for effective decision-making.

It is important to know of the location of any enunciator panel(s) in the building. They may be located on a wall at a specific location or they may be part of the fire control room or station. A full understanding of how to interpret the information given on the panel is critical to effective response.

Manual fire alarm boxes may be located on each floor of a high-rise. Several boxes may be present on each floor. These boxes may be local alarms for the floor, connected to an enunciator panel, or connected to the fire control room/station.

Fire Control Rooms/Stations

Most current codes require that newly constructed high-rise buildings contain a fire control room or station within the building. At a minimum, the room should provide:

- Specific information on alarms that have been activated and

- The status of fire protection systems within the building.

The information available at this location can be extremely useful for determining the exact location of a fire and the status of fire protection systems that may have activated. These rooms or stations frequently have communications systems that allow the transmission of emergency alarms or instructions to building occupants and firefighters.

While a great deal of information is available from a fire control room or station, there are several reasons why it may not be the best place to locate the IC. If the room is on a basement level, then radio communication will probably be difficult. Positioning the IC at the fire control room may also remove that person from face-to-face contact with other officers. In all cases, fire department personnel should be sent to monitor the information available at the fire control room or station and relay it to the IC. This relay can often be established by commercial telephone from the fire control room or station to the fire department dispatch office.

As with other systems installed in high-rise buildings, fire control rooms or stations are not all the same. Monitoring the information that is displayed in these locations or accessing the various systems that they contain requires prior knowledge that can only be gained through pre-incident planning.

Life Safety

Large numbers of people can be exposed to potential danger during a high-rise fire. This requires that immediate attention be given to the issue of life safety. The following life safety issues must be taken into consideration by fire personnel when responding to a high-rise fire.

- Life safety can be enhanced by timely control of the HVAC system.

- Failure to control smoke movement within the building can put many lives at great risk.

- Evacuation takes time—Command staff must anticipate the time needed and assign sufficient personnel to perform the task.

- Occupant behavior during a high-rise fire is largely unpredictable.

- If occupants are going to be evacuated from the building, it is critical that they use stair shafts that are not contaminated with smoke and heat.

Fire Behavior and Spread

There are certain fire behavior and fire spread characteristics that all command officers and operating forces must consider at high-rise fires. These include:

- Stack effect,

- Negative stack effect,

- Vertical extension,

- Core-construction effect,

- Fire loading, and

- Heat buildup.

Stack Effect

Normally, we perceive smoke as being heated and, therefore, lighter than the air surrounding it. Thus, when we cut a hole in the roof of a small structure that is on fire, the hot smoke and gases easily exit the structure. However, this is not so simple when we are dealing with a fire in a high-rise building. Vertical shafts in a tall building tend to act as a chimney or smokestack by channeling heat, smoke, and other products of combustion upward because of convection. As this process occurs, a stratification process also takes place in which hotter smoke moves toward the roof and the cooler smoke stays lower. As long as the air inside the building is hotter than the atmospheric air outside the building, ventilation will occur by having fresh air drawn in through lower building openings and discharged through the top. This effect is considered the “normal” stack effect.

Negative Stack Effect

When the outside air temperature is higher than the inside air temperature (at the building’s upper levels) a negative stack effect may take place. Such a condition is more likely to occur in warm climates. As the smoke leaves the fire area (usually by way of the stair shafts and other vertical openings) it cools. This effect pushes the smoke down the vertical shafts, or it settles to floors below the fire. This situation may cause staging to be relocated farther from the fire, or cause firefighters trying to reach the fire floor to use SCBA earlier than desired.

Vertical Extension

Typical construction methods for high-rise buildings provide common avenues through which fire may vertically extend. The three common methods of fire extension in high-rise buildings include:

- Auto extension,

- Curtain-wall extension, and

- Vertical shaft extension.

Auto extension occurs when the fire generates enough heat to break out windows, after which the fire “rolls out” of the fire floor and up the outside of the building. Heat is transmitted to the floor above causing the window glass to break and combustibles on the floor to ignite.

Curtain-wall extension—As previously discussed, most modern high-rise buildings are constructed of structural steel. Exterior walls (curtain walls) are attached to the structure. A space is created between the floor assemblies and the curtain wall. These spaces are supposed to be sealed during construction. Should there be faulty installation or heavy fire conditions, there may be vertical spread of the fire through this space. This is called curtain-wall extension.

Vertical shaft extension—A number of features incorporated into high-rise design and construction contribute to vertical shaft extension. They include:

- Stair shafts,

- Elevator shafts,

- Electrical chase ways,

- Plumbing/electrical/data cable poke-throughs (holes created through floors for cable or piping distribution),

- Air conditioning supply/return shafts,

- Mail chutes,

- Trash chutes, and

- Access stairs (open, private stairways constructed for tenants who occupy more than one floor of a high-rise).

Core-Construction Effect

A fire that reaches the plenum area around the center core of a high-rise can spread into that plenum area. Firefighters entering the fire floor and advancing on the fire may inadvertently push the fire around the center core. This may cause the fire to circle behind the firefighters cutting off their escape route.

When fire is predicted to be in the plenum area, firefighters entering the corridor from a stair shaft should remove the ceiling tiles in both directions before advancing. This may allow the firefighters to see if fire is in the plenum area. A backup houseline should be in place and operating in the opposite direction to protect the advancing crew(s).

Incident Command System (ICS) for High-Rise Incidents

This section relates the basic organizational approach taken toward high-rise firefighting using the framework of the Incident Command System (ICS). The information in this section provides the firefighter with a clearer understanding of how to specifically organize the command system for a high-rise incident.

High-Rise ICS Support Functions

ICS is used to manage the resources at a high-rise incident. While a high-rise incident may seem to pose the same problems as those in buildings of one or two stories, certain aspects of the high-rise configuration affect ICS.

The location of staging, as well as the need for some special functions within the ICS, is unique to high-rise structures.

Time and Distance Factors

Time factors play an important role in control operations at high-rise fires. At any fire, it takes time to transform orders into actions. At a high-rise fire, time becomes a much more critical element. Being reactive and waiting for things to happen before requesting additional resources, or before moving on-scene resources close to the fire area, can be disastrous for command officers.

The impact of time factors can be greatly reduced by placing personnel and equipment in the staging area (normally two floors below the fire) quickly. Being proactive is the key. Anticipate what may happen, and move resources before the need is actually there. In addition, it is important to know that it takes much longer to perform critical tactical operations during a high-rise fire. The heat of a high-rise fire is physically draining. You should know that rehabilitation with rehydration is critical at a high-rise fire.

Staging reports to the IC in the initial phases of an incident. If, and when, operations is staffed, staging reports directly to the operations section chief.

The purpose of staging is twofold. It is a designated area to pool and quickly deploy personnel and equipment in proximity to the incident. It is also established to manage and control the flow of personnel and equipment to the upper portions of the building. Staging will also provide previously assigned companies an area for rehabilitation, equipment exchange, and first aid.

Staging should be a priority on all working high-rise incidents. It is recommended that a company from the first alarm assignment be used to set up staging. The staging area manager and personnel assigned this responsibility must ascend by a safe route. Some fire departments have pre-assigned companies to establish staging. However, company availability or staffing constraints may not allow the pre-assigned company to establish staging. When this occurs, the IC shall assure that the responsibility for staging is reassigned to another company.

Normally, staging is located two floors below the fire floor to minimize the time-distance factor. The location may be altered because of floor arrangement or incident conditions. Staging shall be the primary point for all fire department personnel who enter the fire area. Staging is also the assembly point where a reserve of personnel and equipment are maintained awaiting deployment within the building.

The staging area manager reports to the IC/operations chief and verifies the location of the staging area. The staging area manager maintains separate stockpiles of reserve and expended equipment, as well as a reserve force at a level specified by the IC. A medical treatment station shall be established in staging to provide medical treatment/rehab care for incident personnel. Resources are dispatched from staging at the direction of the IC/operations chief. Anytime reserves fall below the specified level, additional resources are requested by the staging area manager.

### Functions of Staging

As a minimum, the following will be performed when implementing staging during a high-rise incident:

- Verify location of staging with the IC/operations chief and

- Maintain a complete and accurate record of resource status for personnel accountability.

After the above minimum requirements are met, there are a number of other issues that must be considered. Staging personnel must control stairwell access, and prevent arriving companies from bypassing staging.

If the HVAC system has not been shut down or if the staging floor cannot be isolated from the system, confirm with lobby control that the HVAC system must still be shut down. If possible, open windows for ventilation of the staging floor or have blowers set up to establish a positive-pressure airflow.

Establish effective communications with the IC/operations chief to coordinate personnel deployment, and communicate with the logistics section/base to coordinate equipment movement. Ideally, a separate radio frequency should be used for communications between the IC/operations chief and logistics/base. Cellular phones or regular telephones may be used as an alternative to radios.

Plan the layout of the staging area, using signs taped to walls to identify specific areas. Consider using open storage rooms for fire department equipment. Control reserve and rehabilitation of personnel in separate areas. Maintain a separate stockpile of reserve and expended equipment. Expended equipment should be placed well apart from ready equipment, preferably at the opposite end of the established staging area. Equipment ready for use should be placed in areas closest to stairwells ascending to the fire floor.

Develop an equipment inventory, and order specific quantities from logistics/base. Record what equipment was ordered, the time it was ordered, and the time it was delivered. Equipment that is commonly stockpiled in staging includes:

- SCBA air cylinders,

- Fire hose and fittings,

- Forcible entry tools,

- Ladders,

- Resuscitators,

- Complete SCBA,

- Smoke ejectors/fans,

- Salvage equipment, and

- Medical supplies.

Additional companies directed to staging should bring priority equipment from base/lobby. Don’t come to staging empty handed!

Staging must also make arrangements to take care of the physical needs of the firefighters who will be located there. A medical treatment area should be established to handle any firefighter injuries that occur. Locate and open rest rooms for firefighter use. Secure liquids for firefighters who are in staging. If available, consider using five-gallon drinking water bottles on the staging floor.

The building lighting system should be used to illuminate staging as long as possible. Once this becomes impossible, consider the use of portable generators on the floor below staging. Extension cords may then be run to the staging floor. Plenty of flashlights with spare batteries should be available in the event all other lighting systems fail. They will probably be in high demand by firefighters operating on or above the fire floor.

The staging area manager must anticipate future needs and request appropriate resources at all times. All requests for additional personnel for staging shall be made through the IC/operations chief. It may help the staging area manager to operate from a Staging Responsibility Check-Off Sheet for smoother operations.

Note: In many jurisdictions, the term staging area, as it is applied to structure fires, is used to describe a location one or two blocks from the fire scene where apparatus and equipment are located in a state of readiness for tactical deployment. In a high-rise firefighting operation, the term staging area is applied to a floor where personnel and equipment pools are located, usually two floors below the lowest fire floor. The organizational structure is expanded to include an incident base to facilitate the parking of apparatus, consolidation of equipment, and logistical support.

Initial First Alarm Commitment

The initial-alarm resource commitment dispatched to a reported high-rise fire is critical. Recognize the potential for life loss and the need to have adequate resources on the scene quickly. Most departments increase the amount of resources dispatched to a high-rise fire compared with that sent to other types of structure incidents.

Most fire departments do not dispatch enough first-alarm resources to handle the full potential of a large-scale fire in a high-rise building. First-alarm responses are typically based on the number of resources required to handle the immediate work that has to be done in these situations. Resource needs must take into account the number of personnel needed to perform support operations (for example, base, staging, lobby control, ground support, and systems). The need to frequently relieve and/or rehab personnel assigned to tactical operations must also be considered.

The first-alarm resources should be sufficient to:

- Provide prompt investigation of the reported fire;

- Start an initial fire attack; and

- Handle any immediate support functions required to ensure the safety of building occupants.

The amount of resources dispatched on the first alarm should be standard, regardless of how the alarm is received, to ensure that a planned course of action can be followed by command officers at the incident scene.

If the first-arriving company has any indication that a working fire is in progress, an immediate request should be made for additional resources. This early call for help minimizes the lead-time needed by second-due companies to get to the scene and into operational positions. These resources should be directed to report to base. Base should be located in close proximity to the incident. The base location must be identified and communicated to dispatch. If preliminary investigation indicates that these additional resources are not needed, they can be returned to available status from the incident scene or while en route.

### Example Resource Requirements for Working High-rise Fires

The following example shows how to estimate the companies needed at a working high-rise fire. This example information is offered as a guideline only. The actual number of companies needed by a specific department will depend on company staffing and training levels. However, this example will illustrate the need for:

- Sufficient first-alarm resources on dispatch to the initial report of a high-rise fire and

- A prompt request for additional help when a working fire is encountered.

Note: This example uses four-person staffing on each unit. You should adjust the number of your companies to match approximately the number of response personnel required. It is important to remember that working high-rise incidents are resource intensive, and the number of personnel is the critical issue—not the number of apparatus.

The hypothetical fire situation used to illustrate the need for additional first-alarm resources includes the following factors:

- The fire is on the 12th floor;

- Potential exists for fire extension to the 13th floor;

- The elevators cannot be used;

- Ventilation problems exist on two floors; and

- Rescue/evacuation procedures will be needed.

Basic functional organization:

| 12th Floor Fire Attack | 3 companies |
| --- | --- |
| 13th floor | 1 company |
| Lobby Control, Base, Systems | 1 company (Divided) |
| Staging on 10th floor | 1 company |
| Total basic commitment: | 6 companies |

The fire attack and support resources total six companies. This response totals 24 personnel and a chief. It is recommended that your first-alarm response be similar in numbers of personnel.

Additional resources called on recognition of a working fire may be used as follows:

| Rescue/Evacuation | 2 companies |
| --- | --- |
| Ventilation | 1 company |
| Ground Support | 1 company |
| Basic working fire situation total resources: | 10 companies |

There are seven companies assigned to fire attack, extension control, ventilation, and rescue/evacuation. Three companies are starting to set up the support operations at base, lobby control, ground support, and staging.

Using this example for a moderate fire in a high-rise building with the potential extension problems, a minimum of 10, 4-person firefighting companies are required. Ten companies provide the minimum resource to allow an attack on the fire and initiate the needed support functions. Additional chief officers, resources permitting, should be dispatched to staff command functions. If the fire was prolonged and three companies for each working hose line were used, a commitment of 18 or more companies would be needed to provide a sustained attack on the fire and perform the needed support functions.

Based on response patterns of fire departments that have experience with high-rise fires, it is generally agreed that a minimum of 50 personnel will be required to handle a relatively small working fire in a high rise. These departments place at least this many personnel on-scene with the arrival of:

- The first-alarm assignment (24 or more personnel) and

- The first call for assistance when an indication of a fire in the building is recognized (24 or more personnel + additional chief officers).

Departments with limited resources must have working mutual or automatic aid agreements. In addition, they must train with their mutual or automatic aid companies for high-rise operations.

The Relief Cycle

The objective of the relief cycle is to maintain a constant application of water on the fire. It is based on the use of three companies for each hand line placed in service; one company operating the hand line, one company at the stair shaft landing, and one company at the staging area.

The company at the stair shaft landing moves into position on the hose line soon enough to ensure that the company being relieved has enough air remaining to exit the floor safely and return to staging.

The company being relieved returns to staging to change air cylinders and take a brief rest.

The third company in the relief cycle that has been waiting in staging moves up to the stair shaft landing. This company relieves the company on the hose line at the appropriate time.

The objective of this relief cycle is to maintain constant application of water on the fire. The coordination of the relief cycle should be the responsibility of the tactical level management unit supervisor. The staging area manager must be informed of the relief cycle and must have companies ready to make reliefs at the needed times. The IC/operations chief is accountable for deployment and tracking of all resources. The staging area manager shall maintain a complete and accurate record of resource status for personnel accountability.

### Air Cylinders

Operating in the hostile and humid environment of a high-rise fire will mean that time actually spent attacking the fire will be extremely limited. A 30-minute breathing apparatus cylinder typically is usable for only 15 minutes of work, depending on the skill and physical condition of the individual. Normally, it takes at least 2 or 3 minutes to both reach and exit the fire area. This means that 10 minutes may be the maximum time that can be spent performing the actual fire suppression activity.

The relatively short time that can be spent in actually fighting fire and the debilitating effect of an extremely hot and humid environment are reasons why large numbers of personnel must be committed to suppression activities. The number of times that firefighters can change cylinders and return to firefighting is limited. In most cases, after firefighters have used two SCBA cylinders, they should be assigned to a rehab area in staging for a brief rest before returning to any tactical activities.

First-Arriving Unit Responsibilities

There are certain tasks required of the first-arriving unit at a high-rise fire. Whether an engine company or a truck company, the following tasks must be addressed. If the first-arriving unit is an engine company, it is normal for that company to also be the first unit that ascends to the fire floor.

As a minimum, the first-company should carry out the following tasks:

- Initial size-up—rapidly evaluate the situation. There are a number of considerations that the first-arriving company officer should keep in mind when doing the initial size-up. Do not be fooled by a lack of visible fire conditions outside the building upon arrival. It is possible to have a working fire and not see anything from outside.

- Obviously, if smoke and/or fire are showing from the building, additional resources should be requested immediately. However, there are other indicators that may signal a fire condition within the building. Fire alarm system enunciator panels in the building will indicate if and which smoke alarms or water flow alarms have been tripped. Elevators that have returned to ground level because of a fire alarm activation are another clue of a problem. Lastly, information from building personnel or occupants indicating that there is fire or smoke in the building is usually a reliable method of determining that a fire exists and more resources may be required.

- Give an initial radio report of visible conditions that includes at least the following information:

- Building size;

- Occupancy;

- Obvious conditions (working fire and what levels are involved);

- Any safety concerns (for example, falling glass/debris); and

- Actions being taken.

- Assume and announce Command; for example, “Captain Engine 1 is Wilshire Command.”

Example organization:

The first-arriving company officer should assume initial incident command. This will allow the IC to assign companies and personnel consistent with SOGs. On first sign of a smoke or fire condition, the IC should request additional resources immediately. Preparations must be made to augment the water supply to any fixed fire protection systems in the building. The IC should also try to obtain keys for affected portions of the building. This will make access for firefighting crews much simpler. The IC should be prepared to maintain command until a proper transfer of command can be made to a chief officer.

- Determine the location of the fire or emergency:

- Use the building’s enunciator panel (if equipped with one);

- Gather information from occupants;

- Gather information from the building engineer or fire safety director; and

- Act on visual observations.

- Determine the status of occupants in the immediate fire area, above the fire, and below the fire.

- Control the elevators:

- Return all cars to the ground level and

- Place all cars on firefighter service mode.

There are a number of other functions that can be done if sufficient personnel are available. One helpful thing is to obtain the phone number for a lobby phone. The lobby phone can be used in the event communications are disabled or distorted. The companies operating at different levels can call the lobby phone, and relay information or make requests.

Attempts should also be made to obtain as much information about the building as possible. Referring to the prefire plan, making contact with the building engineer or fire safety director, or reading on-site building diagrams can accomplish this.

Incident Commander Duties

The officer of the first company to remain at ground level will assume command or relieve the officer of the first-ascending unit. It is essential for effective command, control, and support of operating forces that there be an IC at ground level. Some departments prefer to operate command from the building, while others prefer a position on the exterior 200 feet (60 m) from the building. The first IC to stay at ground level may be a company officer. That officer should do the following until relieved:

- Assume or transfer command;

- Obtain a briefing from person being relieved;

- Confirm and announce the transfer of command;

- Announce the location of the command post (if being changed);

- Size up and report conditions, as they are, on assumption of command;

Example organization (fire on 10th floor):

- Confirm the staging floor location;

- Establish resources on this floor (include personnel and equipment);

- Announce location;

- Determine initial strategy and tactics;

- Develop the initial incident strategy;

- Assign tactical objectives to other companies/personnel;

- Evaluate resource needs (Are there enough personnel to accomplish the tactical objectives desired?); and

- Anticipate the need for future resource needs.

Should the first ground level officer be a chief officer, along with the above, the following must be accomplished:

- Determine overall strategy and tactics;

- Develop the incident strategy;

- Assign tactical objectives to other companies/personnel;

- Obtain progress reports on a periodic basis;

- Assign personnel to the planning section as needed to maintain situation and resource status;

- Provide for the safety, accountability, and welfare of personnel. This priority is ongoing throughout the incident; and

- Expand the ICS to include the necessary functions to gain control of the incident.

Example Organization:

First-Ascending Officer Duties

It is imperative that a company ascend to the fire floor as quickly as possible. Based on local SOGs, the company performing this task may or may not be the first company to arrive on the scene.

The initial actions of the first-ascending company at a high-rise fire are extremely critical to the outcome of the incident. This company not only has the responsibility to initiate confinement and suppression efforts, it also provides valuable information that will assist the IC in the development of strategy, tactics, and organization.

The fire department must have SOGs regarding the use of elevators, stair shafts, or combinations of both when ascending to the upper floors in a high-rise building during a fire or reported fire operations. The safest method of ascending to the fire floor is to use a stair shaft that accesses the reported fire floor. However, in some situations, such as extremely tall buildings, this might not be practical. Therefore, it may be necessary to explore the use of elevators for firefighting operations. The determination to use the elevator is ultimately the responsibility of the IC. Information received from the ascending team(s) regarding the safety of elevators and actual floor conditions on the reported fire floor and preceding floors should be relayed immediately to the IC. The IC will then determine if the elevators are safe to use. The initial attack team(s) may need to use stairwells to reach reported fire floors, and make a visual assessment of actual conditions that might affect elevator use.

Note: A fire attack company that uses a stair shaft for fire attack shall pace itself while ascending, and take aloft only necessary equipment, such as SCBA, high-rise hose packs with nozzles, forcible entry tools, radios, and stair shaft keys.

### Determine Floor Configuration

The determination of the floor configuration is critical information that will assist fire attack companies as tactical objectives are assigned. It is important for the first-ascending company to determine the fire floor layout. This can be accomplished using pre-incident plans, floor surveys during ascent, or an assessment of the fire floor.

### Report During Ascent

On the way to the fire floor, the officer should check several floors below the predicted fire floor. This should give the officer an idea of the general layout of the building, specifically the layout of the fire floor. This check will allow the officer to recommend to the IC a habitable floor for staging that is two or three floors below the fire floor. It will also allow the officer to report on the condition of floors other than the fire floor(s).

### Report from the Fire Floor

On arrival at the fire floor, immediately transmit a description of conditions found, including:

- Floor number,

- Occupancy type,

- Percentage of involvement and conditions,

- Rescue problems,

- Other specific problems,

- What your crew is doing, and

- Additional crews needed and for what.

For example, “Engine 14 has reached the 20th floor. This is an office occupancy. I have about ten percent of the total floor involved with possible extension to the offices on each side. There is no obvious rescue; we will perform a primary search. Engine 14 is connecting to the standpipe and moving to attack the fire. I will need two additional engines and one truck company to my location. Companies need to be sent to the floors above.”

### Initial Fire Floor Operations

The primary objectives of the first-ascending company are to provide for rescue and locate and confine the fire. The order in which these objectives should normally be achieved is:

- Determine fire attack stairs,

- Communicate information,

- Conduct primary search, and

- Begin initial fire attack.

### Check the Floor above the Fire Floor

After giving the brief report, the officer should quickly check the floor above. This should provide information necessary for initial determination of fire spread.

Example organization:

Base, Lobby Control, Ground Support, Communications, and System

The first four functions listed above (base, lobby control, ground support, and communications) in the ICS are necessary elements that, when implemented early, allow operations in a high-rise fire situation to continue. However, these functions may be applicable in other types of structure incidents. Base for high-rise situations is simply a more limited definition than the original base used in wildland situations.

### Base

The base area of a high-rise structural incident serves as an assembly and deployment point from which large quantities of personnel and equipment are distributed. The base area serves as the primary point outside the structure to which responding resources report and from which resources receive their initial orders for entering the incident. Base works in coordination with lobby control. The base manager reports to the logistics section chief or to the IC if the logistics section has not been activated.

Logistics

Base

The IC will determine the need for a base at any high-rise incident. The IC will establish the level of resources required in base, and request those resources from the dispatching center. Once the level of resources is established, the base manager will assure that the level is maintained (replenished) until notified by the appropriate incident supervisor. The base manager must maintain communications with the resource status unit (planning section) to assure accountability of resources within the incident.

The responsibilities of the base manager may be summarized as follows:

- Verify location of base with the IC;

- Assure that the base location is a safe distance from the involved high-rise—normally 200 feet (60 m) or more from the structure;

- Determine the most effective access route to base for responding resources—advise dispatch center;

- Establish one or more safe routes to the fire building— coordinate the route(s) with lobby control;

- Maintain an accurate log of apparatus, equipment, and available personnel within base;

- Coordinate movement of equipment and resources into the fire building through lobby control;

- Establish equipment pools by priority of need according to the incident action plan—coordinate with logistics chief;

- Assure that base resources (apparatus, equipment, personnel) are requested before they are actually needed;

- Assure the security of base—use police if necessary; and

- Supply water to the base of the stairwell for use by ground support personnel.

The base manager must control resources as they arrive at base. Strict control must be maintained over the parking location and movement of personnel and equipment through base. The base manager must select a base site that is large enough for the parking and movement of a large number of responding apparatus. Typical base sites include very wide streets or large parking areas. Park apparatus at diagonal angles (\\\\\\\\) to allow easy access and egress in base. If a street is used as a base site, block the street to nonemergency vehicles. If police are not available for this function, use aerial ladder apparatus or other large fire department vehicles. Make sure the apparatus driver(s) remain with the vehicle(s) so that they may be moved when other apparatus need to pass by.

Establish safe traffic flow routes that will assure the effective movement of personnel and equipment into and out of the high rise. Pickup trucks or similar vehicles may be used to move personnel and portable equipment if necessary. Establish a priority order for deployment of personnel and equipment to the incident: spare SCBA air cylinders are always the first priority!

Assure that fire company integrity is maintained. Fire companies must stay together as cohesive units. Maintain an accurate log of fire companies—their arrival in and departure from base—by time interval.

### Lobby Control

The responsibilities for lobby control at a high-rise incident are extensive. Lobby control should be a priority like staging, and it is recommended that it be established on all working high-rise incidents from the first alarm assignment. The lobby control officer reports to the logistics section chief or the IC if the logistics position has not been established.

Logistics

Lobby Control

The lobby control officer shall report to logistics/IC the number of floors in the building (based on elevator floor indicators) and whether the elevators have been recalled. This is valuable information for the IC because of the possibility that people may be trapped in elevators.

The lobby control officer is responsible for the control of fire department personnel and civilians entering and exiting the building. It is very important to direct incoming resources to the correct stairwell when they are ascending to upper floors or staging. All personnel entering or exiting the building should be accounted for by maintaining records that include in and out times and destinations. When directing companies to upper floors, make sure that they are carrying additional equipment.

When the elevators are determined to be safe, the lobby control officer shall designate specific elevators to be used by fire personnel. Lobby control will assign a fire department elevator operator. Any car not equipped with firefighter service should be placed out of service.

The responsibilities of the lobby control officer may be summarized as follows:

- Use the building communications system to address civilian occupants;

- Pressurize the stairwells with fans when the building HVAC cannot be used; and

- Determine occupant egress to ensure a safe corridor for exiting people (consider the use of police officers to control civilians evacuated from the building). Direct personnel to move occupants a minimum of 200 feet (60 m) from the building.

### Ground Support

The ground support unit leader is responsible for the stairwell support duties when equipment cannot be moved to staging by elevators or when an additional water supply is needed. This operation can consume a large number of personnel, not only for the initial set up but also for relief personnel. The ground support unit leader reports to the logistics section chief or the IC if the logistics section has not been activated.

Logistics

Ground Support

The responsibility of ground support is the priority transportation of equipment by way of a stairwell to the staging floor. If equipment is delivered to the roof by helicopter, ground support will handle equipment movement down the stairwell to staging. If an auxiliary water supply is required by way of the stairwell, the officer in charge of ground support will coordinate and supervise this effort. In this situation, a request should be made for base to provide a water supply line to the stairwell entrance.

The following strategies will be helpful in performing ground support.

- Determine the number of personnel necessary to accomplish the task. Consider one person per two floors and one officer per four or five personnel.

- If available, provide a separate radio channel for ground support.

- Officers must remain mobile to supervise the operation. Ground support is very demanding work, and officers must ensure a smooth flow of equipment at a pace that can be sustained.

- Officers must monitor their personnel for signs of undue fatigue or distress. If it is to be an extended operation, arrange for timely relief and consider assigning two-person teams alternating with one carrying and one resting.

- Lobby control or base will deliver equipment to the stairwell entrance at ground level.

Normally, one person picks up equipment at the ground floor entrance to the stairwell and carries it to the third floor landing. That person then returns to the ground floor for another load. The person at the third floor carries the equipment to the fifth floor landing and then returns to the third floor for another load. This process continues until the equipment is delivered to the staging floor hallway. Moving equipment beyond that point is the responsibility of the staging area manager.

If the route involves unusual problems, such as long or crossover hallways or scissor stairwells, supervising officers may need to adjust assignments. Ground support personnel shall have their personal safety equipment (turnouts, helmets, breathing apparatus, and flashlights) available to them in the stairwell. In addition, officers will have their portable radios and, when available, building sound-powered phones.

### Communications Unit

The communications unit leader reports to the logistics section chief and ensures that an effective communications system is maintained between the IC and incident personnel. This includes portable radios, spare batteries, cellular phones, and the building’s sound-powered system. The communications unit leader will also coordinate communication needs with outside agencies.

Logistics

Communications Unit

### Systems Unit

The systems unit leader monitors and maintains built-in fire control, life safety, environmental control, communications, and elevator systems. This unit may operate, support, or augment the systems as required to support the Incident Action Plan (IAP).

The systems unit leader reports to the logistics section chief or the IC if the logistics section chief has not been staffed. This unit may respond to direct requests from the operations section chief.

Logistics

Systems

The unit leader must establish a close liaison with the building engineer or fire safety director, utility company representatives, and the technical specialists.

The major responsibilities of the systems unit leader are:

- Obtaining a briefing from the logistics section chief or the IC and building staff;

- Assessing current situation and requesting needed personnel and resources;

- Requesting response from and making contact with the:

- Building engineer,

- Utility company representative,

- Elevator service company representative, and

- Others as necessary.

- Anticipating the failure of critical building systems;

- Appointing personnel to monitor and operate system display/control panels (Personnel should be radio equipped.);

- Evaluating the status and operation of fire and domestic water pumps and water supply;

- Evaluating and operating, as required, the HVAC and smoke removal and stairwell protection systems;

- Evaluating, supporting, and controlling the building electrical system and emergency power plant;

- Evaluating and supporting the public address, telephone, emergency phone, and other building communications systems;

- Securing operations and demobilizing personnel as determined by the demobilization plan; and

- Maintaining a Unit Log (ICS 214).

Figure 15-1: ICS—8th Floor Fire

Activity 15.1:

High-Rise Simulation Exercise

Student Activity Worksheet

Time: 150 minutes (simulation and debrief) Purpose: To provide practice in using the ICS organization to manage a high-rise incident. Directions: - As a group, you will staff positions within the ICS of the Central City Fire Department (CCFD) for the exercise: - Incident Commander - Incident Commander Aide (not used unless nine students in a group) - Operations Chief - Operations Aide (a second Aide designated for groups with more than 10) - Planning Chief - Logistics Chief - Safety Officer - Information Officer - Liaison Officer - You will record all command decisions and actions during the exercise on the Simulation Action Chart. Entries on the ICS chart are made in response to written or visual messages or in anticipation of problems that could surface during the incident. Simulation outcomes: - Incident Commander: Team report - Operations Chief: Initial action plan and organizational chart - Operations Aide: Command worksheet, which includes objectives, strategies and tactics and an incident map - Planning Chief: Alternate plan - Logistics Chief: Resource list - Safety Officer: Safety message - Information Officer: Press release - At the conclusion of the exercise, all groups will participate in a debriefing facilitated by the instructors. Your group will display both the ICS charts and Simulation Action Charts that you developed during the exercise. Be prepared to explain your reasons for your decisions.

Incident overview: A fire has started on the 8th floor of a 22-story residential high-rise. The fire is fully involving one complete apartment and is threatening to extend into the hallway and to the apartments to either side. Incident priorities: - Life safety - Incident stabilization - Property conservation Strategies: - Rescue  Ventilation - Exposures  Salvage - Confinement - Extinguishment - Overhaul - Using the incident priorities and strategies, develop the control objectives. - Develop an incident organization. - Assign and manage resources.

High-Rise

Quick Access Fire Plan

| Building Address: Tonnalee Apartments - LL Street and 22nd Street |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Building Description: 110’ x 75’, 22-story, noncombustible, high-rise Roof Construction: Prestressed concrete T-beams Floor Construction: Poured reinforced concrete with carpet |  |  |  |  |  |  |  |  |
| Occupancy Type: Elderly High-rise |  |  |  | Initial Resources Required: 1st-alarm assignment |  |  |  |  |
| Hazards to Personnel: Electrical transformer and panel in basement |  |  |  |  |  |  |  |  |
| Location of Water Supply: Hydrant, 12-inch municipal systems, 600’ apart |  |  |  | Available Flow: 1,000 gallons per minute (gpm) per hydrant |  |  |  |  |
|  |  | Estimated Fire Flow |  |  |  |  |  |  |
|  | Level of Involvement | 25% |  | 50% | 75% |  | 100% |  |
|  | Estimated Fire Flow | 75 |  | 150 | 225 |  | 300 |  |
| *Fire in one apartment (22’ x 24’) with two exposures on fire floor and one exposure above. Rounded. |  |  |  |  |  |  |  |  |
| Fire Behavior Prediction: Rapid horizontal spread in one apartment, slow vertical spread |  |  |  |  |  |  |  |  |
| Predicted Strategies: Rescue, Exposures, Confinement, Ventilation |  |  |  |  |  |  |  |  |
| Problems Anticipated: Large numbers of elderly residents. Resource intensive operations |  |  |  |  |  |  |  |  |
| ý Standpipe: Yes—both stair shafts |  |  |  Sprinklers: No |  |  | ¨ Fire Detection: Yes |  |  |

High-Rise Floor Plan

Central City Fire and Emergency Medical Services Fire Resources Overhead Positions—one Fire Chief, one Assistant Chief, one Chief Fire Marshal, seven Fire Inspectors, one Training Captain, two Training Lts., three Deputy Chiefs (1 per shift), six Battalion Chiefs (2 Battalions x 3 shifts). Eleven engine companies—Engines 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, and 11. Each engine company is staffed with one officer and three firefighters. Four truck companies—Trucks 1, 3, 5, and 8. Each truck company is staffed with one officer and three firefighters. Squad Company 1. The squad company is staffed with one officer and two firefighters fully trained in rescue practices. Crash/Fire/Rescue Engine. Airport-staffed with one officer and two firefighters. Engine 103, Engine 104, Engine 105, and Engine 109. Reserve Engines—fully equipped. Truck 103. Reserve Truck—fully equipped. Engine 104 (1,500-gpm foam pumper). Apparatus contains one 500-gallon foam tank; 400 five-gallon cans of foam are stored at Engine 4. All members of Engine 5 and Truck 5 have been fully trained as a Hazardous Materials Response Team. Mask Service Unit 106. Engine 206 (light truck). Boat 2 - 16’ Boston Whaler. EMS Resources (third service operated by the city and stationed in the fire stations): Three EMS supervisors – (one per shift x 3 shifts) operating out of fire station 1. Five ALS ambulances – ALS 2, 4, 7, 9, and 11 Five BLS ambulances – BLS 3, 5, 6, 8, and 10

Central City Fire/EMS Response Criteria 1st-Alarm Resource Card Response time: 5 min. Engine 1 Truck 1 Battalion 1 Engine 2 Ambulance 2 MSU 6 Engine 3 Squad 1 2nd-Alarm Resource Card Response time: 10 min. Engine 4 Truck 3 Battalion 2 Engine 5 Ambulance 3 Deputy Chief Engine 6 EMS Supervisor 3rd-Alarm Resource Card Response time: 15 min. Engine 7 Truck 5 Fire Marshal (Bn. Chief) Engine 8 Trng. Battalion Chief Engine 9 Adm. Battalion Chief 4th-Alarm Resource Card Response time: 15 min. Engine 10 Truck 8 Engine 11 5th-Alarm Resource Card Response time: 20 min. Fisherville Engine 1 Harvest Junct. Engine 2 Kingston Engine 1 6th-Alarm Resource Card Response time: 20 min. Apple Valley Engine 1 Bayport Engine 2 Jasper Engine 2 7th-Alarm Response Card Response time: 25 min. Deep Water Engine 2 Kingston Engine 3

Simulation Action Chart

MSSG.		TIME			ACTION TAKEN

Health and Safety Message

| Incident Name: |  | Date Prepared: |  |  | Time Prepared: |
| --- | --- | --- | --- | --- | --- |
| Operational Period Date: From: To: |  |  | Operational Period Time: From: To: |  |  |
| Major Hazards and Risks: |  |  |  |  |  |
| Narrative: |  |  |  |  |  |
| Prepared By; | Company Name: |  |  | ICS Position: Safety Officer |  |
| Approved By: | Company Name: |  |  | ICS Position: |  |

Press Questions

What started the fire and where did it start?

How many people have been injured or killed?

How many people have been evacuated from the building?

Is the fire under control at this time? If not, when do you expect it to be under control?

Have you transported any injured people to the hospital, and if so, which hospital were they transported to?

Could this have been an arson fire?

How many response personnel are on location and how many of each type of apparatus?

Activity 15.1:

High-Rise Simulation Exercise

Student Assessment sheet

Debriefing: Debriefings are discussions centered on the cues contained in the written and visual messages, the strategies you developed as responses to the cues, and your organization and management of the incident staff. - What critical cues did you observe? - What ICS organization did you develop? - Did your group effectively prioritize responsibilities and actions? - Did you pass information to all who needed to be aware of it? - Did you promptly deal with messages? - Did you identify alternate methods of performing tasks? - Did you recognize the need for additional/outside resources? - Did you coordinate the allocation of resources? - Did you periodically re-evaluate resources? - How did your actions differ from the actions in the matrix? Why did they differ? - What are your strengths and weaknesses in working on an IMT for this incident? - How will you apply what you learned in this exercise to preplanning for a target hazard of this type in your community? - How will you apply what you learned in this exercise to responding to an incident at a target hazard of this type in your community?

Notes:

Module Summary

Effective management of high-rise incidents begins with pre-incident planning and knowledge of:

- Construction characteristics and

- Structural features, including high-rise:

- Building systems and

- Fire control stations/rooms.

Also critical for effective management of high-rise fires is knowledge of fire behavior and spread and ICS functions that normally apply only to high-rise incident management, which are:

- Staging area manager,

- Base manager,

- Lobby control unit leader,

- Systems unit leader, and

- Ground support unit leader.