Module 16:

Dormitory Simulation Exercise

Objective

Upon completing this module, you will be able to apply an appropriate ICS organization to effectively manage a dormitory incident.

Methodology

This module uses lecture, discussion, and a simulation exercise.

NFPA 1021 Standards Cross-Reference Matrix

3-6, 3-7, 4-6, 4-7

Introduction

Dormitory occupancies have always presented a large life hazard concern. Often fires in these occupancies are tragic and cause large property damage. One such fire took the lives of 10 students at Providence College, Providence, Rhode Island. Other recent serious fires have occurred at George Washington University in Washington, DC, Wesley College in Dover, Delaware, and Longwood College in Farmville, Virginia. Building construction features and lack of sound fire training programs led to the loss of life and large property damage during these fires.

Dormitories are found in a variety of building sizes and shapes. They are frequently constructed in a low-rise style with ordinary and mixed combustible construction components. However, newer construction often finds mid-rise and high-rise buildings that have noncombustible construction components. Dormitory building construction frequently may be a combination of both combustible and noncombustible materials.

School buildings designed for another type of occupancy often undergo extensive renovations before being converted to a dormitory occupancy. These renovations often create concealed void spaces that permit the spread of fire and smoke. One very serious fire in a dormitory occupancy occurred in such a converted, renovated, building at the College of William and Mary in Williamsburg, Virginia, in a building fully equipped with sprinklers; it caused $4 million in damages.

Ordinary Construction Features

Ordinary construction describes a variety of buildings. The chief characteristic of this construction type is that the walls are of masonry and they often have wood-frame members. There is a limit to the height of a masonry building. The usual rule is that the solid masonry walls shall be 12-inches thick for the first 35 feet of height and increase in thickness 4 inches for each additional 35 feet or fraction in height. Masonry buildings rarely exceed six stories in height. The tallest old-style masonry building in the United States is 15 stories and is located in Chicago, Illinois. The flooring is wood joist with simple beams spanning from wall to wall. The joists usually run parallel to the street frontage of the building. The roof construction may be similar to the floor construction or may have a peak provided through the use of rafters or simple trusses.

Firefighting Problems in Ordinary Construction

Many masonry walls collapse during fires, while others do not. There are certain indicators of probable collapse. For example, if there is inherent structural instability this will be aggravated by the fire. With the collapse of a floor or roof, a masonry wall also may collapse because of the impact. In some cases, firefighting operations result in retained water and increase the load on a masonry wall; this increased load then results in collapse. Masonry walls can be an effective barrier to the extension of fire to the next building if there are no void spaces through which the fire could travel. Void spaces, however, are inherent in ordinary construction and may be even more prevalent when alterations to the building have been made. In large buildings of ordinary construction, interior masonry walls may be the interior load-carrying structure. Interior columns, girders, and beam systems provide stability during fire conditions. However, since the purpose of these systems was not to stop fire, but merely to carry the interior floor loads, it is unusual to find such a wall through the attic space. The combustible attic may provide a path by which fire may pass over the top of the wall.

Noncombustible Construction Features

Noncombustible construction consists of steel (modern style) or concrete frame (old style) buildings commonly found in high-rise (13 or more stories) and mid-rise (6 to 12 stories) buildings. Fire-resistive requirements include 3-hour columns, 2-hour girders and beams, 2-hour roofs, 2-hour floors, and 4-hour exterior walls.

Firefighting Problems in Noncombustible Construction (Mid-rise/High-rise)

In noncombustible construction, each floor is divided into several small units, and partitions must go from floor to floor to be fully effective. If this is not the case and compartmentation is not complete, fire can spread throughout the plenum areas. Open space around a core area of the structure also may spread fire. Open space areas found in mid-rise and high-rise dormitories often are occupied as student lounge areas and game rooms. Ground-level access normally is not a problem, but access to upper floors may be questionable because of limited use of elevators and required time to walk up the stairs.

Ventilation can be extremely difficult and time consuming. Understanding the Heating, Ventilating, and Air Conditioning (HVAC) system, stairwell configuration, and pressurization capability is extremely critical. During ventilation, consider the problems that falling glass may cause. The water supply may be dependent on a system built into the building. Systems may have been tampered with or vandalized.

Mid-rise and high-rise construction present much greater demands on firefighting resources and require well-defined Standard Operating Guidelines (SOGs). Consider firefighters’ limitations because most fires will be labor intensive. There is a high potential for large life loss, and total evacuation may be difficult.

Mixed Construction Features

Mixed construction often consists of composites of older sections, which could possibly be frame or ordinary construction. They may be combined with newer sections of steel or concrete construction, which contain more noncombustible components. The newer sections are often fire resistive. Attaching a new section to an older section may compromise the newer building, particularly when the two are joined to look like one building.

Often, voids are created when additions are added to existing construction. Blind, covered-up stairwells or shaft ways are common. An alert fire department can present a convincing analysis of the total fire problem and take steps to prevent large fire losses before they occur.

Firefighting problems in mixed construction are similar to those in ordinary construction. They include concealed void spaces, lack of compartmentation, and common lofts or attics.

Life Safety/Fire Training/Fire Prevention Principles for Dormitories

Any delay when an emergency is known to exist should be thoroughly investigated and action taken to avoid the problem in the future. All students and resident assistants should fully understand the proper method for reporting an emergency. Fire evacuation drills should be held on a regular basis (monthly). At the beginning of the school year, student and resident assistants should be given instructions on the proper procedures to follow if a fire is discovered. Continual fire education sessions for students and resident assistants should be held periodically throughout the school year.

Room keys must be accessible. A lock box should be kept on each floor for access by the resident assistant and fire personnel during an emergency. If no lock box is installed at each floor, keys must be available at the main security desk at all times. All interior doors should be self-closing, and exit doors should have panic hardware installed.

Sprinklers should be installed in all dormitory occupancies. Concealed spaces should be considered when sprinklers are installed. Smoke detectors should be installed and tested regularly. Local alarm systems should be checked and tested several times a year.

Students often contribute to the interior fire load. Fishnet decorations, blankets on ceilings, and large posters hanging on walls and windows are highly combustible interior decorations. Unannounced room inspections will help prevent these practices. Changing fire safety attitudes is an ongoing challenge for all fire departments that have dormitories within their communities. Changing the attitudes of students and college officials is an ongoing educational goal. Conducting fire incident simulations periodically will help raise the level of awareness for all students and school officials. It also will prepare the fire department members to be more cognizant of the potential problems that will face them during a true emergency at a dormitory.

Activity 16.1:

Dormitory Simulation Exercise

Student Activity Worksheet

Time: 150 minutes (simulation and debrief) Purpose: To provide practice in using the ICS organization to manage a dormitory incident. Directions: - As a group, you will staff positions within the ICS of the Central City Fire Department (CCFD) for the exercise: - Incident Commander - Incident Commander Aide (not used unless nine students in a group) - Operations Chief - Operations Aide (a second Aide designated for groups with more than 10) - Planning Chief - Logistics Chief - Safety Officer - Information Officer - Liaison Officer - You will record all command decisions and actions during the exercise on the Simulation Action Chart. Entries on the ICS chart are made in response to written or visual messages or in anticipation of problems that could surface during the incident. Simulation outcomes: - Incident Commander: Team report - Operations Chief: Initial action plan and organizational chart - Operations Aide: Command worksheet, which includes objectives, strategies and tactics and an incident map - Planning Chief: Alternate plan - Logistics Chief: Resource list - Safety Officer: Safety message - Information Officer: Press release - At the conclusion of the exercise, all groups will participate in a debriefing facilitated by the instructors. Your group will display both the ICS charts and Simulation Action Charts that you developed during the exercise. Be prepared to explain your reasons for your decisions.

Incident overview: Occupancy is a five-story university dormitory building with the 1st floor partially underground. Initially, the fire is located on the 1st floor of the building, which is of ordinary (balloon) construction, 75’ x 200’. The 1st floor, Side D area is occupied as a storage locker area for students’ personal belongings. The 2nd, 3rd, 4th, and 5th floors are occupied as student dormitory rooms. Each dormitory room and hallway has a smoke detector. The building is not equipped with a sprinkler system. Student occupancy is three students per room. Sixty students occupy each floor. There are three exit stairways located on Side A. A similar dormitory building connects on Side B. There are access doors between the two buildings on each floor. The fire was reported at 0645 hours on a Sunday morning in November. The outside temperature is 34°F (1.1°C). Upon arrival, the IC finds students inside and outside the dormitory building. The custodian reports that the fire is in the 1st-floor locker storage area. Incident priorities: - Life safety - Incident stabilization - Property conservation Strategies: - Rescue  Ventilation - Exposures  Salvage - Confinement - Extinguishment - Overhaul - Using the incident priorities and strategies, develop the control objectives. - Develop an incident organization. - Assign and manage resources.

Dormitory

Quick Access Fire Plan

| Building Address: 240 30th St. |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Building Description: Five-story/ordinary (balloon) construction 75' x 200' with two-story attachment in rear L-shaped design. Roof Construction: Ridgepole and rafter, 1" x 6" sheathing, slate roof gable with dormers. Floor Construction: Beam and joist, 1" x 6" sheathing, hardwood floors. |  |  |  |  |  |  |  |  |
| Occupancy Type: College dormitory. |  |  |  | Initial Resources Required: 1st-alarm assignment. |  |  |  |  |
| Hazards to Personnel: First-floor storage lockers. |  |  |  |  |  |  |  |  |
| Location of Water Supply: Hydrant, 8-inch municipal system 300' apart. |  |  |  | Available Flow: 1,200-gpm per hydrant. |  |  |  |  |
|  |  | Estimated Fire Flow |  |  |  |  |  |  |
|  | Level of Involvement | 25% |  | 50% | 75% |  | 100% |  |
|  | Estimated Fire Flow in gpm | 250 |  | 500 | 750 |  | 1,000 |  |
| *Fire flow based on common area (lobby and hallways) of 1,500 sq. ft. with 4 exposure floors. |  |  |  |  |  |  |  |  |
| Fire Behavior Prediction: Rescue, confinement, ventilation. |  |  |  |  |  |  |  |  |
| Predicted Strategies: Rescue, confinement, extinguishment, ventilation, salvage. |  |  |  |  |  |  |  |  |
| Problems Anticipated: 60 students per occupied floor--3 students per room. |  |  |  |  |  |  |  |  |
|  Standpipe: N/A |  |  |  Sprinklers: N/A |  |  |  Fire Detection: |  |  |

Central City Fire and Emergency Medical Services Fire Resources Overhead Positions—one Fire Chief, one Assistant Chief, one Chief Fire Marshal, seven Fire Inspectors, one Training Captain, two Training Lts., three Deputy Chiefs (1 per shift), six Battalion Chiefs (2 Battalions x 3 shifts). Eleven engine companies—Engines 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, and 11. Each engine company is staffed with one officer and three firefighters. Four truck companies—Trucks 1, 3, 5, and 8. Each truck company is staffed with one officer and three firefighters. Squad Company 1. The squad company is staffed with one officer and two firefighters fully trained in rescue practices. Crash/Fire/Rescue Engine. Airport-staffed with one officer and two firefighters. Engine 103, Engine 104, Engine 105, and Engine 109. Reserve Engines—fully equipped. Truck 103. Reserve Truck—fully equipped. Engine 104 (1,500-gpm foam pumper). Apparatus contains one 500-gallon foam tank; 400 five-gallon cans of foam are stored at Engine 4. All members of Engine 5 and Truck 5 have been fully trained as a Hazardous Materials Response Team. Mask Service Unit 106. Engine 206 (light truck). Boat 2 - 16’ Boston Whaler. EMS Resources (third service operated by the city and stationed in the fire stations): Three EMS supervisors – (one per shift x 3 shifts) operating out of fire station 1. Five ALS ambulances – ALS 2, 4, 7, 9, and 11 Five BLS ambulances – BLS 3, 5, 6, 8, and 10

Central City Fire/EMS Response Criteria 1st-Alarm Resource Card Response time: 5 min. Engine 1 Truck 1 Battalion 1 Engine 2 Ambulance 2 MSU 6 Engine 3 Squad 1 2nd-Alarm Resource Card Response time: 10 min. Engine 4 Truck 3 Battalion 2 Engine 5 Ambulance 3 Deputy Chief Engine 6 EMS Supervisor 3rd-Alarm Resource Card Response time: 15 min. Engine 7 Truck 5 Fire Marshal (Bn. Chief) Engine 8 Trng. Battalion Chief Engine 9 Adm. Battalion Chief 4th-Alarm Resource Card Response time: 15 min. Engine 10 Truck 8 Engine 11 5th-Alarm Resource Card Response time: 20 min. Fisherville Engine 1 Harvest Junct. Engine 2 Kingston Engine 1 6th-Alarm Resource Card Response time: 20 min. Apple Valley Engine 1 Bayport Engine 2 Jasper Engine 2 7th-Alarm Response Card Response time: 25 min. Deep Water Engine 2 Kingston Engine 3

Simulation Action Chart

MSSG.		TIME			ACTION TAKEN

Health and Safety Message

| Incident Name: |  | Date Prepared: |  |  | Time Prepared: |
| --- | --- | --- | --- | --- | --- |
| Operational Period Date: From: To: |  |  | Operational Period Time: From: To: |  |  |
| Major Hazards and Risks: |  |  |  |  |  |
| Narrative: |  |  |  |  |  |
| Prepared By; | Company Name: |  |  | ICS Position: Safety Officer |  |
| Approved By: | Company Name: |  |  | ICS Position: |  |

Press Questions

What is the current situation?

How many students were in the dormitory when the fire occurred?

How many students have been injured?

Where have the injured students been taken?

How can parents check on the condition of their children?

What other agencies have you called in to assist?

We understand that the university is not required to comply with local building codes. Is this statement correct?

Isn’t it a fact that their failure to comply with the city code contributed to this fire?

Activity 16.1:

Dormitory Simulation Exercise

Student Assessment sheet

Debriefing: Debriefings are discussions centered on the cues contained in the written and visual messages, the strategies you developed as responses to the cues, and your organization and management of the incident staff. - What critical cues did you observe? - What ICS organization did you develop? - Did your group effectively prioritize responsibilities and actions? - Did you pass information to all who needed to be aware of it? - Did you promptly deal with messages? - Did you identify alternate methods of performing tasks? - Did you recognize the need for additional/outside resources? - Did you coordinate the allocation of resources? - Did you periodically re-evaluate resources? - How did your actions differ from the actions in the matrix? Why did they differ? - What are your strengths and weaknesses in working on an IMT for this incident? - How will you apply what you learned in this exercise to preplanning for a target hazard of this type in your community? - How will you apply what you learned in this exercise to responding to an incident at a target hazard of this type in your community?

Notes:

Module Summary

Dormitories overload fire department resources by the nature of their construction and their occupancy loads. Students frequently add to the fuel load with room decorations and contribute to fire hazards with propped-opened doors and disabled smoke detectors. Early involvement of mutual-aid resources is required to be able to find and fight the fire, as well as search the building. Pre-incident actions include heavy use of inspections to reduce hazards, and mutual-aid agreement updates.