Module 17:

Grain Elevator Simulation Exercise

Objective

Upon completing this module, you will be able to effectively manage a grain elevator incident.

Methodology

This module uses lecture, discussion, and a simulation exercise.

NFPA 1021 Standards Cross-Reference Matrix

3-6, 3-7, 4-6, 4-7

Introduction

Grain elevator incidents can have a significant negative economic impact on a community and present a commander with a typical hazard. Firefighting strategies need to be developed with the elevator’s specific hazards in mind, particularly considering the problem of dust explosions.

The normal thread of comparability that ties target hazards together is that, typically, they are complex and present a challenge to the command and operational capabilities of most fire and rescue organizations. The consideration here is complex in construction contents, life loads, fire loads, or a combination of all of the above. Scale and magnitude also present unusual challenges.

The mainstay of the fire service is the constant training and upgrading of training that keeps it prepared for everyday, normal responses, such as room and contents fires. We all know we have the training and experience to handle the routine. We not only train for it; we experience it on a regular basis. Normally, we do not have the luxury of routine experience, other than preplanning activities and possibly a target hazard.

Grain Elevator Construction

Typical construction of grain elevators includes:

- Offices:

- Type III—ordinary construction,

- Type II—noncombustible construction, and

- Type V—frame construction.

- Silos:

- Reinforced concrete—multistory; and

- Steel/metal fabrication:

- Site built and

- Prefab and site assembled.

- Head house:

- Steel frame to support conveyors and machinery and

- Corrugated metal roof and siding.

Incident Considerations: Grain Elevators

Grain elevators present unique and unusual problems. Both large and small grain elevators will, like all target hazards, require a thorough and complete preplan.

Preplan Points to Emphasize

- Means of egress (exits and entrances);

- Machinery and related equipment used in the processing and storage of grain; and

- Hazardous materials.

Dust Explosions

Dust explosions are always a threat during a response to a grain elevator complex.

Dust explosions normally consist of a small initial explosion, followed by a second, much more powerful explosion. The second explosion usually is caused when the initial explosion disturbs dust that has built up on equipment and the building itself. Dust explosions cause large-scale damage. Destruction can be widespread. Firefighters in this environment are vulnerable, and safety is always consideration for command. The primary consideration for all responders is to be aware constantly that operations should never disturb large amounts of dust.

Responders could be endangered by large pieces of falling concrete and structural steel that break loose in the extreme pressures generated by explosions. After the explosion occurs, the pressure front follows the path of least resistance, so that one part of a structure is unharmed and another area destroyed.

In recent years, there have been improvements in the prevention of dust explosions. There is an increased awareness of the danger of grain dust. In addition, there is a widespread use of safety equipment designed to prevent explosions and improvements in elevator design.

Hazardous Materials

Pesticides, fumigants, dryer fuels, gasoline or diesel fuel, lubricants, and cleaning solvents are stored at grain-handling facilities. Identification of hazardous materials is important because some materials react rapidly in the presence of moisture-causing explosive gases or fires.

Incident Considerations: Grain Dryers

Normal grain operations require grain to be dried in large dryers. These dryers are usually fueled by propane, fuel oil, or natural gas. These fuels can create additional hazards. Dryers have some safety features, typically an automatic shut-off valve to avoid overheating or over drying the grain product. A typical fire department response to a dryer operation problem is caused when one leg of the dryer discharge chute clogs and backs up, causing grain to overheat and ignite into a smoldering fire. The “hot” grain may remain in the dryer or eventually be conveyed to adjoining elevators or bins. Normally, the elevator staff can and will assist firefighters in locating and extinguishing these types of fires. It is important to shut down all operations during a response of this type. Large fires of this type are seldom easy to control; they rarely show open flame.

Fire Extinguishment

Grain fires generate heat, but locating the seat of the fire is often a challenge. Water is normally used to extinguish grain fires. It always should be applied gently and slowly to avoid raising large amounts of surrounding dust. In large, stubborn elevator fires, the bin or elevator sometimes is sealed off and CO2 is injected from the bottom. This is a very effective alternative.

Interior offensive attack is never recommended. Instead, remove smoldering pockets of grain manually. If it becomes absolutely necessary to enter a grain bin, tank, or silo, use protective equipment and life lines, and always use proper confined-space operating procedures.

Normal Causes or Sources of Ignition for Fire or Explosion

- Bearing friction: overheated grease, oil, or lubricant ignites grain or dust;

- Rubber conveyor belt fires: friction of belt rubs against metal framework;

- Belt/friction slippage;

- Frayed or faulty wiring or electrical connections; and

- Damaged electrical conduit exposes arcing or faulty electrical components.

Activity 17.1:

Grain Elevator Simulation Exercise

Student Activity Worksheet

Time: 150 minutes (simulation and debrief) Purpose: To provide practice in using the ICS organization to manage a grain elevator incident. Directions: - As a group, you will staff positions within the ICS of the Central City Fire Department (CCFD) for the exercise: - Incident Commander - Incident Commander Aide (not used unless nine students in a group) - Operations Chief - Operations Aide (a second Aide designated for groups with more than 10) - Planning Chief - Logistics Chief - Safety Officer - Information Officer - Liaison Officer - You will record all command decisions and actions during the exercise on the Simulation Action Chart. Entries on the ICS chart are made in response to written or visual messages or in anticipation of problems that could surface during the incident. Simulation outcomes: - Incident Commander: Team report - Operations Chief: Initial action plan and organizational chart - Operations Aide: Command worksheet, which includes objectives, strategies and tactics and an incident map - Planning Chief: Alternate plan - Logistics Chief: Resource list - Safety Officer: Safety message - Information Officer: Press release - At the conclusion of the exercise, all groups will participate in a debriefing facilitated by the instructors. Your group will display both the ICS charts and Simulation Action Charts that you developed during the exercise. Be prepared to explain your reasons for your decisions.

Incident overview: A head house conveyor belt has been burning for about 20 minutes as it moves soybeans into the #2 south tower. Grain has been ignited from the belt fire and has been dumped into the tower, which is now about 50 percent full. The fire was reported at 1100 hours on a Wednesday in September. The air temperature is 95°F (35°C), and the humidity is 90 percent. The anticipated high temperature is 105°F (40.5°C), which would make it the third consecutive 100° or higher day. There are 18 employees working at the elevator site. The elevator towers are arranged along a north-south line. There is a double row of 15 towers on each side of the elevator. The wind is less than 5 mph out of the south. Incident priorities: - Life safety - Incident stabilization - Property conservation Strategies: - Rescue  Ventilation - Exposures  Salvage - Confinement - Extinguishment - Overhaul - Using the incident priorities and strategies, develop the control objectives. - Develop an incident organization. - Assign and manage resources.

Grain Elevator

Quick Access Fire Plan

| Building Address: 310 Commerce Road |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Building Description: Grain elevator-60’ x 600’ x 120’ with 180’ center tower. Reinforced concrete grain bins. Roof Construction: Concrete with corrugated metal housing. Floor Construction: Concrete floors in tower. |  |  |  |  |  |  |  |  |
| Occupancy Type: Grain elevator. |  |  |  | Initial Resources Required: 1st-alarm assignment. |  |  |  |  |
| Hazards to Personnel: Heavy operating machinery, dust explosion-hazardous material-high voltage. |  |  |  |  |  |  |  |  |
| Location of Water Supply: Hydrant, 12-inch municipal systems1,000’ apart. |  |  |  | Available Flow: 1,500 gallons per minute (gpm) per hydrant. |  |  |  |  |
|  |  | Estimated Fire Flow |  |  |  |  |  |  |
|  | Level of Involvement |  |  |  |  |  |  |  |
|  | Estimated Fire Flow |  |  |  |  |  |  |  |
| *Fire flow not used in this type of facility. |  |  |  |  |  |  |  |  |
| Fire Behavior Prediction: Hidden fire in bins. Friction fire in conveyor system - product fire in conveyor system. |  |  |  |  |  |  |  |  |
| Predicted Strategies: Confinement, extinguishment. |  |  |  |  |  |  |  |  |
| Problems Anticipated: Access to fire area, height, potential dust explosion. |  |  |  |  |  |  |  |  |
| ý Standpipe: Yes |  |  |  Sprinklers: No |  |  | ¨ Fire Detection: No |  |  |

Central City Fire and Emergency Medical Services Fire Resources Overhead Positions—one Fire Chief, one Assistant Chief, one Chief Fire Marshal, seven Fire Inspectors, one Training Captain, two Training Lts., three Deputy Chiefs (1 per shift), six Battalion Chiefs (2 Battalions x 3 shifts). Eleven engine companies—Engines 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, and 11. Each engine company is staffed with one officer and three firefighters. Four truck companies—Trucks 1, 3, 5, and 8. Each truck company is staffed with one officer and three firefighters. Squad Company 1. The squad company is staffed with one officer and two firefighters fully trained in rescue practices. Crash/Fire/Rescue Engine. Airport-staffed with one officer and two firefighters. Engine 103, Engine 104, Engine 105, and Engine 109. Reserve Engines—fully equipped. Truck 103. Reserve Truck—fully equipped. Engine 104 (1,500-gpm foam pumper). Apparatus contains one 500-gallon foam tank; 400 five-gallon cans of foam are stored at Engine 4. All members of Engine 5 and Truck 5 have been fully trained as a Hazardous Materials Response Team. Mask Service Unit 106. Engine 206 (light truck). Boat 2 - 16’ Boston Whaler. EMS Resources (third service operated by the city and stationed in the fire stations): Three EMS supervisors – (one per shift x 3 shifts) operating out of fire station 1. Five ALS ambulances – ALS 2, 4, 7, 9, and 11 Five BLS ambulances – BLS 3, 5, 6, 8, and 10

Central City Fire/EMS Response Criteria 1st-Alarm Resource Card Response time: 5 min. Engine 1 Truck 1 Battalion 1 Engine 2 Ambulance 2 MSU 6 Engine 3 Squad 1 2nd-Alarm Resource Card Response time: 10 min. Engine 4 Truck 3 Battalion 2 Engine 5 Ambulance 3 Deputy Chief Engine 6 EMS Supervisor 3rd-Alarm Resource Card Response time: 15 min. Engine 7 Truck 5 Fire Marshal (Bn. Chief) Engine 8 Trng. Battalion Chief Engine 9 Adm. Battalion Chief 4th-Alarm Resource Card Response time: 15 min. Engine 10 Truck 8 Engine 11 5th-Alarm Resource Card Response time: 20 min. Fisherville Engine 1 Harvest Junct. Engine 2 Kingston Engine 1 6th-Alarm Resource Card Response time: 20 min. Apple Valley Engine 1 Bayport Engine 2 Jasper Engine 2 7th-Alarm Response Card Response time: 25 min. Deep Water Engine 2 Kingston Engine 3

| International Chemical Safety Cards |  |  |  |
| --- | --- | --- | --- |
| ALUMINIUM PHOSPHIDE |  |  | ICSC: 0472 |
| •INHALATION |  |  |  |
| •SKIN | ACUTE HAZARDS/ SYMPTOMS | PREVENTION | FIRST AID/ FIRE FIGHTING |
| •EYES | Not combustible but forms flammable gas on contact with water or damp air. Gives off irritating or toxic fumes (or gases) in a fire. | NO open flames, NO sparks, and NO smoking. NO contact with acid(s) and water. | Special powder, dry sand, NO other agents. NO water. |
| EXPLOSION | Risk of fire and explosion on contact with acid(s) and water. |  |  |
| EXPOSURE |  | PREVENT DISPERSION OF DUST! STRICT HYGIENE! |  |
|  | Sore throat. Cough. Shortness of breath. Headache. Dizziness. Nausea. Vomiting. | Local exhaust or breathing protection. | Fresh air, rest. Half-upright position. Artificial respiration if indicated. Refer for medical attention. |
|  | Redness. Burning sensation. | Protective gloves. | Remove contaminated clothes. Rinse and then wash skin with water and soap. Wear protective gloves when administering first aid. |
|  | Redness. | Safety goggles, or eye protection in combination with breathing protection. | First rinse with plenty of water for several minutes (remove contact lenses if easily possible), then take to a doctor. |
| •INGESTION | Nausea. Vomiting. Diarrhea. Abdominal pain. Headache. Convulsions. Shock or collapse. Unconsciousness. | Do not eat, drink, or smoke during work. Wash hands before eating. | Induce vomiting (ONLY IN CONSCIOUS PERSONS!). Refer for medical attention. |

| SPILLAGE DISPOSAL | STORAGE | PACKAGING & LABELLING |
| --- | --- | --- |
| Evacuate danger area! Consult an expert! Do NOT wash away into sewer. Sweep spilled substance into sealable containers. Carefully collect remainder, then remove to safe place. Do NOT let this chemical enter the environment. (Extra personal protection: complete protective clothing including self-contained breathing apparatus). | Fireproof. Provision to contain effluent from fire extinguishing. Separated from acids, water, food and feedstuffs. Dry. | Airtight. Do not transport with food and feedstuffs. F symbol T+ symbol R: 15/29-28 S: 1/2-3/9/14-30-36/37-45 UN Hazard Class: 4.3 UN Subsidiary Risks: 6.1 UN Packing Group: I |

| I M P O R T A N T D A T A | PHYSICAL STATE; APPEARANCE: DARK GREY OR DARK YELLOW CRYSTALS. PHYSICAL DANGERS: CHEMICAL DANGERS: The substance decomposes on contact with water, moist air and acids producing highly flammable and toxic gases (phosphine, see ICSC # 0694). OCCUPATIONAL EXPOSURE LIMITS: TLV not established. | ROUTES OF EXPOSURE: The substance can be absorbed into the body by inhalation of its aerosol and by ingestion. INHALATION RISK: Evaporation at 20°C is negligible; a harmful concentration of airborne particles can, however, be reached quickly. Hydrolysis in atmospheric moisture or perspiration may yield gaseous phosphine which can be inhaled. EFFECTS OF SHORT-TERM EXPOSURE: The substance irritates the eyes the skin and the respiratory tract. Inhalation of phosphine liberated from aluminum phosphide may cause lung edema (see Notes). The substance may cause effects on the cardiovascular system, nervous system and respiratory tract , resulting in impaired functions and respiratory failure. Exposure may result in death. |
| --- | --- | --- |
|  | EFFECTS OF LONG-TERM OR REPEATED EXPOSURE: |  |
| PHYSICAL PROPERTIES | Melting point: 1000°C Relative density (water = 1): 2.9 | Solubility in water: reaction |
| ENVIRONMENTAL DATA | The substance is toxic to aquatic organisms. |  |
| N O T E S |  |  |
| Commercial formulations for fumigations usually contain 57% of active ingredient. Reacts violently with fire extinguishing agents such as water. The symptoms of lung edema often do not become manifest until a few hours have passed and they are aggravated by physical effort. Rest and medical observation are therefore essential. Immediate administration of an appropriate spray, by a doctor or a person authorized by him/her, should be considered. Do NOT take working clothes home. Alutal, Celphide, Celphine, Celphos, Delicia Gastoxin, Detia Gas-Ex-B, Detia Gas-Ex-P, Detia Gas-Ex-T, L fume, Phosfume, Phostek, Phostoxin, Quickfos, and Zedesa are trade names. Also consult ICSC # 0694 for Phosphine. |  |  |

chris - Chemical Hazard Response Information System

aluminum phosphide

overview

Material name:

Aluminum phosphide.

CHRIS Code APH.

Common synonyms:

Aluminum monophosphide.

AIP.

Characteristics:

Solid crystals or powder Dark gray; dark yellow, May have “fishy,” phosphine odor.

Sinks and reacts with water to produce poisonous and spontaneously flammable phosphine gas.

Emergency actions:

Avoid contact with solid, dust, or gases FROM REACTION with water.

keep people away.

Wear positive-pressure breathing apparatus and full protective clothing.

Shut off ignition sources and call fire department.

Stop discharge if possible.

Isolate and remove discharged material.

Notify local health and pollution control agencies.

Fire:

Flammable and POISONOUS gas produced on contact with water.

poisonous gases also produced in fire.

Wear positive-pressure breathing apparatus and full protective clothing.

Do not use water or foam.

Extinguish small fires:  dry chemical, soda ash or lime.

Large fires:  withdraw from area and let burn.

Move container from fire area if you can do it without risk.

Exposure:

Call for medical aid.

Poisonous vapor produced in reaction with water may be fatal if inhaled.

severe pulmonary irritant and an acute systemic poison; may cause sudden or delayed death.

Effects may be delayed; Keep under observation.

Dust.

Poisonous; may be fatal if inhaled.

Move to fresh air.

If breathing has stopped, give artificial respiration.

If breathing is difficult, give oxygen.

Effects may be delayed; keep under observation.

Solid.

Poisonous if swallowed or skin EXPOSED.

if in eyes or on skin, flush with running water for at least 15 minutes; hold eyelids open periodically if appropriate.

Remove and isolate contaminated clothing and shoes.

If swallowed and victim is unconscious or having convulsions do nothing except keep warm.

Water pollution:

Effects of low concentration on aquatic life is unknown.

May be dangerous if it enters water intakes.

Notify local health and wildlife officials.

Notify operators of nearby water intakes.

Response to discharge

Issue warning - high flammability, poison, air and water contaminant.  Restrict access, Evacuate area (large discharge only).  Should be removed.

Label

Category:  Flammable Solid; Dangerous when wet.

Class:  4.

CHEMICAL designations

CG compatibility class:  Not listed.

Formula:  AIP.

IMO/UN designation:  4.3/1397.

DOT id no.:  1397.

observable characteristics

Physical state:  Solid (crystals or powder).

Color:  Dark gray; dark yellow.

Odor:  May generate fishy odor of phosphine upon exposure to moisture in air.

health hazards

Personal protective equipment:  Wear positive-pressure breathing apparatus and full protective clothing.

Symptoms following exposure:  Highly toxic.  May be fatal if inhaled (dust) or swallowed.  Also poisonous via skin contact.  (Readily reacts with moisture or acids to produce phosphine gas which is a severe pulmonary irritant, an acute poison and spontaneously flammable. Ingestion may result in respiratory, cardiac, hepatic and renal involvement.  Exposure to phosphine gas may cause headache, fatigue, nausea, vomiting, cough, severe breathing difficulties, shortness of breath, jaundice, paresthesias, ataxia, intention tremor, diplopia and miocardial injury.  Breathing difficulties may be delayed several hours after exposure to phosphine has ceased.)

Treatment of exposure:

INHALATION:  Move victim to fresh air; call for emergency medical care.  If not breathing, give artificial respiration.  If breathing is difficult, give oxygen.  Breathing difficulties may be delayed; Keep under observation.  EYES OR SKIN:  Immediately flush with running water for at least 15 minutes holding upper and lower eyelids open periodically if appropriate.  Remove and isolate contaminated clothing and shoes at the site.  Keep victim quiet and maintain normal body temperature.

INGESTION:  If unconscious or having convulsions, do nothing except maintain normal body temperature.

Threshold limit value:  2mg/m(3) (A1).

Toxicity by ingestion:  Grade 4; LD(50) = 20 mg/kg (human).

Vapor (gas) irritant characteristics:  In the presence of moisture, it generates phosphine gas, a severe pulmonary irritant.

Liquid or solid irritant characteristics:  Data not available.

IDLH value:  Data not available.

fire hazards

Flashpoint:  Not pertinent (Reacts with moisture to produce phosphine gas which is spontaneously flammable at room temperature.)

Flammable limits in air:  Not pertinent.  (Data not available for the phosphine gas produced upon exposure to moisture.)

Fire extinguishing agents:  Small fires:  Dry chemical, soda ash or lime.  Large fires:  Withdraw from area and let fire burn.

Fire extinguishing agents NOT to be used:  Do not use water or foam.

Special hazards of combustion products:  May contain toxic gases and vapors such as oxides of phosphorous and phosphoric acid mist.

Behavior in fire:  Can react with water or acids to produce spontaneously flammable and acutely toxic phosphine gas.

Ignition temperature:  Not pertinent.  (Reacts with water or acids to produce phosphine gas which is spontaneously flammable at room temperature.)

Burning rate:  Not pertinent.

Flame temperature:  Data not available.

chemical reactivity

Reactivity with water:  Reacts with water or steam to produce phosphine gas.  Phosphine is highly toxic and spontaneously flammable; and its combustion products, such as oxides of phosphosus, are also highly toxic.

Reactivity with common materials:  May react with the moisture in wood to produce spontaneously flammable phosphine gas.  Reacts violently with oxidizing substances.

Stability during transport:  Stable (when dry).

Neutralizing agents for acids and caustics:  Not pertinent.

Polymerization: Not pertinent.

Inhibitor of polymerization:  Not pertinent.

water pollution

Aquatic toxicity: Data not available.

Waterfowl toxicity:  Data not available.

hazard classifications

Code of federal regulations:  Flammable solid.

NFPA hazard classification:  Not listed.

PHYSICAL and chemical properties

Physical state at 15 degrees C. and 1 ATM:  Solid.

Molecular weight:  57.96.

Freezing point:  >1,832 degrees F. = > 1,000 degrees C. = >1,273 degrees K.

Specific gravity:  2.85 at 25 degrees C.

Simulation Action Chart

MSSG.		TIME			ACTION TAKEN

Health and Safety Message

| Incident Name: |  | Date Prepared: |  |  | Time Prepared: |
| --- | --- | --- | --- | --- | --- |
| Operational Period Date: From: To: |  |  | Operational Period Time: From: To: |  |  |
| Major Hazards and Risks: |  |  |  |  |  |
| Narrative: |  |  |  |  |  |
| Prepared By; | Company Name: |  |  | ICS Position: Safety Officer |  |
| Approved By: | Company Name: |  |  | ICS Position: |  |

Press Questions

How many injuries are there, and what are you doing to prevent additional injuries?

Is the smoke toxic, and how far do you expect it to spread?

Is there a possibility of a second explosion? If so, what are you doing to prevent a second explosion?

Is it safe for the firefighters to work in the area?

Have you found violations of the fire and building code at this complex?

We understand that there are hazardous chemicals used at this complex. Is the facility permitted to store and handle these materials?

What are you doing to capture the runoff of the contaminated water at the site?

It is extremely hot. What are you doing to protect the firefighters?

Activity 17.1:

Grain Elevator Simulation Exercise

Student Assessment sheet

Debriefing: Debriefings are discussions centered on the cues contained in the written and visual messages, the strategies you developed as responses to the cues, and your organization and management of the incident staff. - What critical cues did you observe? - What ICS organization did you develop? - Did your group effectively prioritize responsibilities and actions? - Did you pass information to all who needed to be aware of it? - Did you promptly deal with messages? - Did you identify alternate methods of performing tasks? - Did you recognize the need for additional/outside resources? - Did you coordinate the allocation of resources? - Did you periodically re-evaluate resources? - What actions did you take in response to these cues? - Size of burning belt, - Air temperature, - Length of burn time, - Fuel mix, - Life load—employees, - Wind direction and velocity, - Fire location, - Power shutdown, - Equipment location, - Inadequate equipment reach, - Access on the catwalk, - Missing employee, - Heavy smoke, - Firefighter collapse, - Fumigant presence, - Injured employee, - Arrival of outside agency, - Loss of water pressure, and - Product level in Tower #2. - How did your actions differ from the actions in the matrix? Why did they differ? - What are your strengths and weaknesses in working on an IMT for this incident? - How will you apply what you learned in this exercise to preplanning for a target hazard of this type in your community? - How will you apply what you learned in this exercise to responding to an incident at a target hazard of this type in your community?

Notes:

Module Summary

The complexity of grain elevator facilities can present an unfamiliar challenge to firefighters unless they have acquainted themselves with the preplan information. Each grain-handling facility is unique in layout, design, construction, operation, equipment, and personnel. Each facility requires individual preplanning. Such plans should be a mutual effort between the facility’s management and the fire department.