Module 7:

Fire Cause Determination I

Objective

Upon completing the module, you will be able to make critical observations about the area of fire origin and fire cause.

Methodology

This module uses lecture, discussion, and activities.

NFPA 1021 Standards Cross-Reference Matrix

3-5, 4-5

Introduction

The fire officer is often in the best position to make critical observations regarding a fire. The officer can compare this fire to similar fires and draw parallels between them for similar fire behavior patterns. The observations of the fire officer during the various stages of an incident are extremely critical. From the moment of dispatch through the overhaul phase, the company officer’s senses are exposed to countless stimuli, many of which can help make a determination of fire origin and cause. The scenes observed can never be duplicated in a courtroom. The fire officer may be the critical link in the prosecution of an accused arsonist in convincing a jury that the fire was unusual.

Critical Observations

We will consider the following phases during which critical observations are possible:

- Responding and arriving,

- Size-up,

- Suppression, and

- Overhaul.

During the incident, the fire officer will see thousands of images and make hundreds of decisions, many instantaneously. The many phases of the operation will provide the officer the opportunity to make particular observations that may be critical to the fire cause determination. On the other hand, the officer may note something casually that later will emerge from memory. This casual note may also be a critical piece of information.

Observations: Responding and Arriving

Unusual Information

The first critical piece of information about the fire usually will come from the person calling in the alarm. This person may be the discoverer of the fire, the occupant of the building on fire, or just a person passing by who saw the fire. In today’s world of instantaneous communications, mobile telephones in many cars make it possible for even a passerby to report an incident.

The person receiving the information from the caller should try to record all of the following pieces of information.

The name of the person reporting the fire—It is important to note any accents or speech impediments, the pitch that would distinguish the voice of a child or a woman from that of a man, the speed of the speech, and any other distinctive traits.

State of the person reporting the fire—The person reporting the call should note whether the caller sounded calm or excited. If the fire was not planned, one would expect some level of excitement in the caller’s voice, especially if it were the owner or an occupant reporting the fire.

Indication of any background noises, especially if they are unusual—If a person is calling from inside a building, one would not expect to hear trucks rumbling by. However, if one is calling from an outside telephone, the likelihood of trucks rumbling by or other traffic noise is much greater.

The exact address of the fire or at least specific information on the area such as street names and landmarks.

Weather Conditions

The officer should note weather conditions. Also, note wind direction and speed and compare with the direction of fire travel. Wind may account for the intensity and/or spread of the fire.

Do not fail to make note of clear weather conditions versus rain, snow, or ice. If the fire was incendiary, it may have been the arsonist’s intention to set the fire in bad weather in order to delay the fire department’s response.

Temperature and humidity readings should be obtained. Freezing temperatures may cause fire hydrants to freeze, or pose other hazards that can delay fire suppression. Climatic conditions and statistics can be obtained by a telephone call to the nearest office of the National Weather Service.

Another key point to note is a change in weather conditions since the arrival of fire companies and the effects of these weather changes on fire suppression operations.

Time of Day/Day of Week

The times and methods of alarms will probably be recorded automatically. Time plays an important role in origin and cause determination.

Alarms allow one to show any abnormal time span between the time the fire was reported and the amount of fire at the time of the first arrival of emergency personnel. Later, the officer might need to identify and interview the person(s) who reported the fire. In addition, it is important to note and record the arrival time of the first-in fire company.

If it was a telephone alarm, most emergency dispatch centers automatically record the location from which the call was made; the caller then can be identified and interviewed. In the event the alarm was received by a municipal alarm box, a neighborhood canvass may turn up a witness to identify who pulled the box. If a private alarm service received the fire alarm, ascertain if there have been any problems with the system or abnormal tests by the client.

Response Route

The next critical phase for making observations begins with the response. While en route, the officer should be aware of anything unusual, especially as he/she nears the fire scene. Vehicles traveling away from the fire scene without lights at night or with persons crouching down in the seat to avoid being observed might be good indications that they were linked with the fire. Pedestrians may be casually or hurriedly leaving the fire scene itself; they may be acting suspiciously.

Of particular interest to the officer are other factors that are not directly associated with the fire itself but which may have some tie-in. Was the timing of the fire such that response was slowed because of a train passing through a grade crossing or a drawbridge raised over the water? There may be no link at all; on the other hand, there may be.

Road construction or repairs can cause delays, especially an unsuspected detour.

Traffic patterns at certain times of the day (morning or evening rush hours) can cause delays. Someone who planned a fire may take advantage of a social event like a parade or an athletic event that can cause huge traffic bottlenecks. The sharp officer will note anything of an unusual nature.

The vehicles in the area of the fire may carry some valuable clues. The officer may find it helpful to question witnesses and any other responding emergency personnel as to their observations while en route to, or after arrival. Ask if they have observed the same vehicles at fire scenes on more than one occasion. Note and identify any vehicles that were damaged by fire, heat, or smoke. Try to find out why these vehicles were there.

Attempt to obtain and record descriptions and license plate numbers. The drivers/owners may be questioned later regarding the reasons their vehicles were at the fire scene.

This listing is not exhaustive. Experienced company officers and firefighters probably can think of many more observable factors that can be noted during the response phase.

Environmental Considerations

Observations about the immediate vicinity of the fire may provide an insight into circumstances surrounding the fire. Some areas of the community may be more prone to fires than others. Fires may occur more often in areas with transient populations than in areas that have had the same owner/occupant for years.

Areas with a high occupancy turnover may not have a fire reported as quickly as other areas. Late reporting may be observed upon arrival in the heavy fire conditions and in the deep charring of structural members. This latter fact may not be known for some time, however.

Uninhabited properties or properties for sale pose interesting possibilities. The fact that the property is for sale or has been for sale for some time is not of itself cause for suspecting an incendiary fire. The fire cause determination must be done just as it would be for any other fire. If the fire is determined to be incendiary, an investigator will need to do a great deal more work to prepare a case against an individual for starting the fire.

However, this activity goes beyond the responsibility of the company officer, although he/she may assist the investigator in a number of ways.

Fires in remote sections of the community or in sections that may have daytime, but not nighttime occupancy may burn longer before being discovered.

As noted above, this listing is not complete. The intent is to get the company officer and firefighters thinking about what we call “environmental factors” that will help to make a correct origin and cause determination.

Observations During Size-Up

The next important phase for making observations is the size-up period. Upon arrival, firefighters seek out available occupants to ascertain the rescue needs of unaccounted-for occupants. This task does not negate the need for a primary search, but does narrow down the responsibilities for search. Without a doubt, the fire scene itself provides the greatest chance for observations. There is a whole range of possibilities. During this activity, one should take note of several items.

Occupants

Questions to ask regarding the occupants include:

- Are the occupants wearing clothing that one would expect for that time of day at that time of the year?

- Did the occupants have time to retrieve special items such as a child’s favorite toy, a pet, the insurance policy, someone’s favorite fishing pole or shotgun, photos, jewelry, or valuable antiques?

- Are there any signs that the occupants have burns, cuts, or singed body hair that may indicate they were close to the fire?

- Is there any sign of soot around their mouths to indicate they were breathing smoke?

- Do any of the occupants need medical attention?

Spectators

Another set of possibilities rests with the spectators in the area. Large numbers of spectators usually will gather to observe any type of disaster. Fires are no exception.

Spectators may show signs of having been involved in starting the fire. They may be wearing fire-damaged clothing or clothing that has an odor of accelerants. They may show visible fire injuries.

Questions concerning spectators include:

- Are there any spectators who have been seen at other fires, especially fires of a similar nature in the same general area?

- Is anyone acting strangely, hiding in shadowed areas, standing out for some reason, or trying too hard to blend into the crowd?

People who intentionally start fires often will stay at the fire scene to see the results of their efforts. Many fire investigations reveal that the same person has been seen at several fires. When arrested, these people state that they stayed at the fire scene to watch the activities.

Unfortunately, this frame of mind also extends to the fire service. Many fires are set by members of fire companies who are seeking thrills associated with responding.

Groups of spectators can be videotaped or photographed for a later detailed review. Things that may prove helpful include:

- Familiar faces or individuals seen at other fires;

- The actions of spectators, such as individuals who appear too concerned about the incident or are too vocal about the incident, displaying criticism about fire and/or police agencies, neighbors, society, and/or governmental agencies;

- Individuals who appear to be too quiet or withdrawn, or overly frightened about the fire; and

- Individuals who appear to be too excited about the emergency, overly brave, helpful, or curious.

The appearance of spectators may provide helpful information. Construction or repair crews should be noted. Accidental fires often result from repair crews working in the structure. Repair crews may be reluctant to be interviewed for fear of the loss of their jobs if they had any part in the fire or fear of liability for damages caused by the fire.

Structure and Location of Fire

During size-up and suppression operations, you can make many observations about the location of the fire, the extent of the fire, and the path of fire travel. These observations may be critical during the fire scene examination to help determine if fire behavior was normal or if it was assisted.

It is critical to determine if the location of the fire upon responder arrival corresponds with the information from the caller. A fire reported in the basement that is a fully involved attic fire upon arrival may be explained because of the type of building construction. On the other hand, it may have been intentionally started in the attic with a trailer from the basement.

It also is important to establish if the fire spread and traveled as anticipated for the type of building and occupancy. The spread of the fire may be very typical or normal, or the spread may indicate it had some assistance.

Observe how the fire vented itself, noting natural ventilation versus fire company venting. Note removal of debris, furnishings, stock, and supplies. Provide security for materials or debris that may aid in identifying the point of origin or fire cause.

If fire suppression (extinguishment) activities still are occurring, note both damaged and undamaged areas of the structure. The structure may suffer extensive damage during the suppression phase. Realizing this fact helps eliminate wasted time and effort in determining the origin of the fire. Areas of extensive damage may not indicate area of origin.

Other fire areas may have required more immediate attention by fire personnel because of the need for protecting dangerous materials, protecting high-value areas, or the process of stopping fire extension to other areas of the building.

The officer should note areas of open burning (visible flames) which may indicate the types of fuel being consumed.

Ascertain whether there was a complete or partial structural collapse. Was there ignition or exposure of flammable or hazardous materials? Were there reports of any explosions? Were firefighters forced to fall back for any reason, or was there a fast fire spread overtaking the fire streams?

Any unusual signs of the occupants’ escape need to be noted. Things like ladders to windows, escape ladders from the windows, bed sheets tied together, or reports from a neighbor that the mother dropped her baby two floors into the arms of a passerby are strong indications that the occupant did not plan a fire.

Flame and Smoke Characteristics

The color of the smoke may indicate the type of material burning. Complete combustion often produces little or no smoke while dense smoke often indicates incomplete combustion.

Flame color also may indicate the type of materials being burned. As the amount of hydrocarbons increases, the flames will become darker or more orange in color. A lack of sufficient oxygen (air) usually causes flames to be darker than when the same fuel is burned in a well-ventilated area.

Extreme caution is required. During the later stages of the fire, smoke colors from materials burning may give false indications. In addition, most structures contain fuels with hydrocarbon bases which, when burning, may produce smoke and/or flames which could give misleading signs.

Here are some examples of smoke and flame colors as indicators of the type of material burning:

| Fuel | Color of Smoke | Color of Flame |
| --- | --- | --- |
| Wood | Gray to brown | Yellow to red |
| Paper | Gray to brown | Yellow to red |
| Cloth | Gray to brown | Yellow to red |
| Gasoline | Black | Yellow to white |
| Naphtha | Black to brown | Straw to white |
| Benzene | White to gray | Yellow to white |
| Lubrication oil | Black | Yellow to white |
| Lacquer | Brownish to black | Yellow to red |
| Turpentine | Brown to black | Yellow to white |
| Acetone | Black | Blue |
| Cooking oil | Brown | Yellow |
| Kerosene | Black | Yellow |

Observations During Suppression Activities

During fire suppression efforts, the firefighters and officers will probably be in the best position to observe the fire. Physical aspects of the fire, such as materials burning, appearance of more than one area of fire origin, and the color of the fire, are critical observations. It also is essential to note the current stage of burning and the intensity with which the fire is burning.

Entry

Among the many other things to be noted are the doors. Was forcible entry necessary or were the doors already open or unlocked? Doors may have been left open accidentally in the occupants’ rush to flee the fire. On the other hand, open doors may be intentional. If the occupants state that they closed the doors, one should attempt to discover how they were opened. Neighbors or police officers who arrive before firefighters and try to help rescue the occupants often open doors to determine if everyone is out.

Difficulty of Extinguishment

Try to determine if the fire went out as would be expected or if the fire was difficult to extinguish. Fires that are difficult to extinguish and do not react normally to the application of water may have had a fuel load higher than expected. This situation should be noted and examined after the fire is out to determine the possible fuel load that caused the difficulty in extinguishment.

Unusual Observations

Was there anything unusual found during suppression, such as windows covered to hide the fire, a means of egress intentionally blocked by furniture, or disabled fire detection and suppression equipment? If the structure has detection equipment, such as smoke detectors, did any firefighter hear it operate? If the structure has fire suppression equipment such as automatic sprinklers, how did it perform in fire control and suppression? Has it been tampered with, or did it operate as it should have?

It is impossible and unnecessary to list here everything the company officer and firefighters need to observe. The items mentioned here are some of the most important ones. Experience will reveal many others. Spending time with your colleagues in a discussion is a good way to generate a list of many more.

Observations During Overhaul

The last critical phase during which more observations can be made is the overhaul phase. In many respects, it is the best because you do not have to worry about the fire and you have plenty of time to carry out a careful examination. The overhaul phase is the first real opportunity to start making critical firsthand observations about the structure, its contents, and the circumstances that caused the fire.

Items Missing or Out of Place

The first thing to establish is whether there appear to be any items missing. On the other hand, are there any items present that shouldn’t normally be there? Do items appear to have been moved around before the fire, for example, a chair moved next to another chair or a chest of drawers repositioned?

A person planning a fire may remove things before the fire, for example, family photographs from the wall or off a table, favorite or expensive items of clothing, pets, or expensive sporting equipment and trophies. The fire officer who makes these observations needs to ask the correct questions of the owner or occupant about those items.

Unusual Prefire Activity

Look for indications of unusual prefire activity. In some situations, fire is used to cover a burglary. The evidence of the burglary may be very apparent after fire suppression. Vending machines or cash registers pried open will still be pried open after the fire. File drawers forced open or left open are another indicator.

Doors that have been forced, broken windows, or other structural damage not caused by the fire may go unnoticed unless you are observant during the overhaul phase. In this respect, it is critical that firefighters disrupt things as little as possible. Sometimes, in their haste to perform overhaul, responders can remove or alter some very critical evidence.

Some clues may be directly related to certain motives. In fires that result from a broken relationship, finding clues to the motive is not uncommon. A person who finds that a spouse or mate has been unfaithful may pile the other’s clothing on the bed and set the bed on fire. Actions like this are also common when someone is seeking attention and feels that there is something standing in the way of a close relationship.

The fire officer may be the only person who connects with the evidence found at the scene with the circumstances that led up to the fire.

Unusual Signs of Fire Activity

The officer should note the features of the structure that allowed the fire, heat, or smoke to travel. Open stairs, pipes or electrical shafts, laundry chutes, or heat ducts are some of the most common paths of fire travel. Building renovations often result in dropped ceilings, covered walls, or new voids. Fire is often hidden in these voids and may burn for a considerable time before being discovered.

Finally, the utilities need to be evaluated to determine if they were operational, shut down at the time of the fire, or shut down by fire suppression personnel. Try to determine if there were any signs of malfunctions, improper usage of the utility, or other factors that could have caused or contributed to the fire.

Activity 7.1:

Observations

Student Activity Worksheet

Time: 30 minutes Purpose: To give you the opportunity to reinforce your observation skills. Directions: - This activity is for the large group. Your instructor will show a set of slides and ask for observations. - Your answers should focus on the following information: - The type of construction, - The location and intensity of the fire, - The color and size of the flame and smoke, and - The exposures affected. - Write your observations in the blanks provided.

Observation one: two-story dwelling with addition Observation two: interior view of a dwelling, smoke and fire Observation three: interior view of a dwelling, smoke and fire Observation four: inside the dwelling, two fires, building under renovation Observation five: two-story, wood-frame dwelling; one room on fire Notes:

Role of the Officer

Make Initial Critical Observations

As one of the first at the scene of a fire, the officer may see things critical to determining the area of fire origin and the fire cause. For example, the officer will observe the color of the flames or smoke, the location of the heaviest fire, and whether flashover has occurred.

Determine if the Fire Behaved as Expected

Careful observation will help discriminate between normal and abnormal fire growth and behavior. Examples of critical discrimination include noting whether the fire reacted violently to water, the fire appeared to spread rapidly from one floor to another with no combustible material in between, and structural elements with limited combustible materials present sustained excessive heat damage.

Describe Observations

Carefully noting observations, and later describing them to a fire investigator or jury, may be critical to a successful fire investigation or criminal prosecution. Examples of critical observations include explaining to a jury observations on how the fire reacted to water; explaining to a fire investigator that the fire was in the smoldering phase upon arrival, and a backdraft occurred soon after arrival; and explaining to a fire investigator how ventilation tactics affected normal fire spread and caused the unusual fire travel.

Fire Behavior

Definition of Fire

A good place to start is with the meaning of combustion.

“Combustion is an exothermic, self-sustaining reaction involving a solid, liquid, and/or gas-phase fuel. The process is usually (but not necessarily) associated with the oxidation of a fuel by atmospheric oxygen. Some solids can burn directly by glowing combustion or smoldering, but in flaming combustion of solids and liquid fuels, vaporization takes place before burning. It is necessary to distinguish between two types of flaming: (1) premixed, in which gaseous fuel is mixed intimately with air before ignition, and (2) diffusive, in which combustion takes place in the regions where the fuel and oxygen are mixing. If premixed burning takes place in a confined space, a rapid pressure rise will occur, giving rise to an explosion.” (NFPA. Fire Protection Handbook, 17th Ed., p. 1-44.)

Although technically correct, such a lengthy and detailed definition of fire (or combustion) is very cumbersome. Therefore, a shorter, more concise definition such as the following may be preferred. Let’s call it a working definition.

Fire is a rapid, self-sustaining oxidation process usually accompanied by the evolution of heat and light in varying intensities.

Of all the terms learned throughout this course, the definition of fire is one of the most important.  This is because the process of combustion underlies everything else we need to consider in reaching our accurate fire origin and cause determination.

Principles of Combustion

### The Fire Triangle

The fire officer must be familiar with basic fire chemistry. First, we will discuss the principles of burning and the elements necessary to produce burning.

The triangle theory is still used for a simple explanation. Until recently, all fires were described or explained as being a triangle of three essential elements. These three elements are heat, fuel, and oxygen. Remove any one of these three elements and the fire goes out.

The fire triangle continues to explain the glowing fire, that is, a glowing fuel mass without flaming combustion. Modern research has expanded the understanding of open or flaming combustion by adding a fourth element.

### The Fire Tetrahedron

This addition of a new element has changed the name of the fire triangle to the “fire tetrahedron.” This fourth element is described as an uninhibited chain reaction. This chain reaction is responsible for the continuous heating of the fuel mass and the continuous production of vapors or flammable gases which the flames ignite.

As defined earlier, fire is often referred to as a rapid self-sustaining oxidation process usually accompanied by the evolution of heat and light of varying intensities.

Oxidation is an exothermic reaction which releases heat. An endothermic reaction absorbs heat. For an exothermic reaction to occur, combustible fuel and an oxidizing agent must come together. Fuel may be almost any material which has not been fully oxidized. Most materials are composed of carbon and hydrogen (hydrocarbons) which usually can be further oxidized.  Most organic materials, flammable gases, and flammable and liquid gases contain large amounts of hydrogen and carbon.

### Air:  The Most Common Oxidizer

Air contains approximately 21 percent oxygen. Other materials may provide oxygen during combustion; these are known as oxidizing agents. Some examples of these oxidizing agents are sodium nitrate and potassium chlorate.

Under certain conditions, combustion can take place without oxygen being present, such as during the burning of magnesium. Such cases are somewhat rare and are not included here.

Only gases or vapors burn during open combustion. Solids and liquids do not actually burn. Actual burning occurs in the flammable gases released by the fuel.

Flashpoint

The flashpoint is the lowest temperature at which a flammable or combustible liquid produces enough vapors to form a flammable mixture at or near the surface of the liquid. This definition is based on experiments done in a laboratory.

Before we go any further, we need to clarify what we mean by flammable since the word is sometimes confusing. The word flammable means that something ignites easily, burns intensely, or has a rapid rate of flame spread. Flammable and inflammable are synonyms. The opposite is nonflammable, which describes materials less likely to ignite, burn, or have a rapid rate of flame spread.

Knowing the flashpoint of something can be critical in making the proper origin and cause determination. Earlier, we mentioned diesel fuel. The flashpoint of diesel fuel differs from that of gasoline. Knowing this might help an officer clearly distinguish between the right and wrong cause if an accelerant was used.

As with so many of the concepts discussed here, the informed officer is the one who knows the principles, can apply them when needed, and knows where to find the specific information to support his or her inquiries. No one could possibly memorize all the supporting information such as flashpoints. This is why we have reference manuals.

Flammable and Combustible Liquids

If we extend the definition of flashpoint to liquids, we come up with two more definitions:

A flammable liquid is any liquid having a flashpoint below 100°F.

A combustible liquid is one having a flashpoint at or above 100°F.

Once again, the informed officer will be able to use this concept with its related information to sort out possible accelerants. The magic line is 100°F.

Ignition Temperature

The ignition temperature is the minimum temperature to which a substance must be heated in air in order to ignite independently of the heated or heating element (self-ignition or auto-ignition).

Often ignition temperature is confused with flashpoint, but there is no relationship between the two terms. Some fuels that have very low flashpoints may have very high ignition temperatures. Here are two examples.

Flashpoint		Ignition Temperature

Gasoline		- 45°F				536°F to 853°F

Kerosene		100°F				410°F

Careful observation will show quite a range in these two examples. The range between the flashpoint and ignition temperature of gasoline is 591 degrees to 908 degrees. The range for kerosene is only 310 degrees. The point is that in some cases knowing that this may be a factor can make a difference in your analysis of origin and cause.

Flammable or Explosive Limits

The next terms we need to look at are flammable or explosive limits. They form a range; that is, they have a lower limit and an upper limit.

These limits are the extreme concentration limits of a combustible or flammable gas in an oxidant through which a flame, once initiated, will continue to burn.

There are upper and lower percentages of an air-gas mixture in which combustion will be supported. A mixture below the lower limit is too lean to burn. A mixture above the upper limit is too rich to burn.

Here are some examples of the flammable limits of common accelerants or fuels. The figures are the percent of vapor in air.

- Gasoline = 1.4 to 7.6 percent.

- No. 1 fuel oil = .7 to 5.0 percent.

- Acetylene = 2.5 to 100 percent.

- Carbon monoxide = 12.5 to 74 percent.

- Hydrogen = 4.0 to 75 percent.

Note the very diverse ranges. For gasoline, it is only 6.2 percent. For acetylene, it is 97.5 percent.

Specific Gravity

Gravity is the weight of something based on the earth’s pull. Specific gravity is the weight of something compared to water, which is given a specific gravity of 1. The weight of the product will either cause it to float on top of water or to sink to the bottom. Gasoline has a specific gravity of .73, is lighter than water, and therefore floats. The specific gravity of asphalt is 1.4, and it will sink in water because it is heavier.

This is another basic fact which may make a difference in some situations involving a flammable liquid fire.

Vapor Density

Vapor density is the ratio of the weight of a volume of a given gas to an equal volume of dry air, air having the value of 1 (as water did for specific gravity). This is used to determine whether a gas will rise or sink in the atmosphere.  This, too, can be very important in determining origin and cause.

If a substance's vapor density is higher than 1, it will tend to sink or collect in low-lying areas. If it is lower than 1, it will tend to rise. A helium balloon tends to rise because its vapor density is lower than that of air. Some toxic gases like methane tend to hover close to the ground because their vapor density is higher.

Solubility

Solubility is the ability of a material to dissolve in water. A common example is alcohol. At a fire involving alcohol, such as at a tavern or distillery, we would expect that when the containers of alcohol break and spill their contents, the product will dissolve in the water. This could cause flammable liquid burns in unusual areas.

For certain types of fires, such as those with an alcohol base, a special type of foam must be used.

The following chart shows the flashpoint, ignition temperature, flammable range, and vapor density of some common fuels the company officer might run into in routine fires. One can find more extensive lists in most fire protection manuals or texts.

Flashpoint, Ignition Temperature, Flammable Limit, 
and Vapor Density

|  | FP(°F) | IT(°F) | FL% | VD |
| --- | --- | --- | --- | --- |
| Acetone | 0 | 869 | 2.6 - 12.8 | 2.0 |
| Acetylene | Gas | 581 | 2.5 - 100.0 | 0.9 |
| Anhydrous Ammonia | Gas | 1204 | 16 - 25 | 0.6 |
| Benzene | 12 | 1040 | 1.3 - 7.1 | 2.8 |
| Carbon Monoxide | Gas | 1128 | 12.5 - 74 | 1.0 |
| Ethyl Alcohol | 55 | 689 | 3.3 - 19 | 1.6 |
| Fuel Oil #1 | 100 | 410 | 0.7 - 5 | --- |
| Fuel Oil #6 | 150 | 765 | --- | --- |
| Gas (natural) | Gas | 900 - 1170 | 3.8/6.5 - 13/17 | --- |
| Gasoline | - 45 | 536 | 1.4 - 7.6 | 3.4 |
| Heptane | 25 | 419 | 1.05 - 6.7 | 3.5 |
| Hexane | - 7 | 437 | 1.1 - 7.5 | 3.0 |
| Hydrogen | Gas | 752 | 4.0 - 7.5 | 0.1 |
| Motor Oil | 300 - 450 | 500 - 700 | --- | --- |
| Turpentine | 95 | 488 | 0.8 | --- |
| O-Xylene | 90 | 869 | 1.0 - 6.0 | 3.7 |

Pyrolysis

Pyrolysis begins when fuel is heated. Pyrolysis is the chemical decomposition of matter through the action of heat. This process begins when fuel is heated: pyrolysis continues through specific stages. Let’s use wood as a fuel example.

Various gases slowly form during decomposition. The combustibility of these gases increases with the material's decomposition. The decomposition moves deeper into the fuel mass as the fuel surface is attacked and charring occurs.

Evolved gases ignite when they reach their lower flammable limit, assuming an ignition source is available. Ignition may be delayed if ignition energy (source) is not available.

Heat from an open flame starts the secondary pyrolysis reaction. Heat balance and heat feedback are important to continued burning. A positive heat balance occurs when more heat is produced than is lost to heat conduction, convection, or radiation and the fire continues to burn. A negative heat balance occurs when most of the heat produced is lost to conduction, convection, or radiation, and the fire normally goes out.

Feedback of heat preheats nearby fuels and allows the fire to continue and spread.

### Example

To illustrate the many physical and chemical processes involved in fires, we will use the ignition, burning, and eventual extinction of a wood slab in a typical fireplace.

Suppose the wood slab is heated initially by thermal radiation. As its surface temperature approaches the boiling point of water, gases (principally steam) slowly evolve from the wood. These initial gases have little, if any, combustible content. As the slab temperature increases above the boiling point of water, the “drying” process penetrates deeper into the wood interior.

With continued heating, the wood surface begins to discolor when the surface temperature approaches 575°F. This discoloration is visible evidence of pyrolysis, the chemical decomposition of matter through the action of heat. When wood pyrolyzes, it releases combustible gases while leaving behind a black, carbonaceous residue called “char.” This pyrolysis process penetrates “char” and continues deeper into the wood slab interior as the heating continues.

Soon after active pyrolysis begins, combustible gases typically evolve rapidly enough to support gas-phase combustion. However, combustion will occur only if there is a pilot sufficient to support a piloted ignition. If no such pilot is present, the wood surface often must be heated to a much higher temperature before autoignition occurs.

Once ignition occurs, a diffusion flame rapidly covers the pyrolyzing surface from direct contact with oxygen. Meanwhile, the flame heats the fuel surface and causes an increase in the rate of pyrolysis. If the original radiant heat source is withdrawn at ignition, the burning will continue, provided the wood slab is thin enough (less than 3/4 inch).  Otherwise, the flames will go out because the slab surface loses too much heat by thermal radiation and thermal conduction into its interior. If there is an adjacent parallel wood surface (or insulating material) facing the ignited slab, it can recapture radiation loss, so that the ignited slab can continue burning even after the withdrawal of the initial heat source. This explains why one cannot burn a single large log in a fireplace, but must use several logs to recapture the radiant heat losses.

As the burning continues, a char layer builds up. This char layer, which is a good thermal insulator, restricts the flow of heat to the wood interior, and consequently tends to reduce the rate of pyrolysis. The pyrolysis rate also will decrease when the supply of unpyrolyzed wood runs out. When the pyrolysis rate decreases to the point of not being able to sustain gas-phase combustion, oxygen from the air will come into direct contact with the char, permitting it to undergo direct glowing combustion, provided the radiant heat losses are not too large.

The above discussion presumes there is an ample (but not excessive) supply of air (oxidant) for combustion. If there were insufficient oxidant to burn the available fuel vapor, the excess vapors would travel with the flow and possibly burn where they eventually find sufficient oxidant. For example, this happens when fuel vapors emerge and burn outside a window of a fully involved but underventilated room fire.

If, on the other hand, one was to impose a forced draft onto the pyrolyzing surface, the oxidant supply may exceed that required for complete combustion of the fuel vapors. In this case, the excess oxidant can cool the flames sufficiently to suppress their chemical reaction and extinguish them. This happens, for example, when one blows out a match or candle. In the case of larger fires with ample supply of fuel vapors, blowing on them simply increases their rate of burning by increasing the flame to fuel-surface heat transfer, which in turn enhances the fuel supply rate.

Following the ignition of a certain portion of our wood slab, the flames are likely to spread over the entire fuel. Flame spread can be thought of as a continuous succession of piloted ignitions where the flames themselves provide the heat source. One commonly observes that upward flame spread is much more rapid than downward or horizontal flame spread. This is because hot flames generally travel upward and distribute their heat over a greater area in an upward direction. Thus, each successive “upward ignition” adds a much greater burning area to the fire than a corresponding “downward or horizontal ignition.”

Generally, materials which ignite easily or rapidly also propagate flames rapidly. The ignitability of a material is controlled by its resistance to heating (thermal inertia) and the temperature rise required for it to begin to pyrolyze. Materials with low thermal inertia, such as foamed plastics or balsa wood, heat up rapidly when subjected to a given heat flux. These materials are often easy to ignite and can cause very rapid flame spread. On the other hand, dense materials such as ebony wood tend to have relatively high thermal inertia and are difficult to ignite.

The burning rates of larger, more hazardous fires are principally governed by the radiant heat transfer from the flames to the pyrolyzing fuel surface. The flame radiation comes primarily from the luminous soot particles in the flames. Combustibles which tend to produce large amounts of soot or smoke (such as polystyrene) also tend to support more intense fires, despite the fact that their fuel vapors burn less completely, as evidenced by their higher smoke production.

Activity 7.2:

Predicting Fire Behavior

Student Activity Worksheet

Time: 30 minutes Purpose: To apply the principles of fire behavior to a simulated emergency response. From the scenario, you will be able to make predictions about fire behavior. Directions: - Your instructor will divide the class into small groups. Appoint a spokesperson to report your group’s findings at the end of the activity. - Read the scenario and answer the questions. - After 15 minutes, you will report your answers.

Scenario: The temperature is 62°F on a clear day. There is no measurable wind and the time is 1830 hours. A tank truck carrying 8,000 gallons of gasoline was coming down a hill into town when the driver lost control of the vehicle. The truck overturned on Main Street and spilled part of its contents. The contents ran down Main Street and into the basement of the post office. The post office had a gas-fueled hot water heater in the basement. Soon afterward, there was an explosion, and three buildings, along with the truck, were involved in fire. As you approach the scene, you observe a large cloud of heavy black smoke and dark red flames. The exterior walls of two dwellings and the entire one-story post office are involved. The post office walls are partially blown from their foundation. During extinguishment, you observe that the burning gasoline is floating down the street on top of the water. You also are informed that gasoline vapors in the storm sewer are above 40 percent but have not ignited. No one was injured during the incident.

Questions: - Is the gasoline at or above its flashpoint? - Is the gasoline at or above its ignition temperature? - Is the gasoline vapor in the storm sewer within its flammable limits? - Why did the gasoline float on top of the water? - Why did the gasoline vapors collect in the storm sewer down the street in an area lower than the leaking tank truck? - What could have been the ignition source for the spilled gasoline? - What evidence may indicate that the fire may have started in the basement of the post office? - If the tank had been loaded with diesel fuel, would you have expected the same results?

Rate of Heat Release

Heat is measured in British thermal units (Btus). One Btu is the amount of heat required to raise one pound of water one degree Fahrenheit at 60 degrees. The total number of Btus which might evolve from a fire becomes important in relation to fire load and the amount of damage in the structure.

Once again, it is important to underscore that the knowledge of this technical point is not what matters; what counts is the officer's ability to use this knowledge with all of the other bits of information at the scene.

Having some sense of the number of Btus produced by various combustibles may be important information when related to the damage inflicted on a structure.

The following chart shows the number of Btus produced by the combustion of various materials.

| Product | Btu/lb |
| --- | --- |
| Wood | 7,000 |
| Polyurethane | 14,700 |
| Polyvinyl chloride | 17,900 |
| Polystyrene | 18,750 |
| Polypropylene | 20,000 |
| Polyethylene | 20,100 |

With the discussion of Btus, we come to the end of this first section dealing with some basic principles of fire science. To repeat, the purpose is not to make the officer and firefighter fire experts. The goal is to make the initial responders more aware of some of the principles that underlie the origin and spread of fire. This knowledge will, in turn, help the responders make more accurate determinations as to origin and cause.

Reaction of Fire to Water Application

This is an important aspect of fire suppression and a critical observation the fire officer can make in terms of fire cause determination. Normal items in today’s environment will produce a significant amount of heat. The application of the appropriate amounts of water should cause the fire to diminish and go out. If the fire does not react as one has seen at previous, similar fires, naturally one should be curious as to the reasons.

A common reason might be the use of an accelerant. Another could be the combustion of a substance that burns hot and fiercely when water is applied. Whatever the case, this fact could be another clue that will help figure out the correct answer.

Heat Transfer and Temperature Factors

The properties of heat are such that its route of travel and method of transfer can reveal interesting information about the origin and cause. Therefore, it is a critical evaluation while making a fire cause determination.

As the fire travels from room to room or from one portion of a building to another, the methods of travel should be fairly obvious. This observation is critical to determine if the fire was spread intentionally with devices such as accelerants, trailers, or separate fires.

There are three methods of heat transfer.

Conduction is the transfer of heat by direct contact of one body with another. There are many examples of heat transferred and the spread of fire from involved to uninvolved areas by heating some structural element. Some common examples include:

- Pipes and other conduits;

- Steel structural members; or

- Walls, doors, ceilings, or floors.

Convection is the transfer of heat by some circulating medium, either gas or liquid. This form of heat transfer is the one most responsible for fire spread in structures.

An example of heat transfer by convection is the spreading of fire from lower to upper structural areas when the upper areas become heated to their ignition temperatures.

Radiation is the transfer of heat from one area to another without direct contact, and without any circulating hot gases to help “bathe” the area in heat. It is heat in the form of energy that travels through space or materials as waves, as when the sun warms the earth. An example of heat transfer by radiation is an exposure fire.

Structural fires usually involve all three types of heat transfer or combinations of all three. Being able to differentiate one from the other may be critical for the officer in working a cause and origin determination.

Observing fire spread may be vital to reconstructing where the fire started and how it traveled to another location. Certain fire travel predictions also can be made for some classifications of buildings.

Consider the following example. A fire starts in the basement of a wood-frame balloon construction building. Shortly after, the fire appears in the attic. This may be a result of convected heat traveling up through open wall cavities to the attic level and igniting combustible materials.

Many fire officers may quickly and erroneously assume that the second fire was the work of an arsonist.

Flashover and Backdraft

There are two very important and related topics that we need to look at now. A clear understanding of each and the ability to recognize their signs play an important part in accurate origin and cause determination.

Flashover

Flashover can be defined as the state of a fire when a room or other area becomes heated to the point when flames flash over the entire surface of the room or area simultaneously.

Flashover is caused when heat produced by the fire collects at the ceiling level and returns to lower areas by thermal radiation. Combustible materials are heated to their ignition temperatures and fire flashes over large areas.

A more recent term used to describe the rapid spread of flame over one or more surfaces is “flameover.” This should not be confused with flashover.

Flashover may cause the area to appear to have been exposed to flammable accelerants. However, during the fire scene examination, discovery of the following would indicate flashover rather than the use of an accelerant:

- Burning over top surfaces of materials,

- Lack of normal fire spread from point of origin,

- Lack of accelerant residue, and

- Demarcation line around the area.

The other related term is backdraft.

Backdraft

Backdraft is the explosion or rapid burning of heated gases that results when air is admitted to a building heavily filled with smoke from a fire that has depleted the oxygen content of the building.

What causes a backdraft? Carbon monoxide is one of the major components of most structural fires, and it is highly flammable (explosive limits = 12.5 to 74 percent). The ignition temperature of carbon monoxide (1,128°F) is well below the normal temperatures found at ceiling levels of most structural fires.  The carbon monoxide usually collects at the upper areas of a fire- involved structure, and the introduction of air (oxygen) from below frequently results in an air-to-carbon monoxide mixture within the flammable range.

Backdraft explosions often cause injuries to firefighters. Knowing this fact will play an important role in fire scene investigation. Interviews with the fire personnel who were present at the scene are critical. Information that should be obtained includes the following items.

- What was the extent of smoke at the time of arrival?

- Was there a lack of visible flame or presence of blue flames?

- What method was used to open or enter the structure?

- Was there venting of the roof before entry to help eliminate smoke?

- Did entry before venting provide oxygen and possibly produce a backdraft?

- Was there movement of smoke immediately preceding the explosion?

- Smoke may appear to change its direction of travel.

- The structure may appear to breathe.

- There may be sounds like a whistle, a jet, or a train before the explosion.

Backdrafts also can occur during the later stages of the fire if pockets of smoke remain after venting.

Classes of Fire

As part of a fire scene examination, one should note the methods of extinguishment. A question to ponder is whether the proper extinguishment was used for the type of fire that occurred. There are four classes of fire with which every officer should be familiar.

Class “A” fires are those fires involving ordinary combustible materials or materials which produce an ash or glowing coals. Some common examples are wood, paper, cloth, and rubber.

Class “B” fires are those fires involving flammable or combustible liquids. Some common examples are gasoline, kerosene, fuel oil, and alcohol.

Class “C” fires are those fires involving energized electrical equipment.

Class “D” fires are those fires involving certain combustible metals. Some common examples are magnesium, titanium, sodium, and potassium.

Normal Fire Behavior

Fires usually will go through at least two stages. If the building is tightly closed and remains closed, it may go through a third stage and eventually self-extinguish. Knowing the stages and being able to distinguish each is another important skill for the officer.

### The Incipient Stage

This is the first or beginning stage. All fires start with a single point of ignition and expand from that point. A class B fire (flammable liquid) expands rapidly, while a class A fire expands slowly. As a class A fire starts, the fire gases and heat start to rise upward and outward. This often will be referred to as the “V” pattern with the bottom of the “V” pointing to the area of fire origin. The hotter and the faster the fire, the narrower the “V” pattern.

Class A fires can accelerate with flammable liquids and also can have their normal fire patterns altered. As the fire burns, it also produces a wide variety of fire gases.

Carbon monoxide (CO) is not the most toxic fire gas, but it is the most abundant. As the oxygen level goes down, the production of CO goes up. Carbon dioxide (CO2) also is produced in every fire.

Two other commonly produced gases are hydrogen sulfide (H2S), made when organic materials containing sulfur burn (such as hair and wool), and benzene (C6H6), produced when vinyl materials burn.

### Free-Burning Stage

This is the second stage. It follows flashover of the area or building, which now appears to be fully involved. In accidental fires this stage typically will occur in about 3 to 5 minutes from the time of ignition.

The type of fuel or arrangement of the fuel may affect the time it takes the fire to develop fully. A fire accelerated with a flammable liquid or other methods, such as a trailer device of combustible materials arranged to quickly cause fire spread, may alter this normal fire growth and the normal three to five minute growth pattern.

### Smoldering Stage

When a fire has passed through stages one (incipient) and two (free-burning), it will likely smolder if the area or building is closed.

For this to happen, the oxygen level must fall to 16 percent or lower. The free-burning phase is reduced, and the fire may retreat to its area of origin (most heavily burned part) and continue to smolder for a long while.

Various things happen during the third state of burning:

- Carbon monoxide production rises.

- The temperature of the area or building rises.

- Large amounts of heavy black carbon are deposited on the walls and windows.

- Moisture may appear on the inside of the window glass as the outside air cools the glass.

- The building may appear to be puffing and breathing.

- Smoke may appear around windows, through the siding, or through the roofing materials.

From this one can infer that the level of the structure where the smoke stops puffing is probably where the seat of the fire is. If there is puffing smoke on the second floor but none on the first, the area of fire origin may well be located on the second floor. This is not an absolute, but is a good rule of thumb.

If the area or building remains tightly closed, eventually the fire will suffocate from lack of oxygen. In some instances, fire departments have responded to an apparently significant fire which has gone out by itself after causing considerable heat and smoke damage.

As buildings are tightly constructed and insulated, the potential for stage three fires increases.

Combustion Byproducts

Four categories of combustion byproducts are produced as the result of combustion.  They are gases, heat, flames, and smoke. We need to discuss each of these and how they relate to fire behavior.

Fire gases produced in most fires depend on variables including the chemical makeup of the burning materials, the available oxygen during burning, and/or the temperature of the fire and the fire area.

The toxicity of these gases is determined by variables such as the concentration (percent) of gas in air, the length of exposure, and the physical condition of any victims.

The toxic effects on personnel are greater during the fire because of increased respiration by the victim from exertion, heat, and excess carbon dioxide. What ordinarily appear to be harmless amounts of toxicity may become dangerous during the fire.

Here are some examples of common toxic fire gases.

Carbon monoxide (CO). This is considered the most dangerous in most fires. CO is not the most toxic, but it is the most abundant. The amount being produced can increase as available oxygen decreases.

Carbon dioxide (CO2). This gas is usually produced in large amounts in most fires.

Hydrogen sulfide (H2S). This gas is often produced by the incomplete combustion of organic materials containing sulfur. Some examples of materials which may release hydrogen sulfide are wool, rubber, hides, meat, and hair.

Ammonia (NH3) is often produced by the burning of combustible materials containing nitrogen. Examples of materials which may release ammonia are wool, silk, acrylic plastic, and melamine resins.

Hydrogen chloride (HCl) is often produced by the burning of combustible plastic containing chlorine. Polyvinylchloride (PVC) is the most notable example of materials that release hydrogen chloride.

Nitrogen dioxide (NO2) is the gas often produced by the decomposition and combustion of cellulose nitrate in fires involving ammonium nitrate and other inorganic nitrates.

When in a combustion stage, this gas can often be identified by a reddish-brown smoke or vapor. The toxic effect may be delayed for up to several hours.

Acrolein (CH2:CHCHO) is often produced by the combustion of petroleum products.

Other fire gases which are usually found only in small quantities in most fires are hydrogen dioxide, hydrogen cyanide, and phosgene.

Flame, Heat, and Smoke

Flame results from the burning of most materials in an oxygen-rich atmosphere. This produces luminosity (flame). Therefore, flame is considered a byproduct of combustion.

Heat is the byproduct which is most responsible for the spread of fire in buildings. The presence of moisture in heated air increases the danger and the damage.

Smoke consists of very fine solid particles and condensed vapor. Gases that are produced by the heating of combustible materials are contained in flammable tar droplets and carried upward with the thermal column. Very little smoke is produced during complete combustion. As combustion diminishes, the smoke density increases.

Oxygen and Fire

Air contains 21 percent oxygen. Without an air supply, insufficient oxygen often results during combustion. Flaming combustion usually ceases when the available oxygen is less than 16 percent.

Activity 7.3:

Stages of Burning

Student Activity Worksheet

Time: 45 minutes Purpose: To provide the opportunity to determine various stages of fire progression. The ability of a fire officer to read a fire accurately is not only critical to the fire-suppression efforts, but to the fire cause determination and fire origin efforts as well. Directions: - Your instructor will divide the class into small groups. Your group should select a spokesperson to report your findings to the class. - Your group will read the three fire scenarios and determine the stage of fire progression. - You will have 10 minutes to discuss each scenario and determine the stage of fire progression. - Your instructor will ask you to provide answers and describe what factors led to your decision.

Scenario 1 A fire has occurred in a two-story, wood-frame, single-family dwelling of balloon-type construction. The area of fire origin appears to be around the bed of the master bedroom. There is heavy fire damage in the bedroom and especially around the bed. The family states that it had left to go shopping approximately 5 minutes before the fire. The bedroom door where the fire started was open, as were the doors of the three other bedrooms on the second floor. All of the bedroom walls had been finished with wood paneling. The fire had spread from the room of origin to all the other second-floor bedrooms before fire department arrival. Upon your arrival, flames are coming out of several windows and the fire has burned through the roof over the master bedroom. Questions: - At what stage was the fire upon your arrival? - Are there any unusual circumstances that would require further investigation?

Scenario 2 The building is a one-story, wood-frame, platform-type construction. It was built in 1993 as a single-family unit. The fire started in a pile of newspapers and spread to an overstuffed easy chair placed against the living room wall. From there a very distinct “V” pattern is recognized in the char pattern on the wall. In the fire debris next to the chair are the broken remains of a glass ashtray. The occupant stated that he had not smoked for at least 12 hours and was very careful about extinguishing his cigarettes. He was at work and had left the dwelling about 3 hours earlier. A neighbor had called the fire department to report something unusual next door. The windows appeared to be dark and she thought she smelled smoke in the area. Upon your arrival, you observe darkened window glass that is hot to the touch, a light puffing around the eaves, and the strong odor of smoke. Questions: - At what stage was the fire upon your arrival? - Are there any unusual circumstances that would require further investigation?

Scenario 3 Your dispatcher has sent you to investigate a report of three boys playing in a garage. They were seen lighting small pieces of paper and then stepping on them to put them out. As you approach the address, you observe three boys running from the garage and into a neighboring home. From the driveway, you observe a small amount of gray smoke coming from the partially opened side door. Upon entering the garage, you see a fire that involves the workbench area and the wall behind the bench. There is no electrical equipment on the bench area or other sources of ignition nearby. Questions: - At what stage was the fire upon your arrival? - Are there any unusual circumstances that would require further investigation?

Module Summary

Careful and methodical observations on the part of the company officer and firefighters probably will play a big role in helping to make a determination of origin and cause. The observations made at the time of the alarm and during response, size-up, suppression, and overhaul will contribute a vast amount of useful information. Later, this information will be sorted out and evaluated to see which parts can help determine origin and cause.

This early part of the process cannot be underestimated and cannot be done too carefully. In many ways, it can make or break the ultimate investigation.

The time spent on collecting, recording, and analyzing this information is well spent. It can make all the difference if there is litigation later. Even if there is no litigation, there will be satisfaction from the knowledge that you were able to make a final determination on origin and cause.

As mentioned a number of times during this module, the officer does not need to memorize a lot of information to function effectively at the fire scene. What is required is a good grasp of some of the basic fire principles and how these affect an accurate origin and cause determination.

This module has presented the major fire principles to help the officer avoid making some erroneous conclusions by misreading the facts.