Operations

Module 13: Enclosed Mall Simulation Exercise

Answer Key

- Mall fires require long attack line stretches. How long?

- 200 to 500 feet

- 200 to 400 feet

- 300 to 500 feet (SM 13-7)

- 300 to 600 feet

- The best type of preplanning for malls is participation in the planning and construction phases of the project. Ensure that adequate water supply, fire department access, and 					 have been considered.

- Security

- Ventilation

- Fire load

- Life safety (SM 13-4 to 13-5)

- Most major building codes require building components to be protected. Protection can range from 					.

- 2 to 4 hours

- 1 to 3 hours (SM 13-3)

- 1 to 5 hours

- 2 to 5 hours

- may allow for rapid, undetected fire spread and require effective search for the fire.

- Breached fire walls

- Maze-like storage rooms

- High-rise structures opening directly into the mall

- Open ceilings (SM 13-4)

- Place the letter of the term on the right in the blank in front of the phrase on the left that describes it.

A (SM 13-7)	False fronts						A.	Roof top dangers

B (SM 13-5)	Haz mat							B.	Fire department can control

C (SM 13-4)	Class A combustibles			C.	Fire load