Operations

Module 15: High-Rise Simulation Exercise

Test

Name:														Date:

Directions: Circle the letter of the correct answer in the following questions.

- Roofs in high-rise buildings are required to have 					 fire-resistive rating.

- At least a 2-hour

- A 1- to 3-hour

- A 2- to 4-hour

- At least a 3-hour

- Which of the following statements is false?

- In older buildings, the emergency power may supply only exit lighting in the stair shafts.

- An elevator equipped with firefighter service is a safe elevator.

- Safety procedures in high-rise fires include using split-bank elevators that terminate at least five floors below the lowest reported fire floor.

- A good rule-of-thumb is to give the Incident Commander an update every two to three floors during ascent.

- Place the letter of the fire behavior on the right in the blank in front of its description on the left.

When the fire generates enough heat to break out 	A. Core-construction effect

windows and roll out the fire floor and 				B. Negative stack effect

up the outside of the building							C. Auto extension

When the fire circles behind firefighters,

cutting off their escape route

When the smoke is pushed down vertical shafts

or settles on floors below the fire

- Normally, staging is located 					 floors below the fire floor to minimize the time-distance factor.

- Two

- Three

- Four

- Five

- Place the letter of the style of high-rise construction on the right in the blank in front of its description on the left.

Upper-level walls tapered										A.	Core

May have scissor stair shafts									B.	New style

Stair shafts commonly placed in center of steel skeleton	C.	Old style