unit 4:

Building Construction types

objectives

The students will:

1.	List ways to collect data and manage information.

2.	List and describe the five types of building construction.

3.	Identify the strengths, weaknesses, characteristics, and collapse potential for each of the five methods of building construction.

4.	Identify special safety concerns.

5.	Given a scenario, identify the strengths and weaknesses in different building construction types.

methodology

This unit uses lecture and discussion.

(Total Time:  2 hr., 25 min.)

145 min.	Lecture/Discussion

Objectives	IG 4-3

Management of Information	IG 4-3

Type I--Fire-Resistive Construction	IG 4-5

Type II--Noncombustible/Limited Combustible 
Construction	IG 4-9

Type III--Ordinary Construction	IG 4-13

Type IV--Heavy-Timber/Mill Construction	IG 4-18

Type V--Wood-Frame Construction	IG 4-20

What are the Firefighting Safety Concerns?	IG 4-24

What is the Brief Initial Report?	IG 4-29

Summary	IG 4-31

audiovisual

# Slides 4-1 to 4-115

| Slide 4-1 |  |  |
| --- | --- | --- |
| 145 min. |  |  |
| Lecture/Discussion |  |  |
| Slide 4-2 |  |  |
|  |  | i. objectives (5 min.) |
|  |  | The students will: |
|  |  | A. List ways to collect data and manage information. |
|  |  | B. List and describe the five types of building construction. |
|  |  | C. Identify the strengths, weaknesses, characteristics, and collapse potential for each of the five methods of building construction. |
|  |  | D. Identify special safety concerns. |
|  |  | E. Given a scenario, identify the strengths and weaknesses in different building construction types. |
| Slide 4-3 |  |  |
|  |  | ii. Management of information (5 min.) |
|  |  | A. Struggle for safety. |
|  |  | 1. Improvements in safety hinges on the management of information. |
|  |  | 2. To increase the ability to survive, all responders must know their jurisdictions. |
|  |  | 3. Various construction methods and materials will create extreme risk. |
|  |  | 4. Each day must be a training day. |
|  |  | 5. There will always be changes in the materials used. |
| Slide 4-4 |  |  |
|  |  | B. Code enforcement. |
|  |  | Need training to spot building and fire code violations. |
|  |  | C. Preincident planning data. |
|  |  | 1. Collection injury and death reports show 100 line-of-duty deaths (LODDs) each year. |
|  |  | 2. Fireground changes are necessary. |
|  |  | D. Sizeup. |
|  |  | 1. Improves ability to make correct decisions at an actual emergency. |
|  |  | 2. Must be able to retrieve information in a timely manner. |
|  |  | 3. Must allot time to gathering information. |
| Slide 4-5 |  |  |
|  |  | The National Fire Protection Association (NFPA) 220, Standard on Types of Building Construction--explains about the NFPA standard, revision policy, and consensus of committee process used by the fire service. Others agencies may only look at combustible and noncombustible features. |
| Slide 4-6 |  |  |
|  |  | E. Identification of construction classes. |
|  |  | 1. A thorough understanding of the strengths and characteristics of various construction types is critical to effective and safe operations. |
|  |  | 2. As noted by Francis Brannigan, responders must understand that a building code is "intended." |
| Slide 4-7 |  |  |
|  |  | iii. Type i--fire-resistive construction (20 min.) |
|  |  | A. Characteristics. |
|  |  | 1. Structural steel, sprayed or encased. |
|  |  | 2. All vertical openings protected with approved devices. |
|  |  | 3. Exterior walls are normally nonsupporting (e.g., precast, glass, foam, concrete, and aluminum). |
|  |  | 4. Exterior/Interior nonbearing walls will vary in fire resistance. |
|  |  | 5. Stairways are enclosed in fire-resistive materials. |
|  |  | 6. Floors may be poured-in-place or prefabricated concrete. |
|  |  | 7. Roof construction is similar to floors. |
|  |  | 8. "Center core" construction allows perimeter to be used for revenue producing areas. Large open areas often contain heavy fire loading. Utilities are in a common shaft. |
|  |  | 9. "End core" buildings provide stairs at the end of each floor and a center corridor. |
|  |  | B. Strengths. |
|  |  | 1. Stairways and corridors are enclosed in fire-rated materials. |
|  |  | 2. Structural members are not immediately exposed to direct flames. |
|  |  | 3. Generally have built-in fire protection systems. |
|  |  | 4. Components may vary depending on age of building and codes in force at the time of construction. |
|  |  | 5. Compartmentalization provided by fire-resistive walls and floors. |
|  |  | C. Firefighting/Safety concerns. |
|  |  | 1. Central air-conditioning penetrates all parts of the building creating a potential for fire and smoke spread. Must be aware of poke-through construction that violates fire protective barriers. |
|  |  | 2. Fire has a tendency to "lap" from floor to floor. |
|  |  | 3. Ventilation is a significant problem. |
| Slide 4-8 |  |  |
|  |  | D. Fire-resistive requirements. |
|  |  | 1. Exterior bearing walls--3 to 4 hours. |
|  |  | 2. Interior bearing walls--2 to 4 hours. |
|  |  | 3. Columns, beams, girders, trusses, arches--2 to 4 hours. |
|  |  | 4. Floors--2 to 3 hours. |
|  |  | 5. Roof--1-1/2 to 2 hours. |
|  |  | 6. Exterior nonbearing walls--0 hours. |
| Slide 4-9 |  |  |
|  |  | Show Slide 4-9. Signs on a roof may be an example of a load that was not designed into original building. Explain the differences between dead load and live load. |

| Slide 4-10 |  |  |
| --- | --- | --- |
|  |  | Show Slide 4-10. |
| Slide 4-11 |  |  |
|  |  | Show Slide 4-11. |
| Slide 4-12 |  |  |
|  |  | Show Slide 4-12. |
| Slide 4-13 |  |  |
|  |  | Show Slide 4-13. |
| Slide 4-14 |  |  |
|  |  | E. MGM Grand. |
|  |  | 1. Design of steel can vary from midrise to highrise. |
|  |  | 2. Large loss of life can occur remotely from the fire when heating, ventilating, and air conditioning (HVAC), water supply, and electrical are not maintained. |

|  |  | Case study: smoke spread to upper floors in Las Vegas via the HVAC system. |
| --- | --- | --- |
| Slide 4-15 |  |  |
|  |  | F. Spray on fire protection. |
|  |  | 1. On steel truss and ceiling to provide 2 to 4 hours of protection under normal fire conditions. |
| Slide 4-16 |  |  |
|  |  | 2. On steel joist/ceiling. |
|  |  | 3. In fire-resistive materials. |
| Slide 4-17 |  |  |
| Slide 4-18 |  |  |
|  |  | 4. Gap will allow fire and smoke to spread to upper floors. This can also be called a safing gap. |

| Slide 4-19 |  |  |
| --- | --- | --- |
|  |  | 5. Nonload bearing wall. |
| Slide 4-20 |  |  |
|  |  | iv. Type II--Noncombustible/Limited Combustible construction (20 min.) |
|  |  | Type II construction will have structural elements with little or no protection from the effects of fire. In the event of a fire, these unprotected steel structural members may fail and collapse with little warning. |
| Slide 4-21 |  |  |
|  |  | A. Characteristics. |
|  |  | 1. Structural steel may be exposed to the effects of fire. |
|  |  | 2. Noncombustible refers to structural members, not content or interior finish. |
|  |  | 3. Structural framework is made of steel that is bolted, riveted, or welded. |
|  |  | 4. Susceptible to expansion or contraction of steel members from heat resulting in early distortion. |
|  |  | 5. Wall enclosures may be masonry, steel, glass, aluminum, or other materials. |
|  |  | 6. Floor and roof support systems often will be lightweight bar joists or trusses. |
|  |  | B. Strengths. |
|  |  | 1. Structural members will not add to the fire load. |
|  |  | 2. Easily recognizable characteristics. |
|  |  | 3. Means of egress may be enclosed in fire-resistive materials. |
|  |  | 4. Often built with fire protection systems in place. |
|  |  | C. Firefighting/Safety concerns--NFPA ratings. |
|  |  | ASK: What are NFPA ratings? |
|  |  | Suggested response: NFPA ratings are based on test procedures. |
| Slide 4-22 |  |  |
|  |  | 1. Exterior/Interior bearing walls--0 to 2 hours. |
|  |  | 2. Columns, beams, girders, trusses, arches-- 0 to 2 hours. |
|  |  | 3. Floors--0 to 2 hours. |
|  |  | 4. Roof--0 to 1 hour. |
| Slide 4-23 |  |  |
|  |  | D. Design. |
|  |  | 1. Single story--flat roof. |
|  |  | Large space above top of window and door could be an indicator of a truss roof assembly. |
| Slide 4-24 |  |  |
|  |  | 2. Multistory--bow string truss. |
|  |  | a. Indicator of a truss roof assembly and flat roof case study. |
|  |  | - Truss roof assemblies have killed firefighters in many States. |
|  |  | b. In 1988, after the Hackensack Ford Fire, the State of New Jersey created a new building code mandating that all new construction will have exterior signage indicating a truss floor or roof. |
| Slide 4-25 |  |  |
|  |  | 3. Multistory--steel framework. |
|  |  | Different shapes and configuration. |
| Slide 4-26 |  |  |
|  |  | 4. Metal retail structures. |
|  |  | a. Generally high fire load. |
|  |  | b. Could result in early failure of structure. |
| Slide 4-27 |  |  |
|  |  | 5. Metal industrial structures. |
|  |  | Building with skylights. |
| Slide 4-28 |  |  |
|  |  | 6. Metal peaked roof. |
|  |  | Used as office space. |

| Slide 4-29 |  |  |
| --- | --- | --- |
|  |  | 7. Concrete block exterior walls--typical in noncombustible building. |
| Slide 4-30 |  |  |
|  |  | 8. Tilt-up construction--failure in pancake collapse. |
| Slide 4-31 |  |  |
|  |  | 9. Cinder block--metal frame--used for support. |
| Slide 4-32 |  |  |
|  |  | 10. Steel with columns--helps carry the load. |
| Slide 4-33 |  |  |
|  |  | 11. Unprotected steel frame with trusses--support the metal roof deck. |
| Slide 4-34 |  |  |
|  |  | 12. Unprotected steel--will twist when heated. |
| Slide 4-35 |  |  |
|  |  | 13. Steel loses strength at 1,100 °F (593 °C). |
|  |  | a. Steel cannot support own weight at 1,500 °F (816 °C). |
|  |  | b. Steel elements can transmit heat to uninvolved portions of a building. |
| Slide 4-36 |  |  |
|  |  | 14. Stairs fastened to unprotected structural elements. |
| Slide 4-37 |  |  |
|  |  | v. Type III--Ordinary Construction (20 min.) |
|  |  | These building types provide many challenges to firefighters, such as void spaces and common walls allowing rapid fire extension and little structural protection. Fire in concealed spaces can travel in many directions. |
| Slide 4-38 |  |  |
|  |  | A. Characteristics. |
|  |  | 1. Exterior walls are masonry. |
|  |  | 2. Structural members and interior construction are made of wood. This includes floor and roof supports. |

|  |  | 3. Floor and roof decking is usually a wood tongue-and-groove composition. Many new lightweight and durable materials can be found. |
| --- | --- | --- |
|  |  | 4. Height rarely exceeds six stories and is normally two or three stories. |
|  |  | 5. Common walls may share wall sockets for floor joists and roof rafters. |
|  |  | 6. These buildings often have parapet walls, signs, cornices, and marquees. |
|  |  | B. Strengths. |
|  |  | 1. Full cut lumber used in the structural members. |
|  |  | 2. Masonry walls. |
|  |  | C. Firefighting/Safety concerns. |
|  |  | 1. Collapse is a real possibility under prolonged heating. |
|  |  | 2. Concealed spaces that can contribute to smoke and fire spread. These open spaces or voids can be accessible as a common attic or inaccessible as in a cockloft. |
|  |  | 3. These buildings often have undergone renovations, creating voids, dropped ceilings, and shafts that allow fire/smoke spread. |
|  |  | 4. Walls may be tied to floors with metal bolts or wall plates; it is uncommon to use both. The observation of stars is a common indicator of a reinforced wall. |
| Slide 4-39 |  |  |
|  |  | D. Design. |
|  |  | 1. Ordinary construction--Main Street USA. |
|  |  | a. Can be found in most communities. |
|  |  | b. Also called Main Street USA construction. |
| Slide 4-40 |  |  |
|  |  | 2. Masonry exterior walls--Exterior walls can be brick or stone and do not add to the fire load. |
| Slide 4-41 |  |  |
|  |  | 3. Contains concealed spaces. |
|  |  | a. Firefighter safety depends on the right equipment. |
|  |  | b. The thermal imaging camera (TIC) is a required piece of equipment to uncover fire in concealed spaces. |
| Slide 4-42 |  |  |
|  |  | 4. Commonly found in strip mall construction. Malls are one of the most common uses of ordinary construction with masonry and lightweight steel or wood trusses. |
| Slide 4-43 |  |  |
|  |  | 5. Wood lath over plaster--can create a channel for fire spread. |
| Slide 4-44 |  |  |
|  |  | 6. Wooden flooring. |
|  |  | a. Floors can burn through when exposed to fire. |
|  |  | b. Lightweight components have resulted in many firefighter injures and deaths. |

| Slide 4-45 |  |  |
| --- | --- | --- |
|  |  | 7. Wooden joists and rafters--can be full dimension wood and still fail. |
| Slide 4-46 |  |  |
|  |  | 8. Roof covering adds to fuel load. Asphalt and other materials can add to fuel load. |
| Slide 4-47 |  |  |
|  |  | 9. Ridge pole used as main roof support. Often an attic can be found in the space under a peaked roof. |
| Slide 4-48 |  |  |
|  |  | 10. Fire cut--designed to permit a floor to fall downward and reduces the risk of pushing out a bearing wall. |
| Slide 4-49 |  |  |
|  |  | 11. Toe-nailed joints--In older construction, nails were used as connectors. |
| Slide 4-50 |  |  |
|  |  | 12. Gang-nailer and cleat--In new construction, lightweight materials are used as connectors. |
| Slide 4-51 |  |  |
|  |  | 13. Wooden cornice. |
|  |  | a. Cornice overhangs can burn, leading to the spread of fire and collapse. |
|  |  | b. A safe corridor for firefighters to operate is necessary. |
| Slide 4-52 |  |  |
|  |  | 14. Large window renovations. |
|  |  | a. Renovations can be used to update the appearance of an older building. |
|  |  | b. Cornice is masonry. |
| Slide 4-53 |  |  |
|  |  | 15. Marquee overhang--can collect a lot of water and may fail due to excessive weight. |
| Slide 4-54 |  |  |
|  |  | 16. Do not trust the truss. Experience has shown that truss assemblies can quickly fail. |

| Slide 4-55 |  |  |
| --- | --- | --- |
|  |  | 17. Heavy-timber/Mill construction. |
|  |  | a. Heavy timber is an ideal design. |
|  |  | b. The heavy timber will resist collapse from fire and there are few concealed spaces. |
| Slide 4-56 |  |  |
|  |  | vi. Type IV--Heavy-Timber/mill Construction (15 min.) |
|  |  | A. Characteristics. |
|  |  | 1. Common in older factories and mills. |
|  |  | 2. Masonry exterior walls have large dimension lumber as structural supports. |
|  |  | 3. Minimum size lumber, usually 8 inches, is used in columns, beams, girders, and arches. |
|  |  | 4. Minimum dimensions of all exposed wood is 2 inches. |
|  |  | 5. Wood floors have a minimum thickness of 3 inches. |
|  |  | 6. Roof supports have minimum dimensions of 4 by 6 inches with decking of 1-1/8-inch thickness. |
|  |  | B. Strengths. |
|  |  | 1. Support members have a large mass and can sustain fire for a long time before being destroyed. |
|  |  | 2. Heavy timber roof and floor elements should support the building during an offensive attack. |
|  |  | 3. Floors usually will have scupper to help with water runoff. |
|  |  | 4. Often will have a built-in sprinkler system, especially in those buildings recently renovated. |
|  |  | C. Firefighting/Safety concerns. |
|  |  | 1. Often contains an unprotected opening for freight elevators or utility shafts. Potential for rapid fire development. |
|  |  | 2. Large open area. |
|  |  | 3. Floors may be soaked from years of supporting machinery. |
|  |  | 4. Often has a heavy fire load of stock waiting to be shipped. |
| Slide 4-57 |  |  |
|  |  | D. Designs. |
|  |  | 1. Renovated structures. |
|  |  | a. Renovations are very popular. |
|  |  | b. Energy efficiency and handicap access changes are common. |
| Slide 4-58 |  |  |
|  |  | 2. Occupancy change--Heavy timber buildings can be changed in retail outlets. |
| Slide 4-59 |  |  |
|  |  | 3. Sealed windows--can be sealed for security and energy efficiency. |

| Slide 4-60 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | 4. Interior wooden columns. |  |
|  |  |  |  | a. Heavy-timber buildings can be renovated without creating concealed spaces. |  |
|  |  |  |  | b. Note open columns, beams, and flooring. |  |
| Slide 4-61 |  |  |  |  |  |
|  |  |  |  | 5. Heavy timber--large water supply requirements. |  |
|  |  |  |  | a. Heavy-timber buildings need a good water supply for extinguishment, use of master streams to prevent extension, and can quickly escalate into a campaign-type incident. |  |
|  |  |  |  | b. Firefighter's rehab is essential for long-duration incidents. |  |
| Slide 4-62 |  |  |  |  |  |
|  |  |  |  | 6. Potential collapse zone. |  |
|  |  |  |  | In the initial stages, proper placement of the apparatus and establishment of a collapse zone are very important. |  |
| Slide 4-63 |  |  |  |  |  |
|  |  |  |  | 7. Large dimensional lumber. |  |
|  |  |  |  | Large dimensional lumber is characteristic of heavy-timber construction. |  |
| Slide 4-64 |  |  |  |  |  |
|  |  |  |  | vii. Type V--Wood-Frame construction (20 min.) |  |
|  |  |  |  | The wood-frame structure is the most common type of construction in America. |  |
| Slide 4-65 |  |  |  |  |  |
|  |  |  |  | A. Characteristics. |  |
|  |  |  |  | Walls, floors, and roof structures are all constructed from combustible wooden framing. |  |
|  |  |  |  | B. Strengths. |  |
|  |  |  |  | The adjoining sidewalls in a townhouse design are more stable than free standing walls. |  |
|  |  |  |  | C. Firefighter/Safety concerns. |  |
|  |  |  |  | 1. Technology is changing. |  |
|  |  |  |  | 2. Most of the technological changes are related to adding lightweight components. These components fail at an early rate and create many toxic gases. |  |
|  |  |  |  | 3. Roof leak or plumbing pipe leakage will weaken wood beams/joists. |  |
| Slide 4-66 |  |  |  |  |  |
|  |  |  |  | D. Comparison of wood frame to heavy timber. |  |
| Slide 4-67 |  |  |  |  |
|  |  |  | 1. Wooden-frame exterior will add to the fire load. |  |

| Slide 4-68 |  |  |
| --- | --- | --- |
|  |  | 2. Wall studs provide strength and support for structural elements. |
| Slide 4-69 |  |  |
|  |  | 3. Firefighters must be aware of residential burn time and anticipate structural failure. |
| Slide 4-70 |  |  |
|  |  | 4. Firefighters must be aware of commercial property burn time and anticipate structural failure. |
| Slide 4-71 |  |  |
|  |  | 5. Plywood can be used for the exterior wall. |
|  |  | a. It is normally covered to protect against the weather. |
|  |  | b. Soffits/Overhangs can permit fire to enter the inside of the structure. |
| Slide 4-72 |  |  |
|  |  | 6. Exterior covering. |
|  |  | a. Brick veneers in a townhouse design provide more strength than freestanding bearing walls. |

| Slide 4-73 |  |  |
| --- | --- | --- |
|  |  | b. Vinyl siding. |
| Slide 4-74 |  |  |
|  |  | 7. Balloon frame has studs that run from the foundation to the attic. |
| Slide 4-75 |  |  |
|  |  | 8. Platform frame has the walls of each story built on a platform formed by the preceding floor. |
| Slide 4-76 |  |  |
| Slide 4-77 |  |  |
|  |  | 9. Post and beam has structural members of substantial dimensions and is sided with a lightweight covering such as wood board. |
| Slide 4-78 |  |  |
|  |  | 10. Log cabins have wooden logs that are interlocked by notching the ends. |
| Slide 4-79 |  |  |
|  |  | 11. A log cabin design can have many new energy efficient features. Heavy logs will add to the fuel load. |
| Slide 4-80 |  |  |
|  |  | viii. What are the firefighting safety concerns? (25 min.) |
| Slide 4-81 |  |  |
|  |  | A. Building being constructed. |
|  |  | Construction phase has new concrete and accountability of workers. |
| Slide 4-82 |  |  |
|  |  | B. Building being demolished. |
|  |  | 1. Many times explosives/cutting equipment are used. |
|  |  | 2. Plastic wraps can easily ignite causing early failure of the structure when columns are altered or removed. |

| Slide 4-83 |  |  |
| --- | --- | --- |
|  |  | C. Changes in occupancy result in commodities that exceed the capacity of the sprinkler system. |
| Slide 4-84 |  |  |
|  |  | D. Mixed construction adds many dangers to responders. |
|  |  | Risks of concealed spaces can add to suppression, search, rescue, and ventilation challenges. |
| Slide 4-85 |  |  |
|  |  | E. Combustible outer coverings and balconies. |
| Slide 4-86 |  |  |
|  |  | F. Wooden lintel will burn and fail. |
| Slide 4-87 |  |  |
|  |  | G. Metal lintel will expand and fail. |

| Slide 4-88 |  |  |
| --- | --- | --- |
|  |  | H. Overhangs add much fuel load. |
| Slide 4-89 |  |  |
|  |  | I. Mansard roof adds a significant fuel load. |
| Slide 4-90 |  |  |
|  |  | J. Parapet walls are free standing and can collapse without much advance warning. |
| Slide 4-91 |  |  |
|  |  | K. Star indicator of an unstable wall. |
| Slide 4-92 |  |  |
|  |  | L. Angle iron to support exterior wall. |
| Slide 4-93 |  |  |
|  |  | M. Void spaces can spread fire problems. |
| Slide 4-94 |  |  |
|  |  | N. Poke through can spread fire problems. |
| Slide 4-95 |  |  |
|  |  | O. Missing ceiling tiles will reduce fire protection rating of a ceiling. |
| Slide 4-96 |  |  |
|  |  | P. Suspended ceilings are attached to the ceiling with light gauge metals. |
|  |  | 1. These ceiling assemblies can hide a spreading fire problem and quickly fail. |
|  |  | 2. The use of a TIC is necessary to pinpoint heat sources. |
| Slide 4-97 |  |  |
|  |  | Q. Large open spaces can lead to firefighters getting disoriented and lost. |

| Slide 4-98 |  |  |
| --- | --- | --- |
|  |  | R. Different building materials react to heat in different ways. |
|  |  | 1. Steel expands and lightweight pre-engineered may collapse. |
|  |  | 2. Wood gives off toxic gases. |
|  |  | 3. Both suddenly fail. |
|  |  | S. Danger of steel that expands and wood that burns. |
| Slide 4-99 |  |  |
|  |  | T. New technology can create less stable building components under fire conditions. |
|  |  | U. The strength of lightweight construction is the floor-by-floor approach. |
| Slide 4-100 |  |  |
|  |  | V. Method of construction. |
|  |  | 1. "All at once." |
|  |  | a. Resulted in many large-scale fires. |
|  |  | b. The exposed wooden walls create the potential for a conflagration. |
| Slide 4-101 |  |  |
|  |  | 2. Lightweight truss without firestops will create a significant fuel load. |
|  |  | Lightweight connectors will loosen and the structural integrity of the entire roof will be lost. |
| Slide 4-102 |  |  |
|  |  | 3. Truss and columns have various type fasteners and are made of materials that react differently during a fire. |
| Slide 4-103 |  |  |
|  |  | W. Hazards and safety scale. |
|  |  | New building construction methods and materials equal new rules and new tactics. |
| Slide 4-104 |  |  |
|  |  | X. Risk/Benefit evaluation based on code enforcement inspections, company level preplans, and onscene sizeup skills. |
|  |  | Y. Knowledge of the different building construction types will prevent serious and fatal injuries. |
| Slide 4-105 |  |  |
|  |  | iX. What is the Brief Initial Report? (10 min.) |
|  |  | A. Knowledge of the different building construction types will prevent serious and fatal injuries. |
|  |  | B. What do I have? |
| Slide 4-106 |  |  |
|  |  | 1. Building and occupancy type. |
|  |  | a. Size of structure--stories and dimension. |
|  |  | b. Type of construction. |
|  |  | c. Occupied or vacant. |
| SM p. 4-10 |  |  |
| Slide 4-107 |  |  |
|  |  | Refer students to their Student Manual (SM). They can review the plot plan and follow the example Brief Initial Report (BIR) used by the instructor for this section. |
| Slide 4-108 |  |  |
|  |  | 2. Description of conditions. |
|  |  | a. Describe situation or incident conditions and where those conditions are seen. |
|  |  | - Nothing showing. |
| Slide 4-109 |  | - Smoke/Fire--location and density. |
|  |  | - Status of occupants if known. |
|  |  | - Exposures--Sides A, B, C, D, or other. |
|  |  | b. Example. |
|  |  | - Heavy smoke and fire showing from the second floor, Side A. |
| Slide 4-110 |  |  |
|  |  | Emphasize the critical importance of recognizing building construction types. |
| Slide 4-111 |  |  |
|  |  | C. Who is in Command? |
|  |  | While it is assumed that the first-arriving officer is in Command until Command is transferred, the initial report should make it clear whether that first officer is keeping Command or if it is being transferred, and to whom. |

| Slide 4-112 |  |  |
| --- | --- | --- |
|  |  | D. Example. |
|  |  | 1. Engine arrived on location at Side of a (describe structure or facility). |
|  |  | 2. Describe incident conditions, e.g., fire, smoke conditions, and apparent location. |
| Slide 4-113 |  | 3. Tell what tactical operation your crew is doing. |
|  |  | 4. (Rank) Engine is Command. |
|  |  | 5. Call for additional resources, if needed. |
| Slide 4-114 |  |  |
| Slide 4-115 |  |  |
|  |  | X. Summary (5 min.) |
|  |  | If you think building construction has changed, you are right! Now, you must change your thinking and decision making process. |