Objectives-Strategy-Tactical Direction

Single Family House Fire Scenario

Nature/Conditions:                            Initial Report:

Two-story single family dwelling, 20’ x 45’, occupied.

Heavy fire first floor kitchen, side C, heavy smoke 2nd floor.

Report of occupants trapped on the second floor side B bedroom.

| Objective | Strategy | Tactical Direction |
| --- | --- | --- |
| Objective # 1. Safe removal of all occupants from fire building within 10 minutes. | Strategy # 1. Remove all occupants from fire building. | Tactic # 1: S/R # 1 group to second floor via interior for occupant removal. Tactic # 2: S/R # 1 group to second floor via exterior for occupant removal. |
|  | Strategy # 2: Confine fire to room of origin and prevent upward extension. | Tactic # 1: Develop a 1 ¾ “hose line to 1st floor kitchen area for confinement. Tactic # 2: Develop a 1 ¾” hose line to first floor stairway for occupant and FF protection. |
|  | Strategy #3: Ventilate for safe removal of occupants. | Tactic #1: Ventilation group to ventilate the center stairway and side B second floor. |
|  | Strategy # 4: Employ Special Equipment to assist in occupant removal. | Tactic #1: Employ an imaging camera for 2nd floor search and rescue operations. |
| Objective #2: Confine fire to room/building of origin within 10 minutes. | Strategy #1: Extinguish fire in room of origin and prevent extension. | Tactic # 1: Develop a 1 ¾” hose line to first floor kitchen area for fire extinguishment. Tactic# 2. Develop a 2nd - 1 ¾” hose line to first floor kitchen area as a backup line. |
|  | Strategy # 2: Protect lateral and upward internal exposures. | Tactic # 1: Develop a 1 ¾ “hose line to second floor, side C for internal exposure protection. |
|  | Strategy # 3: Ventilate structure for fire control and smoke removal. | Tactic# 1: Ventilation group to ventilate all roof apertures then cross ventilate first and second floors. |
|  | Strategy # 4: Employ sound salvage techniques and safe overhaul procedures. | Tactic # 1: Salvage group to provide covers for perishables and valuables in basement and prevent unnecessary water damage. Tactic # 2: Employ an imaging camera to 1st and 2nd floors to expose any hidden fire. |