UNIT 5:

THE ANALYTICAL SIZEUP PROCESS

objectives

The students will:

1.	Discuss the scientific method.

2.	Describe the primary sizeup factors and determine their impact on objectives and strategies.

3.	Analyze the Command Sequence Action Panning Cycle.

points for the instructor

The content of this unit is based on the theories and information presented in Firefighting Strategy and Leadership, by Charles V. Walsh and Leonard G. Marks, published by Gregg Division of McGraw Hill Book Company (2nd ed., 1977).

methodology

This unit uses lecture, discussion and small group activities.

(Total Time:  3 hr., 30 min.)

120 min.	Lecture/Discussion

Objectives	IG 5-3

The Scientific Method	IG 5-3

The Primary Factors Chart	IG 5-4

Making Decisions	IG 5-5

Limits	IG 5-6

Evaluating Primary Factors	IG 5-6

The Command Sequence Cycle	IG 5-17

45 min.	Small Group Activity 5.1

Primary Factors Chart Exercise	IG 5-27

45 min.	Small Group Activity 5.2

Objectives-Strategy-Tactics Chart Exercise	IG 5-57

audiovisual

Slides 5-1 to 5-122

| Slide 5-1 |  |  |
| --- | --- | --- |
| 60 min. |  |  |
| Lecture/Discussion |  |  |
| Slide 5-2 |  |  |
|  |  | I. Objectives (5 min.) |
|  |  | The students will: |
|  |  | A. Discuss the scientific method. |
|  |  | B. Describe the primary sizeup factors and determine their impact on objectives and strategies. |
|  |  | C. Analyze the Command Sequence Action Planning Cycle. |
| Slide 5-3 |  |  |
|  |  | II. the Scientific Method (5 min.) |
|  |  | A. Observe the facts (sizeup). |
|  |  | B. Test the accuracy of these facts through observation. |
|  |  | C. If these facts are accurate, a logical hypothesis can be deduced. |
|  |  | D. If these hypotheses are proven true, outcomes may be predicted in similar situations. |
|  |  | E. These hypotheses are called principles. |

| Slide 5-4 |  |  |
| --- | --- | --- |
|  |  | III. the Primary Factors Chart (10 min.) |
|  |  | As an instructor, you should have read the material on primary factors and became familiar with the Student Manual (SM) text before beginning this section of the course. |
| SM p. 5-6 Slide 5-5 |  | Refer students to the Primary Factors Chart in the SM. Take time to explain how the chart is used and why it is important. The information on Primary Factors, Objectives, and Strategies is explained in detail in the SM. Recommend to students that they read this material after class. |
|  |  | Columns on the Primary Factors Chart. |
| Slide 5-6 |  |  |
|  |  | A. Column 1: Primary Factors. |
|  |  | 1. Primary factors are the conditions and elements that should be recognized and evaluated on arrival and during operations. |
|  |  | 2. Some examples of primary factors: |
|  |  | a. Life hazard. |
|  |  | b. Location/Fire. |
|  |  | c. Construction. |
|  |  | d. Structural collapse. |
|  |  | e. Time/Duration of operations. |
|  |  | B. Column 2: Incident Objectives. |
|  |  | 1. Safe Removal of Occupants: When the life hazard for occupants is the limiting or strategic factor. |

|  |  | 2. Contain and Control Fire to Room/ Building of Origin: No life hazard for occupants and fire is considered to be controllable by a direct or indirect attack. |
| --- | --- | --- |
|  |  | 3. Contain, Control, and Limit Fire in Exposures: No life hazard for occupants and the situation is temporarily uncontrollable. |
|  |  | C. Column 3: Incident Strategies. |
|  |  | D. Column 4: Evaluate Strategies. |
| Slide 5-7 |  |  |
|  |  | IV. Making Decisions (5 min.) |
|  |  | A. Example: Effective ventilation facilitates the advancement of hoselines, yet the ineffective stretching or laying of hoselines nullifies the effectiveness of ventilation. Ineffective stretching or lying of hoselines can be harmful if it results in spreading the fire before a line is ready to operate. |
| Slide 5-8 |  |  |
|  |  | B. Strategies can also affect primary factors such as the extent of fire after arrival, heat and smoke conditions, exposure requirement hazards, duration of operation, requirements to operate, etc. |
| Slide 5-9 |  |  |
|  |  | C. Each of the primary factors has an effect on strategies. |
|  |  | 1. We determine objectives and strategies based on the sizeup we conduct evaluating the primary factors. |
|  |  | 2. When we implement actions to meet our objectives, these actions affect the primary factors. This is a circular process. |

| Slide 5-10 |  |  |
| --- | --- | --- |
|  |  | V. Limits (5 min.) |
|  |  | For practical purposes, there are two limits. |
|  |  | A. Life hazard for occupants. Risks to firefighters, ranging from merely unusual to extreme, may also be warranted. |
|  |  | B. If there is no life hazard for occupants, personnel are never to be jeopardized unnecessarily. |
| Slide 5-11 |  |  |
|  |  | VI. Evaluating Primary Factors (50 min.) |
|  |  | A. Life hazard for occupants. |
|  |  | 1. Rescue work. |
|  |  | Forcible entry is made with less regard for structural damage, exposure hazards, or the availability of a hose stream to protect personnel. |
|  |  | 2. Covering exposures. |
|  |  | Life hazard may make the task of covering exposures more difficult and may delay the attack on the fire itself. |
| Slide 5-12 |  |  |
|  |  | B. Life hazard for firefighters. |
|  |  | Acceptance of warranted risks is essential for good results in carrying out fire activities. |
| Slide 5-13 |  |  |
|  |  | C. Location of fire on arrival. |
|  |  | 1. Entry. |
|  |  | a. It is preferable to force entry near the location of the fire, especially when the area involved is large. |

|  |  | b. This enables firefighters to get water on the fire most quickly and minimizes the physical hardship of advancing hoselines. |
| --- | --- | --- |
| Slide 5-14 |  |  |
|  |  | 2. Ventilation. |
|  |  | a. The main objective of ventilation is to localize the fire and stop its horizontal spread within a structure. |
|  |  | b. For example, if a fire is extending into a cockloft via pipe recess or similar channel, the roof should be opened. If this were done in the wrong place, it could be disastrous. |
| Slide 5-15 |  |  |
|  |  | 3. Removal of occupants. |
|  |  | a. Location of fire is critical. A fire on the first floor of a five-story residential building could endanger all the occupants and necessitate their removal. |
|  |  | b. However, if the same fire originated on the fourth floor, it might be better to move occupants off the fifth floor to the first or second floor. This is especially true if the fire occurs on a cold night and occupants are scantily clothed. |
| Slide 5-16 |  |  |
|  |  | 4. Checking for extension of fire. |
|  |  | a. A fire near a vertical or horizontal structure channel will spread readily. |
|  |  | b. Officers assigned to check for fire extension should note the location of the fire and keep in mind how heat travels by conduction, convection, and radiation via exposed channels. |

| Slide 5-17 |  |  |
| --- | --- | --- |
|  |  | 5. Placement and use of hoselines. |
|  |  | The location of the fire determines the amount of hoseline to be stretched and, in some cases, the size. |
|  |  | 6. Use of special equipment. |
|  |  | a. High-level fires may require the use of standpipe systems, ladder pipes, or other high-caliber streams. |
|  |  | b. The fire also may influence the decision to use sprinkler systems or fixed systems of various types. |
| Slide 5-18 |  |  |
|  |  | 7. Heat transfer. |
|  |  | a. Radiation. |
|  |  | b. Conduction. |
|  |  | c. Convection. |
|  |  | D. Extent of fire after arrival. |
| Slide 5-19 |  |  |
|  |  | ASK: What effect would extent of the fire have on objectives and strategies? |
|  |  | Suggested responses: |
|  |  | - Forcible entry--In light fire, entry should be made in the manner least damaging to property. The use of a key may be a solution. However, in situations where the extent of the fire is obviously substantial, such consideration is not warranted. Speed in getting an efficient operation underway is more important. |
|  |  | - Ventilation--The amount of structural damage done in ventilating should have a reasonable relationship to the extent of the fire. |

|  |  | E. Construction. |
| --- | --- | --- |
|  |  | 1. Ordinary construction. |
| Slide 5-20 |  |  |
|  |  | ASK: What is the effect of ordinary construction on the objectives and strategies? |
|  |  | Suggested responses: |
|  |  | - Concealed spaces can contribute to smoke and fire spread--cocklofts, common attics, etc. |
|  |  | - These buildings often have undergone renovation, creating voids, dropped ceilings, etc., that allow fire spread. |
|  |  | - Walls may be tied to floors with metal bolts, stars, and/or wall plates. |
|  |  | - Collapse is a real possibility under prolonged, heated fire conditions. |
|  |  | - While cooling is a good tactic, beware lest unprotected metal columns, especially cast iron, fail when very hot and struck with water. |
|  |  | 2. Wood-frame construction. |
| Slide 5-21 |  |  |
|  |  | ASK: What is the effect of wood-frame construction on objectives and strategies? |
|  |  | Suggested responses: |
|  |  | - Balloon frame rarely has firestops and floor joists are tied to walls, allowing for fire spread in any direction. |
|  |  | - Unprotected vertical openings for heating/air-conditioning vents, plumbing, utilities, etc., create a path for fire extension. |

|  |  | - Structure is totally combustible. |
| --- | --- | --- |
|  |  | - Roof and floor supports may be of lightweight construction. |
|  |  | 3. Fire-resistive (modern highrise) construction. |
| Slide 5-22 |  |  |
|  |  | ASK: What is the effect of fire-resistive construction on objectives and strategies? |
|  |  | Suggested responses: |
|  |  | - Central air-conditioning penetrates all parts of the building, creating a potential for smoke and fire spread. |
|  |  | - Fire has a tendency to "lap" from floor to floor. |
|  |  | - In the Las Vegas, NV, Hilton fire in 1981, fire "lapped" 22 floors in 20 minutes (8th to 30th floors). |
|  |  | - Office buildings of this type of construction often contain large open areas with heavy fire loading. |
|  |  | - Ventilation is a significant problem. |
|  |  | - Operations are extremely labor-intensive, demanding extensive resources. |
| Slide 5-23 |  |  |
|  |  | ASK: What effect would noncombustible construction have on objectives and strategies? |
|  |  | Suggested responses: |
|  |  | - Buildings may be used as office buildings, warehouses, automobile repair, etc. |
|  |  | - Structural members are noncombustible or limited-combustible materials: |
|  |  | - walls; |
|  |  | - floors; |
|  |  | - roofs; and |
|  |  | - support members. |
|  |  | - Unprotected steel is a common structural component. While noncombustible, it is not resistive to bending, flexing, expanding, or relaxing. |
| Slide 5-24 |  |  |
|  |  | ASK: What is the effect of buildings under construction on objectives and strategies? |
|  |  | Suggested responses: |
|  |  | - There is a minimal need for forcible entry and ventilation. |
|  |  | - Stress is placed on the placement and use of hoselines. |
|  |  | - Operative standpipes should be supplied and used, or lines have to be stretched up the outside of the structure, in which case, sufficient personnel must be anticipated. |
|  |  | - If fire is within reach from ladder pipes or tower ladder platforms, fog from the windward side can be effective. Solid streams would be advisable, however, if greater penetration were needed. |
|  |  | - Exterior exposure hazard in buildings under construction can present multiple problems, because besides the danger to nearby buildings from radiation and convection of heat, a spark ember hazard may exist. |

| Slide 5-25 |  |  |
| --- | --- | --- |
|  |  | ASK: What is the effect of buildings under demolition on objectives and strategies? |
|  |  | Suggested response: No life hazard for occupants--the effects are similar to those of buildings under construction, assuming comparable heights. |
| Slide 5-26 |  |  |
|  |  | F. Occupancy and contents. |
|  |  | 1. Ventilation. |
|  |  | a. Some materials give off gases that are toxic or injurious to the eyes or skin. Ventilation is achieved more slowly. |
|  |  | b. Where the presence of explosive mixtures or substances is suspected, exterior ventilation measures should be taken to prevent an explosion or to minimize its results. |
|  |  | 2. Placement of hoselines. |
|  |  | Difficulty in ventilating can reduce the effectiveness of hoselines. |
|  |  | 3. Selecting an extinguishing agent. |
|  |  | In some cases, water will spread the fire or may cause an explosion or release of gases that may intensify and exacerbate extension of the fire. |
|  |  | 4. Overhauling. |
|  |  | The quantity of material involved, the manner in which it is stored, its nature, and the degree to which it has been subjected to the fire all affect overhauling. |

| Slide 5-27 |  |  |
| --- | --- | --- |
|  |  | G. Height. |
|  |  | 1. Ventilation. |
|  |  | a. Height can affect activities at fires in highrise buildings. |
|  |  | b. At lower-level fires, roof or window ventilation may be possible, thus facilitating the advancement of lines from either side of the fire, as well as the search for and removal of occupants. |
|  |  | 2. Placement of hoselines. |
|  |  | a. Exterior lines also may be used. |
|  |  | b. In either case, control is likely to be established more quickly at a lower-level fire than at a similar but higher fire, which can only be attacked from the interior. |
| Slide 5-28 |  |  |
|  |  | H. Area. |
|  |  | 1. If the fire can be confined to a small room, the fact that the total floor area is 200 by 200 feet (40,000 square feet) hardly matters. |
|  |  | 2. However, if such an area is not effectively subdivided and there is no small room, sooner or later, the extent of the fire can cover the total floor area. |
|  |  | 3. Proximity of exposures. |
|  |  | a. Proximity alone does not make an exposure vulnerable. |

|  |  | b. To evaluate the effects of proximity in selecting objectives and activities, it must be considered in conjunction with other contributing factors--such as construction, location of fire, occupancy, wind direction, and velocity. |
| --- | --- | --- |
|  |  | I. Structural collapse. |
|  |  | In assessing the effects of other factors on structural collapse, officers should carefully consider the type of construction involved in the fire. |
| Slide 5-29 |  |  |
|  |  | J. Weather. |
|  |  | 1. Low temperatures retard the initial development of fire, but once a fire has started, they impair firefighters' efficiency. |
|  |  | 2. High temperatures generally are classified as temperatures in the 80s and 90s. |
|  |  | a. High humidity and inversion conditions are characterized by dense smoke and poor visibility. |
|  |  | b. High humidity and high moisture content of the air make it more difficult for a vigorous fire to become established, but do not slow its spread once it is well started. |
|  |  | 3. Rain greatly reduces the probability of fire spreading from building to building. |
|  |  | 4. Wind. |
|  |  | a. Velocity is an important factor. With winds less than 15 miles per hour (mph), the effects of the wind can usually be controlled by defensive measures. |
|  |  | b. With winds 15 to 30 mph, the rate of fire propagation increases dramatically. |

|  |  | - A wind of 30 mph threatens exposures downwind. |
| --- | --- | --- |
|  |  | - Winds of 30 mph are conducive to conflagrations. |
|  |  | 5. Visibility. |
|  |  | Impaired visibility makes it more difficult to recognize and evaluate pertinent factors properly, thereby hampering decision making and increasing the possibility of error. |
| Slide 5-30 |  |  |
|  |  | K. Resource requirements. |
|  |  | The water, apparatus, equipment, personnel, and special extinguishing agents required and available for an effective fire operation. |
| Slide 5-31 |  |  |
|  |  | L. Auxiliary appliances. |
|  |  | 1. Sprinkler systems. |
|  |  | 2. Pressurization of stairways or other building areas. |
| Slide 5-32 |  |  |
|  |  | M. Topography. |
|  |  | 1. When operating at a fire on steeply-graded streets, it may be advisable to position aerial trucks or tower ladders on the high side of the fire to ensure maximum reach. |
|  |  | 2. Wedges can be used effectively to level portable ladders to allow for safe climbing on hilly terrains. |
|  |  | 3. Streets that are one-way, congested by vehicular traffic, or covered by snow or ice tend to delay response of the fire department. |

| Slide 5-33 |  |  |
| --- | --- | --- |
|  |  | N. Explosions. |
|  |  | 1. Smoke explosions or backdrafts at fires are essentially caused by the rapid combustion of a mixture of flammable gas, vapor, mist, or dust and air under certain conditions. |
|  |  | 2. Bomb explosions. |
|  |  | a. Warnings of the impending explosion may or may not be given. |
|  |  | b. Chief officers responding to the designated target must assume the warning is authentic and conduct operations accordingly. |
| Slide 5-34 |  |  |
|  |  | O. Duration of operations. |
|  |  | 1. Fire operations of long duration are generally difficult to deal with from the beginning. |
|  |  | 2. They may feature heavy involvement and structural collapse. |
|  |  | 3. They may maximize the exposure hazard. |
|  |  | 4. Consider relief for personnel. |
| Slide 5-35 |  |  |

| Slide 5-36 |  |  |
| --- | --- | --- |
|  |  | VII. The Command Sequence Cycle (40 min.) |
|  |  | A. Overview. |
|  |  | 1. Critical factors of incident operations often are overlooked (or are not given enough emphasis). This can result in: |
|  |  | a. Poor use of resources. |
| Slide 5-37 |  | b. Inappropriate strategies and tactics. |
|  |  | c. Safety problems. |
|  |  | d. High-incident costs. |
|  |  | e. Lower effectiveness. |
|  |  | 2. A simple (but thorough) process for planning can be used for small, short-term, long-term, or more complex incidents and events. |
| Slide 5-38 |  |  |
|  |  | 3. This process is referred to as the Command Sequence Cycle and consists of the following sequential steps. |
|  |  | a. Understanding the situation (sizeup the primary factors). |
|  |  | b. Establishing Incident Objectives. |
|  |  | c. Developing Incident Strategy (activities). |
|  |  | d. Tactical direction and resource assignments. |
|  |  | e. Implementing the plan. |
|  |  | f. Evaluating the plan. |

| Slide 5-39 |  |  |
| --- | --- | --- |
|  |  | B. Understanding the situation. Sizeup primary factors. |
|  |  | 1. In general, the essential elements of information needed to understand the situation fully consist of: |
| Slide 5-40 |  |  |
|  |  | a. What has occurred? |
|  |  | b. Whether the incident will get bigger (or smaller). |
|  |  | c. Present (and future) resources and needs. |
|  |  | 2. The analysis and comparison of incident primary factors and cues are the basis for problem identification. |
|  |  | a. Report of occupants trapped. |
|  |  | b. Fire extending to exposures. |
|  |  | c. Presence of flammable vapors. |
| Slide 5-41 |  |  |
|  |  | 3. Primary factors. |
|  |  | a. Life hazard. |
|  |  | b. Location of incident/fire. |
|  |  | c. Construction. |
|  |  | d. Occupancy (contents). |
|  |  | e. Height. |
|  |  | f. Area. |
|  |  | g. Structural collapse. |
|  |  | h. Weather. |
|  |  | i. Apparatus/Personnel. |
|  |  | j. Auxiliary appliances. |
|  |  | k. Explosions/Backdraft. |
|  |  | l. Topography. |
|  |  | m. Time. |
| Slide 5-42 |  |  |
|  |  | C. Establishing Incident Objectives. |
| Slide 5-43 |  |  |
|  |  | ASK: What is an Incident Objective? |
|  |  | Suggested responses: |
|  |  | - Incident Objectives are statements of guidance and direction necessary for the selection of appropriate Strategy(ies) and the tactical direction of resources. |
| Slide 5-44 |  |  |
|  |  | - Answering the question of what can be accomplished when all allocated resources have been deployed effectively (based on realistic expectations). |
|  |  | - Determining Incident Objectives and Strategies is an essential prerequisite to developing a plan. |
| Slide 5-45 |  |  |
|  |  | 1. Incident Objectives must be (SMART) |
|  |  | a. Specific--to determine the needs to be accomplished. |
|  |  | b. Measurable--so that a final evaluation can determine whether Objectives were achieved. |

|  |  | c. Achievable/Attainable--with the resources available to the agency. |
| --- | --- | --- |
|  |  | d. Realistic/Results Oriented--enough to allow for consideration of strategic and tactical alternatives. |
|  |  | e. Time Specific--when should this be accomplished/how long to complete? |
|  |  | 2. Establishing Incident Strategy (activities). |
| Slide 5-46 |  |  |
|  |  | ASK: What is an Incident Strategy? |
|  |  | Suggested responses: |
|  |  | - Incident Strategies are a general plan or direction selected to accomplish Incident Objectives. |
| Slide 5-47 |  | - They generally answer the question of what needs to be done and describe the method (or methods) that should be used--either singly or in combination--that will result in achieving the Incident Objectives. |
|  |  | - It is essential to also consider alternative strategies that may be employed (should the primary strategy fail to achieve the intended results). |
| Slide 5-48 |  |  |
|  |  | a. For small incidents, Incident Objectives and Strategies are the sole responsibility of the Incident Commander (IC)/Company Officer (CO) and may take only a few minutes to complete. |
|  |  | b. For larger incidents, members of the General Staff (and others) will contribute to this process. |

| Slide 5-49 |  |  |  |  |
| --- | --- | --- | --- | --- |
|  |  |  | c. Communication. |  |
|  |  |  | - Communicate assignments clearly. |  |
|  |  |  | - Ensure assignments are understood. |  |
|  |  |  | - Obtain timely feedback. |  |
| Slide 5-50 |  |  |  |  |  |
|  |  |  |  | D. Tactical direction and resource assignments. |  |
| Slide 5-51 |  |  |  |  |  |
|  |  |  |  | ASK: What is a Tactic? |  |
|  |  |  |  | Suggested responses: |  |
|  |  |  |  | - Tactics refer to the deployment and direction of incident resources to accomplish the strategy--as guided by the objectives. |  |
| Slide 5-52 |  |  |  |  |  |
|  |  |  |  | - Tactics generally answer the question of how we are going to accomplish the strategy. |  |
|  |  |  |  | - They are normally established to be conducted within an operational period. |  |
| Slide 5-53 |  |  |  |  |  |
|  |  |  |  | 1. Tactical direction includes |  |
|  |  |  |  | a. Determining the Tactics and operations necessary for the selected strategy. |  |
|  |  |  |  | b. Determining and assigning appropriate resources. |  |
|  |  |  |  | 2. The tactical direction is developed around a specific operational period and must have measurable results. |  |
|  |  |  |  | 3. For large incidents that may last for some time, there is a limit to what may be achieved (in terms of accomplishing an Incident Objective in a single operational period). |  |
|  |  |  |  | 4. Therefore, tactical directions should be stated in terms of accomplishments that can be achieved realistically within the timeframe of an operational period. |  |
|  |  |  |  | 5. Resource assignments will be made for each of the specific work tasks. |  |
|  |  |  |  | a. These assignments will consist of the kind and number of resources needed to achieve the tactical operations for each operational period. |  |
|  |  |  |  | b. If resources are not available for a specific tactical operation, then the IC may need to prioritize tactical assignments or reassess the tactics (and perhaps the overall strategy). |  |
| Slide 5-54 |  |  |  |  |  |
|  |  |  |  | E. Evaluating the plan. |  |
| Slide 5-55 |  |  |  |  |  |
|  |  |  |  | 1. Is the Strategy attaining the overall Objectives? |  |
|  |  |  |  | 2. Are appropriate Tactics facilitating the Strategy? |  |
|  |  |  |  | 3. Does the plan need to be modified or adjusted? |  |
|  |  |  |  | 4. Continue the sizeup and evaluate the effectiveness of the Incident Action Plan (IAP) (evaluate every 10 minutes). |  |
|  |  |  |  | 5. Identify alternative plans for firefighter safety when occupant safety has been determined not to be a primary factor. |  |
|  |  |  | Use this example to illustrate Objectives, Strategies, and Tactics. |  |
| Slide 5-56 |  |  |  |  |
|  |  |  | Occupied house fire: |  |
|  |  |  | Two-story occupied dwelling, 20 by 40 foot ordinary construction, fire located in living room area first floor, Side D, report of occupants trapped in second-floor bedroom, Side A. No external exposures. |  |
| Slide 5-57 |  |  |  |  |
|  |  |  | Objectives |  |
|  |  |  | - safe removal of occupants within 10 minutes; and |  |
|  |  |  | - contain and control fire to the room/building of origin. |  |
| Slide 5-58 |  |  |  |  |
|  |  |  | Strategies |  |
|  |  |  | Example of Strategies for Objective #1: safe removal of occupants. |  |
|  |  |  | - Strategy #1: rescue occupants; |  |
|  |  |  | - Strategy #2: confinement/extinguishment; and |  |
|  |  |  | - Strategy #3: ventilation. |  |

| Slide 5-59 |  |  |
| --- | --- | --- |
|  |  | Tactics |
|  |  | For the rescue Strategies, the Tactics might be |
|  |  | - Tactic #1: deploy search/rescue group to upper floors; |
|  |  | - Tactic #2: deploy one 1-3/4-inch hoseline to first floor for stairwell/occupant protection; and |
|  |  | - Tactic #3: ventilate stairwell (if possible) and Side A to support occupant removal. |
| Slide 5-60 |  |  |
|  |  | Strategies |
|  |  | Example of Strategies for Objectives #2: contain and control fire to room/building of origin. |
|  |  | - Strategy #1: confine/extinguish; |
|  |  | - Strategy #2: ventilation; and |
|  |  | - Strategy #3: salvage. |
| Slide 5-61 |  |  |
|  |  | Tactics |
|  |  | For the confine/extinguish Strategies, the Tactics might be |
|  |  | - Tactic #1: deploy one 1-3/4-inch hoseline to first floor, Side D, for confinement/extinguishment; |
|  |  | - Tactic #2: deploy one 1-3/4-inch hoseline to second floor, Side D, for interior exposure protection; and |
|  |  | - Tactic #3: vertical ventilation of the roof apertures and horizontal ventilation of first and second floors. |
| Slide 5-62 |  |  |
|  |  | Alternatives |
|  |  | - Tactic #4: positive-pressure ventilation with proper use of fans; and |
|  |  | - Tactic #5: use of salvage covers and control of water runoff. |

| Slide 5-63 |  |  |
| --- | --- | --- |
|  |  | F. Summary. |
|  |  | 1. Scientific method of sizeup. |
|  |  | 2. Using the Primary Factors Chart. |
|  |  | 3. Making decisions. |
|  |  | 4. Evaluating primary factors. |
|  |  | 5. Command Sequence Cycle. |

This page intentionally left blank.

| 45 min. |  |  |
| --- | --- | --- |
| Small Group |  |  |
| Activity 5.1 |  |  |
| Slide 5-64 |  |  |
|  |  | Activity 5.1 |
|  |  | Primary Factors Chart Exercise |
|  |  | Purpose |
|  |  | To identify primary factors at an incident scenario. |
|  |  | Directions to Students |
|  |  | 1. Place students into groups of five or six. |
|  |  | 2. There are five primary factor exercises. You can choose one of the following options. |
|  |  | a. Select one primary factor exercise for all of the student groups. |
|  |  | b. Select one primary factor exercise for each student group. |
| SM p. 5-35 |  |  |
|  |  | 3. Review the exercise or exercises with the student group. Point out construction features and types (hip roofs, knee walls, etc.). |
| SM p. 5-37 |  |  |
| IG p. 5-35 |  | 4. Refer students to the Primary Factors Chart. |
|  |  | 5. Walk students through the Primary Factors Chart application. |
|  |  | 6. Explain that the exercise objective is to concentrate on Column 1 (students will address Columns 2 to 4 of the Primary Factors Chart). |
|  |  | 7. Conduct a walkaround of untouched slides for Sides A-B-C-D for each selected scenario. |
|  |  | 8. Conduct second walkaround of touched up slides for Sides A-B-C-D. |

|  |  | 9. Student groups shall develop a Primary Factors Exercise Chart report for their assigned scenario. |  |  |
| --- | --- | --- | --- | --- |
|  |  | a. Identify the most pertinent 13 primary factors for each scenario. |  |  |
|  |  | b. Place findings on Column 1 of the Primary Factors Exercise Chart. |  |  |
|  |  | c. Identify the most pertinent subfactors for each identified primary factor for the exercise scenario. Place findings on Column 2 of the Primary Factors Exercise Chart. |  |  |
|  |  | d. Identify pertinent precautions to be taken to address each identified subfactor for the exercise scenario. Place findings on Column 3 of the Primary Factors Exercise Chart. |  |  |
|  |  | e. Each group shall select a spokesperson to report Primary Factors Exercise Chart findings to the class. |  |  |
| Exercise #1 Slides |  |  |  |  |
| Slide 5-65 |  |  | Slide 5-66 | Slide 5-67 |
| Slide 5-68 |  |  | Slide 5-69 | Slide 5-70 |
| Slide 5-71 |  |  | Slide 5-72 | Slide 5-73 |
| Slide 5-74 |  |  | Slide 5-75 | Slide 5-76 |
| Slide 5-77 |  |  |  |  |
| Exercise #2 Slides |  |  |  |  |
| Slide 5-78 |  |  | Slide 5-79 | Slide 5-80 |
| Slide 5-81 |  |  | Slide 5-82 | Slide 5-83 |

| Slide 5-84 |  |  | Slide 5-85 | Slide 5-86 |
| --- | --- | --- | --- | --- |
| Slide 5-87 |  |  | Slide 5-88 |  |
| Exercise #3 Slides |  |  |  |  |
| Slide 5-89 |  |  | Slide 5-90 | Slide 5-91 |
| Slide 5-92 |  |  | Slide 5-93 | Slide 5-94 |
| Slide 5-95 |  |  | Slide 5-96 | Slide 5-97 |
| Slide 5-98 |  |  | Slide 5-99 | Slide 5-100 |
| Exercise #4 Slides |  |  |  |  |
| Slide 5-101 |  |  | Slide 5-102 | Slide 5-103 |
| Slide 5-104 |  |  | Slide 5-105 | Slide 5-106 |
| Slide 5-107 |  |  | Slide 5-108 | Slide 5-109 |
| Slide 5-110 |  |  |  |  |
| Exercise #5 Slides |  |  |  |  |
| Slide 5-111 |  |  | Slide 5-112 | Slide 5-113 |
| Slide 5-114 |  |  | Slide 5-115 | Slide 5-116 |
| Slide 5-117 |  |  | Slide 5-118 | Slide 5-119 |
| Slide 5-120 |  |  | Slide 5-121 |  |

student activity worksheet

Activity 5.1

Primary Factors Chart Exercise

Purpose

To identify primary factors at an incident scenario.

Directions

1. You will be placed into groups of five or six.

2. The instructor will select the exercise or exercises for your group to work on.

3. Review the exercise or exercises within your group.

4. You will develop a Primary Factors Exercise Chart report for your assigned scenario.

a. Identify the most pertinent 13 primary factors for each scenario.

b. Place findings on Column 1 of the Primary Factors Exercise Chart.

c. Identify the most pertinent subfactors for each identified primary factor for your exercise scenario. Place findings on Column 2 of the Primary Factors Exercise Chart.

d. Identify pertinent precautions that will be taken to address each identified subfactor for your exercise scenario. Place findings on Column 3 of the Primary Factors Exercise Chart.

e. You will select a spokesperson to report Primary Factors Exercise Chart findings to the class.

This page intentionally left blank.

student activity worksheet

Activity 5.1 (cont'd)

Primary Factors Chart

This page intentionally left blank.

student activity worksheet

Activity 5.1 (cont'd)

Residential--Type V--Balloon Frame--Exercise #1

Structure: Three-story, 18 by 40 foot

Building Construction: Type V--balloon frame

Roof Construction: 2- by 6-inch beam-and-rafter system

Floors: 2- by 6-inch plywood flooring system

Alarm System: No smoke detectors installed

Occupants: Six occupants per occupancy

Three children, three adults

each occupancy

Special Concerns: One bedridden occupant on O2--located front bedroom--third floor--Side A/D

Situation Report:

Fire Building:

It is November 30, 0600 hours, temperature is 23 °F (-5 °C), wind from east at 10 mph. Upon arrival, one adult and three children are outside of the fire building. Occupants report two adults unaccountable from fire building including a bedridden occupant in third-floor front bedroom. The other missing occupant slept on second floor, but could not be located by occupants.

Exposure Building:

Two adults and three children from attached exposure occupancy are also outside.

This page intentionally left blank.

student activity worksheet

| Activity 5.1 (cont'd) |  |  |
| --- | --- | --- |
| Primary Factors Exercise Chart |  |  |
| Pertinent Primary Factors | Pertinent Subfactors | Pertinent Precautions To Be Taken |
| 1. | 1. 2. | 1. 2. |
| 2. | 1. 2. 3. | 1. 2. 3. |
| 3. | 1. 2. | 1. 2. |
| 4. | 1. 2. | 1. 2. |
| 5. | 1. 2. | 1. 2. |
| 6. | 1. 2. | 1. 2. |
| 7. | 1. 2. 3. 4. | 1. 2. 3. 4. |
| 8. | 1. 2. 3. | 1. 2. 3. |
| 9. | 1. 2. | 1. 2. |
| 10. | 1. 2. | 1. 2. |
| 11. | 1. | 1. |
| 12. | 1. 2. | 1. 2. |
| 13. | 1. 2. | 1. 2. |

This page intentionally left blank.

student activity worksheet

Activity 5.1 (cont'd)

Residential--Type V--Platform Frame--Exercise #2

Structure: Two-story, 40 by 60 foot

Building Construction: Type V--platform frame

Roof Construction: 2- by 6-inch truss roof support system

Floors: 2- by 4-inch truss floor system

Alarm System: No smoke detectors installed

Occupants: Eight occupants

Two adults--three children

Ages--8, 10, and 12 years

One dog (collie)

Special Concerns: One 12-year-old child has a lower leg cast suffering from a dislocated left knee

Situation Report:

Fire Building:

It is Sunday, February 2, 0800 hours, temperature is 18 °F (-8 °C), wind from east at 12 mph. Upon arrival, two adults and one child are outside of the fire building. Occupants report two children, ages 10 and 12, are unaccounted for along with their collie dog. Missing children slept in bedrooms #2 and #3 downstairs.

Exposure Building:

No immediate exposures.

This page intentionally left blank.

student activity worksheet

| Activity 5.1 (cont'd) |  |  |
| --- | --- | --- |
| Primary Factors Exercise Chart |  |  |
| Pertinent Primary Factors | Pertinent Subfactors | Pertinent Precautions To Be Taken |
| 1. | 1. 2. | 1. 2. |
| 2. | 1. 2. 3. | 1. 2. 3. |
| 3. | 1. 2. | 1. 2. |
| 4. | 1. 2. | 1. 2. |
| 5. | 1. 2. | 1. 2. |
| 6. | 1. 2. | 1. 2. |
| 7. | 1. 2. 3. 4. | 1. 2. 3. 4. |
| 8. | 1. 2. 3. | 1. 2. 3. |
| 9. | 1. 2. | 1. 2. |
| 10. | 1. 2. | 1. 2. |
| 11. | 1. | 1. |
| 12. | 1. 2. | 1. 2. |
| 13. | 1. 2. | 1. 2. |

This page intentionally left blank.

student activity worksheet

Activity 5.1 (cont'd)

Commercial--Type III--Ordinary--Exercise #3

Structure: Two-story, 20 by 40 foot

Building Construction: Type III--ordinary

Roof Construction: Post-and-beam

Floors: 2- by 6-inch flooring system tongue and groove

Alarm System: Smoke detectors installed

Occupants: Eight employees

Special Concerns: High fuel load--all floors

Flammable materials--basement

Situation Report:

Fire Building:

It is Saturday, March 5, 1300 hours, temperature 25 °F (-4 °C), and wind from north at 7 mph. Upon arrival, the Art Gallery manager reports two employees are unaccounted for; they were refinishing wooden antiques in the basement. He reports that he has a large amount of flammable paints and solvents in 5- to 10-gallon cans in the basement. Three employees were in the basement working at the time. They report that a small explosion occurred in the finishing room. Two of those employees are outside with severe arm and face burns. One employee is still missing. All other customers and employees are accounted for.

Exposure Building:

Occupied as Richmond Art Gallery Side B--two-story, 20 by 40 foot--Type III--ordinary balloon frame.

This page intentionally left blank.

student activity worksheet

| Activity 5.1 (cont'd) |  |  |
| --- | --- | --- |
| Primary Factors Exercise Chart |  |  |
| Pertinent Primary Factors | Pertinent Subfactors | Pertinent Precautions To Be Taken |
| 1. | 1. 2. | 1. 2. |
| 2. | 1. 2. 3. | 1. 2. 3. |
| 3. | 1. 2. | 1. 2. |
| 4. | 1. 2. | 1. 2. |
| 5. | 1. 2. | 1. 2. |
| 6. | 1. 2. | 1. 2. |
| 7. | 1. 2. 3. 4. | 1. 2. 3. 4. |
| 8. | 1. 2. 3. | 1. 2. 3. |
| 9. | 1. 2. | 1. 2. |
| 10. | 1. 2. | 1. 2. |
| 11. | 1. | 1. |
| 12. | 1. 2. | 1. 2. |
| 13. | 1. 2. | 1. 2. |

This page intentionally left blank.

student activity worksheet

Activity 5.1 (cont'd)

Commercial--Type II--Noncombustible--Exercise #4

Structure: One-story, 40 by 50 foot

Building Construction: Type II--noncombustible

Roof Construction: Metal truss roof support system

Floors: Concrete

Alarm System: Smoke detectors installed

Sprinklers: Yes--partial playroom area

Occupants: Thirty preschool children

Five adults

Special Concerns: Locked fence--rear play area

Situation Report:

Fire Building:

It is January 25, 1015 hours, temperature 15 °F (-9 °C), wind from north at 10 mph. Upon arrival, several children are outside along with four adults. Two children and one adult are unaccounted for. All three were last seen in the children's lavatory.

Exposure Building:

No immediate exposures.

This page intentionally left blank.

student activity worksheet

| Activity 5.1 (cont'd) |  |  |
| --- | --- | --- |
| Primary Factors Exercise Chart |  |  |
| Pertinent Primary Factors | Pertinent Subfactors | Pertinent Precautions To Be Taken |
| 1. | 1. 2. | 1. 2. |
| 2. | 1. 2. 3. | 1. 2. 3. |
| 3. | 1. 2. | 1. 2. |
| 4. | 1. 2. | 1. 2. |
| 5. | 1. 2. | 1. 2. |
| 6. | 1. 2. | 1. 2. |
| 7. | 1. 2. 3. 4. | 1. 2. 3. 4. |
| 8. | 1. 2. 3. | 1. 2. 3. |
| 9. | 1. 2. | 1. 2. |
| 10. | 1. 2. | 1. 2. |
| 11. | 1. | 1. |
| 12. | 1. 2. | 1. 2. |
| 13. | 1. 2. | 1. 2. |

This page intentionally left blank.

student activity worksheet

Activity 5.1 (cont'd)

Commercial/Residential--Type III--Ordinary--Exercise #5

Structure: Three-story, 20 by 40 foot

Building Construction: Type III--ordinary

Roof Construction: 2- by 12-foot flat plywood with asphalt

Floors: 2- by 6-inch plywood flooring

Alarm System: Smoke detectors installed

Sprinklers: Kitchen cooking area

Occupants: Eight employees

Eight diners

Special Concerns: Two commercial kitchens--first and second floors

Situation Report:

Fire Building:

It is April 1, 1830 hours, temperature 40 °F (4 °C), wind from southwest at 23 mph. Upon arrival, the manager of Tim's Restaurant reports that a candle fell from a table on a second floor dining room igniting the fire. All diners and six employees are accounted for, but two cooks from the second floor kitchen area are unaccounted for. He further stated that he did not think anyone was on the third floor at the time of the fire, but was not sure.

Exposure Building:

Exposure buildings are similar commercial/residential occupancies, first floor commercial, second and third floors residential. Status of occupants unknown upon arrival.

This page intentionally left blank.

student activity worksheet

| Activity 5.1 (cont'd) |  |  |
| --- | --- | --- |
| Primary Factors Exercise Chart |  |  |
| Pertinent Primary Factors | Pertinent Subfactors | Pertinent Precautions To Be Taken |
| 1. | 1. 2. | 1. 2. |
| 2. | 1. 2. 3. | 1. 2. 3. |
| 3. | 1. 2. | 1. 2. |
| 4. | 1. 2. | 1. 2. |
| 5. | 1. 2. | 1. 2. |
| 6. | 1. 2. | 1. 2. |
| 7. | 1. 2. 3. 4. | 1. 2. 3. 4. |
| 8. | 1. 2. 3. | 1. 2. 3. |
| 9. | 1. 2. | 1. 2. |
| 10. | 1. 2. | 1. 2. |
| 11. | 1. | 1. |
| 12. | 1. 2. | 1. 2. |
| 13. | 1. 2. | 1. 2. |

This page intentionally left blank.

| 45 min. |  |  |
| --- | --- | --- |
| Small Group |  |  |
| Activity 5.2 |  |  |
| Slide 5-122 |  |  |
|  |  | Activity 5.2 |
|  |  | Objectives-Strategy-Tactics Chart Exercise |
|  |  | Purpose |
|  |  | To identify objectives, strategy(ies), and tactics at an incident scenario. |
|  |  | Directions to Students |
| SM p. 5-69 |  |  |
|  |  | 1. Place students into groups of five or six. |
| SM p. 5-35 |  |  |
|  |  | 2. Refer students back to their Activity 5.1 materials. Have each group use the same scenario. |
| SM p. 5-37 |  |  |
| IG p. 5-35 |  | 3. Refer students to Primary Factors Chart Columns 2 to 4. |
|  |  | 4. Walk students through the Primary Factors Chart application for Columns 2 to 4. |
|  |  | 5. Explain that this portion of the exercise is to concentrate on Columns 2 to 4, and develop the Objectives, Strategies, and Tactics for their scenario. |
| SM p. 5-71 IG p. 5-61 |  | 6. Refer students to the Central City Fire/EMS Dispatch Criteria sheet in their SM for initial 1st Alarm Assignment. Refer students to their Plot Plans--Situation Report for each selected scenario. |
|  |  | 7. Conduct second walkaround of touched up slides for Sides A-B-C-D. |
|  |  | 8. Student groups shall develop an Objectives-Strategy-Tactics Chart report for their assigned scenario. |
|  |  | a. Column 1--identify the 1st Operational Period Objectives for the scenario. |
|  |  | b. Column 2--identify the 1st Operational Period Strategies for each objective. |

|  |  | - Column 3--identify the 1st Operational Period Tactics for each strategy. |
| --- | --- | --- |
|  |  | d. Column 4--assign companies to perform tactics. |
|  |  | e. Each group shall select a spokesperson to report Objectives-Strategy-Tactics Chart findings to the class. |
|  |  | Each scenario has 1st Alarm Structural Resources available at start of the exercise. |
|  |  | - Engine 1 |
|  |  | - Engine 2 |
|  |  | - Engine 3 |
|  |  | - Ladder 1 |
|  |  | - Basic Life Support (BLS) 1 |
|  |  | Each scenario will have working incident structural resources arrive 10 minutes into incident. |
|  |  | - Engine 5 |
|  |  | - Engine 6 |
|  |  | - Squad 1 |
|  |  | - Engine 6 (Rapid Intervention Crew (RIC)) |
|  |  | - Air Cascade-1 FF |
|  |  | - Safety Officer |

student activity worksheet

Activity 5.2

Objectives-Strategy-Tactics Chart Exercise

Purpose

To identify objectives, strategy(ies), and tactics at an incident scenario.

Directions

1. You will be placed into groups of five or six.

2. Your instructor will review the Primary Factors Chart Exercise and the Primary Factors Chart.

3. Use the Central City Fire/EMS Dispatch Criteria sheet for initial 1st Alarm Assignment.

4. Your groups will develop an Objectives-Strategy-Tactics Chart report for their assigned scenario.

a. Column 1--identify the 1st Operational Period Objectives for the scenario.

b. Column 2--identify the 1st Operational Period Strategies for each objective.

c. Column 3--identify the 1st Operational Period Tactics for each strategy.

d. Column 4--assign companies to perform tactics.

e. Each group shall select a spokesperson to report Objectives-Strategy-Tactics Chart findings to class.

This page intentionally left blank.

student activity worksheet

| Activity 5.2 (cont'd) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Central City Fire/EMS Dispatch Criteria |  |  |  |  |  |  |  |  |  |
| Assignment | Alarm Type | Engine | Ladder | Squad | Battalion Chief | EMS Unit | RIT | Safety Officer | Air Cascade |
| 1st Alarm | Structural | 3 | 1 |  |  | 1 BLS |  |  |  |
| Working Incident | Structural | 2 |  | 1 | 1 |  | 1 | 1 | 1 |
| 1st Alarm | Target Hazard | 3 | 2 | 1 | 1 | 1 BLS |  |  |  |
| Working Incident | Target Hazard | 2 |  |  |  | 1 ALS | 1 | 1 | 1 |
| 2nd Alarm | Structural/ Target Hazard | 2 | 1 |  | 1 | 1 EMS |  | 1 | 1 |
| 3rd Alarm | Structural/ Target Hazard | 2 | 1 |  |  | 1 EMS |  |  |  |
| 4th Alarm | Structural/ Target Hazard | 2 | 1 |  | 1 |  |  |  |  |

This page intentionally left blank.

student activity worksheet

| Activity 5.2 (cont'd) |  |  |  |
| --- | --- | --- | --- |
| Objectives-Strategy-Tactics Chart |  |  |  |
| Objectives | Strategy(ies) | Tactics | Assigned to: |
| Objective #1 | 1. | 1. 2. 3. | 1. 2. 3. |
|  | 2. | 1. 2. 3. | 1. 2. 3. |
|  | 3. | 1. 2. 3. | 1. 2. 3. |
| Objective #2 | 1. | 1. 2. 3. | 1. 2. 3. |
|  | 2. | 1. 2. 3. | 1. 2. 3. |
|  | 3. | 1. 2. 3. | 1. 2. 3. |
| Objective #3 | 1. | 1. 2. 3. | 1. 2. 3. |
|  | 2. | 1. 2. 3. | 1. 2. 3. |
|  | 3. | 1. 2. 3. | 1. 2. 3. |