unit 6:

Burn Time considerations And line-of-duty deaths from collapse incidents

objectives

The students will:

1.	Given examples of different types of structures and different fire loads, predict the time of collapse.

2.	Discuss the impact of the primary factor, construction, on line-of-duty deaths (LODDs).

methodology

This unit uses lecture and discussion.

(Total Time: 2 hr., 15 min.)

135 min.	Lecture/Discussion

Objectives	IG 6-3

Wood Frame	IG 6-3

Ordinary Warehouse	IG 6-14

Line-of-Duty Deaths 1994 to 2002	IG 6-22

Summary	IG 6-30

audiovisual

Slides 6-1 to 6-84

| Slide 6-1 |  |  |
| --- | --- | --- |
| 135 min. |  |  |
| Lecture/Discussion |  |  |
| Slide 6-2 |  |  |
|  |  | I. objectives (5 min.) |
|  |  | The students will: |
|  |  | A. Given examples of different types of structures and different fire loads, predict the time of collapse. |
|  |  | B. Discuss the impact of the primary factor, construction, on line-of-duty deaths (LODDs). |
| Slide 6-3 |  |  |
|  |  | II. Wood Frame (55 min.) |
|  |  | A. Predicting a potential structural collapse is one of the most challenging tasks facing an Incident Commander (IC) at a fire scene. |
|  |  | B. Usually the lack of information on the construction of the building, fire size, fire location, fire burn time, condition of the building, fuel load, etc., makes the task nearly impossible. |
|  |  | C. Phoenix, AZ, example: |
|  |  | 1. A series of fire tests were conducted to collect data for a project examining the feasibility of predicting structural collapse. |
|  |  | 2. The fire test scenario was selected as part of a training video being prepared by the Phoenix Fire Department. |
|  |  | 3. Multiple fires were started in each structure to facilitate collapse; the fires were not intended to test the fire endurance of the structures. |
|  |  | 4. Four structures with different roof constructions were used for the fire tests. |
|  |  | a. Some were scheduled for demolition and other structures were built specifically for the fire tests. |
|  |  | b. Each structure was allowed to burn until some portion of the structure collapsed. |
|  |  | c. These structures were constructed for test experiments in order to examine several issues related to firefighter health and safety. |
|  |  | d. Temperatures were measured as a function of time in four locations within each structure. |
|  |  | e. Furniture items were placed in the front and back of each structure to simulate living room and bedroom areas. |
|  |  | f. The living room and bedroom areas of each structure were ignited simultaneously using electric matches. |
|  |  | g. Peak temperatures obtained during the tests ranged from approximately 800 °C (1,470 °F) to 1,000 °C (1,800 °F). |
|  |  | 5. The roof of each structure collapsed approximately 17 minutes after ignition. |
|  |  | D. The fire department in Phoenix conducted a series of live fire training exercises in various structures in an effort to better educate firefighters about structural collapse. |
|  |  | 1. The first goal of the tests was to obtain temperature data from a burning structure during a collapse. |
|  |  | 2. Second, various techniques and tools were evaluated for use in predicting structural collapse. |
|  |  | Specifically, the use of thermal imaging techniques was examined as a means to predict collapse. |
|  |  | 3. In addition, the exterior of the building was observed prior to and during collapse to identify any visual indicators of impending collapse. |
|  |  | 4. In collaboration with the Phoenix Fire Department, researchers from the National Institute of Standards and Technology (NIST) provided measurement support during the fire tests. |
|  |  | 5. These tests were not designed to evaluate the fire endurance of wood trusses, gypsum wallboard, wood studs, or any other structural elements used in the construction of the four structures. |
|  |  | 6. The fire scenario used in this study was designed to reach flashover conditions rapidly to force a partial or complete collapse of the structure. |
|  |  | E. Using video and data obtained from two different fire test series, the fire department developed a set of three videotapes dealing with fireground command and collapse issues. |

|  |  | This unit presents the results obtained from a set of tests that were conducted in single-story wood-frame residential structures. |
| --- | --- | --- |
|  |  | F. Many factors influence the failure of structural elements including the load on the element, protection of the element, fire intensity, and fire duration. |
|  |  | 1. More or less time may be available before failure of the element depending on the particular fire scenario. |
|  |  | 2. The fire resistance and ability of a structural member to maintain its load during an actual fire are influenced by a number of factors. |
|  |  | 3. Many of these factors can vary significantly depending on the specific fire scenario. |
|  |  | 4. Fire-resistance ratings, and measured or predicted times to failure or collapse should not be relied on as absolute indicators of time available for operating on or within a burning structure. |
|  |  | 5. Each structure must be evaluated and reevaluated during the fire to determine whether or not it is safe for firefighters to remain within any potential collapse zone. |
| Slide 6-4 |  |  |
|  |  | G. Asphalt shingles over oriented strand board (OSB) structure was used in this test. |
|  |  | H. Photograph showing the front (Side A) and (Side D) of test structure with asphalt shingle roof over OSB prior to start of test. |
| Slide 6-5 |  | 1. Two firefighter mannequins were placed on the roof. |
|  |  | 2. Each firefighter weighed 280 pounds with gear. |
|  |  | 3. Air-conditioning unit weighed 500 pounds. |
|  |  | I. Photograph showing rear (Side C) and (Side B) of test structure with asphalt shingle roof over OSB prior to start of test. (Note air-conditioning unit.) |
| Slide 6-6 |  |  |
|  |  | J. Plan view of generic test structure showing approximate placement of furniture and other items within structure (not to scale), and showing locations of measurement instruments and dimensions (all dimensions in meters). |
| Slide 6-7 |  |  |
|  |  | K. Diagram showing typical construction of the structure. |
|  |  | 1. Two rooms with attic space. |
|  |  | 2. Interior walls were constructed of 2- by 4-inch wood studs with 1/2-inch gypsum board nailed to each side. Doorways were 2.9 by 6.5 feet. |
|  |  | 3. Interior walls and ceilings had 1/2-inch gypsum board nailed to studs. All joints and nails were taped and covered with joint compound. |
|  |  | 4. Exterior walls were composed of 2- by 4-inch wood studs on 1.3-foot centers, covered with 5/8-inch wood siding. |
| Slide 6-8 |  |  |
|  |  | L. Photograph of "living room" area showing furniture arrangement. |
|  |  | Note: Ignition point will be on couch under paper. |
|  |  | 1. Room contained a couch, love seat, and two chairs consisting of wood frames with polyurethane foam cushioning material. |
|  |  | 2. Two wooden end tables, coffee table, and two table lamps were also placed in the living room. |
|  |  | 3. A hollow core wood door was mounted in the doorway opening. |
| Slide 6-9 |  |  |
|  |  | M. Photograph showing "living room" with furniture, cabinets, and thermocouple array. |
|  |  | 1. Two walls in living room were covered with 1/8-inch press board paneling attached with adhesive and brads. |
|  |  | 2. Two living room walls were covered with gypsum board. |
|  |  | 3. In the living room, the wall adjacent to the front wall had an approximate 4- by 3-foot section removed to let air into the room. |
|  |  | 4. A piece of plywood on a slide track allowed it to be moved back and forth to regulate the flow of air into the room. |
| Slide 6-10 |  |  |
|  |  | N. Photograph showing "living room" ceiling with plastic vent to help facilitate fire into the attic space, electrical box is noted, and a 2- by 2-1/2-foot attic access is cut into the ceiling and covered with 1/2-inch drywall with molding strips holding it in place. |
| Slide 6-11 |  |  |
|  |  | O. Photograph showing furniture in "bedroom" area of test structure. A thermocouple array is visible in the foreground. |
|  |  | 1. The bedroom walls were covered with 1/8-inch press board and paneling attached with adhesive brads. |
|  |  | 2. One wall in the bedroom had 4- by 3-foot high section removed and the rear wall had a 3- by 3-foot window with glass. |
|  |  | 3. A piece of plywood on a slide track allowed it to be moved back and forth to regulate the flow of air into the room. |
| Slide 6-12 |  |  |
|  |  | P. Photograph showing two double beds, nightstands, and lamps in the "bedroom." |
|  |  | 1. Bedroom contained two sets of foam mattresses and box springs on metal frames. |
|  |  | 2. Wooden nightstands were placed adjacent to each bed. |
|  |  | 3. Two wooden dressers were located in the room. |
|  |  | 4. One dresser was located along the wall opposite the end of the two beds while the second dresser was adjacent to the side of the second bed. |
|  |  | 5. A chair with polyurethane padding on a wood frame was positioned in the bedroom diagonally opposite the end corner of the first bed. |
|  |  | 6. Table lamps were placed on top of the two nightstands. |
|  |  | 7. Both rooms had nylon wall-to-wall carpet laid over on the floor over a 3 pound pad. |
| Slide 6-13 |  |  |
|  |  | Q. Photograph showing roof structure. A thermocouple array is also visible. |
|  |  | 1. Gable ends were studded with 2- by 3-inch lumber and covered with 1/2-inch high density fiberboard. |
|  |  | 2. Trusses were nailed on each end and attached to the top plate with metal hurricane ties. |
|  |  | 3. Structure had 2- by 6-inch boards nailed to the truss tails, and the tails on the trusses were cut off at 1.7 feet. |
|  |  | 4. The outside rafts were nailed to 2- by 4-inch boards and attached to the second truss at 4-inch intervals. |
| Slide 6-14 |  |  |
|  |  | R. Photograph of "living room" showing ignition location in corner of couch under newspaper. |
| Slide 6-15 |  |  |
|  |  | S. Photograph showing "bedroom" with ignition location in corner of chair under the newspaper. |
| Slide 6-16 |  |  |
|  |  | Point out roof construction asphalt shingles over OSB. Point out electric match ignition to chair in bedroom (Sides B and D). |
|  |  | For the following slides, point out the burn time scale. |
|  |  | T. Timeline. |
| Slide 6-17 |  |  |
|  |  | 1. Photograph of test structure at "1:00 minute" after ignition. |

| Slide 6-18 |  |  |
| --- | --- | --- |
|  |  | 2. Photograph of test structure at "1:15 minutes" after ignition. |
| Slide 6-19 |  |  |
|  |  | 3. Photograph of test structure at "2:00 minutes" after ignition. |
| Slide 6-20 |  |  |
|  |  | 4. Photograph of test structure at "3:00 minutes" after ignition, living room reached flash over temperature. |
| Slide 6-21 |  |  |
|  |  | 5. Photograph Side D fire increasing in living room at "4:00 minutes." |
|  |  | Point out fire now showing Side D. Flashover has occurred. Heavier smoke conditions showing on Side B. |
| Slide 6-22 |  |  |
|  |  | 6. Photograph showing conditions on Side D--fire begins diminishing in living room at "5:00 minutes." Most likely a result of the falling ceiling causing a temporary decrease in temperatures in living room. Side B maintains heavy smoke conditions. |

| Slide 6-23 |  |  |
| --- | --- | --- |
|  |  | 7. Photograph showing conditions on Side D--fire and temperatures still diminishing in living room at "6:00 minutes." Side B maintains heavy smoke conditions. |
| Slide 6-24 |  |  |
|  |  | 8. Photograph of test structure at "7:00 minutes" after ignition. |
| Slide 6-25 |  |  |
|  |  | 9. Photograph showing temperatures increasing on Sides B and D. Fire appears to have entered the attic space at approximately "8:00 minutes." |
| Slide 6-26 |  |  |
|  |  | 10. Photograph of test structure at "9:00 minutes" after ignition. |
| Slide 6-27 |  |  |
|  |  | 11. Photograph of test structure at "10:00 minutes" after ignition. |
| Slide 6-28 |  |  |
|  |  | 12. Photograph of test structure at "11:00 minutes" after ignition. |
|  |  | Point out that conditions are not showing much change from the 9:00-minute mark. |
| Slide 6-29 |  |  |
|  |  | 13. Photograph of test structure at "12:00 minutes" after ignition. Fire now extending to roof/attic area Side A. |
| Slide 6-30 |  |  |
|  |  | 14. Photograph of test structure at "13:00 minutes" after ignition; increased fire showing from Sides A-B-D with fire extending into roof/attic area on Sides A and D. |
| Slide 6-31 |  |  |
|  |  | 15. Photograph of test structure at "14:00 minutes" after ignition; increased temperatures and volume of fire all sides and roof area. |
| Slide 6-32 |  |  |
|  |  | 16. Photograph of test structure at "15:00 minutes" after ignition; front door burned away with portion of roof structure burning. Structure is fully involved. |
| Slide 6-33 |  |  |
|  |  | 17. Photograph of test structure at "16:00 minutes" after ignition as internal collapse has started. Roof/Attic area is fully involved. |
| Slide 6-34 |  |  |
|  |  | 18. Photograph of test structure at "17:00 minutes" after ignition; roof collapse has begun. |
| Slide 6-35 |  |  |
|  |  | 19. Photograph of test structure at "19:00 minutes" after ignition; full roof collapse. |
| Slide 6-36 |  |  |
|  |  | iii. ordinary warehouse (40 min.) |
|  |  | A. Overview. |
|  |  | 1. Two fire tests were conducted in a warehouse located in Phoenix to develop data for evaluation of a methodology for predicting structural collapse. |
|  |  | 2. A firewall was constructed to divide the warehouse into two fire compartments. |
|  |  | 3. Temperatures were measured as a function of time in three locations during the first test and in two locations during the second test. |
|  |  | 4. In addition, the volume fraction of carbon monoxide was measured at selected locations during each test. |
|  |  | 5. Stacks of wood pallets were used as the primary fuel source and were ignited using paper and an electric match. |
|  |  | 6. Some combustible debris and building structural elements provided the remainder of the fuel load. |
|  |  | 7. Peak temperatures obtained at different elevations ranged from approximately 300 °C (570 °F) to 800 °C (1,470 °F). |
|  |  | 8. Peak carbon monoxide volume fraction reached 4 percent in the first test and 5 percent during the second test. |
|  |  | 9. The roof of the front half of the structure burned through approximately 18 minutes after ignition of the fire for the first test. |
|  |  | 10. The roof of the back half of the structure burned through about 15 minutes after the start of the second test. |
| Slide 6-37 |  |  |
|  |  | B. Elevation views showing details of wood trusses supporting the roof of the warehouse. |
|  |  | 1. Building was a single story with peaked roof composed of rolled roofing material laid over 2- by 8-inch boards supported on wood trusses. |
|  |  | 2. Trusses used 2- by 12-inch lumber for the top and bottom chord and various lumber sizes for the web members. |
|  |  | 3. Trusses were spaced 15 feet apart and oriented perpendicular to the long dimension of the building. |
|  |  | 4. Peak of the roof was 18 feet above the floor with the bottom chord of truss located 10 feet above the floor. |
|  |  | 5. Trusses rested on bearing walls composed of brick and block. |
|  |  | C. Test #1. |
| Slide 6-38 |  |  |
|  |  | 1. Plan view of warehouse showing layout and dimensions (dimensions in meters). |
|  |  | a. Arrow shows wall separating two warehouse sections. |
|  |  | - Front entrance Side A and front section are Test Area #1. |
|  |  | - Fifty- by 90-foot roll up door Side C and the rear section are Test Area #2. |
|  |  | - Forty-five- by 50-foot warehouse was separated into two halves with a wall constructed of 2- by 4-inch wood studs with 1/2-inch plywood nailed to one side. |
|  |  | b. Plywood was covered with two layers of 5/8-inch fire-rated gypsum board. |
|  |  | c. Separation wall was built along a truss 90 feet from the front of the building. |
|  |  | d. Front half of building was 50-feet wide and 90-inches long overall. |
|  |  | e. Area had been subdivided into smaller areas. |
|  |  | f. Primary separation wall was composed of gypsum board on wood studs and located along the truss 60 feet from the front of the building. |
|  |  | g. Wall went only to the underside of the bottom chord of the truss. |
|  |  | h. A door 7.5-feet high by 6-feet wide, was located in this wall 12 feet from the east wall. |
|  |  | i. There were several holes cut through the wall at the 10 foot level. |
|  |  | j. Two small rooms were also located along the east wall. |
|  |  | k. One room served as an office, the other room served as a restroom. |
|  |  | 2. Plan and elevation views showing details of wood trusses supporting the roof of the warehouse. |
|  |  | 3. Top view--elevation view of wall dividing front and rear sections. Plywood/Gypsum wall materials on 2- by 4-inch studding separating sections. |
| Slide 6-39 |  |  |
|  |  | 4. Bottom view--elevation view showing front wall (Side A) of area used for first test. |
|  |  | a. View of front wall of the warehouse structure had a doorway approximately 6-feet wide and 10-feet high, located 13 feet from east wall. |
|  |  | b. A glass window 7.7-feet high and 28-feet wide was located in the front wall approximately 21 feet from east wall. The sill of the window was 2.3-feet high. |
|  |  | c. The first burn area was 50 by 90 feet with a single entrance in the front wall. |
|  |  | d. The second burn area was 45 by 50 feet with a single doorway in the east wall. The doorway was 8-feet wide and 8-feet high. It was located 96 feet from the front wall of the building and 6 feet from the separating wall. |
| Slide 6-40 |  |  |
|  |  | 5. Plan view of warehouse showing locations of measuring instruments and fuel packages. |
|  |  | Point out pallet locations on view. One set of three and one single stack for first burn area. |
| Slide 6-41 |  |  |
|  |  | 6. Photograph showing the placement of wood pallets in the front part of the warehouse used for the first test. |
|  |  | a. Warehouse dimensions: |
|  |  | - One story--50 by 135 feet. |
|  |  | - Ordinary construction. |
|  |  | - Peaked roof. |
|  |  | - Supported on wood trusses. |
|  |  | - Front section 50 by 90 feet. |
|  |  | - Rear section 45 by 50 feet. |
| Slide 6-42 |  |  |
|  |  | 7. Photograph of front of warehouse with flames visible through front door at the start of the first test. Warehouse closed tight "40 seconds" after ignition. |
|  |  | Point out burn time scale for the following slides. |

| Slide 6-43 |  |  |
| --- | --- | --- |
|  |  | 8. Photograph of smoke coming from roof top ventilator during the first test. Heavy smoke from ventilator; flames reached warehouse ceiling level in "1:30 seconds" after ignition. |
| Slide 6-44 |  |  |
|  |  | 9. Photograph of roof area showing flames coming from ventilator during first test. Roof area around ventilator began to burn between the "3:00 and 4:00 minute" mark after ignition. Inside temperatures are at flashover conditions at the "4:00 minute" mark. |
| Slide 6-45 |  |  |
|  |  | 10. Photograph showing smoke coming from roof and front of warehouse as fire is becoming oxygen deficient during the first test in "5:00 minutes" after ignition. Front window broken at the "8:00 minute" mark to allow oxygen into building. |
| Slide 6-46 |  |  |
|  |  | 11. Photograph showing fire involving significant portion of roof structure with partial collapse beginning during first test in "11:00 minutes." |
| Slide 6-47 |  |  |
|  |  | 12. Photograph showing the warehouse at the conclusion of the first test with the roof structure over the front half completely gone. Portions of the roof collapse at the "18:00 minute" mark. Full collapse in "18:20 minutes" after ignition. |
|  |  | Front wall collapses at "19:20 minutes." |
| Slide 6-48 |  |  |
|  |  | D. Test #2. |
|  |  | 1. Structural collapse fire test--single-story ordinary construction warehouse. |
| Slide 6-49 |  |  |
|  |  | 2. Plan view of warehouse showing layout and dimensions (dimensions in meters). |
|  |  | a. Arrow shows wall separating two warehouse sections. |
|  |  | b. Second burn area was 45 by 50 feet with a single doorway in the east wall. |
|  |  | c. The doorway was 8-feet wide and 8-feet high. |
|  |  | d. It was located 96 feet from the front wall of the building and 6 feet from the separating wall. |
| Slide 6-50 |  |  |
|  |  | 3. Plan and elevation views showing details of wood trusses supporting the roof of the warehouse. |
| Slide 6-51 |  |  |
|  |  | 4. Top view--elevation view of wall dividing front and rear sections. Plywood/Gypsum wall materials on 2- by 4-inch studding separating sections. |
| Slide 6-52 |  |  |
|  |  | 5. Bottom view--elevation view showing front wall (Side A) of area used for first test. |
|  |  | 6. Plan view of warehouse showing locations of measuring instruments and fuel packages. |
|  |  | a. Point out pallet set used for second test. |
|  |  | b. Point out doorway on Side B. |
| Slide 6-53 |  |  |
|  |  | 7. Photograph showing placement of wood pallets and adjacent thermocouple tree in the front portion of the warehouse used for the second test. |
| Slide 6-54 |  |  |
|  |  | 8. Photograph of fully-involved fuel package through open door after the start of the second test. Roll up door view, fire is free burning in "2:30 minutes." |
|  |  | Point out burn time scale for the following slides. |
| Slide 6-55 |  |  |
|  |  | 9. Photograph of smoke coming from open door during second test in "3:15 minutes" after ignition. Flashover temperatures are reached at the "4:00 minute" mark. |
| Slide 6-56 |  |  |
|  |  | 10. Photograph of smoke coming from open door during second test in "6:30 minutes." |
| Slide 6-57 |  |  |
|  |  | 11. Photograph of rooftop ventilator after it collapsed due to severe heat exposure during second test in "9:00 minutes" after ignition. |
| Slide 6-58 |  |  |
|  |  | 12. Photograph showing flames through open door and burning roof during the second test. Fire showing through portions of roof in "13:30 minutes." |
| Slide 6-59 |  |  |
|  |  | 13. Photograph showing partial collapse of the rear and side walls after the second test in "15:20 minutes." At "16:30 minutes" additional portions of the roof collapsed. |
| Slide 6-60 |  |  |
|  |  | iv. line-of-duty deaths 1994 to 2002 (30 min.) |
|  |  | A. Overview. |
|  |  | 1. As part of a project funded by the U.S. Fire Administration (USFA), the Building and Fire Research Laboratory (BFRL) at the National Institute of Standards and Technology (NIST) is exploring the feasibility of developing a system for use by firefighters to predict structural collapse during fireground operations. |
|  |  | a. Predicting a potential structural collapse is one of the most challenging tasks facing an IC at a fire scene. |
|  |  | b. Usually the lack of information on the construction of the building, fire size, fire location, fire burn time, condition of building, fuel load, etc., makes the task nearly impossible. |
|  |  | 2. They examined records to determine if there were any trends or patterns that could be detected in firefighter fatalities due to structural collapse. |
|  |  | 3. If so, these trends could be brought immediately to the attention of training officers and ICs and investigated further to determine probable causes. |
|  |  | 4. This study examined data from structural collapse incidents that occurred from 1994 to 2002 involving one or more firefighter fatalities in which fire weakened to failure structural members of a building resulting in the complete or partial collapse of any part of the structure, excluding the cases described above. |
|  |  | 5. Several data parameters were analyzed for each incident, including |
|  |  | a. The time of the incident. |
|  |  | b. The property type of the structure. |
|  |  | c. The firefighter's age. |
|  |  | d. Years of experience. |
|  |  | e. Status (i.e., career or volunteer). |
|  |  | f. Nature and cause of death. |
|  |  | g. Activity at the time of death. |
|  |  | 6. In the case of rank, individuals ranked higher than captain were designated chief officers. Any dual-trained firefighter (e.g., firefighter/paramedic) was designated as firefighter. |
|  |  | 7. The 1994 to 2002 data were also compared to results reported in two earlier National Fire Protection Association (NFPA) reports addressing firefighter fatalities due to structural collapse. |
|  |  | 8. These reports analyzed data from incidents that occurred from 1979 to 1988 and 1983 to 1992, respectively. |
|  |  | 9. The raw data from these reports were not available. |
|  |  | 10. In order to make comparisons, several categories of the data were restructured for the more recent years in order to fit the criteria established in the NFPA reports. |
|  |  | a. For instance, in the historical reports the category for cause of death only had the two parameters "caught or trapped" and "struck by contact with object." |
|  |  | b. Data collected for the years 1994 to 2002 included "exposure" and "fell or jumped" as additional causes of death (as designated by the USFA Memorial database). |
|  |  | c. Cases that belonged to the latter two categories were realigned with the criteria established in the NFPA reports. |
|  |  | d. For example, the cause of death of a firefighter who fell through a floor during a fire attack and was unable to escape or be rescued would be designated in the USFA database as "fell or jumped." |
|  |  | e. To compare this case to the NFPA reports the cause of death would be reassigned to "caught or trapped." |
|  |  | B. Between the years 1979 and 2002 there were over 180 firefighter fatalities due to structural collapse, not including those firefighters lost in 2001 in the collapse of the World Trade Center Towers. |
|  |  | 1. Structural collapse is an insidious problem within the firefighting community. |
|  |  | 2. It often occurs without warning and can easily cause multiple fatalities. |
| Slide 6-61 |  |  |
|  |  | C. Between the years 1994 and 2002 there were 63 deaths caused by structural collapse in a total of 47 fires. |
| Slide 6-62 |  |  |
|  |  | 1. Of these deaths, over two-thirds occurred within the first 6 months of the year and over one-half occurred in the first 3 months. |
| Slide 6-63 |  |  |
|  |  | 2. Over 42 percent of deaths with known incident times occurred in the first eight hours of the day (12:00 a.m. to 7:59 a.m.). |
| Slide 6-64 |  |  |
|  |  | 3. Victims of structural collapse were part of several age groups, experience levels, and ranks, and were involved in several activities. |
|  |  | a. Fatalities by age. |
| Slide 6-65 |  |  |
|  |  | b. Fatalities by years of experience. |
|  |  | - Over 44 percent of deaths involved career firefighters with 6 or more years of experience. |
| Slide 6-66 |  |  |
|  |  | c. Fatalities by rank. |
| Slide 6-67 |  |  |
|  |  | d. Fatalities by activity. |
|  |  | Point out the different categories. Is this a trend to unsupervised activities? |
|  |  | - A majority of the victims (over 65 percent) were involved in fire attack or advancing hose. |
| Slide 6-68 |  |  |
|  |  | e. Fatalities by nature of death. |
|  |  | - The nature of firefighter deaths in collapsed structures is categorized as asphyxiation, burns, internal trauma, and other causes. |
|  |  | - Over 42 percent of fatalities (27 deaths) were by asphyxiation. |
| Slide 6-69 |  |  |
|  |  | f. Fatalities by activity and duty type. |
| Slide 6-70 |  |  |
|  |  | g. Fatalities by cause of death and duty death. |
| Slide 6-71 |  |  |
|  |  | h. Fatalities by month and rank. |
| Slide 6-72 |  |  |
|  |  | i. Fatalities by activity and cause of death. |
| Slide 6-73 |  |  |
|  |  | j. Fatalities by cause of death and property type. |
| Slide 6-74 |  |  |
|  |  | k. Fatalities by cause of death and nature of death. |
| Slide 6-75 |  |  |
|  |  | l. Fatalities by property type and status. |
| Slide 6-76 |  |  |
|  |  | m. Fatalities by activities and status. |
| Slide 6-77 |  |  |
|  |  | n. Fatalities by activity and property type. |
| Slide 6-78 |  |  |
|  |  | o. Fatalities by month and nature of death. |
| Slide 6-79 |  |  |
|  |  | D. Fatalities due to structural collapse per year (1979 to 2002). |
|  |  | Point out categories--decrease from 1989 to 1990. (Why?) Possibly enactment of NFPA 1500. |
| Slide 6-80 |  |  |
|  |  | 1. Fatalities by cause of death. |
| Slide 6-81 |  |  |
|  |  | 2. Fatalities by fixed property type. |
|  |  | Point out categories (1979 to 1988): |
|  |  | Stores/Offices--32 percent Residential--15 percent |

| Slide 6-82 |  |  |
| --- | --- | --- |
|  |  | Point out categories (1983 to 1992): |
|  |  | Stores/Offices--32 percent Residential--13 percent |
| Slide 6-83 |  |  |
|  |  | Point out categories (1994 to 2002): |
|  |  | Stores/Offices--21 percent Residential--51 percent |
| Slide 6-84 |  |  |
|  |  | v. SUMMARY (5 min.) |
|  |  | A. In this lesson you were shown burn times and the importance of understanding burn time for crew safety. |
|  |  | B. You must always consider the length of time a structure has been burning when you arrive on the scene. |
|  |  | C. The LODD statistics provided with compelling information regarding how and where firefighters become fatalities. Use the information you learned to keep you and your crew safe. |