UNIT 7:

FIREGROUND DECISION MAKING EXERCISES

objectives

The students will:

1.	State the objectives when confronted with a Type I, II, III, IV, or V occupancy when conducting an analytical sizeup of the structure.

2.	State the strategies when confronted with a Type I, II, III, IV, or V occupancy when conducting an analytical sizeup of the structure.

3.	State the tactics when confronted with a Type I, II, III, IV, or V occupancy when conducting an analytical sizeup of the structure.

4.	Identify and write the findings from the walkaround in Columns 1, 2, and 3 of the Primary Factors Chart.

5.	As a group, prepare a briefing of the findings from the Initial Company Officer (ICO), the Initial Company Officer Assistant (ICOA), the Initial Incident Safety Officer (IISO), Initial Planning Section, and Initial Logistics Section.

6.	Given a simulated incident where building construction is a critical issue, develop a sizeup report of fireground conditions, complete the Primary Factors Chart, and document progress using the Command Sequence Tactical Chart.

This page intentionally left blank.

building construction classifications

Basic knowledge of how buildings are constructed and how they will react when subjected to fire is of vital importance to every Company Officer (CO).  A building that is on fire is physically being destroyed.  The mission of the CO is to determine if it is possible to enter the structure, how long the structure can last under fire conditions, what strengths the building can offer, or what weaknesses may exist.  Each type of building will react differently under fire conditions, and certain predictions can be made.  As an example, a wood-frame building with a lightweight roof structure of truss rafters can be expected to lose its structural strength faster than a building with a roof structure of standard rafter and ridge board design.

Construction Classifications

Generally, it is not possible to distinguish construction classifications from the exterior of a building.  Most often, even the experienced observer must look at the bearing members (wall, floor, and roof assemblies) in order to tell the construction classification.  At the end of each section describing different construction classifications we will provide a brief summary describing cues and rules of thumb that correspond with that classification.

Fire Resistive

Fire resistive is a method of construction where all key structural elements that hold the building up will withstand normal fire conditions for a minimum of 3 hours.  The structural elements generally will be reinforced concrete or steel with a fire protection covering applied.  In addition, the floors will be fire resistive and designed to limit fire spread.  This type of structure has demonstrated fire after fire that it can withstand complete devastation of the contents and still remain structurally sound.  While the structural components are a very positive feature, there may be some concerns that a CO should be aware of.  As an example, fire in many buildings may spread from floor to floor at the area where the outer wall of the building attaches to the floor segments.  Many designs provide for a space between the floor and the wall.  This area may be closed off with insulation or may be totally open.  In addition, the windows are very often the vehicle for fire spread with fire leaping from floor to floor.  To manage this problem some architects have staggered the windows or placed eyebrows over the tops of the windows.

Fire-resistive cues:  Bearing members are either reinforced, poured, or prestressed concrete assemblies or skeletal steel with the steel protected by sufficient layers of drywall or a sprayed-on, fire-resistive coating.  Special fire suppression problems for COs would include open floor plans that have large open areas without separations or compartmentation; limited opportunities for ventilating; and high heat levels inside the structure.

Noncombustible

Noncombustible is a method of construction where the structural components will not burn, but may be susceptible to early collapse under fire conditions.  The walls may be constructed of steel or masonry with steel floor and roof structure.  This steel will be unprotected from the products of combustion and may be vulnerable to early failure.  This method of construction is very popular in commercial or industrial structures.  While the structural elements will not contribute to the fuel load, unprotected steel will expand as it warms and eventually it will not be able to support itself.  As it expands, it has the capacity to push walls or to twist and collapse, and may drop the structural members that it was supporting.

The strength of this construction is in the load-carrying capacity and the long areas that it can span without support posts.  It is an easily constructed type of building with large steel beams or trusses put in place with cranes.  Steel is attached easily to other components by bolting, riveting, or welding, and a frame can be assembled quickly.  The weakness of this construction is in the reaction to fire conditions, where the steel expands and weakens with the potential for collapse.  This building generally is considered a candidate for early deterioration under fire conditions.  A CO must pay close attention to this classification in order to ensure the safety of firefighters.

Noncombustible cues:  Bearing members are made of noncombustible materials such as metal, concrete, stone, etc.  Most often these buildings are skeletal steel assemblies where the steel is exposed and unprotected from the effects of fire.

Heavy Timber

Heavy timber (mill) is a method of construction using substantial wood structural elements for floor and roof supports and with masonry exterior walls.  This method was used heavily in the northeastern United States to construct mills.  The mills often would be built near natural sources of waterpower, and would be constructed up to six stories in height.  As the exterior walls were masonry, they would be wider at the bottom than at the top.  As the walls rise, they diminish in size, since they will be carrying less load.  In many of these structures the walls at the ground floor could be up to 36-inches thick.  The structural timber would be a minimum of 8 by 8 inches, and they were spaced based upon the load they would be supporting.  Under fire conditions, the floors and support timbers burn slowly and remain strong for a considerable time.  In many of these structures the floors were several inches thick to support heavy machinery and goods.  To minimize water damage to floors below, the floors would be equipped with scuppers.

The strength of this type of building is in the size of the wooden structural members that held the floor and roof in place, in addition to the masonry walls.  While these buildings often had firewalls with fire doors, they also posed massive fire problems due to large open areas with heavy fire loads, oil-soaked floors, and large quantities of combustible stock.  In general, this classification is considered a strong building to work in during fire conditions, but one in which fire can quickly surpass a fire department's ability to suppress.

Heavy-timber (mill) cues:  These buildings have masonry walls.  The floors and roof assemblies are wood.  The wooden members are much larger than nominal lumber sizes.  Look for a minimum of 4- by 6-inch wood joists, 6- by 8-inch wood columns, and thick floor decking.

Ordinary

Ordinary construction has been termed "Main Street USA."  This type of building also gained the name "taxpayer" because often the owner would operate a store on the first floor and live on the second floor.  The business would pay the taxes on the property and the utilities, while the owner lived in the building virtually free.  This building has masonry exterior walls, and the floors and roof are wood joist.  The structural members for floor joists and roof rafters were often 3 by 10 inches and typically would span 12 to 14 feet, supported by a post-and-beam arrangement for interior walls.  Since many of the streets on which these buildings were constructed were narrow, an effort had to be made to limit collapse of the masonry wall.  A technique called "fire cut" would be used, where the end of the floor joist or rafter going into a bearing wall would be cut on an angle so that the bottom of the rafter or joist would be longer than the top.  The idea was that when the wood member burned off on the inner portion of the structure it would pull out of the wall and fall into the structure, rather than lifting the wall directly above it and pushing the masonry wall into the street.  Through the years these buildings typically were renovated several times with ceilings dropped, new voids created for new plumbing fixtures, and walls removed between occupancies in order to expand floor space.

The strength of these structures is in the masonry walls and the full-dimension lumber used to construct the floor and roof components.  In addition, the floor and roof elements were installed with a fire cut so that they could drop out of the walls without bringing the walls into the street.  The number of renovations that the building has had often will cause unexpected fire travel and multiple-floor involvement.

Ordinary (masonry/wood joist) cues:  These buildings have masonry walls.  The floor and roof assemblies are wooden.  The floor joists often sit in the masonry walls in sockets that hold the joist ends.  To determine whether or not the joists have been fire cut, normally one must go to the basement level and examine the first floor joists where they sit in the wall sockets.

Wood Frame

Wood frame is a method of construction where the structural components are framed out of wood.  The use of a combustible structural element poses a special concern as it will lose its load-carrying capacity as it burns; eventually gravity will take over and pull whatever it was supporting to the ground.

Post and beam is a method of wood-frame construction and typically is used in barn construction.  A modern method is called "pole barn" construction, where large pressure-treated poles are set into the ground and the framework of the building is hung from these poles.  The poles themselves will last considerably longer under fire conditions than the materials used for the roof or walls.

Balloon is a method of wood-frame construction that was popular when long structural materials were available.  The common characteristic of this type of building is that the wall studs extend from the foundation of the structure to the roof.  When it was time to attach the second floor, the floor joists were simply nailed to the wall studs.  This created an open area the entire length of the wall's studs, and across the floors to the opposite side of the building as well.  If a fire got into the walls, ceiling, or floor space it was free to go wherever it pleased.  Firefighters often tell of being inside the basements of these buildings and shining a light at the foundation with a fellow firefighter in the attic reporting that he/she saw the light shining through.  Generally, the interior walls were constructed of wood lath over wood studs with plaster attached; the lath was said to resemble kindling, and was arranged in a very desirable manner for rapid fire extension up through a stud channel. This structure typically used full-dimension lumber and had close spacing of structural elements.  If a CO was not reading the building correctly, he could become fooled quickly as the fire worked the building in all directions.  Extension must be checked aggressively in this type of structure.

Platform is an open method of construction that has been popular since the late 1940s.  The structure is built one floor or story at a time.  Each floor has a floor deck, sill plate, wall studs, and a plate at the top of the wall.  For a fire to travel from one floor to another through the walls, it has a great deal of material in its path to burn through.  More often a fire will find another route of extension.  Fire may extend via ventilation shafts for dryer, bathroom, and kitchen vents.  Areas around plumbing pipes or heating ducts also will be vulnerable to fire extension.  Generally the interior wall construction will use drywall material, which is inherently fire resistive and provides for compartmentation of a fire.  The weak components most often will be the floor or roof.

Lightweight methods of construction have become popular, with truss construction or a sandwich-beam method of floor or roof support.  Generally wooden trusses will be made of smaller dimension lumber, and are held together with metal gusset plate fasteners.  Under fire conditions the plates may loosen and the structural integrity of the entire component may be lost.  Another technique in use today is the process of ripping a 3/8- or 1/2-inch groove into a 2 by 4 and inserting a piece of 3/8- or 1/2-inch plywood.  The size of the plywood is dependent upon the area to be spanned.  These are commonly found in floor joists or as rafters on a flat or limited pitch roof, and are even becoming popular in strip mall construction. The strength of this method of construction is the floor-by-floor method of building.  A weakness may very well be the lightweight floor or roof design.

Wood-frame cues:  Wood-frame buildings normally have a masonry foundation with all floor, wall, and roof assemblies composed of nominal-sized lumber.  The great percentage of private, detached dwellings are wood-frame construction.

FIRE BEHAVIOR factors

An important part of your job will be the ability to make an accurate fire behavior prediction.  Understanding fire behavior factors will assist you greatly in determining what is happening and what is likely to happen.  They will have an impact on safety, strategy, and the use of resources.

Those factors are

    - heat release;

    - thermal stratification;

    - rollover;

    - flashover; and

    - backdraft.

Heat Release

Heat is described in several ways, all of which bear a definite relationship to each other.  In order to better understand the concept of heat, the following definitions are necessary:

      - British thermal unit (Btu):  One Btu is the amount of heat required to raise the temperature of 1 pound of water 1 °F (-17.2 °C) (when the measurement is performed at 60 °F (15.6 °C)).

      - Heat of combustion:  the amount of heat that will be released by a substance when it is completely consumed by fire.

A number of variables influence the output of heat from burning materials:

    - the amount of area of solid combustibles exposed to heat and oxygen (the state of subdivision);

    - the area of free surface of the liquid (in case of flammable substances, to give off vapor pressure); and

    - the conductivity of solids (wood, etc.), which can influence the amount of heat given off when materials burn.

Knowledge of the types of materials present in a given fire situation and their heat values is important, and can assist you in determining the amount of water to apply, as well as the behavior of other materials within the environment.

Even though the heat values (in Btus) of various materials are not precise, they provide us with necessary information for developing the concepts of "fireloading" and the heat absorption qualities of water.  Some examples of the heat of combustion values of various materials are shown in the following table.

| Materials | Btu/lb. |
| --- | --- |
| Asphalt | 17,150 |
| Cotton batting | 7,000 |
| Gasoline | 19,250 |
| Paper | 7,900 |
| Polystyrene | 18,000 |
| Polyvinyl chloride | 7,500 to 9,500 |
| Wood | 7,500 to 9,050 |

Thermal Stratification

Thermal stratification is the layering of heat in a given enclosed area.  The ceiling or upper area will be a higher temperature.  Floor covering materials are potentially less hazardous than ceiling or wall surfaces.

In the prefire inspection, you should not ignore the degree of combustibility of materials used throughout the occupancy.  In the MGM Grand Hotel fire, the use of plastic materials in ceiling areas dramatically affected the fire behavior.

The introduction of water through a nozzle will change the thermal stratification of the enclosed area rapidly.  In most cases, a thermal balance will occur following the introduction of water.  The temperature in the room will equalize.

If the thermal balance is disturbed, temperatures can be raised beyond the point where any victims trapped inside would have a chance of surviving.  Wide fogs applied into a room can turn into superheated steam, endangering both potential victims and firefighters.  Full protective clothing and self-contained breathing apparatus (SCBA) are a must for firefighters.

Rollover

The term rollover is used to describe the fire or flame front that often is observed rolling along in front of the materials that are actually burning.  As a combustible gas is produced and liberated from combustible materials it must mix with air (oxygen) in order to burn.  Since the material that is burning consumes tremendous amounts of air (oxygen) there may be a limited amount of air (oxygen) in the upper levels of the room to support combustion of all the fuel being produced.  This fuel-rich atmosphere will be pushed in front of the fire by the thermal column of heat from the fire and may not come within its flammable limits until several feet away from the main body of the fire.  This is especially true in confined areas such as hallways.  Often fire seems to be rolling along at ceiling level at a distance up to 10 to 20 feet ahead of the main fire.  What actually is being witnessed is a fuel-rich mixture being pushed well ahead of the fire; when it comes into its flammable limits (mixture of air and fuel gas) it burns.  This is often described as the fire rolling over.

Flashover

A very basic definition of flashover is the ignition of combustibles in an area heated by convection or radiation, or a combination of the two.  The combustible substances in a room are heated to their ignition points and almost simultaneous combustion of the material occurs.  In other words, the entire area is preheated to its ignition temperature and can become fully involved in fire in a matter of seconds.

Some of the warning signs of imminent flashover are intense heat, free-burning fire, unburned articles starting to smoke, and fog streams turning to steam a short distance from the nozzle.

To reduce the chance of flashover, temperatures need to be lowered quickly by ventilation and water application.

Backdraft

As a fire develops, the combustion process creates an atmosphere that is deficient in oxygen and can lead to the possibility of backdraft occurring.  This also is referred to as a smoke explosion.  The difference between flashover and backdraft is the amount of oxygen present.  In flashover there is adequate oxygen available for combustion, and the fire is free-burning prior to flashover.  In backdraft, there is insufficient oxygen for active burning, and the fire is smoldering.  It is an oxygen-deficient atmosphere.

Normally, sufficient oxygen is present during most fires so that the conditions leading to backdraft are minimized.  However, when oxygen is depleted and the fire begins to smolder, an oxygen-deficient atmosphere is created in the fire area.  When conditions like this develop, gases such as carbon monoxide and carbonaceous-particle smoke or suspensions are produced.  These are capable of reacting with oxygen.

This poses an explosion threat if oxygen is allowed to enter the structure improperly.  The accumulated gases will ignite readily, spreading fire or causing a violent explosion.  Due to temperatures in the room, the fuel is evolving into ignitable vapors at or above their ignition temperatures.  All that is needed is oxygen to complete the fire triangle.

When backdraft conditions are present and oxygen is introduced before the inside pressure is relieved, the resultant explosion can eliminate firefighters and their hoses.  The potential for backdraft exists in buildings, rooms, attics, or any other confined space.  An indication of backdraft is when a fire has depleted the oxygen content in an area, yet has preheated that space above the ignition temperatures of the combustibles in it.  Another indicator is hot, heavy smoke issuing from the building (smoke is sometimes described as lazy, or sick-looking).  This may be accompanied by dark carbonization on the window glass.  In this situation, the building may seem to be breathing (drawing smoke back in the opening followed by expelling smoke from the opening).  Backdrafts may occur during the incipient phase as well as the smoldering phase.

Ventilation is the first priority and must precede fire attack under backdraft conditions.

fire travel predictions

Heat and Smoke Travel

Checking fire extension requires a knowledge of how fire spreads, along with a knowledge of building construction features and the effects of concealed vertical and horizontal spaces.  Whenever and wherever openings are made, hoselines should be ready.  While every effort should be made to minimize damage to the building and its contents, openings have to be large enough for inspection, hose manipulation, and ventilation.

Until determined otherwise, it is a safe assumption that when a working fire exists inside a building, fire has entered concealed vertical channels.

Personnel should be looking for indicators such as blisters and discolorations on walls, smoke patterns at molding, walls hot to the touch, or smoke (or fire) showing around roof features, such as vent pipes, etc.  If these are present, checking vertical extension is a must.

The tendency for most fire to travel vertically does not preclude horizontal travel.  Fire will follow any path available: void spaces between ceilings and floors, over false or hanging ceilings, around utility conduits, etc.  Extension occurs not only within the structure, but also from building to building.  Here again, hoselines must be in place prior to opening up these areas.  Most of the time, the indicators of fire in these areas are difficult to read, but look for some of the indicators present in vertical spread.  These areas should not be overlooked and have been responsible for fatalities, as well as for fires getting out of control.

Tactical operations in large and complex occupancies will have to be coordinated carefully in order to accomplish a reduction or change in heat and smoke travel.  Ventilation is a key tactical operation that will affect how, when, and where heat and smoke spread through a structure.

In your prefire inspections and plans, always look at all the possibilities of heat and smoke travel in a specific occupancy.  The time of the fire is not the time to study heat and smoke probabilities.

Based on the fire behavior factors and resource capabilities, you must make a fire behavior prediction that answers the following questions:

- Where is the fire at this time?

- In what direction is it likely to spread?

- Is there a probability that flashover is imminent?

- Is there a probability that backdraft is imminent?

- Is collapse likely to occur within the time required for offensive operations?

Once you have the answers to these questions, you have identified the problems, and can have a much clearer idea of what the resource needs are.

How Fires Extend

The travel of fire in structures can be predicted based on a good understanding of building construction and fire behavior factors.  Fire, smoke, and heat travel are dependent on many factors, such as void areas within the structure, the effect of the wind, and the positioning of hoselines.

The building layout and design can be an advantage or a disadvantage to your fire confinement and suppression efforts.  Generally, fires in large open areas will be more difficult to confine than fires in a compartmented area.  Fire load and built-in fire protection features will affect your efforts.  Items such as firewalls, fire doors, and automatic sprinkler systems can play a major role in the amount of resources you will need and the efforts it will take.

Fires generally spread from room to room via open doorways or through lightweight doors that do not last more than a few minutes under fire conditions.  Fires generally spread from floor to floor via open stairways, or open shafts and voids.

Predict Fire Travel

Fire travel predictions can be made by asking a few basic questions:

- Where is the fire now?

- Where is the smoke showing?

- What is in place to stop the spread of the fire and smoke to other areas of the structure, such as firewalls or other fire-resistant materials?

- What signs do I see, such as discoloration of paint, bubbling tar, or other building reactions, that will provide me with clues as to the fire travel?

The ability to predict fire travel will provide a CO accurate predictions that can lead to successful operations, realistic and proactive decisions, and adjustments as needed.

A good CO is an informed CO. An informed CO will be a well-trained CO.  A well-trained CO will be a safe CO.  Be careful out there!

This page intentionally left blank.

Activity 7.1

Large Exercise Overview

Large Group Exercise #1:  Frame Townhouse

Purpose

To allow you to demonstrate your knowledge, skills, and abilities (KSAs) you have gained as a result of attending this course.

Conduct a walkaround of exposures A-B-C-D of the structures that will be seen in the exercise.  There can be up to seven structures used in the activity.

Directions

1.	You will be working in small groups.

2.	Each group will do each construction type.  Exercise is designed to place the ICO as the Incident Commander (IC) until relieved by the next level of authority arriving at the incident.  (This will be your instructor.)

3.	For this simulation the ICO will have additional students assigned to Command and General Staff positions, which under normal field conditions would be the sole responsibility of the ICO.

4.	Student group assignments for each group:

a.	ICO.

b.	ICOA--scribe.

c.	IISO.

d.	Initial Planning Section--Situation Unit.

e.	Initial Logistics Section--service and support.

5.	Your group will record your predictions on an easel pad or paper.

6.	Your group will select a spokesperson to explain to the class how fire would travel in this building.

7.	Throughout the activity you will receive messages from the instructor.

8.	In this portion of exercise your group is to concentrate on Column 1 Primary Factors Chart.  You will have 10 minutes to complete this portion of the activity.

9.	The ICOA-scribe shall develop a Primary Factors Exercise Chart on an easel pad for his/her assigned exercise scenario.

a.	Identify the most pertinent primary factors for each exercise scenario.

b.	Place findings on Column 1 of the Primary Factors Exercise Chart.

c.	Identify the most pertinent subfactors for each identified primary factor for exercise scenario.  Place findings on Column 2 of the Primary Factors Exercise Chart.

d.	Identify pertinent precautions that will be taken to address each identified sub-factor for exercise scenario.  Place findings on Column 3 of the Primary Factors Exercise Chart.

Activity 7.1 (cont'd)

Primary Factors Chart

This page intentionally left blank.

Activity 7.1 (cont'd)

Primary Factors Exercise Chart

| Pertinent Primary Factors | Pertinent Subfactors | Pertinent Precautions To Be Taken |
| --- | --- | --- |
| 1. | 1. 2. | 1. 2. |
| 2. | 1. 2. 3. | 1. 2. 3. |
| 3. | 1. 2. | 1. 2. |
| 4. | 1. 2. | 1. 2. |
| 5. | 1. 2. | 1. 2. |
| 6. | 1. 2. | 1. 2. |
| 7. | 1. 2. 3. 4. | 1. 2. 3. 4. |
| 8. | 1. 2. 3. | 1. 2. 3. |
| 9. | 1. 2. | 1. 2. |
| 10. | 1. 2. | 1. 2. |
| 11. | 1. | 1. |
| 12. | 1. 2. | 1. 2. |
| 13. | 1. 2. | 1. 2. |

This page intentionally left blank.

LARGE EXERCISE #1

Vital Building Information

Situation Report

RESIDENTIAL--TYPE V--BALLOON FRAME

Structure:	Two-story--18 by 40 foot townhouses

Five townhouse units

Building Construction:	Type V--balloon frame

Roof Construction:	2- by 6-inch truss roof support system

Floors:	2- by 6-inch plywood flooring system

Alarm System:	No smoke detectors installed

Occupants:	Two adult occupants per occupancy

Units 1, 3, and 5 each have one young child

Special Concerns:	Unit 2 has two senior citizen occupants

Situation Report:

Fire Building:

It is January 30, 0730 hours, temperature is 23 °F (-5 °C), wind from west at 10 miles per hour (mph).

Upon arrival, four adults and two children are outside of the townhouses.  Occupants report two adults and one child are unaccounted for from Unit 3 and two occupants each from Units 2 and 4 are unaccounted for.

Exposures:

Each occupancy has attic storage.  Attic access is located in middle bedrooms on second floor.

This page intentionally left blank.

Activity 7.1 (cont'd)

Objectives-Strategy-Tactics Chart

1.	Use the enlarged Objectives-Strategy-Tactics Chart or easel pad.

2.	Refer back the Primary Factors Exercise materials in Unit 5:  The Analytical Sizeup Process if necessary.

3.	Place Slide Iteration #2 for Sides A-B-C-D for each scenario chosen.

4.	Receive and act on Exercise Message #2 and Plot Plan #2.

5.	Concentrate on Columns 2 and 3 and develop the Objectives-Strategy-Tactics for the scenario.

6.	ICOA scribe shall develop an easel pad for Objectives-Strategy-Tactics for assigned exercise scenario.  (You have 20 minutes to complete.)

a.	Column 1:  Identify the first operational period objectives for the scenario.

b.	Column 2:  Identify the first operational period strategies for each objective.

c.	Column 3:  Identify the first operational period tactics for each strategy.

d.	Column 4:  Assign companies to perform tactics.

Each scenario has first alarm structural resources available at start of exercise:

- Engine 1

- Engine 2

- Engine 3

- Ladder 1

- Basic Life Support (BLS) 1

Each scenario will have working incident structural resources arrive 10 minutes into incident.

- Engine 5

- Engine 6

- Squad 1

- Engine 6 (Rapid Intervention Crew (RIC))

- Air Cascade-1 FF

- Safety Officer

Central City Fire EMS Dispatch Criteria

| Assignment | Alarm Type | Engine | Ladder | Squad | Battalion Chief | EMS Unit | RIC | Safety Officer | Air Cascade |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| First Alarm | Structural | 3 | 1 |  |  | 1 BLS |  |  |  |
| Working Incident | Structural | 2 |  | 1 | 1 |  | 1 | 1 | 1 |
| First Alarm | Target Hazard | 3 | 2 | 1 | 1 | 1 BLS |  |  |  |
| Working Incident | Target Hazard | 2 |  |  |  | 1 ALS | 1 | 1 | 1 |
| Second Alarm | Structural/ Target Hazard | 2 | 1 |  | 1 | 1EMS |  | 1 | 1 |
| Third Alarm | Structural/ Target Hazard | 2 | 1 |  |  | 1 MS |  |  |  |
| Fourth Alarm | Structural/ Target Hazard | 2 | 1 |  | 1 |  |  |  |  |

Central City Fire/EMS Staffing:

- Engine-Ladder-Squad Company--1 Officer--3 Firefighters

- Rapid Intervention Crew (RIC)--One Engine Company

- EMS BLS Unit--2 EMT Basic

- EMS ALS Unit--2 EMT Paramedic

- Air Cascade--1 Firefighter

A second alarm may be requested but will not arrive on scene before end of exercise.

Activity 7.1 (cont'd)

Objectives-Strategy-Tactics Chart

| Objectives | Strategy(ies) | Tactics | Assigned to: |
| --- | --- | --- | --- |
| Objective #1 | 1. | 1. 2. 3. | 1. 2. 3. |
|  | 2. | 1. 2. 3. | 1. 2. 3. |
|  | 3. | 1. 2. 3. | 1. 2. 3. |
| Objective #2 | 1. | 1. 2. 3. | 1. 2. 3. |
|  | 2. | 1. 2. 3. | 1. 2. 3. |
|  | 3. | 1. 2. 3. | 1. 2. 3. |
| Objective #3 | 1. | 1. 2. 3. | 1. 2. 3. |
|  | 2. | 1. 2. 3. | 1. 2. 3. |
|  | 3. | 1. 2. 3. | 1. 2. 3. |

This page intentionally left blank.

Activity 7.1 (cont'd)

ICS Form 201, Incident Debriefing and ICS Form 214, Unit Log

1.	Use the enlarged ICS Form 214, Unit Log or easel pad.

2.	The ICOA will maintain ICS Form 214 for all tactical decisions made by the ICO along with other pertinent messages and decisions made by the ICO during the exercise.  Entries should be recorded under Time and Major Events columns.

3.	Distribute ICS Form 214.

a.	IISO.

b.	Initial Planning Section (Situation Unit).

c.	Initial Logistics Section (Service and Support).

4.	Distribute ICS Form 201, Incident Debriefing (enlarged to 11 by 17).

a.	Initial Planning Section (Situation Unit).

b.	Initial Logistics Section (Service and Support).

5.	Begin exercise scenario message distribution.

6.	Follow message distribution timeframes until all messages have been distributed.  (You have 30 minutes.)

This page intentionally left blank.

Activity 7.1 (cont'd)

Preparing for Exercise Debriefing

You have 15 minutes for each group to prepare their debriefing materials for presentation.

1.	ICO.

a.	Primary Factors Chart.

b.	Objectives-Strategy-Tactics Chart.

2.	Initial Company Officer Assistant--ICS Form 214.

3.	Initial Safety Officer--ICS Form 214.

4.	Initial Planning Section.

5.	Initial Logistics Section--ICS Form 201.

This page intentionally left blank.

Activity 7.1 (cont'd)

ICS Form 214, Unit Log

| UNIT LOG |  | 1. Incident Name | 2. Date Prepared |  | 3. Time Prepared |
| --- | --- | --- | --- | --- | --- |
| 4. Unit Name/Designators |  | 5. Unit Leader (Name and Position) |  |  | 6. Operational Period |
| 7. Personnel Roster Assigned |  |  |  |  |  |
| Name |  | ICS Position |  | Home Base |  |
| 8. Activity Log |  |  |  |  |  |
| Time | Major Events |  |  |  |  |
| 9. Prepared by (Name and Position) |  |  |  |  |  |

Instructions for completing the Unit Log (ICS Form 214)

| Item Number | Item Title | Instructions |
| --- | --- | --- |
| 1. | Incident Name | Print the name assigned to the incident. |
| 2. | Date Prepared | Enter date prepared (month, day, year). |
| 3. | Time Prepared | Enter time prepared (24-hour clock). |
| 4. | Unit Name | Enter the title of the organizational unit or resource designator (Engine 1 - Safety Officer, Situation Unit – Logistics Section). |
| 5. | Unit Leader | Enter the name of the individual in charge of the Unit. |
| 6. | Operational Period | Enter the time span covered by the log (e.g., 1800 Oct. 12 to 0600 Oct. 13). |
| 7. | Personnel Roster | List the name, position, and home base of each member assigned to the unit during the operational period. |
| 8. | Activity Log | Enter the time and briefly describe each significant occurrence or event (e.g., task assignments, task completions, injuries, difficulties encountered, etc.). |
| 9. | Prepared By | Enter the name and title of the person approving the log. Provide log to immediate supervisor at the end of each operational period. |

Activity 7.1 (cont'd)

ICS Form 201, Incident Debriefing

| incident briefing |  | 1. incident name | 2. date prepared | 3. time prepared |
| --- | --- | --- | --- | --- |
| 4. map sketch |  |  |  |  |
| ICS 201 Page 1 of 4 | 5. prepared by (name and position) |  |  |  |

| 6. summary of current actions |  |
| --- | --- |
| ICS 201 | page 2 |

| 7. current organization |  |
| --- | --- |
| ICS 201 | page 3 |

| 8. resources summary |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| resources ordered |  | resource identification | eta | on scene √ | location/assignment |
| ICS 201 | page 4 |  |  |  |  |

Instructions for completing the Incident Briefing (ICS Form 201)

| Item Number | Item Title | Instructions |
| --- | --- | --- |
| 1. | Incident Name | Print the name assigned to the incident. |
| 2. | Date Prepared | Enter date prepared (month, day, year). |
| 3. | Time Prepared | Enter time prepared (24-hour clock). |
| 4. | Map Sketch | Show perimeter and control lines, resources assignments, incident facilities, and other special information on a sketch map or attached to the topographic or orthophoto map. |
| 5. | Resources Summary | Enter the following information about the resources allocated to the incident. Enter the number and type of resource ordered. |
|  | Resources Ordered | Enter the number and type of resource ordered. |
|  | Resource Identification | Enter the agency three-letter designator, S/T, Kind/ Type and resource designator. |
|  | ETA/On Scene | Enter the estimated arrival time and place the arrival time or a checkmark in the "on scene" column upon arrival. |
|  | Location/ Assignment | Enter the assigned location of the resource and/or the actual assignment. |
| 6. | Current Organization | Enter on the organization chart the names of the individuals assigned to each position. Modify the chart as necessary. |
| 7. | Summary of Current Actions | Enter the strategy and tactics used on the incident and note any specific problem areas. |
| 8. | Prepared By | Enter the name and position of the person completing the form. |
| *Note |  | Additional pages may be added to ICS Form 201 if needed. |

This page intentionally left blank.

Activity 7.1 (cont'd)

Debriefing Procedures

1.	The instructor will debrief exercise scenarios with each group.

2.	The instructor will display the slides for your group's scenario when debriefing.

3.	Assigned ICO for each group shall present the following:

a.	Primary Factors Chart.

b.	Objectives-Strategy-Tactics Chart.

c.	ICS Form 214.

4.	Each group will have 15 minutes to debrief.

5.	The instructor will address any specific issues that were omitted from charts and identify issues where improvement is required.

This page intentionally left blank.

Activity 7.1 (cont'd)

Plot Plans

Exercise #1

Frame Townhouse--Walkaround

Iteration 1--Message 1

Iteration 2--Message 2

Iteration 2--Message 3

Iteration 3--Message 4

Iteration 3--Message 5

Iteration 4--Message 6

Iteration 4--Message 7

Iteration 4--Message 8

Activity 7.1 (cont'd)

Large Group Exercise #2:  Office Complex

Purpose

To allow you to demonstrate your knowledge, skills, and abilities (KSAs) you have gained as a result of attending this course.

Conduct a walkaround of exposures A-B-C-D of the structures that will be seen in the exercise.  There can be up to seven structures used in the activity.

Directions

1.	You will be working in small groups.

2.	Each group will do each construction type.  Exercise is designed to place the ICO as the Incident Commander (IC) until relieved by the next level of authority arriving at the incident.  (This will be your instructor.)

3.	For this simulation the ICO will have additional students assigned to Command and General Staff positions, which under normal field conditions would be the sole responsibility of the ICO.

4.	Student group assignments for each group:

a.	ICO.

b.	ICOA--scribe.

c.	IISO.

d.	Initial Planning Section--Situation Unit.

e.	Initial Logistics Section--service and support.

5.	Your group will record your predictions on an easel pad or paper.

6.	Your group will select a spokesperson to explain to the class how fire would travel in this building.

7.	Throughout the activity you will receive messages from the instructor.

8.	In this portion of exercise your group is to concentrate on Column 1 Primary Factors Chart.  You will have 10 minutes to complete this portion of the activity.

9.	The ICOA-scribe shall develop a Primary Factors Exercise Chart on an easel pad for his/her assigned exercise scenario.

a.	Identify the most pertinent primary factors for each exercise scenario.

b.	Place findings on Column 1 of the Primary Factors Exercise Chart.

c.	Identify the most pertinent subfactors for each identified primary factor for exercise scenario.  Place findings on Column 2 of the Primary Factors Exercise Chart.

d.	Identify pertinent precautions that will be taken to address each identified sub-factor for exercise scenario.  Place findings on Column 3 of the Primary Factors Exercise Chart.

Activity 7.1 (cont'd)

Primary Factors Chart

This page intentionally left blank.

Activity 7.1 (cont'd)

Primary Factors Exercise Chart

| Pertinent Primary Factor | Pertinent Subfactor | Pertinent Precautions To Be Taken |
| --- | --- | --- |
| 1. | 1. 2. | 1. 2. |
| 2. | 1. 2. 3. | 1. 2. 3. |
| 3. | 1. 2. | 1. 2. |
| 4. | 1. 2. | 1. 2. |
| 5. | 1. 2. | 1. 2. |
| 6. | 1. 2. | 1. 2. |
| 7. | 1. 2. 3. 4. | 1. 2. 3. 4. |
| 8. | 1. 2. 3. | 1. 2. 3. |
| 9. | 1. 2. | 1. 2. |
| 10. | 1. 2. | 1. 2. |
| 11. | 1. | 1. |
| 12. | 1. 2. | 1. 2. |
| 13. | 1. 2. | 1. 2. |

This page intentionally left blank.

LARGE EXERCISE #2

Vital Building Information

Situation Report

COMMERCIAL--TYPE V--BALLOON FRAME

Structure:	Office complex, 50 by 120 feet

12 one-story offices

20 by 25 feet various office tenants

Special care occupancies

Building Construction:	Type V--frame

Roof Construction:	2- by 6-inch truss roof support system

Common attic storage

Floors:	Concrete slab

Alarm System:	Smoke detectors installed

Occupants:	Tenant occupancy varies

4 to 8 employees per occupancy

Special Concerns:	Units 3 and 4--methadone clinic

Units 11 and 12--daycare center

Situation Report:

Fire Building:

It is March 5, 1030 hours, temperature is 27 °F (-3 °C), wind from east at 8 miles per hour (mph).

Upon arrival, several people are outside the office complex on Sides A and C.  Occupants from Offices 3 and 4 report that they are not sure everyone is accounted for.  The daycare center is evacuating children from Offices 11 and 12.  All offices are reported occupied.

Exposures:

Each occupancy has attic storage.  Attic access is located in near the rear of office.

This page intentionally left blank.

Activity 7.1 (cont'd)

Objectives-Strategy-Tactics Chart

1.	Use the enlarged Objectives-Strategy-Tactics Chart or easel pad.

2.	Refer back the Primary Factors Exercise materials in Unit 5:  The Analytical Sizeup Process if necessary.

3.	Place Slide Iteration #2 for Sides A-B-C-D for each scenario chosen.

4.	Receive and act on Exercise Message #2 and Plot Plan #2.

5.	Concentrate on Columns 2 and 3 and develop the Objectives-Strategy-Tactics for the scenario.

6.	ICOA scribe shall develop an easel pad for Objectives-Strategy-Tactics for assigned exercise scenario.  (You have 20 minutes to complete.)

a.	Column 1:  Identify the first operational period objectives for the scenario.

b.	Column 2:  Identify the first operational period strategies for each objective.

c.	Column 3:  Identify the first operational period tactics for each strategy.

d.	Column 4:  Assign companies to perform tactics.

Each scenario has first alarm structural resources available at start of exercise:

- Engine 1

- Engine 2

- Engine 3

- Ladder 1

- Basic Life Support (BLS) 1

Each scenario will have working incident structural resources arrive 10 minutes into incident.

- Engine 5

- Engine 6

- Squad 1

- Engine 6 (Rapid Intervention Crew (RIC))

- Air Cascade-1 FF

- Safety Officer

Central City Fire EMS Dispatch Criteria

| Assignment | Alarm Type | Engine | Ladder | Squad | Battalion Chief | EMS Unit | RIC | Safety Officer | Air Cascade |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| First Alarm | Structural | 3 | 1 |  |  | 1 BLS |  |  |  |
| Working Incident | Structural | 2 |  | 1 | 1 |  | 1 | 1 | 1 |
| First Alarm | Target Hazard | 3 | 2 | 1 | 1 | 1 BLS |  |  |  |
| Working Incident | Target Hazard | 2 |  |  |  | 1 ALS | 1 | 1 | 1 |
| Second Alarm | Structural/ Target Hazard | 2 | 1 |  | 1 | 1EMS |  | 1 | 1 |
| Third Alarm | Structural/ Target Hazard | 2 | 1 |  |  | 1 MS |  |  |  |
| Fourth Alarm | Structural/ Target Hazard | 2 | 1 |  | 1 |  |  |  |  |

Central City Fire/EMS Staffing:

- Engine-Ladder-Squad Company--1 Officer--3 Firefighters

- Rapid Intervention Crew (RIC)--One Engine Company

- EMS BLS Unit--2 EMT Basic

- EMS ALS Unit--2 EMT Paramedic

- Air Cascade--1 Firefighter

A second alarm may be requested but will not arrive on scene before end of exercise.

Activity 7.1 (cont'd)

Objectives-Strategy-Tactics Chart

| Objectives | Strategy(ies) | Tactics | Assigned to: |
| --- | --- | --- | --- |
| Objective #1 | 1. | 1. 2. 3. | 1. 2. 3. |
|  | 2. | 1. 2. 3. | 1. 2. 3. |
|  | 3. | 1. 2. 3. | 1. 2. 3. |
| Objective #2 | 1. | 1. 2. 3. | 1. 2. 3. |
|  | 2. | 1. 2. 3. | 1. 2. 3. |
|  | 3. | 1. 2. 3. | 1. 2. 3. |
| Objective #3 | 1. | 1. 2. 3. | 1. 2. 3. |
|  | 2. | 1. 2. 3. | 1. 2. 3. |
|  | 3. | 1. 2. 3. | 1. 2. 3. |

This page intentionally left blank.

Activity 7.1 (cont'd)

ICS Form 201, Incident Debriefing and ICS Form 214, Unit Log

1.	Use the enlarged ICS Form 214, Unit Log or easel pad.

2.	The ICOA will maintain ICS Form 214 for all tactical decisions made by the ICO along with other pertinent messages and decisions made by the ICO during the exercise.  Entries should be recorded under Time and Major Events columns.

3.	Distribute ICS Form 214.

a.	IISO.

b.	Initial Planning Section (Situation Unit).

c.	Initial Logistics Section (Service and Support).

4.	Distribute ICS Form 201, Incident Debriefing (enlarged to 11 by 17).

a.	Initial Planning Section (Situation Unit).

b.	Initial Logistics Section (Service and Support).

5.	Begin exercise scenario message distribution.

6.	Follow message distribution timeframes until all messages have been distributed.  (You have 30 minutes.)

This page intentionally left blank.

Activity 7.1 (cont'd)

Preparing for Exercise Debriefing

You have 15 minutes for each group to prepare their debriefing materials for presentation.

1.	ICO.

a.	Primary Factors Chart.

b.	Objectives-Strategy-Tactics Chart.

2.	Initial Company Officer Assistant--ICS Form 214.

3.	Initial Safety Officer--ICS Form 214.

4.	Initial Planning Section.

5.	Initial Logistics Section--ICS Form 201.

This page intentionally left blank.

Activity 7.1 (cont'd)

ICS Form 214, Unit Log

| UNIT LOG |  | 1. Incident Name | 2. Date Prepared |  | 3. Time Prepared |
| --- | --- | --- | --- | --- | --- |
| 4. Unit Name/Designators |  | 5. Unit Leader (Name and Position) |  |  | 6. Operational Period |
| 7. Personnel Roster Assigned |  |  |  |  |  |
| Name |  | ICS Position |  | Home Base |  |
| 8. Activity Log |  |  |  |  |  |
| Time | Major Events |  |  |  |  |
| 9. Prepared by (Name and Position) |  |  |  |  |  |

Instructions for completing the Unit Log (ICS Form 214)

| Item Number | Item Title | Instructions |
| --- | --- | --- |
| 1. | Incident Name | Print the name assigned to the incident. |
| 2. | Date Prepared | Enter date prepared (month, day, year). |
| 3. | Time Prepared | Enter time prepared (24-hour clock). |
| 4. | Unit Name | Enter the title of the organizational unit or resource designator (Engine 1 - Safety Officer, Situation Unit – Logistics Section). |
| 5. | Unit Leader | Enter the name of the individual in charge of the Unit. |
| 6. | Operational Period | Enter the time span covered by the log (e.g., 1800 Oct. 12 to 0600 Oct. 13). |
| 7. | Personnel Roster | List the name, position, and home base of each member assigned to the unit during the operational period. |
| 8. | Activity Log | Enter the time and briefly describe each significant occurrence or event (e.g., task assignments, task completions, injuries, difficulties encountered, etc.). |
| 9. | Prepared By | Enter the name and title of the person approving the log. Provide log to immediate supervisor at the end of each operational period. |

Activity 7.1 (cont'd)

ICS Form 201, Incident Debriefing

| incident briefing |  | 1. incident name | 2. date prepared | 3. time prepared |
| --- | --- | --- | --- | --- |
| 4. map sketch |  |  |  |  |
| ICS 201 Page 1 of 4 | 5. prepared by (name and position) |  |  |  |

| 6. summary of current actions |  |
| --- | --- |
| ICS 201 | page 2 |

| 7. current organization |  |
| --- | --- |
| ICS 201 | page 3 |

| 8. resources summary |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| resources ordered |  | resource identification | eta | on scene √ | location/assignment |
| ICS 201 | page 4 |  |  |  |  |

Instructions for completing the Incident Briefing (ICS Form 201)

| Item Number | Item Title | Instructions |
| --- | --- | --- |
| 1. | Incident Name | Print the name assigned to the incident. |
| 2. | Date Prepared | Enter date prepared (month, day, year). |
| 3. | Time Prepared | Enter time prepared (24-hour clock). |
| 4. | Map Sketch | Show perimeter and control lines, resources assignments, incident facilities, and other special information on a sketch map or attached to the topographic or orthophoto map. |
| 5. | Resources Summary | Enter the following information about the resources allocated to the incident. Enter the number and type of resource ordered. |
|  | Resources Ordered | Enter the number and type of resource ordered. |
|  | Resource Identification | Enter the agency three-letter designator, S/T, Kind/ Type and resource designator. |
|  | ETA/On Scene | Enter the estimated arrival time and place the arrival time or a checkmark in the "on scene" column upon arrival. |
|  | Location/ Assignment | Enter the assigned location of the resource and/or the actual assignment. |
| 6. | Current Organization | Enter on the organization chart the names of the individuals assigned to each position. Modify the chart as necessary. |
| 7. | Summary of Current Actions | Enter the strategy and tactics used on the incident and note any specific problem areas. |
| 8. | Prepared By | Enter the name and position of the person completing the form. |
| *Note |  | Additional pages may be added to ICS Form 201 if needed. |

This page intentionally left blank.

Activity 7.1 (cont'd)

Debriefing Procedures

1.	The instructor will debrief exercise scenarios with each group.

2.	The instructor will display the slides for your group's scenario when debriefing.

3.	Assigned ICO for each group shall present the following:

a.	Primary Factors Chart.

b.	Objectives-Strategy-Tactics Chart.

c.	ICS Form 214.

4.	Each group will have 15 minutes to debrief.

5.	The instructor will address any specific issues that were omitted from charts and identify issues where improvement is required.

This page intentionally left blank.

Activity 7.1 (cont'd)

Plot Plans

Exercise #2

Office Complex--Walkaround

Iteration 1--Message 1

Iteration 2--Message 2

Iteration 2--Message 3

Iteration 3--Message 4

Iteration 3--Message 5

Iteration 4--Message 6

Iteration 4--Message 7

Iteration 4--Message 8

This page intentionally left blank.

Activity 7.1 (cont'd)

Large Group Exercise #3:  Highrise

Purpose

To allow you to demonstrate your knowledge, skills, and abilities (KSAs) you have gained as a result of attending this course.

Conduct a walkaround of exposures A-B-C-D of the structures that will be seen in the exercise.  There can be up to seven structures used in the activity.

Directions

1.	You will be working in small groups.

2.	Each group will do each construction type.  Exercise is designed to place the ICO as the Incident Commander (IC) until relieved by the next level of authority arriving at the incident.  (This will be your instructor.)

3.	For this simulation the ICO will have additional students assigned to Command and General Staff positions, which under normal field conditions would be the sole responsibility of the ICO.

4.	Student group assignments for each group:

a.	ICO.

b.	ICOA--scribe.

c.	IISO.

d.	Initial Planning Section--Situation Unit.

e.	Initial Logistics Section--service and support.

5.	Your group will record your predictions on an easel pad or paper.

6.	Your group will select a spokesperson to explain to the class how fire would travel in this building.

7.	Throughout the activity you will receive messages from the instructor.

8.	In this portion of exercise your group is to concentrate on Column 1 Primary Factors Chart.  You will have 10 minutes to complete this portion of the activity.

9.	The ICOA-scribe shall develop a Primary Factors Exercise Chart on an easel pad for his/her assigned exercise scenario.

a.	Identify the most pertinent primary factors for each exercise scenario.

b.	Place findings on Column 1 of the Primary Factors Exercise Chart.

c.	Identify the most pertinent subfactors for each identified primary factor for exercise scenario.  Place findings on Column 2 of the Primary Factors Exercise Chart.

d.	Identify pertinent precautions that will be taken to address each identified sub-factor for exercise scenario.  Place findings on Column 3 of the Primary Factors Exercise Chart.

Activity 7.1 (cont'd)

Primary Factors Chart

This page intentionally left blank.

Activity 7.1 (cont'd)

Primary Factors Exercise Chart

| Pertinent Primary Factor | Pertinent Subfactor | Pertinent Precautions To Be Taken |
| --- | --- | --- |
| 1. | 1. 2. | 1. 2. |
| 2. | 1. 2. 3. | 1. 2. 3. |
| 3. | 1. 2. | 1. 2. |
| 4. | 1. 2. | 1. 2. |
| 5. | 1. 2. | 1. 2. |
| 6. | 1. 2. | 1. 2. |
| 7. | 1. 2. 3. 4. | 1. 2. 3. 4. |
| 8. | 1. 2. 3. | 1. 2. 3. |
| 9. | 1. 2. | 1. 2. |
| 10. | 1. 2. | 1. 2. |
| 11. | 1. | 1. |
| 12. | 1. 2. | 1. 2. |
| 13. | 1. 2. | 1. 2. |

This page intentionally left blank.

LARGE EXERCISE #3

Vital Building Information

Situation Report

RESIDENTIAL--TYPE I--FIRE RESISTIVE

Structure:	Residential highrise, 50 by 100 feet

22-Story Senior Citizen Occupancy

Building Construction:	Type I--fire resistive--steel frame

Roof Construction:	Concrete slab

Floors:	2-inch concrete over Q decking

Alarm System:	Smoke detectors installed

Stairwell:	Rear of building--Side C

Standpipe:	6-inch dry system--stairwell connection in rear of building

Elevators:	Double bank

Rear of building

Occupants:	6 apartments per floor

2 occupants per apartment

Special Concerns:	Wheelchair occupant--Apt. 1002

Nonambulatory occupant--Apt. 1105

Blind occupant--Apt. 1306

Situation Report:

Fire Building:

It is July 4, 1430 hours, temperature is 94 °F (34 °C), wind from east at 3 mph.

Upon arrival, several people are outside the highrise, Side C and more are evacuating down the rear stairwell.  The alarm system is sounding.  Status of occupants in Apartment 801 is unknown.

Exposures:

No immediate exposures.  Internal exposures only.

This page intentionally left blank.

Activity 7.1 (cont'd)

Objectives-Strategy-Tactics Chart

1.	Use the enlarged Objectives-Strategy-Tactics Chart or easel pad.

2.	Refer back the Primary Factors Exercise materials in Unit 5:  The Analytical Sizeup Process if necessary.

3.	Place Slide Iteration #2 for Sides A-B-C-D for each scenario chosen.

4.	Receive and act on Exercise Message #2 and Plot Plan #2.

5.	Concentrate on Columns 2 and 3 and develop the Objectives-Strategy-Tactics for the scenario.

6.	ICOA scribe shall develop an easel pad for Objectives-Strategy-Tactics for assigned exercise scenario.  (You have 20 minutes to complete.)

a.	Column 1:  Identify the first operational period objectives for the scenario.

b.	Column 2:  Identify the first operational period strategies for each objective.

c.	Column 3:  Identify the first operational period tactics for each strategy.

d.	Column 4:  Assign companies to perform tactics.

Each scenario has first alarm structural resources available at start of exercise:

- Engine 1

- Engine 2

- Engine 3

- Ladder 1

- Basic Life Support (BLS) 1

Each scenario will have working incident structural resources arrive 10 minutes into incident.

- Engine 5

- Engine 6

- Squad 1

- Engine 6 (Rapid Intervention Crew (RIC))

- Air Cascade-1 FF

- Safety Officer

Central City Fire EMS Dispatch Criteria

| Assignment | Alarm Type | Engine | Ladder | Squad | Battalion Chief | EMS Unit | RIC | Safety Officer | Air Cascade |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| First Alarm | Structural | 3 | 1 |  |  | 1 BLS |  |  |  |
| Working Incident | Structural | 2 |  | 1 | 1 |  | 1 | 1 | 1 |
| First Alarm | Target Hazard | 3 | 2 | 1 | 1 | 1 BLS |  |  |  |
| Working Incident | Target Hazard | 2 |  |  |  | 1 ALS | 1 | 1 | 1 |
| Second Alarm | Structural/ Target Hazard | 2 | 1 |  | 1 | 1EMS |  | 1 | 1 |
| Third Alarm | Structural/ Target Hazard | 2 | 1 |  |  | 1 MS |  |  |  |
| Fourth Alarm | Structural/ Target Hazard | 2 | 1 |  | 1 |  |  |  |  |

Central City Fire/EMS Staffing:

- Engine-Ladder-Squad Company--1 Officer--3 Firefighters

- Rapid Intervention Crew (RIC)--One Engine Company

- EMS BLS Unit--2 EMT Basic

- EMS ALS Unit--2 EMT Paramedic

- Air Cascade--1 Firefighter

A second alarm may be requested but will not arrive on scene before end of exercise.

Activity 7.1 (cont'd)

Objectives-Strategy-Tactics Chart

| Objectives | Strategy(ies) | Tactics | Assigned to: |
| --- | --- | --- | --- |
| Objective #1 | 1. | 1. 2. 3. | 1. 2. 3. |
|  | 2. | 1. 2. 3. | 1. 2. 3. |
|  | 3. | 1. 2. 3. | 1. 2. 3. |
| Objective #2 | 1. | 1. 2. 3. | 1. 2. 3. |
|  | 2. | 1. 2. 3. | 1. 2. 3. |
|  | 3. | 1. 2. 3. | 1. 2. 3. |
| Objective #3 | 1. | 1. 2. 3. | 1. 2. 3. |
|  | 2. | 1. 2. 3. | 1. 2. 3. |
|  | 3. | 1. 2. 3. | 1. 2. 3. |

This page intentionally left blank.

Activity 7.1 (cont'd)

ICS Form 201, Incident Debriefing and ICS Form 214, Unit Log

1.	Use the enlarged ICS Form 214, Unit Log or easel pad.

2.	The ICOA will maintain ICS Form 214 for all tactical decisions made by the ICO along with other pertinent messages and decisions made by the ICO during the exercise.  Entries should be recorded under Time and Major Events columns.

3.	Distribute ICS Form 214.

a.	IISO.

b.	Initial Planning Section (Situation Unit).

c.	Initial Logistics Section (Service and Support).

4.	Distribute ICS Form 201, Incident Debriefing (enlarged to 11 by 17).

a.	Initial Planning Section (Situation Unit).

b.	Initial Logistics Section (Service and Support).

5.	Begin exercise scenario message distribution.

6.	Follow message distribution timeframes until all messages have been distributed.  (You have 30 minutes.)

This page intentionally left blank.

Activity 7.1 (cont'd)

Preparing for Exercise Debriefing

You have 15 minutes for each group to prepare their debriefing materials for presentation.

1.	ICO.

a.	Primary Factors Chart.

b.	Objectives-Strategy-Tactics Chart.

2.	Initial Company Officer Assistant--ICS Form 214.

3.	Initial Safety Officer--ICS Form 214.

4.	Initial Planning Section.

5.	Initial Logistics Section--ICS Form 201.

This page intentionally left blank.

Activity 7.1 (cont'd)

ICS Form 214, Unit Log

| UNIT LOG |  | 1. Incident Name | 2. Date Prepared |  | 3. Time Prepared |
| --- | --- | --- | --- | --- | --- |
| 4. Unit Name/Designators |  | 5. Unit Leader (Name and Position) |  |  | 6. Operational Period |
| 7. Personnel Roster Assigned |  |  |  |  |  |
| Name |  | ICS Position |  | Home Base |  |
| 8. Activity Log |  |  |  |  |  |
| Time | Major Events |  |  |  |  |
| 9. Prepared by (Name and Position) |  |  |  |  |  |

Instructions for completing the Unit Log (ICS Form 214)

| Item Number | Item Title | Instructions |
| --- | --- | --- |
| 1. | Incident Name | Print the name assigned to the incident. |
| 2. | Date Prepared | Enter date prepared (month, day, year). |
| 3. | Time Prepared | Enter time prepared (24-hour clock). |
| 4. | Unit Name | Enter the title of the organizational unit or resource designator (Engine 1 - Safety Officer, Situation Unit – Logistics Section). |
| 5. | Unit Leader | Enter the name of the individual in charge of the Unit. |
| 6. | Operational Period | Enter the time span covered by the log (e.g., 1800 Oct. 12 to 0600 Oct. 13). |
| 7. | Personnel Roster | List the name, position, and home base of each member assigned to the unit during the operational period. |
| 8. | Activity Log | Enter the time and briefly describe each significant occurrence or event (e.g., task assignments, task completions, injuries, difficulties encountered, etc.). |
| 9. | Prepared By | Enter the name and title of the person approving the log. Provide log to immediate supervisor at the end of each operational period. |

Activity 7.1 (cont'd)

ICS Form 201, Incident Debriefing

| incident briefing |  | 1. incident name | 2. date prepared | 3. time prepared |
| --- | --- | --- | --- | --- |
| 4. map sketch |  |  |  |  |
| ICS 201 Page 1 of 4 | 5. prepared by (name and position) |  |  |  |

| 6. summary of current actions |  |
| --- | --- |
| ICS 201 | page 2 |

| 7. current organization |  |
| --- | --- |
| ICS 201 | page 3 |

| 8. resources summary |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| resources ordered |  | resource identification | eta | on scene √ | location/assignment |
| ICS 201 | page 4 |  |  |  |  |

Instructions for completing the Incident Briefing (ICS Form 201)

| Item Number | Item Title | Instructions |
| --- | --- | --- |
| 1. | Incident Name | Print the name assigned to the incident. |
| 2. | Date Prepared | Enter date prepared (month, day, year). |
| 3. | Time Prepared | Enter time prepared (24-hour clock). |
| 4. | Map Sketch | Show perimeter and control lines, resources assignments, incident facilities, and other special information on a sketch map or attached to the topographic or orthophoto map. |
| 5. | Resources Summary | Enter the following information about the resources allocated to the incident. Enter the number and type of resource ordered. |
|  | Resources Ordered | Enter the number and type of resource ordered. |
|  | Resource Identification | Enter the agency three-letter designator, S/T, Kind/ Type and resource designator. |
|  | ETA/On Scene | Enter the estimated arrival time and place the arrival time or a checkmark in the "on scene" column upon arrival. |
|  | Location/ Assignment | Enter the assigned location of the resource and/or the actual assignment. |
| 6. | Current Organization | Enter on the organization chart the names of the individuals assigned to each position. Modify the chart as necessary. |
| 7. | Summary of Current Actions | Enter the strategy and tactics used on the incident and note any specific problem areas. |
| 8. | Prepared By | Enter the name and position of the person completing the form. |
| *Note |  | Additional pages may be added to ICS Form 201 if needed. |

This page intentionally left blank.

Activity 7.1 (cont'd)

Debriefing Procedures

1.	The instructor will debrief exercise scenarios with each group.

2.	The instructor will display the slides for your group's scenario when debriefing.

3.	Assigned ICO for each group shall present the following:

a.	Primary Factors Chart.

b.	Objectives-Strategy-Tactics Chart.

c.	ICS Form 214.

4.	Each group will have 15 minutes to debrief.

5.	The instructor will address any specific issues that were omitted from charts and identify issues where improvement is required.

This page intentionally left blank.

Activity 7.1 (cont'd)

Plot Plans

Exercise #3

Highrise--Walkaround

Iteration 1--Message 1

Iteration 2--Message 2

Iteration 2--Message 3

Iteration 3--Message 4

Iteration 3--Message 5

Iteration 4--Message 6

Iteration 4--Message 7

Iteration 4--Message 8

Activity 7.1 (cont'd)

Large Group Exercise #4:  Mill Building

Purpose

To allow you to demonstrate your knowledge, skills, and abilities (KSAs) you have gained as a result of attending this course.

Conduct a walkaround of exposures A-B-C-D of the structures that will be seen in the exercise.  There can be up to seven structures used in the activity.

Directions

1.	You will be working in small groups.

2.	Each group will do each construction type.  Exercise is designed to place the ICO as the Incident Commander (IC) until relieved by the next level of authority arriving at the incident.  (This will be your instructor.)

3.	For this simulation the ICO will have additional students assigned to Command and General Staff positions, which under normal field conditions would be the sole responsibility of the ICO.

4.	Student group assignments for each group:

a.	ICO.

b.	ICOA--scribe.

c.	IISO.

d.	Initial Planning Section--Situation Unit.

e.	Initial Logistics Section--service and support.

5.	Your group will record your predictions on an easel pad or paper.

6.	Your group will select a spokesperson to explain to the class how fire would travel in this building.

7.	Throughout the activity you will receive messages from the instructor.

8.	In this portion of exercise your group is to concentrate on Column 1 Primary Factors Chart.  You will have 10 minutes to complete this portion of the activity.

9.	The ICOA-scribe shall develop a Primary Factors Exercise Chart on an easel pad for his/her assigned exercise scenario.

a.	Identify the most pertinent primary factors for each exercise scenario.

b.	Place findings on Column 1 of the Primary Factors Exercise Chart.

c.	Identify the most pertinent subfactors for each identified primary factor for exercise scenario.  Place findings on Column 2 of the Primary Factors Exercise Chart.

d.	Identify pertinent precautions that will be taken to address each identified sub-factor for exercise scenario.  Place findings on Column 3 of the Primary Factors Exercise Chart.

Activity 7.1 (cont'd)

Primary Factors Chart

This page intentionally left blank.

Activity 7.1 (cont'd)

Primary Factors Exercise Chart

| Pertinent Primary Factor | Pertinent Subfactor | Pertinent Precautions To Be Taken |
| --- | --- | --- |
| 1. | 1. 2. | 1. 2. |
| 2. | 1. 2. 3. | 1. 2. 3. |
| 3. | 1. 2. | 1. 2. |
| 4. | 1. 2. | 1. 2. |
| 5. | 1. 2. | 1. 2. |
| 6. | 1. 2. | 1. 2. |
| 7. | 1. 2. 3. 4. | 1. 2. 3. 4. |
| 8. | 1. 2. 3. | 1. 2. 3. |
| 9. | 1. 2. | 1. 2. |
| 10. | 1. 2. | 1. 2. |
| 11. | 1. | 1. |
| 12. | 1. 2. | 1. 2. |
| 13. | 1. 2. | 1. 2. |

This page intentionally left blank.

LARGE EXERCISE #4

Vital Building Information

Situation Report

HEAVY TIMBER--TYPE IV--MILL CONSTRUCTION

Structure:	Two-story--clothing outlet

Building Construction:	Type IV--mill construction

Roof Construction:	Beam and rafter, 1-1/4-inch wooden roof decking, metal roof covering

Floors:	Beam and joist, 2-inch wooden floor decking

Alarm System:	Smoke detectors installed

Sprinkler:	Basement, first and second floors

Occupants:	Office and sales employees

Special Concerns:	Heavy fuel load

Situation Report:

Fire Building:

It is May 25, 1330 hours, temperature is 75 °F (24 °C), wind from northeast at 5 mph.

Upon arrival, several employees are outside the building on Sides A.  Store manager reports that plumbers were working in the basement repairing a broken water pipe when the fire broke out. The plumbing foreman reported the sprinkler system was shut down for the repairs.  One plumber is unaccounted for and was last seen in the basement and one employee working on the second floor is missing.

Exposures:

Side B--two-story retail clothing store--no other immediate exposures.

This page intentionally left blank.

Activity 7.1 (cont'd)

Objectives-Strategy-Tactics Chart

1.	Use the enlarged Objectives-Strategy-Tactics Chart or easel pad.

2.	Refer back the Primary Factors Exercise materials in Unit 5:  The Analytical Sizeup Process if necessary.

3.	Place Slide Iteration #2 for Sides A-B-C-D for each scenario chosen.

4.	Receive and act on Exercise Message #2 and Plot Plan #2.

5.	Concentrate on Columns 2 and 3 and develop the Objectives-Strategy-Tactics for the scenario.

6.	ICOA scribe shall develop an easel pad for Objectives-Strategy-Tactics for assigned exercise scenario.  (You have 20 minutes to complete.)

a.	Column 1:  Identify the first operational period objectives for the scenario.

b.	Column 2:  Identify the first operational period strategies for each objective.

c.	Column 3:  Identify the first operational period tactics for each strategy.

d.	Column 4:  Assign companies to perform tactics.

Each scenario has first alarm structural resources available at start of exercise:

- Engine 1

- Engine 2

- Engine 3

- Ladder 1

- Basic Life Support (BLS) 1

Each scenario will have working incident structural resources arrive 10 minutes into incident.

- Engine 5

- Engine 6

- Squad 1

- Engine 6 (Rapid Intervention Crew (RIC))

- Air Cascade-1 FF

- Safety Officer

Central City Fire EMS Dispatch Criteria

| Assignment | Alarm Type | Engine | Ladder | Squad | Battalion Chief | EMS Unit | RIC | Safety Officer | Air Cascade |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| First Alarm | Structural | 3 | 1 |  |  | 1 BLS |  |  |  |
| Working Incident | Structural | 2 |  | 1 | 1 |  | 1 | 1 | 1 |
| First Alarm | Target Hazard | 3 | 2 | 1 | 1 | 1 BLS |  |  |  |
| Working Incident | Target Hazard | 2 |  |  |  | 1 ALS | 1 | 1 | 1 |
| Second Alarm | Structural/ Target Hazard | 2 | 1 |  | 1 | 1EMS |  | 1 | 1 |
| Third Alarm | Structural/ Target Hazard | 2 | 1 |  |  | 1 MS |  |  |  |
| Fourth Alarm | Structural/ Target Hazard | 2 | 1 |  | 1 |  |  |  |  |

Central City Fire/EMS Staffing:

- Engine-Ladder-Squad Company--1 Officer--3 Firefighters

- Rapid Intervention Crew (RIC)--One Engine Company

- EMS BLS Unit--2 EMT Basic

- EMS ALS Unit--2 EMT Paramedic

- Air Cascade--1 Firefighter

A second alarm may be requested but will not arrive on scene before end of exercise.

Activity 7.1 (cont'd)

Objectives-Strategy-Tactics Chart

| Objectives | Strategy(ies) | Tactics | Assigned to: |
| --- | --- | --- | --- |
| Objective #1 | 1. | 1. 2. 3. | 1. 2. 3. |
|  | 2. | 1. 2. 3. | 1. 2. 3. |
|  | 3. | 1. 2. 3. | 1. 2. 3. |
| Objective #2 | 1. | 1. 2. 3. | 1. 2. 3. |
|  | 2. | 1. 2. 3. | 1. 2. 3. |
|  | 3. | 1. 2. 3. | 1. 2. 3. |
| Objective #3 | 1. | 1. 2. 3. | 1. 2. 3. |
|  | 2. | 1. 2. 3. | 1. 2. 3. |
|  | 3. | 1. 2. 3. | 1. 2. 3. |

This page intentionally left blank.

Activity 7.1 (cont'd)

ICS Form 201, Incident Debriefing and ICS Form 214, Unit Log

1.	Use the enlarged ICS Form 214, Unit Log or easel pad.

2.	The ICOA will maintain ICS Form 214 for all tactical decisions made by the ICO along with other pertinent messages and decisions made by the ICO during the exercise.  Entries should be recorded under Time and Major Events columns.

3.	Distribute ICS Form 214.

a.	IISO.

b.	Initial Planning Section (Situation Unit).

c.	Initial Logistics Section (Service and Support).

4.	Distribute ICS Form 201, Incident Debriefing (enlarged to 11 by 17).

a.	Initial Planning Section (Situation Unit).

b.	Initial Logistics Section (Service and Support).

5.	Begin exercise scenario message distribution.

6.	Follow message distribution timeframes until all messages have been distributed.  (You have 30 minutes.)

This page intentionally left blank.

Activity 7.1 (cont'd)

Preparing for Exercise Debriefing

You have 15 minutes for each group to prepare their debriefing materials for presentation.

1.	ICO.

a.	Primary Factors Chart.

b.	Objectives-Strategy-Tactics Chart.

2.	Initial Company Officer Assistant--ICS Form 214.

3.	Initial Safety Officer--ICS Form 214.

4.	Initial Planning Section.

5.	Initial Logistics Section--ICS Form 201.

This page intentionally left blank.

Activity 7.1 (cont'd)

ICS Form 214, Unit Log

| UNIT LOG |  | 1. Incident Name | 2. Date Prepared |  | 3. Time Prepared |
| --- | --- | --- | --- | --- | --- |
| 4. Unit Name/Designators |  | 5. Unit Leader (Name and Position) |  |  | 6. Operational Period |
| 7. Personnel Roster Assigned |  |  |  |  |  |
| Name |  | ICS Position |  | Home Base |  |
| 8. Activity Log |  |  |  |  |  |
| Time | Major Events |  |  |  |  |
| 9. Prepared by (Name and Position) |  |  |  |  |  |

Instructions for completing the Unit Log (ICS Form 214)

| Item Number | Item Title | Instructions |
| --- | --- | --- |
| 1. | Incident Name | Print the name assigned to the incident. |
| 2. | Date Prepared | Enter date prepared (month, day, year). |
| 3. | Time Prepared | Enter time prepared (24-hour clock). |
| 4. | Unit Name | Enter the title of the organizational unit or resource designator (Engine 1 - Safety Officer, Situation Unit – Logistics Section). |
| 5. | Unit Leader | Enter the name of the individual in charge of the Unit. |
| 6. | Operational Period | Enter the time span covered by the log (e.g., 1800 Oct. 12 to 0600 Oct. 13). |
| 7. | Personnel Roster | List the name, position, and home base of each member assigned to the unit during the operational period. |
| 8. | Activity Log | Enter the time and briefly describe each significant occurrence or event (e.g., task assignments, task completions, injuries, difficulties encountered, etc.). |
| 9. | Prepared By | Enter the name and title of the person approving the log. Provide log to immediate supervisor at the end of each operational period. |

Activity 7.1 (cont'd)

ICS Form 201, Incident Debriefing

| incident briefing |  | 1. incident name | 2. date prepared | 3. time prepared |
| --- | --- | --- | --- | --- |
| 4. map sketch |  |  |  |  |
| ICS 201 Page 1 of 4 | 5. prepared by (name and position) |  |  |  |

| 6. summary of current actions |  |
| --- | --- |
| ICS 201 | page 2 |

| 7. current organization |  |
| --- | --- |
| ICS 201 | page 3 |

| 8. resources summary |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| resources ordered |  | resource identification | eta | on scene √ | location/assignment |
| ICS 201 | page 4 |  |  |  |  |

Instructions for completing the Incident Briefing (ICS Form 201)

| Item Number | Item Title | Instructions |
| --- | --- | --- |
| 1. | Incident Name | Print the name assigned to the incident. |
| 2. | Date Prepared | Enter date prepared (month, day, year). |
| 3. | Time Prepared | Enter time prepared (24-hour clock). |
| 4. | Map Sketch | Show perimeter and control lines, resources assignments, incident facilities, and other special information on a sketch map or attached to the topographic or orthophoto map. |
| 5. | Resources Summary | Enter the following information about the resources allocated to the incident. Enter the number and type of resource ordered. |
|  | Resources Ordered | Enter the number and type of resource ordered. |
|  | Resource Identification | Enter the agency three-letter designator, S/T, Kind/ Type and resource designator. |
|  | ETA/On Scene | Enter the estimated arrival time and place the arrival time or a checkmark in the "on scene" column upon arrival. |
|  | Location/ Assignment | Enter the assigned location of the resource and/or the actual assignment. |
| 6. | Current Organization | Enter on the organization chart the names of the individuals assigned to each position. Modify the chart as necessary. |
| 7. | Summary of Current Actions | Enter the strategy and tactics used on the incident and note any specific problem areas. |
| 8. | Prepared By | Enter the name and position of the person completing the form. |
| *Note |  | Additional pages may be added to ICS Form 201 if needed. |

This page intentionally left blank.

Activity 7.1 (cont'd)

Debriefing Procedures

1.	The instructor will debrief exercise scenarios with each group.

2.	The instructor will display the slides for your group's scenario when debriefing.

3.	Assigned ICO for each group shall present the following:

a.	Primary Factors Chart.

b.	Objectives-Strategy-Tactics Chart.

c.	ICS Form 214.

4.	Each group will have 15 minutes to debrief.

5.	The instructor will address any specific issues that were omitted from charts and identify issues where improvement is required.

This page intentionally left blank.

Activity 7.1 (cont'd)

Plot Plans

Exercise #4

Mill Building--Walkaround

Iteration 1--Message 1

Iteration 2--Message 2

Iteration 2--Message 3

Iteration 2--Message 4

Iteration 3--Message 5

Iteration 4--Message 6

Iteration 4--Message 7

Iteration 4--Message 8

Activity 7.1 (cont'd)

Large Group Exercise #5:  Shopping Center

Purpose

To allow you to demonstrate your knowledge, skills, and abilities (KSAs) you have gained as a result of attending this course.

Conduct a walkaround of exposures A-B-C-D of the structures that will be seen in the exercise.  There can be up to seven structures used in the activity.

Directions

1.	You will be working in small groups.

2.	Each group will do each construction type.  Exercise is designed to place the ICO as the Incident Commander (IC) until relieved by the next level of authority arriving at the incident.  (This will be your instructor.)

3.	For this simulation the ICO will have additional students assigned to Command and General Staff positions, which under normal field conditions would be the sole responsibility of the ICO.

4.	Student group assignments for each group:

a.	ICO.

b.	ICOA--scribe.

c.	IISO.

d.	Initial Planning Section--Situation Unit.

e.	Initial Logistics Section--service and support.

5.	Your group will record your predictions on an easel pad or paper.

6.	Your group will select a spokesperson to explain to the class how fire would travel in this building.

7.	Throughout the activity you will receive messages from the instructor.

8.	In this portion of exercise your group is to concentrate on Column 1 Primary Factors Chart.  You will have 10 minutes to complete this portion of the activity.

9.	The ICOA-scribe shall develop a Primary Factors Exercise Chart on an easel pad for his/her assigned exercise scenario.

a.	Identify the most pertinent primary factors for each exercise scenario.

b.	Place findings on Column 1 of the Primary Factors Exercise Chart.

c.	Identify the most pertinent subfactors for each identified primary factor for exercise scenario.  Place findings on Column 2 of the Primary Factors Exercise Chart.

d.	Identify pertinent precautions that will be taken to address each identified sub-factor for exercise scenario.  Place findings on Column 3 of the Primary Factors Exercise Chart.

Activity 7.1 (cont'd)

Primary Factors Chart

This page intentionally left blank.

Activity 7.1 (cont'd)

Primary Factors Exercise Chart

| Pertinent Primary Factor | Pertinent Subfactor | Pertinent Precautions To Be Taken |
| --- | --- | --- |
| 1. | 1. 2. | 1. 2. |
| 2. | 1. 2. 3. | 1. 2. 3. |
| 3. | 1. 2. | 1. 2. |
| 4. | 1. 2. | 1. 2. |
| 5. | 1. 2. | 1. 2. |
| 6. | 1. 2. | 1. 2. |
| 7. | 1. 2. 3. 4. | 1. 2. 3. 4. |
| 8. | 1. 2. 3. | 1. 2. 3. |
| 9. | 1. 2. | 1. 2. |
| 10. | 1. 2. | 1. 2. |
| 11. | 1. | 1. |
| 12. | 1. 2. | 1. 2. |
| 13. | 1. 2. | 1. 2. |

This page intentionally left blank.

LARGE EXERCISE #5

Vital Building Information

Situation Report

COMMERCIAL--TYPE V--FRAME

Structure:	Shopping center, 40 by 100 feet

5 one-story occupancies

20 by 40 feet--various tenants

Building Construction:	Type V--frame

Roof Construction:		2- by 6-inch truss roof support system

Common attic storage

Floors:	Concrete slab

Alarm System:	Smoke detectors installed

Occupants:	Restaurants/Offices

Special Concerns:	Commercial cooking facilities

High-occupancy restaurant

Situation Report:

Fire Building:

It is October 15, Friday 1230 hours, temperature is 47 °F (8 °C), wind from south at 12 mph.

Upon arrival, several people are outside the building on Side A.  The manager of Quizno's Sub reports an explosion occurred in Quizno's.  He is suffering from severe burns of his arms, neck, and back.  He said just before the explosion that he smelled gas.  One employee is missing.  Four Quizno customers are outside suffering from smoke inhalation.  Occupants of Powell's Insurance and Chris' Steak House are exiting the building.

Exposures:

Each occupancy has attic storage.  Attic access is located near the rear of the building.

This page intentionally left blank.

Activity 7.1 (cont'd)

Objectives-Strategy-Tactics Chart

1.	Use the enlarged Objectives-Strategy-Tactics Chart or easel pad.

2.	Refer back the Primary Factors Exercise materials in Unit 5:  The Analytical Sizeup Process if necessary.

3.	Place Slide Iteration #2 for Sides A-B-C-D for each scenario chosen.

4.	Receive and act on Exercise Message #2 and Plot Plan #2.

5.	Concentrate on Columns 2 and 3 and develop the Objectives-Strategy-Tactics for the scenario.

6.	ICOA scribe shall develop an easel pad for Objectives-Strategy-Tactics for assigned exercise scenario.  (You have 20 minutes to complete.)

a.	Column 1:  Identify the first operational period objectives for the scenario.

b.	Column 2:  Identify the first operational period strategies for each objective.

c.	Column 3:  Identify the first operational period tactics for each strategy.

d.	Column 4:  Assign companies to perform tactics.

Each scenario has first alarm structural resources available at start of exercise:

- Engine 1

- Engine 2

- Engine 3

- Ladder 1

- Basic Life Support (BLS) 1

Each scenario will have working incident structural resources arrive 10 minutes into incident.

- Engine 5

- Engine 6

- Squad 1

- Engine 6 (Rapid Intervention Crew (RIC))

- Air Cascade-1 FF

- Safety Officer

Central City Fire EMS Dispatch Criteria

| Assignment | Alarm Type | Engine | Ladder | Squad | Battalion Chief | EMS Unit | RIC | Safety Officer | Air Cascade |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| First Alarm | Structural | 3 | 1 |  |  | 1 BLS |  |  |  |
| Working Incident | Structural | 2 |  | 1 | 1 |  | 1 | 1 | 1 |
| First Alarm | Target Hazard | 3 | 2 | 1 | 1 | 1 BLS |  |  |  |
| Working Incident | Target Hazard | 2 |  |  |  | 1 ALS | 1 | 1 | 1 |
| Second Alarm | Structural/ Target Hazard | 2 | 1 |  | 1 | 1EMS |  | 1 | 1 |
| Third Alarm | Structural/ Target Hazard | 2 | 1 |  |  | 1 MS |  |  |  |
| Fourth Alarm | Structural/ Target Hazard | 2 | 1 |  | 1 |  |  |  |  |

Central City Fire/EMS Staffing:

- Engine-Ladder-Squad Company--1 Officer--3 Firefighters

- Rapid Intervention Crew (RIC)--One Engine Company

- EMS BLS Unit--2 EMT Basic

- EMS ALS Unit--2 EMT Paramedic

- Air Cascade--1 Firefighter

A second alarm may be requested but will not arrive on scene before end of exercise.

Activity 7.1 (cont'd)

Objectives-Strategy-Tactics Chart

| Objectives | Strategy(ies) | Tactics | Assigned to: |
| --- | --- | --- | --- |
| Objective #1 | 1. | 1. 2. 3. | 1. 2. 3. |
|  | 2. | 1. 2. 3. | 1. 2. 3. |
|  | 3. | 1. 2. 3. | 1. 2. 3. |
| Objective #2 | 1. | 1. 2. 3. | 1. 2. 3. |
|  | 2. | 1. 2. 3. | 1. 2. 3. |
|  | 3. | 1. 2. 3. | 1. 2. 3. |
| Objective #3 | 1. | 1. 2. 3. | 1. 2. 3. |
|  | 2. | 1. 2. 3. | 1. 2. 3. |
|  | 3. | 1. 2. 3. | 1. 2. 3. |

This page intentionally left blank.

Activity 7.1 (cont'd)

ICS Form 201, Incident Debriefing and ICS Form 214, Unit Log

1.	Use the enlarged ICS Form 214, Unit Log or easel pad.

2.	The ICOA will maintain ICS Form 214 for all tactical decisions made by the ICO along with other pertinent messages and decisions made by the ICO during the exercise.  Entries should be recorded under Time and Major Events columns.

3.	Distribute ICS Form 214.

a.	IISO.

b.	Initial Planning Section (Situation Unit).

c.	Initial Logistics Section (Service and Support).

4.	Distribute ICS Form 201, Incident Debriefing (enlarged to 11 by 17).

a.	Initial Planning Section (Situation Unit).

b.	Initial Logistics Section (Service and Support).

5.	Begin exercise scenario message distribution.

6.	Follow message distribution timeframes until all messages have been distributed.  (You have 30 minutes.)

This page intentionally left blank.

Activity 7.1 (cont'd)

Preparing for Exercise Debriefing

You have 15 minutes for each group to prepare their debriefing materials for presentation.

1.	ICO.

a.	Primary Factors Chart.

b.	Objectives-Strategy-Tactics Chart.

2.	Initial Company Officer Assistant--ICS Form 214.

3.	Initial Safety Officer--ICS Form 214.

4.	Initial Planning Section.

5.	Initial Logistics Section--ICS Form 201.

This page intentionally left blank.

Activity 7.1 (cont'd)

ICS Form 214, Unit Log

| UNIT LOG |  | 1. Incident Name | 2. Date Prepared |  | 3. Time Prepared |
| --- | --- | --- | --- | --- | --- |
| 4. Unit Name/Designators |  | 5. Unit Leader (Name and Position) |  |  | 6. Operational Period |
| 7. Personnel Roster Assigned |  |  |  |  |  |
| Name |  | ICS Position |  | Home Base |  |
| 8. Activity Log |  |  |  |  |  |
| Time | Major Events |  |  |  |  |
| 9. Prepared by (Name and Position) |  |  |  |  |  |

Instructions for completing the Unit Log (ICS Form 214)

| Item Number | Item Title | Instructions |
| --- | --- | --- |
| 1. | Incident Name | Print the name assigned to the incident. |
| 2. | Date Prepared | Enter date prepared (month, day, year). |
| 3. | Time Prepared | Enter time prepared (24-hour clock). |
| 4. | Unit Name | Enter the title of the organizational unit or resource designator (Engine 1 - Safety Officer, Situation Unit – Logistics Section). |
| 5. | Unit Leader | Enter the name of the individual in charge of the Unit. |
| 6. | Operational Period | Enter the time span covered by the log (e.g., 1800 Oct. 12 to 0600 Oct. 13). |
| 7. | Personnel Roster | List the name, position, and home base of each member assigned to the unit during the operational period. |
| 8. | Activity Log | Enter the time and briefly describe each significant occurrence or event (e.g., task assignments, task completions, injuries, difficulties encountered, etc.). |
| 9. | Prepared By | Enter the name and title of the person approving the log. Provide log to immediate supervisor at the end of each operational period. |

Activity 7.1 (cont'd)

ICS Form 201, Incident Debriefing

| incident briefing |  | 1. incident name | 2. date prepared | 3. time prepared |
| --- | --- | --- | --- | --- |
| 4. map sketch |  |  |  |  |
| ICS 201 Page 1 of 4 | 5. prepared by (name and position) |  |  |  |

| 6. summary of current actions |  |
| --- | --- |
| ICS 201 | page 2 |

| 7. current organization |  |
| --- | --- |
| ICS 201 | page 3 |

| 8. resources summary |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| resources ordered |  | resource identification | eta | on scene √ | location/assignment |
| ICS 201 | page 4 |  |  |  |  |

Instructions for completing the Incident Briefing (ICS Form 201)

| Item Number | Item Title | Instructions |
| --- | --- | --- |
| 1. | Incident Name | Print the name assigned to the incident. |
| 2. | Date Prepared | Enter date prepared (month, day, year). |
| 3. | Time Prepared | Enter time prepared (24-hour clock). |
| 4. | Map Sketch | Show perimeter and control lines, resources assignments, incident facilities, and other special information on a sketch map or attached to the topographic or orthophoto map. |
| 5. | Resources Summary | Enter the following information about the resources allocated to the incident. Enter the number and type of resource ordered. |
|  | Resources Ordered | Enter the number and type of resource ordered. |
|  | Resource Identification | Enter the agency three-letter designator, S/T, Kind/ Type and resource designator. |
|  | ETA/On Scene | Enter the estimated arrival time and place the arrival time or a checkmark in the "on scene" column upon arrival. |
|  | Location/ Assignment | Enter the assigned location of the resource and/or the actual assignment. |
| 6. | Current Organization | Enter on the organization chart the names of the individuals assigned to each position. Modify the chart as necessary. |
| 7. | Summary of Current Actions | Enter the strategy and tactics used on the incident and note any specific problem areas. |
| 8. | Prepared By | Enter the name and position of the person completing the form. |
| *Note |  | Additional pages may be added to ICS Form 201 if needed. |

This page intentionally left blank.

Activity 7.1 (cont'd)

Debriefing Procedures

1.	The instructor will debrief exercise scenarios with each group.

2.	The instructor will display the slides for your group's scenario when debriefing.

3.	Assigned ICO for each group shall present the following:

a.	Primary Factors Chart.

b.	Objectives-Strategy-Tactics Chart.

c.	ICS Form 214.

4.	Each group will have 15 minutes to debrief.

5.	The instructor will address any specific issues that were omitted from charts and identify issues where improvement is required.

This page intentionally left blank.

Activity 7.1 (cont'd)

Plot Plans

Exercise #5

Shopping Center--Walkaround

Iteration 1--Message 1

Iteration 2--Message 2

Iteration 2--Message 3

Iteration 3--Message 4

Iteration 3--Message 5

Iteration 4--Message 6

Iteration 4--Message 7

Iteration 4--Message 8

This page intentionally left blank.

Activity 7.1 (cont'd)

Large Group Exercise #6:  Bank Mansard Roof

Purpose

To allow you to demonstrate your knowledge, skills, and abilities (KSAs) you have gained as a result of attending this course.

Conduct a walkaround of exposures A-B-C-D of the structures that will be seen in the exercise.  There can be up to seven structures used in the activity.

Directions

1.	You will be working in small groups.

2.	Each group will do each construction type.  Exercise is designed to place the ICO as the Incident Commander (IC) until relieved by the next level of authority arriving at the incident.  (This will be your instructor.)

3.	For this simulation the ICO will have additional students assigned to Command and General Staff positions, which under normal field conditions would be the sole responsibility of the ICO.

4.	Student group assignments for each group:

a.	ICO.

b.	ICOA--scribe.

c.	IISO.

d.	Initial Planning Section--Situation Unit.

e.	Initial Logistics Section--service and support.

5.	Your group will record your predictions on an easel pad or paper.

6.	Your group will select a spokesperson to explain to the class how fire would travel in this building.

7.	Throughout the activity you will receive messages from the instructor.

8.	In this portion of exercise your group is to concentrate on Column 1 Primary Factors Chart.  You will have 10 minutes to complete this portion of the activity.

9.	The ICOA-scribe shall develop a Primary Factors Exercise Chart on an easel pad for his/her assigned exercise scenario.

a.	Identify the most pertinent primary factors for each exercise scenario.

b.	Place findings on Column 1 of the Primary Factors Exercise Chart.

c.	Identify the most pertinent subfactors for each identified primary factor for exercise scenario.  Place findings on Column 2 of the Primary Factors Exercise Chart.

d.	Identify pertinent precautions that will be taken to address each identified sub-factor for exercise scenario.  Place findings on Column 3 of the Primary Factors Exercise Chart.

Activity 7.1 (cont'd)

Primary Factors Chart

This page intentionally left blank.

Activity 7.1 (cont'd)

Primary Factors Exercise Chart

| Pertinent Primary Factor | Pertinent Subfactor | Pertinent Precautions To Be Taken |
| --- | --- | --- |
| 1. | 1. 2. | 1. 2. |
| 2. | 1. 2. 3. | 1. 2. 3. |
| 3. | 1. 2. | 1. 2. |
| 4. | 1. 2. | 1. 2. |
| 5. | 1. 2. | 1. 2. |
| 6. | 1. 2. | 1. 2. |
| 7. | 1. 2. 3. 4. | 1. 2. 3. 4. |
| 8. | 1. 2. 3. | 1. 2. 3. |
| 9. | 1. 2. | 1. 2. |
| 10. | 1. 2. | 1. 2. |
| 11. | 1. | 1. |
| 12. | 1. 2. | 1. 2. |
| 13. | 1. 2. | 1. 2. |

This page intentionally left blank.

LARGE EXERCISE #6

Vital Building Information

Situation Report

COMMERCIAL--TYPE III--ORDINARY

Structure:			Three-story--40 by 50 feet

Building Construction:	Type III--ordinary

Roof Construction:		2- by 12-inch mansard roof with

asphalt shingles

Floors:			2- by 6-inch flooring system

tongue and groove

Alarm System:		Smoke detectors installed

Occupants:			Twenty employees and

customers

Special Concerns:		Bank security locations

Situation Report:

Fire Building:

It is Friday, December 23, 1230 hours, temperature 25 °F (-4 °C), wind from north at 9 mph.

Upon arrival, several employees and customers are outside the building.  Three employees who attempted to fight the fire are suffering from minor burns and smoke inhalation.  The bank manager reports a few employees were eating lunch in the third break/lunch room and they are unaccounted for.  He reports that the fire is in the second floor copy/printing room.

Exposures:

No immediate exposures.

This page intentionally left blank.

Activity 7.1 (cont'd)

Objectives-Strategy-Tactics Chart

1.	Use the enlarged Objectives-Strategy-Tactics Chart or easel pad.

2.	Refer back the Primary Factors Exercise materials in Unit 5:  The Analytical Sizeup Process if necessary.

3.	Place Slide Iteration #2 for Sides A-B-C-D for each scenario chosen.

4.	Receive and act on Exercise Message #2 and Plot Plan #2.

5.	Concentrate on Columns 2 and 3 and develop the Objectives-Strategy-Tactics for the scenario.

6.	ICOA scribe shall develop an easel pad for Objectives-Strategy-Tactics for assigned exercise scenario.  (You have 20 minutes to complete.)

a.	Column 1:  Identify the first operational period objectives for the scenario.

b.	Column 2:  Identify the first operational period strategies for each objective.

c.	Column 3:  Identify the first operational period tactics for each strategy.

d.	Column 4:  Assign companies to perform tactics.

Each scenario has first alarm structural resources available at start of exercise:

- Engine 1

- Engine 2

- Engine 3

- Ladder 1

- Basic Life Support (BLS) 1

Each scenario will have working incident structural resources arrive 10 minutes into incident.

- Engine 5

- Engine 6

- Squad 1

- Engine 6 (Rapid Intervention Crew (RIC))

- Air Cascade-1 FF

- Safety Officer

Central City Fire EMS Dispatch Criteria

| Assignment | Alarm Type | Engine | Ladder | Squad | Battalion Chief | EMS Unit | RIC | Safety Officer | Air Cascade |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| First Alarm | Structural | 3 | 1 |  |  | 1 BLS |  |  |  |
| Working Incident | Structural | 2 |  | 1 | 1 |  | 1 | 1 | 1 |
| First Alarm | Target Hazard | 3 | 2 | 1 | 1 | 1 BLS |  |  |  |
| Working Incident | Target Hazard | 2 |  |  |  | 1 ALS | 1 | 1 | 1 |
| Second Alarm | Structural/ Target Hazard | 2 | 1 |  | 1 | 1EMS |  | 1 | 1 |
| Third Alarm | Structural/ Target Hazard | 2 | 1 |  |  | 1 MS |  |  |  |
| Fourth Alarm | Structural/ Target Hazard | 2 | 1 |  | 1 |  |  |  |  |

Central City Fire/EMS Staffing:

- Engine-Ladder-Squad Company--1 Officer--3 Firefighters

- Rapid Intervention Crew (RIC)--One Engine Company

- EMS BLS Unit--2 EMT Basic

- EMS ALS Unit--2 EMT Paramedic

- Air Cascade--1 Firefighter

A second alarm may be requested but will not arrive on scene before end of exercise.

Activity 7.1 (cont'd)

Objectives-Strategy-Tactics Chart

| Objectives | Strategy(ies) | Tactics | Assigned to: |
| --- | --- | --- | --- |
| Objective #1 | 1. | 1. 2. 3. | 1. 2. 3. |
|  | 2. | 1. 2. 3. | 1. 2. 3. |
|  | 3. | 1. 2. 3. | 1. 2. 3. |
| Objective #2 | 1. | 1. 2. 3. | 1. 2. 3. |
|  | 2. | 1. 2. 3. | 1. 2. 3. |
|  | 3. | 1. 2. 3. | 1. 2. 3. |
| Objective #3 | 1. | 1. 2. 3. | 1. 2. 3. |
|  | 2. | 1. 2. 3. | 1. 2. 3. |
|  | 3. | 1. 2. 3. | 1. 2. 3. |

This page intentionally left blank.

Activity 7.1 (cont'd)

ICS Form 201, Incident Debriefing and ICS Form 214, Unit Log

1.	Use the enlarged ICS Form 214, Unit Log or easel pad.

2.	The ICOA will maintain ICS Form 214 for all tactical decisions made by the ICO along with other pertinent messages and decisions made by the ICO during the exercise.  Entries should be recorded under Time and Major Events columns.

3.	Distribute ICS Form 214.

a.	IISO.

b.	Initial Planning Section (Situation Unit).

c.	Initial Logistics Section (Service and Support).

4.	Distribute ICS Form 201, Incident Debriefing (enlarged to 11 by 17).

a.	Initial Planning Section (Situation Unit).

b.	Initial Logistics Section (Service and Support).

5.	Begin exercise scenario message distribution.

6.	Follow message distribution timeframes until all messages have been distributed.  (You have 30 minutes.)

This page intentionally left blank.

Activity 7.1 (cont'd)

Preparing for Exercise Debriefing

You have 15 minutes for each group to prepare their debriefing materials for presentation.

1.	ICO.

a.	Primary Factors Chart.

b.	Objectives-Strategy-Tactics Chart.

2.	Initial Company Officer Assistant--ICS Form 214.

3.	Initial Safety Officer--ICS Form 214.

4.	Initial Planning Section.

5.	Initial Logistics Section--ICS Form 201.

This page intentionally left blank.

Activity 7.1 (cont'd)

ICS Form 214, Unit Log

| UNIT LOG |  | 1. Incident Name | 2. Date Prepared |  | 3. Time Prepared |
| --- | --- | --- | --- | --- | --- |
| 4. Unit Name/Designators |  | 5. Unit Leader (Name and Position) |  |  | 6. Operational Period |
| 7. Personnel Roster Assigned |  |  |  |  |  |
| Name |  | ICS Position |  | Home Base |  |
| 8. Activity Log |  |  |  |  |  |
| Time | Major Events |  |  |  |  |
| 9. Prepared by (Name and Position) |  |  |  |  |  |

Instructions for completing the Unit Log (ICS Form 214)

| Item Number | Item Title | Instructions |
| --- | --- | --- |
| 1. | Incident Name | Print the name assigned to the incident. |
| 2. | Date Prepared | Enter date prepared (month, day, year). |
| 3. | Time Prepared | Enter time prepared (24-hour clock). |
| 4. | Unit Name | Enter the title of the organizational unit or resource designator (Engine 1 - Safety Officer, Situation Unit – Logistics Section). |
| 5. | Unit Leader | Enter the name of the individual in charge of the Unit. |
| 6. | Operational Period | Enter the time span covered by the log (e.g., 1800 Oct. 12 to 0600 Oct. 13). |
| 7. | Personnel Roster | List the name, position, and home base of each member assigned to the unit during the operational period. |
| 8. | Activity Log | Enter the time and briefly describe each significant occurrence or event (e.g., task assignments, task completions, injuries, difficulties encountered, etc.). |
| 9. | Prepared By | Enter the name and title of the person approving the log. Provide log to immediate supervisor at the end of each operational period. |

Activity 7.1 (cont'd)

ICS Form 201, Incident Debriefing

| incident briefing |  | 1. incident name | 2. date prepared | 3. time prepared |
| --- | --- | --- | --- | --- |
| 4. map sketch |  |  |  |  |
| ICS 201 Page 1 of 4 | 5. prepared by (name and position) |  |  |  |

| 6. summary of current actions |  |
| --- | --- |
| ICS 201 | page 2 |

| 7. current organization |  |
| --- | --- |
| ICS 201 | page 3 |

| 8. resources summary |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| resources ordered |  | resource identification | eta | on scene √ | location/assignment |
| ICS 201 | page 4 |  |  |  |  |

Instructions for completing the Incident Briefing (ICS Form 201)

| Item Number | Item Title | Instructions |
| --- | --- | --- |
| 1. | Incident Name | Print the name assigned to the incident. |
| 2. | Date Prepared | Enter date prepared (month, day, year). |
| 3. | Time Prepared | Enter time prepared (24-hour clock). |
| 4. | Map Sketch | Show perimeter and control lines, resources assignments, incident facilities, and other special information on a sketch map or attached to the topographic or orthophoto map. |
| 5. | Resources Summary | Enter the following information about the resources allocated to the incident. Enter the number and type of resource ordered. |
|  | Resources Ordered | Enter the number and type of resource ordered. |
|  | Resource Identification | Enter the agency three-letter designator, S/T, Kind/ Type and resource designator. |
|  | ETA/On Scene | Enter the estimated arrival time and place the arrival time or a checkmark in the "on scene" column upon arrival. |
|  | Location/ Assignment | Enter the assigned location of the resource and/or the actual assignment. |
| 6. | Current Organization | Enter on the organization chart the names of the individuals assigned to each position. Modify the chart as necessary. |
| 7. | Summary of Current Actions | Enter the strategy and tactics used on the incident and note any specific problem areas. |
| 8. | Prepared By | Enter the name and position of the person completing the form. |
| *Note |  | Additional pages may be added to ICS Form 201 if needed. |

This page intentionally left blank.

Activity 7.1 (cont'd)

Debriefing Procedures

1.	The instructor will debrief exercise scenarios with each group.

2.	The instructor will display the slides for your group's scenario when debriefing.

3.	Assigned ICO for each group shall present the following:

a.	Primary Factors Chart.

b.	Objectives-Strategy-Tactics Chart.

c.	ICS Form 214.

4.	Each group will have 15 minutes to debrief.

5.	The instructor will address any specific issues were omitted from charts and identify issues where improvement is required.

This page intentionally left blank.

Activity 7.1 (cont'd)

Plot Plans

Exercise #6

Bank--Walkaround

Iteration 1--Message 1

Iteration 2--Message 2

Iteration 2--Message 3

Iteration 3--Message 4

Iteration 3--Message 5

Iteration 4--Message 6

Iteration 4--Message 7

Iteration 4--Message 8

Activity 7.1 (cont'd)

Large Group Exercise #7:  Noncombustible Warehouse

Purpose

To allow you to demonstrate your knowledge, skills, and abilities (KSAs) you have gained as a result of attending this course.

Conduct a walkaround of exposures A-B-C-D of the structures that will be seen in the exercise.  There can be up to seven structures used in the activity.

Directions

1.	You will be working in small groups.

2.	Each group will do each construction type.  Exercise is designed to place the ICO as the Incident Commander (IC) until relieved by the next level of authority arriving at the incident.  (This will be your instructor.)

3.	For this simulation the ICO will have additional students assigned to Command and General Staff positions, which under normal field conditions would be the sole responsibility of the ICO.

4.	Student group assignments for each group:

a.	ICO.

b.	ICOA--scribe.

c.	IISO.

d.	Initial Planning Section--Situation Unit.

e.	Initial Logistics Section--service and support.

5.	Your group will record your predictions on an easel pad or paper.

6.	Your group will select a spokesperson to explain to the class how fire would travel in this building.

7.	Throughout the activity you will receive messages from the instructor.

8.	In this portion of exercise your group is to concentrate on Column 1 Primary Factors Chart.  You will have 10 minutes to complete this portion of the activity.

9.	The ICOA-scribe shall develop a Primary Factors Exercise Chart on an easel pad for his/her assigned exercise scenario.

a.	Identify the most pertinent primary factors for each exercise scenario.

b.	Place findings on Column 1 of the Primary Factors Exercise Chart.

c.	Identify the most pertinent subfactors for each identified primary factor for exercise scenario.  Place findings on Column 2 of the Primary Factors Exercise Chart.

d.	Identify pertinent precautions that will be taken to address each identified sub-factor for exercise scenario.  Place findings on Column 3 of the Primary Factors Exercise Chart.

Activity 7.1 (cont'd)

Primary Factors Chart

This page intentionally left blank.

Activity 7.1 (cont'd)

Primary Factors Exercise Chart

| Pertinent Primary Factor | Pertinent Subfactor | Pertinent Precautions To Be Taken |
| --- | --- | --- |
| 1. | 1. 2. | 1. 2. |
| 2. | 1. 2. 3. | 1. 2. 3. |
| 3. | 1. 2. | 1. 2. |
| 4. | 1. 2. | 1. 2. |
| 5. | 1. 2. | 1. 2. |
| 6. | 1. 2. | 1. 2. |
| 7. | 1. 2. 3. 4. | 1. 2. 3. 4. |
| 8. | 1. 2. 3. | 1. 2. 3. |
| 9. | 1. 2. | 1. 2. |
| 10. | 1. 2. | 1. 2. |
| 11. | 1. | 1. |
| 12. | 1. 2. | 1. 2. |
| 13. | 1. 2. | 1. 2. |

This page intentionally left blank.

LARGE EXERCISE #7

Vital Building Information

Situation Report

COMMERCIAL--TYPE II--NONCOMBUSTIBLE

Structure:			One-story--metal fabricating and finishing

Building Construction:	Type II--noncombustible

Roof Construction:		Structural steel with asphalt covering over metal decking

Floors:			Concrete slab

Alarm System:		Smoke detectors installed

Sprinkler:			Paint spraying refinishing room only

Special Concerns:		Metal fabricating and spray paint

Finishing compressed air bottles oxygen and acetylene

Solvents and paint containers

Situation Report:

Fire Building:

It is August 14, 1530 hours, temperature is 92 °F (33 °C), wind from north at 3 mph.

Upon arrival, several employees are outside the building on Side A.  Plant foreman reports a small explosion occurred near the paint finishing room and fire broke out after the explosion.  Two employees are suffering from arm and neck burns and need further treatment.  Three employees are unaccounted for.  They worked in the manufacturing finishing area.  Four office employees are complaining of smoke inhalation.  All office employees are accounted for.

Exposures:

No immediate exposures.

This page intentionally left blank.

Activity 7.1 (cont'd)

Objectives-Strategy-Tactics Chart

1.	Use the enlarged Objectives-Strategy-Tactics Chart or easel pad.

2.	Refer back the Primary Factors Exercise materials in Unit 5:  The Analytical Sizeup Process if necessary.

3.	Place Slide Iteration #2 for Sides A-B-C-D for each scenario chosen.

4.	Receive and act on Exercise Message #2 and Plot Plan #2.

5.	Concentrate on Columns 2 and 3 and develop the Objectives-Strategy-Tactics for the scenario.

6.	ICOA scribe shall develop an easel pad for Objectives-Strategy-Tactics for assigned exercise scenario.  (You have 20 minutes to complete.)

a.	Column 1:  Identify the first operational period objectives for the scenario.

b.	Column 2:  Identify the first operational period strategies for each objective.

c.	Column 3:  Identify the first operational period tactics for each strategy.

d.	Column 4:  Assign companies to perform tactics.

Each scenario has first alarm structural resources available at start of exercise:

- Engine 1

- Engine 2

- Engine 3

- Ladder 1

- Basic Life Support (BLS) 1

Each scenario will have working incident structural resources arrive 10 minutes into incident.

- Engine 5

- Engine 6

- Squad 1

- Engine 6 (Rapid Intervention Crew (RIC))

- Air Cascade-1 FF

- Safety Officer

Central City Fire EMS Dispatch Criteria

| Assignment | Alarm Type | Engine | Ladder | Squad | Battalion Chief | EMS Unit | RIC | Safety Officer | Air Cascade |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| First Alarm | Structural | 3 | 1 |  |  | 1 BLS |  |  |  |
| Working Incident | Structural | 2 |  | 1 | 1 |  | 1 | 1 | 1 |
| First Alarm | Target Hazard | 3 | 2 | 1 | 1 | 1 BLS |  |  |  |
| Working Incident | Target Hazard | 2 |  |  |  | 1 ALS | 1 | 1 | 1 |
| Second Alarm | Structural/ Target Hazard | 2 | 1 |  | 1 | 1EMS |  | 1 | 1 |
| Third Alarm | Structural/ Target Hazard | 2 | 1 |  |  | 1 MS |  |  |  |
| Fourth Alarm | Structural/ Target Hazard | 2 | 1 |  | 1 |  |  |  |  |

Central City Fire/EMS Staffing:

- Engine-Ladder-Squad Company--1 Officer--3 Firefighters

- Rapid Intervention Crew (RIC)--One Engine Company

- EMS BLS Unit--2 EMT Basic

- EMS ALS Unit--2 EMT Paramedic

- Air Cascade--1 Firefighter

A second alarm may be requested but will not arrive on scene before end of exercise.

Activity 7.1 (cont'd)

Objectives-Strategy-Tactics Chart

| Objectives | Strategy(ies) | Tactics | Assigned to: |
| --- | --- | --- | --- |
| Objective #1 | 1. | 1. 2. 3. | 1. 2. 3. |
|  | 2. | 1. 2. 3. | 1. 2. 3. |
|  | 3. | 1. 2. 3. | 1. 2. 3. |
| Objective #2 | 1. | 1. 2. 3. | 1. 2. 3. |
|  | 2. | 1. 2. 3. | 1. 2. 3. |
|  | 3. | 1. 2. 3. | 1. 2. 3. |
| Objective #3 | 1. | 1. 2. 3. | 1. 2. 3. |
|  | 2. | 1. 2. 3. | 1. 2. 3. |
|  | 3. | 1. 2. 3. | 1. 2. 3. |

This page intentionally left blank.

Activity 7.1 (cont'd)

ICS Form 201, Incident Debriefing and ICS Form 214, Unit Log

1.	Use the enlarged ICS Form 214, Unit Log or easel pad.

2.	The ICOA will maintain ICS Form 214 for all tactical decisions made by the ICO along with other pertinent messages and decisions made by the ICO during the exercise.  Entries should be recorded under Time and Major Events columns.

3.	Distribute ICS Form 214.

a.	IISO.

b.	Initial Planning Section (Situation Unit).

c.	Initial Logistics Section (Service and Support).

4.	Distribute ICS Form 201, Incident Debriefing (enlarged to 11 by 17).

a.	Initial Planning Section (Situation Unit).

b.	Initial Logistics Section (Service and Support).

5.	Begin exercise scenario message distribution.

6.	Follow message distribution timeframes until all messages have been distributed.  (You have 30 minutes.)

This page intentionally left blank.

Activity 7.1 (cont'd)

Preparing for Exercise Debriefing

You have 15 minutes for each group to prepare their debriefing materials for presentation.

1.	ICO.

a.	Primary Factors Chart.

b.	Objectives-Strategy-Tactics Chart.

2.	Initial Company Officer Assistant--ICS Form 214.

3.	Initial Safety Officer--ICS Form 214.

4.	Initial Planning Section.

5.	Initial Logistics Section--ICS Form 201.

This page intentionally left blank.

Activity 7.1 (cont'd)

ICS Form 214, Unit Log

| UNIT LOG |  | 1. Incident Name | 2. Date Prepared |  | 3. Time Prepared |
| --- | --- | --- | --- | --- | --- |
| 4. Unit Name/Designators |  | 5. Unit Leader (Name and Position) |  |  | 6. Operational Period |
| 7. Personnel Roster Assigned |  |  |  |  |  |
| Name |  | ICS Position |  | Home Base |  |
| 8. Activity Log |  |  |  |  |  |
| Time | Major Events |  |  |  |  |
| 9. Prepared by (Name and Position) |  |  |  |  |  |

Instructions for completing the Unit Log (ICS Form 214)

| Item Number | Item Title | Instructions |
| --- | --- | --- |
| 1. | Incident Name | Print the name assigned to the incident. |
| 2. | Date Prepared | Enter date prepared (month, day, year). |
| 3. | Time Prepared | Enter time prepared (24-hour clock). |
| 4. | Unit Name | Enter the title of the organizational unit or resource designator (Engine 1 - Safety Officer, Situation Unit – Logistics Section). |
| 5. | Unit Leader | Enter the name of the individual in charge of the Unit. |
| 6. | Operational Period | Enter the time span covered by the log (e.g., 1800 Oct. 12 to 0600 Oct. 13). |
| 7. | Personnel Roster | List the name, position, and home base of each member assigned to the unit during the operational period. |
| 8. | Activity Log | Enter the time and briefly describe each significant occurrence or event (e.g., task assignments, task completions, injuries, difficulties encountered, etc.). |
| 9. | Prepared By | Enter the name and title of the person approving the log. Provide log to immediate supervisor at the end of each operational period. |

Activity 7.1 (cont'd)

ICS Form 201, Incident Debriefing

| incident briefing |  | 1. incident name | 2. date prepared | 3. time prepared |
| --- | --- | --- | --- | --- |
| 4. map sketch |  |  |  |  |
| ICS 201 Page 1 of 4 | 5. prepared by (name and position) |  |  |  |

| 6. summary of current actions |  |
| --- | --- |
| ICS 201 | page 2 |

| 7. current organization |  |
| --- | --- |
| ICS 201 | page 3 |

| 8. resources summary |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| resources ordered |  | resource identification | eta | on scene √ | location/assignment |
| ICS 201 | page 4 |  |  |  |  |

Instructions for completing the Incident Briefing (ICS Form 201)

| Item Number | Item Title | Instructions |
| --- | --- | --- |
| 1. | Incident Name | Print the name assigned to the incident. |
| 2. | Date Prepared | Enter date prepared (month, day, year). |
| 3. | Time Prepared | Enter time prepared (24-hour clock). |
| 4. | Map Sketch | Show perimeter and control lines, resources assignments, incident facilities, and other special information on a sketch map or attached to the topographic or orthophoto map. |
| 5. | Resources Summary | Enter the following information about the resources allocated to the incident. Enter the number and type of resource ordered. |
|  | Resources Ordered | Enter the number and type of resource ordered. |
|  | Resource Identification | Enter the agency three-letter designator, S/T, Kind/ Type and resource designator. |
|  | ETA/On Scene | Enter the estimated arrival time and place the arrival time or a checkmark in the "on scene" column upon arrival. |
|  | Location/ Assignment | Enter the assigned location of the resource and/or the actual assignment. |
| 6. | Current Organization | Enter on the organization chart the names of the individuals assigned to each position. Modify the chart as necessary. |
| 7. | Summary of Current Actions | Enter the strategy and tactics used on the incident and note any specific problem areas. |
| 8. | Prepared By | Enter the name and position of the person completing the form. |
| *Note |  | Additional pages may be added to ICS Form 201 if needed. |

This page intentionally left blank.

Activity 7.1 (cont'd)

Debriefing Procedures

1.	The instructor will debrief exercise scenarios with each group.

2.	The instructor will display the slides for your group's scenario when debriefing.

3.	Assigned ICO for each group shall present the following:

a.	Primary Factors Chart.

b.	Objectives-Strategy-Tactics Chart.

c.	ICS Form 214.

4.	Each group will have 15 minutes to debrief.

5.	The instructor will address any specific issues that were omitted from charts and identify issues where improvement is required.

This page intentionally left blank.

Activity 7.1 (cont'd)

Plot Plans

Exercise #7

Noncombustible--Walkaround

Iteration 1--Message 1

Iteration 2--Message 2

Iteration 2--Message 3

Iteration 3--Message 4

Iteration 3--Message 5

Iteration 4--Message 6

Iteration 4--Message 7

Iteration 4--Message 8