STATE OF NEW JERSEY
DIVISION DEPARTMENT OF COMMUNITY AFFAIRS

DIVISION OF FIRE SAFETY

FIRE INSPECTOR CERTIFICATION COURSE

Module 11

Fire Protection System 
Code Requirements

in cooperation with

Kean University Fire Safety Training Program

Time: 3 hours

Teaching/Learning Level: Cognitive- knowledge, comprehension, application.

## Learning/Teaching Aids:

- Syllabus

- Board/easel pad

- Chalk/markers

- PC/laptop

- Projection unit

- Projection screen

- Power Point presentation

- New Jersey Uniform Fire Code NJAC 5:70-1 et seq.

- NJIFC 2015

- NJ UCC as adopted and referenced

- IFSTA ‘Fire Inspection and Code Enforcement’ 6th ed.

- NFPA Automatic Sprinkler Systems Handbook

- NFPA Fire Pump Handbook

- NFPA National Fire Alarm Code Handbook

- Student Manual

Behaviors to Foster

Encourage students to listen carefully, take notes and to actively participate by asking questions and offering experiences that will foster discussion during lectures. It is imperative that they absorb and retain as much information as possible to assist them in having a solid foundation of knowledge for application in this field successfully completion the exam for this course and the State Certification exam.

Tasks:

.

- Understanding the requirements of the NJIFC as it applies to inspection, testing, and maintenance of fire protection, fire suppression and smoke control systems and devices.

- Understanding the application of referenced standards in the maintenance of fire protection and fire suppression systems and devices.

Given in a classroom setting:

- The student handout

- New Jersey Uniform Fire Code NJAC 5:70-1 et seq.

- NJIFC -  2015 International Fire Code

- NJ UCC adopted Code  IBC 2015 NJ

- IFSTA ‘Fire Inspection and Code Enforcement’8th ed., Chapters 13 & 14 Power Point presentation

Standards:

- NJAC 5:70-1 et seq.

- NJAC 5:23-1 et seq.

- NFPA 10 Portable Fire Extinguishers

- NFPA 12 Carbon Dioxide Extinguishing Systems

- NFPA 12A Halon 1301 Fire Extinguishing Systems

- NFPA 13 Installation of Sprinkler Systems

- NFPA 14 Installation of Standpipe and Hose Systems

- NFPA 17 Dry Chemical Extinguishing Systems

- NFPA 17A Wet Chemical Extinguishing Systems

- NFPA 20 Installation of Centrifugal Fire Pumps

- NFPA 22 Water Tanks for Private Fire Protection

- NFPA 24 Installation of Private Fire Service Mains

- NFPA 25 Inspection, Testing and Maintenance of Water-Based Fire Protection Systems

- NFPA 230 Fire Protection of General Storage

- NFPA 231 General Storage

- NFPA 231C Rack Storage of Materials

- NFPA 231D Storage of Rubber Tires

- NFPA 231E Storage of Baled Cotton

- NFPA 231F Storage of Rolled Paper

- NFPA 72 National Fire Alarm Code

Prerequisite Knowledge: Module 7, 8, 10

Prerequisite Skills: Ability to listen effectively; follows directions; take notes; and retains knowledge.

## Resources/References:

- New Jersey Uniform Fire Code NJAC 5:70-1 et seq. NJ UCC IBC 2015 edition

- IFSTA ‘Fire Inspection and Code Enforcement’ 8th ed. Chapters 13&14 NFPA Automatic Sprinkler Systems Handbook

- NFPA Fire Pump Handbook

- NFPA National Fire Alarm Code Handbook NFPA Fire Alarm Signaling Systems

- Student Manual

## Attention: (Call to Order)

Motivation: (State Need to Know)

(A reminder to instructors; it is best if the instructor uses an example from a recent or current event that is pertinent to the lesson. In this case you can refer to the need to understand the basics of when the systems were required to be installed pursuant to the UCC and the provisions of the UFC applying to the maintenance of the fire protection and fire suppression systems. Fire losses due to missing or un-maintained systems and/or success stories would be appropriate.)

## Student Performance Objective (SPO):

- SPO-1 The student will have an understanding of when fire sprinkler systems are required to be installed and requirements for maintenance.

- SPO-2 The student will have an understanding of when fire standpipe systems are required to be installed and requirements for maintenance.

- SPO-3 The student will have an understanding of when fire alarm systems are required to be installed and requirements for maintenance.

- SPO-4 The student will have an understanding of when alarm system monitoring methods are required to be installed and requirements for maintenance.

- SPO-5 The student will have an understanding of when commercial kitchen exhaust hood protection systems are required to be installed and requirements for maintenance.

- SPO-6 The student will have an understanding of when smoke control systems are required to be installed and requirements for maintenance.

## Enabling Objectives (EO):

.

- EO 1-1-1 Working from the Power Point presentation and the NJ Adopted UFC (FC 2015), review when the UFC requires testing and maintenance the installation of fire alarm systems and monitoring.

- EO 1-1-2 Working from the Power Point presentation and the NJ Adopted UFC (IFC 2015), review when the UFC requires the maintenance of Automatic Kitchen Systems smoke control systems and components.

- EO 1-1-3 Working from the Power Point presentation and the NJ Adopted UFC (IFC 2015), review when the UFC requires the maintenance of aqueous and non-aqueous  fire suppression  systems and components

## Overview/Main Points:

- Fire inspectors must have an understanding of when fire protection and fire suppression systems and their components are required to be installed and the requirements for maintaining them in a functioning condition.

- The fire inspector must have an understanding of when fire alarms systems and monitoring of them is required to be installed and the requirements for maintaining them in a functioning condition.

- The fire inspector must have a basic knowledge of when smoke removal systems are required to be installed and the requirements for maintaining them in a functioning condition.

## Initial Instructions:

Prior to this lesson the instructor shall have reviewed the lesson plan for this module, the PowerPoint presentation with notes, reference materials, and the student handouts and have read UFC NJAC 5:70-3, Chapter 5, NJ Adopted UCC (IBC 2015) to refresh the instructor’s knowledge on this topic.

## Opener: Call to order; start with a motivator (need to know) related to objectives and the lesson; state objectives and main points.

Teaching points

The purpose of this module is to ensure the student is familiar with the code language covering the requirements for the installation of the fire protection systems and the language covering the requirements for maintenance of the systems and/or devices.

It is essential that the instructor work directly from the codebooks during this module and that the students follow along in their own books. The PowerPoint presentation is only to be used as a placeholder as you work your way through the codebooks.

The students must learn where items are covered within the codebooks to enable them to pass the State certification exam. The only way that will occur is if the modules covering code language are taught directly from the codebooks.

If a question arises that requires a graphic representation to properly present the answer, the instructor can refer to a slide from the previous module or to a pictorial example in the IFSTA textbook.

## Summary:

## Summarize the material covered in this lesson by reviewing the SPOs listed and the Main Points during the Evaluation. At the end of the Summary/Evaluation remind the students of the next class date/time and the homework that must be completed to successfully participate in class and comprehend the material provided during the next lesson.

## Module 11 Fire Protection System Code Requirements

Student Performance Objective (SPO):

- SPO-1 The student will have an understanding of when fire sprinkler systems are required to be installed and requirements for maintenance.

- SPO-2 The student will have an understanding of when fire standpipe systems are required to be installed and requirements for maintenance.

- SPO-3 The student will have an understanding of when fire alarm systems are required to be installed and requirements for maintenance.

- SPO-4 The student will have an understanding of when alarm system monitoring methods are required to be installed and requirements for maintenance.

- SPO-5 The student will have an understanding of when commercial kitchen exhaust hood protection systems are required to be installed and requirements for maintenance.

- SPO-6 The student will have an understanding of when smoke control systems are required to be installed and requirements for maintenance.

## Enabling Objectives (EO):

- EO 1-1-1 Working from the Power Point presentation and the NJ Adopted UCC (IBC 2015), review when the UCC requires the installation of fire protection and fire suppression systems.

- EO 1-1-2 Working from the Power Point presentation and the NJ Adopted UCC (IBC 2015), review when the UCC requires the installation of fire alarm systems and monitoring.

- EO 1-1-3 Working from the Power Point presentation and the NJ Adopted UCC (IBC 2015), review when the UCC requires the installation of smoke control systems and components.

## Overview/Main Points:

- Fire inspectors must have an understanding of when fire protection and fire suppression systems and their components are required to be installed and the requirements for maintaining them in a functioning condition.

- The fire inspector must have an understanding of when fire alarms systems and monitoring of them is required to be installed and the requirements for maintaining them in a functioning condition.

- The fire inspector must have a basic knowledge of when smoke removal systems are required to be installed and the requirements for maintaining them in a functioning condition.

## EVALUATION

Oral Review: Utilizing the SPOS and Main Points, orally assess the students’ comprehension of the material provided during this lesson.

Other Evaluation: (If there are assigned quizzes for this lesson, state that fact here.)

Instructors may use course quizzes, or create and use lesson quizzes and other learning reinforcements. Quizzes are diagnostic and may be given as in-class group assignments to generate discussion or as home assignments and used as review prior to starting the next session.

HOMEWORK

# Readings

UFC 5:70-3 Chapter 9

# Assignments

- End Module  Quiz

APPENDIX

# LEVEL OF INSTRUCTION

| Cognitive - Knowledge - Comprehension - Application - Analysis - Synthesis - Evaluation | Psychomotor - Perception - Set - Guided Response. - Mechanism - Complex Overt Response - Adaptation - Origination |
| --- | --- |

### DESCRIPTIONS

Cognitive:

- Knowledge:  remembers, recalls; the lowest learning level.  Defines, describes, identifies, labels, lists, matches, names, outlines, reproduces, selects, states.

- Comprehension:  grasps meaning, interprets material, estimates future trends; the lowest level of understanding.  Converts, defends, gives examples, distinguishes, estimates, explains, extends, generalizes, infers, paraphrases, predicts, rewrites, summarizes.

- Application:  uses material in new and concrete situations, applies rules, methods, concepts, principles, laws, and theories; requires higher understanding level.  Changes, computes, demonstrate, solve, discover, manipulate, modify, operate, predict, prepare, uses, produces, relates.

- Analysis:  breaks material into components to understand structural organizational; higher intellectual level than comprehension and application requiring understanding of both structure and content.  Breaks down, diagrams, differentiates, infers, discriminates, relates, distinguishes, identifies, illustrates, outlines, points out, selects, separates, subdivides.

- Synthesis:  able to put parts together to form a new whole, stresses creative behaviors, emphasizes forming new patterns or structures.  Categorizes, combines, complies, composes, creates, devises, designs, explains, generates, modifies, organizes, plans, relates, revises, reconstructs, reorganizes, writes/rewrites, summarizes, tells.

- Evaluation:  able to judge value of material for a given purpose based on definite criteria.  Highest in cognitive hierarchy as this contains elements of all other categories plus conscious value judgments based on clearly defined criteria.  Appraises, compares, concludes, relates, contrasts, criticizes, describes, discriminates, explains, justifies, interprets, summarizes, supports.

Psychomotor:

- Perception:  uses organs or sense to obtain cues to guide motor activity.  Chooses, describes, detects, differentiates, distinguishes, identifies, isolates, relates, selects, separates

- Set:  readiness to take a particular type of action; includes mental, physical or emotional set.  Begins, displays, explains, moves, shows, proceeds, reacts, responds, starts, volunteers.

- Guided Response:  early stages in learning a complex skill; includes imitation, trial and error.  Assembles, builds, calibrates, displays, constructs, dismantles, dissects, fastens, fixes, grinds, heats, manipulates, measures, mends, mixes, organizes, sketches, works.

- Mechanism:  performs acts where learned responses have become habitual and moves with confidence and proficiency; same as guided response.

- Complex Overt Response:  skillful motor performance of complex movement.  Performs proficiently, quickly, smoothly, accurately with minimum energy, without hesitation.  Same as guided response.

- Adaptation:  skills are so well developed that movement patterns can be modified to fit special requirements or meet problem situations.  Adapts, alters, changes, rearranges, reorganizes, revises, varies.

- Origination:  creates new movement patterns to fit a unique situation or problem.  Emphasizes creativity based on highly developed skills.  Arranges, combines, composes, constructs, designs, originates