FIRE INSPECTOR CERTIFICATION COURSE

Student Manual

Module 2

Theory of Code Enforcement

in cooperation with

Kean University Fire Safety Training Program

COURSE OBJECTIVES

Student Performance Objectives (SPO):

- SPO-1: The student will shall define the path of legal authority.

- SPO-2: The Student will understand how buildings are designed and constructed

- SPO-3: The student shall identify the difference between a UFC & UCC.

- SPO-4: The student shall identify the difference between use groups.

Module Outline

- Building classifications

  - Plan review

  - Use group determination

  - Code applications

- Code Enforcement in New Jersey

  - Pre 1977

  - Post 1977

- Use groups defined

- Use group examples

SUPPLEMENTAL MATERIALS

New Jersey Adopted Construction Codes

The following codes are adopted through the Uniform Construction Code (N.J.A.C. 5:23). The Uniform Construction Code can be obtained from the Department of Community Affairs, Publications Unit at (609) 984-0040.

| Code | Adoption Date |
| --- | --- |
| International Building Code ‐NJ ed., 2015 Obtained From: International Code Council, Inc. [888] 422‐ 7233 | Sept‐21‐2015(w/tech. amendments at NJAC 5:23‐3.14 http://www.lexisnexis.com/hottopics/njcode/ |
| National Electrical Code, 2014 Obtained From: National Fire Protection Association [617] 770‐3000 | Sept‐21‐2015 (w/tech. amendments at NJAC 5:23‐3.16) |
| National Standard Plumbing Code, 2015 Obtained From: National Assoc. of Plumbing‐Heating‐Cooling Contractors [800] 533‐7694 or in New Jersey at [800] 652‐7422 | January 4, 2016 (w/tech. amendments at NJAC 5:23‐3.15) |
| International Energy Conservation Code‐NJ ed,2015(Residential) Obtained From: International Code Council [888] 422‐7233 and ASHRAE 90.1‐2013(Commercial) Obtained From: American Society of Heating, Refrigerating and Air‐Conditioning Engineers, Inc. [800] 527‐4723 | Sept‐21‐2015 (w/tech. amendments at NJAC 5:23‐3.18) |
| International Mechanical Code,2015 Obtained From: International Code Council [888] 422‐7233 | Sept‐21‐2015 (w/tech. amendments at NJAC 5:23‐3.20) |
| International Fuel Gas Code,2009 Obtained From: International Code Council [888] 422‐7233 | Sept‐21‐2015 (w/tech. amendments at NJAC 5:23‐3.22) |
| International Residential Code ‐NJ ed., 2015 Obtained From: International Code Council [888] 422‐7233 | Sept‐21‐2015 (w/tech. amendments at NJAC 5:23‐3.21) |
| Rehabilitation Subcode N.J.A.C. 5:23‐6 Obtained From: State of NJ, Department of Community Affairs, Division of Codes and Standards [609] 984‐0040 | Updated Yearly |
| Barrier‐Free Subcode 5:23‐7 Obtained From : State of NJ, Department of Community Affairs, Division of Codes and Standards [609] 984‐0040 ICC/ANSI A117.1‐ 2009 Obtained From: ANSI 11 West 42nd St New York, NY 10036 or International Code Council [888] 422‐7233 | Sept‐21‐2015 (w/tech. amendments at NJAC 5:23‐7.2) |

http://www.state.nj.us/dca/divisions/codes/codreg/

Grace Period

For a period of six months following the operative date of a subcode revision, applicants may submit a complete permit application, including all prior approvals, to be reviewed under the code in force immediately preceding the subcode revision. Provided that the application is complete, the construction official and applicable subcode officials shall perform the plan review and issue construction permit(s) based on the code in force immediately prior to the operative date of the subcode revision.  This grace period shall apply only to revisions of subcodes.

http://www.state.nj.us/dca/divisions/codes/codreg/ 5‐2018

Contact Information - Division of Codes and Standards

New Jersey Department of Community Affairs

Division of codes and Standards

101 South Broad St.

PO Box 802

Trenton, NJ   08625-0802

Director's Office:

Phone (609) 292- 7899 
Fax (609) 633-6729

Code Assistance:

Phone (609) 984-7609
Fax (609) 984-7717

Publications:

Phone (609) 984-0040

UNIFORM CONSTRUCTION CODE USE GROUPS

Assembly

| A-1-A | This Use Group shall include all theaters and other buildings used primarily for theatrical or operatic performances and exhibitions, arranged with a raised stage, proscenium curtain, fixed or portable scenery loft, lights, motion picture booth, mechanical appliances or other theatrical accessories and equipment, and provided with fixed seats. |
| --- | --- |
| A-1-B | This Use Group shall include all theaters without a stage and equipped with fixed seats used for motion picture performances |
| A-2 | This Use Group shall include all buildings and places of public assembly, without theatrical stage accessories, designed for use as dance halls, night clubs, and eating and/or drinking establishments, and similar occupancies, in which the established maximum permitted occupant load exceeds the number of seats provided by more than 30 percent, and shall include all rooms, lobbies and other spaces connected thereto with a common means of egress and entrance. |
| A-3 | This Use Group shall include all buildings with or without an auditorium in which persons assemble for amusement, entertainment or recreation, and incidental motion picture, dramatic or theatrical presentations, lectures or other similar purposes without theatrical stage other than a raised platform; and principally used without permanent seating facilities, including art galleries, exhibition halls, museums, lecture halls, libraries, restaurants other than night clubs, and recreation centers; and buildings designed for other similar assembly purposes including passenger terminals and places of worship. |
| A-4 | This Use Group shall include all buildings used arenas |
| A-5 | This Use Group shall include grandstands, bleachers, coliseums, stadiums, tents and similar structures for outdoor assembly uses |
| Use Group B | All buildings and structures, or parts thereof, shall be classified in Use Group B which are used for the transaction of business, for the rendering of professional services, or for other services that involve stocks of goods, wares or merchandise in limited quantities for use incidental to office uses or sample purposes |
| Use Group E | This Use Group shall include all buildings and structures serving 50 or more students from kindergarten through grade 12 and also means and includes any educational building serving 50 or more students in some, but not all, of the grades from kindergarten to grade 12, inclusive. |
| Use Group F | All buildings and structures, or parts thereof, in which occupants are engaged in performing work or labor in the fabricating, assembling or processing of products or materials shall be classified in Use Group F; including, among others, factories, assembling plants, industrial laboratories and all other industrial and manufacturing uses, except those of. |

| F—1 | Factory and industrial uses which are not otherwise classified as low-hazard Use Group F-2, shall be classified as a moderate-hazard factory and industrial use, Use Group F-1. |
| --- | --- |
| F—2 | Factory and industrial uses which involve the fabrication or manufacturing of non-combustible materials that, during finishing, packing or processing, do not contribute to a significant fire hazard, shall be classified as Use Group F—2. The following manufacturing processes are indicative of, and shall be classified as, Use Group F—2: beverages, nonalcoholic; brick and masonry; ceramic products; foundries; glass products; gypsum; ice; metal fabrication and assembly; and water pumping plants. |
| Use Group H | All buildings and structures, or parts thereof, shall be classified in Use Group H which are used for the manufacturing, processing, generation or storage of corrosive, highly toxic, highly combustible, flammable or explosive materials that constitute a high fire or explosion hazard, including loose combustible fibers, dust and unstable materials. |
| H-1 | High (H-1) to low (H-5) hazards |
| H-2 |  |
| H-3 |  |
| H-4 |  |
| H-5 |  |
| I-2 | This Use Group shall include all buildings used for housing people suffering from physical limitations because of health or age, including, among others, day nurseries, hospitals, sanitariums, infirmaries, orphanages and homes for aged and infirm. |
| I-3 | This Use Group shall include all buildings designed for the detention of people under restraint, including, among others, jails, prisons, reformatories, mental detention and similar uses |

| Use Group M | All buildings and structures, or parts thereof, shall be classified in Use Group M which are used for display and sales purposes involving stocks of goods, wares or merchandise incidental to such purposes and accessible to the public; including, among others, retail stores, motor fuel service stations, shops and sales-rooms and markets. |
| --- | --- |
| R-1 | This Use Group shall include all hotels, motels, and similar buildings arranged for shelter and sleeping accommodations and in which the occupants are primarily transient in nature, making use of the facilities for a period of less than 30 days. This definition shall also mean and include bed and breakfast guesthouses and bed and breakfast inns. |
| R-2 | This Use Group shall include all multiple family dwellings having more than two dwelling units and shall also include all dormitories, rooming houses, group rentals where the occupants are living independently of each other and similar buildings arranged for shelter and sleeping accommodations in which the occupants are primarily not transient in nature. This Use Group shall also include attached one- and two-family dwelling units which do not meet the definition for Use Group R—3. |
| R-3 | This Use Group shall include all buildings arranged for the use of detached one- and two-family dwelling units, including not more than five lodgers or boarders per family. This Use Group shall also mean and include: Bed and breakfast homestays; and ii. Attached one- and two-family dwellings constructed in accordance with the Uniform Construction Code requirements for multiple single-family dwellings. |
| R-4 | (IRC) This group shall include all detached one or two family dwellings not more than three stories in height and accessory structures as indicated in the IRC |
| S-1 | All buildings and structures, or parts thereof, which are used primarily for the storage of moderate hazard contents which are likely to burn with moderate rapidity, but which do not produce either poisonous gases, fumes or explosives; including, among others, warehouses, storehouses and freight depots |
| S-2 | All buildings and structures, or parts thereof, which are used primarily for the storage of noncombustible materials, and of low hazard wares that do not ordinarily burn rapidly such as products on wood pallets or in paper cartons without significant amounts of combustible wrappings; including, among others, warehouses, storehouses and freight depots. Such products may have a negligible amount of plastic trim such as knobs, handles or film wrapping. |
| Use Group U | - Building and structures of an accessory character and miscellaneous structures not classified in any specific use group shall be constructed, equipped and maintained to conform to the requirements of this code commensurate with fire and life hazard incidental to their occupancy. Shall include structures that are accessory to a residential occupancy; Agricultural buildings, livestock shelters, barns, detached garages, carports, greenhouses, grain silos, and Sheds greater than 2,500 sq.ft. |

Key terms

- Use Group

- Life Hazard use

- Uniform Fire Code

- Uniform Construction Code

Review Questions

- Define UCC

- Define UFC

- Define Use Group

- N.J.A.C means:

- N.J.S.A. means

- The UCC is designed and intended to be used for __________?

- The UFC is designed and intended to be used for ___________?

- T or F	The Uniform Fire Code is applicable to all building in the state of New Jersey.

HOMEWORK ASSIGNMENTS

This work is to be completed by the next class.

Readings:

- IFSTA 8th Ed. (FI&CE) Chapter 4

- N.J.S.A. 52:27D et seq.

Assignment

- IFSTA Worksheet #3

- On the sheet provided on the next page, list ten (10) buildings or structures in your home town or an area that you know.  Determine the building construction type, and the building code use group. The space entitled “LHU Code” should be left blank.

- Name________________________________________________               Assignment #2

| 1 | Building name | Building Construction |
| --- | --- | --- |
|  | Use Group | LHU Code |
| 2 | Building name | Building Construction |
|  | UCC Use Group | LHU Code |
| 3 | Building name | Building Construction |
|  | UCC Use Group | LHU Code |
| 4 | Building name | Building Construction |
|  | UCC Use Group | LHU Code |
| 5 | Building name | Building Construction |
|  | UCC Use Group | LHU Code |
| 6 | Building name | Building Construction |
|  | UCC Use Group | LHU Code |
| 7 | Building name | Building Construction |
|  | UCC Use Group | LHU Code |
| 8 | Building name | Building Construction |
|  | UCC Use Group | LHU Code |
| 9 | Building name | Building Construction |
|  | UCC Use Group | LHU Code |
| 10 | Building name | Building Construction |
|  | UCC Use Group | LHU Code |