- What is a riser on a sprinkler system?

  - A feeder which originates from a city or private water supply

  - The discharge of water from a sprinkler head at time of activation

  - Supplies water to a cross main that directly serves a number of branch lines

  - A devise to prevent water from backflowing into the potable water supply

- Which of the following are commonly used release mechanisms for sprinkler heads?

  - Chemical pellets

  - Fusible links

  - Glass bulbs

  - All – A, B & C

  - Only – B & C

- What temperature classification does a white colored sprinkler head represent?

  - Ordinary

  - Intermediate

  - Extra high

  - Ultra high

- What is the temperature rating of a sprinkler head with a red colored bulb?

  - 325°F - 375°F

  - 500°F - 575°F

  - 250°F - 300°F

  - 204°C - 246°C

- Which type of sprinkler system employs a deluge-type valve, fire detection devices and closed sprinklers?

  - Wet-pipe

  - Dry-pipe

  - Deluge

  - Preaction

- Which type of sprinkler system maintains air under pressure, which escapes upon sprinkler activation, and water is automatically admitted to the system?

  - Wet-pipe

  - Dry-pipe

  - Deluge

  - Preaction

- Which type of sprinkler system employs a deluge-type valve, fire detection devices and open sprinklers?

  - Wet-pipe

  - Dry-pipe

  - Deluge

  - Preaction

- Which of the following is considered an indicating control valve?

  - All – C, D & E

  - Only – C & D

  - OS&Y

  - PIV

  - WPIV

- Which of the following indicating valves displays a sign indicating “Open”, “Closed or Shut”?

  - OS&Y

  - PIV

  - Both A & B

  - Neither A nor B

- Which type of sprinkler head is designed to suppress fires in high challenge hazards such as warehouses?

  - Standard sprinklers

  - QRES

  - RES

  - ESFR

- Water mist sprinkler systems provide very fine mist of water to absorb large quantities of heat. Intermediate water mist systems provide operating pressures of __________.

  - 175 psi or less

  - 175-200 psi

  - 175-500 psi

  - 500-750 psi

- Fixed water spray sprinkler systems are often used to protect what type of hazards?

  - All – C, D & E

  - Only – C & E

  - Flammable & gaseous liquids

  - Transformers

  - Motors

- What clearance under sprinklers should be maintained from stock, lights or other objects to prevent obstruction of water discharge pattern?

  - 18 inches

  - 20 inches

  - 24 inches

  - 30 inches

- The minimum number of spare sprinkler heads required by NFPA 13 is __?

  - 24

  - 12

  - 6

  - 3

- What is the purpose of fire department connections as part of automatic sprinkler systems?

a.	Protect firefighters in the structure

b.	Allow for use of any secondary water supply

c.	Boost the pressure of the primary water supply

d.	Serve as a backup protection if sprinkler system fails

16.	Which of the following are used to turn off or isolate the water supply to the system when it is necessary to perform maintenance, change sprinklers, or interrupt operation?

a. Key locks

b.	 Remote sensing valves

c.	 Water flow control valves

d. 	Main system control panels

17.	Which type of valve is a gate valve that has a yoke on the outside with a threaded stem or screw?

a. 	Post indicator valve

b. 	Outside stem and yoke

c.	 Wall post indicator valve

d.	 Post indicator valve assembly

18.	Which type of valve is an underground gate valve that has a hollow metal post attached to the valve housing?

a. 	Post indicator valve

b. 	Outside stem and yoke

c.	 Wall post indicator valve

d.	 Post indicator valve assembly

19.	Which operating valve is located on the riser and used to flow water for testing the water flow alarm?

a. Drain valves

b. Check valves

c.	Alarm-test valves

d.	Drip check or drip ball valves

20.	Which valve is used to drain water from piping when pressure is relieved in the pipe?

a. Drain valves

b. Check valves

c. 	Alarm-test valves

d. 	Drip check or drip ball valves

21.	Which valve is usually a globe valve and may sometimes be found at a remote location within a sprinkler system?

a.	Secondary valve

b.	Main check valve

c.	Firefighter’s valve

d.	Inspector’s test valve

22.	Which are vertical sections of pipe that connect the underground supply to the rest of the piping in the system?

a. Hangers

b. Main loops

c. Branch lines

d. System risers

23.	The pipe schedule table method is limited to:

a.	All known hazards.

b.	Light and ordinary hazards.

c.	Occupancies under a certain size.

d.	Occupancies with no known risks.

24.	Which method allows for calculations to be made to protect all types of hazards, including extra hazards, with any type of piping and any type of sprinkler discharge device?

a.	Previous usage

b.	Pipe schedule tables

c.	Hydraulic calculations

d.	Correlation of hazards

25.	Which type of sprinkler is recessed and covered with a removable decorative cover plate that releases when exposed to a specific level of heat?

a. Flush

b. Pendent

c. Recessed

d. Concealed

26.	Which type of sprinkler is mounted in a ceiling with the body of the sprinkler, including the threaded shank, above the plane of the ceiling?

a. Flush

b. Pendent

c.	Recessed

d.	Concealed

27.	Which type of sprinkler is installed downward from the branch line so that water flowing from the sprinkler strikes the deflector and is distributed over the protected area?

a. Flush

b. Pendent

c. Recessed

d. Concealed

28.	Which type of sprinkler is installed upward from a line so that water is discharged up against the deflector?

a. Upright

b. Sidewall

c. Recessed

d. Concealed

29.	Which type of sprinkler is a type of spray sprinkler capable of producing characteristic large water droplets and is listed for its capability of providing fire control of specific high-challenge fire hazards?

a. 	Extended coverage

b. 	Early suppression fast-response

c.	 Control mode specific application

d.	 Quick-response early suppression

30.	Which type of sprinkler is designed to rapidly suppress fires in high-challenge hazards such as warehouses?

a.	Extended coverage

b.	Early suppression fast-response

c.	Control mode specific application

d.	Quick-response early suppression

31.	Which type of sprinkler is normally provided in a deluge sprinkler system?

a. Open Head

b. 	Early suppression fast-response

c.	 Control mode specific application

d.	 Quick-response early suppression

32.	Which fire suppression system provides protection to specific hazards by applying water droplets of a predetermined pattern, particle size, velocity, and density through specially designed nozzles?

a.	Water-mist system

b.	Foam-water system

c.	Combination system

d.	Water-spray fixed system

33.	Which fire suppression system uses a very fine spray that controls or extinguishes fire by displacing oxygen and blocking radiant heat production?

a.	Water-mist system

b.	Foam-water system

c.	Combination system

d.	Water-spray fixed system

34.	Water-mist fire suppression systems are designed to be operated at:

- Intermittent high and low pressures.

- The same pressures as standard sprinkler systems.

- Considerably lower pressures than standard sprinkler systems.

- Considerably higher pressures than standard sprinkler systems

35.	Water-mist fire suppression systems utilize air cylinders that must be hydrostatically tested before recharging if it has been more than:

a.	Three years since their last tests.

b.	Five years since their last tests.

c.	Seven years since their last tests.

d.	Ten years since their last tests.

36.	Water-mist fire suppression systems that utilize air cylinders that have not been discharged should be emptied and tested:

a.	 Annually.

b.	 Every five years.

c.	 Every seven years.

d.	 Every twelve years.

37.	Which fire suppression system is commonly used to protect Class B fire hazards and can be a wet-pipe, dry-pipe, preaction, or deluge system?

a. 	Water-mist system

b.	 Foam-water system

- Combination system

- Water-spray fixed system

38.	As part of the maintenance requirements of NFPA® 25, standpipe and hose fire suppression systems must be flow and hydrostatic tested:

a. Annually.

b.	 Every three years.

c.	 Every five years.

d.	 Every ten years.

39.	Which of the following is a component of standpipe and hose fire suppression systems?

a.	  Control panel

- Water purifying filter

- Pressure-regulating device

- Emergency backflow indicator

40.	Which classification of standpipe and hose fire suppression systems must be capable of supplying effective fire streams during the more advanced stages of fire within a structure?

a.  Class I

b.  Class II

c.	Class III

d.  Class IV