- Who shall issue a permit before any fire alarm; detection or suppression system is installed, enlarged or extended?

  - zoning officer

  - fire official

  - construction official

  - fire protection subcode official

- Who shall be notified before disconnection or interruption of protection and when tests, repairs, alterations or additions are started on fire protection systems?

  - fire chief

  - fire official

  - construction official

  - fire protection subcode official

- Who must approve the removal of nonrequired fire protection equipment?

  - fire official

  - business owner’s insurance carrier

  - fire protection subcode official

  - both A & C

- How shall disconnected nonrequired fire protection equipment be handled?

  - It shall be removed so as not to give a false indication that the area is protected

  - It shall be reconnected and placed back into service

  - It shall be evaluated by the business owner’s insurance carrier

  - It shall be returned to the installation vendor

- What standard is used for the inspection, testing and maintenance of smoke and heat vents?

  - NFPA 10

  - NFPA 12

  - NFPA 204

  - NFPA 750

- Which of the following is a step in the procedure for a preplanned impairment program?

  - The fire official and the fire department have been notified

  - Necessary tools and materials have been assembled on the impairment site

  - The areas or buildings involved have been inspected and the increased risk determined

  - All - A, B & C

  - Only A and C

- When restoring impaired equipment to normal working order, which of the following need to occur?

  - The impairment tag(s) have been removed

  - Necessary inspections and tests have been conducted to verify that effected systems are operational

  - The building owner/manger, insurance carrier, alarm company and other involved parties have been advised that protection is restored

  - All – A, B & C

  - Only A and C

- Automatic sprinklers shall be maintained with due regard for obstructions. Which of the following could be caused by an obstruction?

  - Delay of activation

  - Water pattern distribution problem

  - Excessive water damage

  - All of the above

  - A & B above

- Which of the following sprinkler system components require monitoring?

  - C, D & E – below

  - Only D & E

  - Water levels & temperature

  - Water flow switches

  - Critical air pressures

- Alarm, supervisory and trouble signals shall be distinctly different and shall be automatically transmitted to which of the following?

  - An approved central station

  - A remote supervisory station

  - A proprietary supervisory station

  - All - A, B & C

  - Only A & C

- Which Uniform Construction Code form is used to placard hydraulically calculated sprinkler systems?

  - C-217

  - F-380

  - F-278

  - S-447

- In dealing with fire protection systems, what devices may be interlocked to activate or shut-down?

  - Fuel shut-offs

  - Smoke & heat vents

  - Window shutters

  - All – A, B & C

  - None - interlocking is not allowed

- Which of the following is used for the installation of automatic fire-extinguishing systems?

  - The National Standard Plumbing Code

  - The fire official enforcing the Code

  - The fire protection subcode of the Uniform Construction Code

  - The building owner’s insurance policy

- Which of the following is used to monitor fire-extinguishing systems when a building fire alarm is installed?

  - NFPA 462

  - NFPA 77

  - NFPA 72

  - NFPA 30

- At what interval shall wet-chemical extinguishing systems be inspected and tested for proper operation?

  - 6-month

  - 3-month

  - 12-month

  - 4-month

- In addition to their listing, which standard is used to maintain, periodically inspect and test foam-extinguishing systems?

  - NFPA 11

  - NFPA 16

  - NFPA 11A

  - All – A, B & C

  - None of the above

- In addition to their listing, which standard is used to maintain, periodically inspect and test wet-chemical extinguishing systems?

  - NFPA 17

  - NFPA 17A

  - NFPA 2001

  - NFPA 12

- In addition to their listing, which standard is used to maintain, periodically inspect and test dry-chemical extinguishing systems?

  - NFPA 17

  - NFPA 17A

  - NFPA 2001

  - NFPA 12

- In addition to their listing, which standard is used to maintain, periodically inspect and test carbon dioxide extinguishing systems?

  - NFPA 17

  - NFPA 17A

  - NFPA 2001

  - NFPA 12

- Which of the following standards shall be used to inspect and test foam-extinguishing systems?

  - NFPA 2001

  - NFPA 12A

  - NFPA 25

  - NFPA 72

- At what time intervals shall all system hoses be tested for carbon dioxide systems?

  - 12 months

  - 5 years

  - 6 months

  - 10 years

- In dealing with a clean agent system, what percentage in loss of original weight of the extinguishing agent container shall cause the container to be refilled or replaced?

  - more than 5 percent

  - more than 10 percent

  - more than 2 percent

  - more than 15 percent

- Where shall the minimum and maximum manual means of activating the fire-extinguishing system for commercial cooking hood systems be located in relation to the kitchen exhaust system?

  - 48 inches ;  42 inches

  - 20 feet  ;  10 feet

  - 10 feet  ;  20 feet

  - 42 inches  ;  48 inches

- Where shall the manual actuation device for a commercial cooking hood fire-extinguishing system be located?

  - near the sink

  - immediately next to the stove

  - in the managers office

  - at or near a means of egress

- Which of the following standards is used to test pre-engineered wet chemical extinguishing systems used for commercial cooking?

  - UL 270

  - NFPA 204

  - NFPA 1492

  - UL 300

- What shall simultaneously occur with the activation of the fire-extinguishing system for a commercial cooking hood system?

  - shut off all HVAC

  - shut down the fuel or electrical supply to the cooking equipment

  - turn on all emergency lighting

  - turn on exhaust fans

- Which standard is used to test automatic sprinklers used for the protection of fryers?

  - UL 199E

  - UL 300FR

  - NFPA 12

  - NFPA 80

- What rating of portable fire extinguisher shall be used to protect cooking equipment involving vegetable or animal oils and fats?

  - Class D

  - Class B

  - Class K

  - Class C

- What is the maximum travel distance from commercial-type cooking equipment to a portable fire extinguisher?

  - 50-foot

  - 75-foot

  - 40-foot

  - 30-foot

- What minimum number of Class K portable fire extinguishers (1.5 gal min) shall be provided for six fryers with a maximum cooking medium capacity of 80 pounds each?

  - 1

  - 2

  - 3

  - 6

- How often shall fusible links and automatic sprinkler heads be replaced when used as part of a commercial cooking hood fire-extinguishing system?

  - 12 months

  - 3 months

  - 6 months

  - 24 months

- How often shall a commercial cooking hood fire-extinguishing system be serviced?

  - 12 months

  - 3 months

  - 6 months

  - 24 months

- What is the minimum size letter, in a color that contrast with the background, used to identify cabinets containing fire-fighting equipment such as standpipes, fire hose, fire extinguishers or fire department valves?

  - 1 inch

  - 2 inch

  - 4 inch

  - 6 inch

- Which of the following standard shall be used for the selection, installation and maintenance of portable fire extinguishers?

  - UL 300

  - NFPA 72

  - NFPA 13

  - NFPA 10

- Which of the following occupancies would require portable fire extinguishers?

  - A bank equipped throughout with quick response sprinklers

  - A high school wood shop equipped with quick response sprinklers

  - A warehouse storing metal parts

  - Only B & C

  - Only A & B

- What is the maximum travel distance allowed for fire extinguisher placement when dealing with a Class A fire hazard?

  - 3,000 square feet

  - 75 foot

  - 30 foot

  - 1,000 square feet

- What color shall manual fire alarm boxes be?

  - transparent

  - green

  - red

  - white

- When shall automatic fire-extinguishing systems be connected to the building fire alarm?

  - When required by the code in effect at the time the fire alarm was installed

  - When required by the construction official

  - When required by insurance regulations

  - When required by the fire chief

- Which of the following standards governs the maintenance / testing schedules and procedures for fire alarm and fire detection systems?

  - NFPA 16

  - NFPA 72

  - NFPA 10

  - NFPA 13

- How soon after installation shall smoke detector sensitivity shall be checked?

  - within one year

  - within two years

  - within five years

  - within ten years

- Who is responsible for ensuring that the fire and life safety systems are maintained in an operable condition at all times?

  - fire official

  - fire chief

  - fire protection subcode official

  - building owner

- How often shall an engine-driven generator, required to provide a secondary power source, be exercised under load for a minimum of ½ hour in a continuous period?

  - at least daily

  - at least weekly

  - at least monthly

  - every 15 days

- At what interval shall non battery-powered single-station smoke detectors be tested?

  - weekly

  - monthly

  - bi-weekly

  - bi-monthly

- At what interval shall battery-powered single-station smoke detectors, in other than R-1, R-2 and bed & breakfast homestays, be tested?

  - weekly

  - monthly

  - bi-weekly

  - bi-monthly

- At what interval shall batteries in single-station smoke detectors, In Group R-1, R-2 and bed & breakfast homestays, be changed?

  - as necessary

  - at change of occupancy

  - at least once per year

  - Only A & C

  - All - A, B & C

- What does an illuminated GREEN pilot-lamp-type indicator on the fire-fighter’s smoke control system’s panel indicates?

  - Fans, dampers and other operating equipment in their normal status

  - Fans, dampers and other operating equipment in their off or closed status

  - Fans, dampers and other operating equipment in their on or open status

  - Fans, dampers and other operating equipment in a fault status

- How often shall dedicated smoke control systems be operated for each control sequence?

  - monthly

  - bi-monthly

  - annually

  - semi-annually

- How often shall non dedicated smoke control systems be operated for each control sequence?

  - monthly

  - bi-monthly

  - annually

  - semi-annually

- Where shall explosion control be provided?

  - Where quantities of hazardous materials specified in Table 911.1 exceed the maximum allowable quantities in Table 2703.1.1(1).

  - Where the quantities of flammable liquids specified in Table 906.3(2) exceed the maximum allowable travel distance specified in Table 906.3(1)

  - Where a structure, room or space is occupied for purposes involving explosion hazards as identified in Table 911.1.

  - Only A & C

  - All – A, B & C

- Which reference standards would be useful in dealing with explosion control?

  - NFPA 68, 69, 495

  - NFPA 70, 72, 2001

  - UL 300, 1515. 701

  - NFPA 31, 17, 17A

- Which of the following would require barricade construction explosion control method?

  - Explosives Division 1.3

  - Flammable gas - Gaseous

  - Oxidizer liquids and solids

  - Only A & C

  - All – A, B & C

- Which of the following would require a deflagration venting or deflagration prevention system explosion control method?

  - Unstable (reactive)   3 detonable

  - Pyrophoric

  - Explosives Division 1.1

  - Only A & C

  - All – A, B & C

- What is the minimum distance required to provide immediate access to fire department connections without obstruction by any object?

  - 20 feet

  - 10 feet

  - 5 feet

  - 3 feet

- Which of the following standards governs the periodic inspection, test and maintenance of all fire department connections?

  - NPPA 13

  - NPFA 25

  - NFPA 14

  - NFPA 20

- Which of the following standards shall govern the maintenance of fire pumps?

  - NFPA 13 & 14

  - NFPA 17 & 17A

  - NFPA 30 & 31

  - NFPA 20 & 25

- What is the minimum temperature that shall be kept within a pump room?

  - 32 degrees

  - 35 degrees

  - 40 degrees

  - 45 degrees

- Which of the following methods shall be employed to supervise, in the open position, fire pump valves and backflow prevention device or assembly?

  - Any of the following could be used – B, C, D or E

  - Central-station, proprietary or remote-station signaling service

  - Local signaling service that will cause the sounding of an audible signal at a constantly attended location

  - Locking valves open

  - Sealing of  valves and approved weekly recorded inspection where valves are located within fenced enclosures under the control of the owner

- What position shall the fire pump test outlet valves normally be in?

  - left in “churn” mode

  - supervised in the closed position

  - supervised in the open position

  - None of the above - there should be no valve in the fire pump test outlet

- Which of the following standards govern periodic test of engine generator sets supplying emergency or standby power to fire pump assemblies?

  - NFPA 13

  - NFPA 20

  - NFPA 25

  - NFPA 110

- Which of the following standards govern periodic tests and inspection of private water tanks and water service mains?

  - NFPA 13

  - NFPA 20

  - NFPA 25

  - NFPA 110

- How often shall elevators be tested for recall and emergency or fire service operation?

  - monthly

  - bi-monthly

  - semi -annually

  - annually