- Who shall determine, in a disputed case, whether a Uniform Construction Code requirement involves fire safety?

  - the Division of Fire Safety

  - the fire official

  - the Division of Codes and Standards

  - the construction official

- What is the meaning of the phrase “current fire safety requirements” as set forth in Subchapter 4?

  - the New Jersey Uniform Construction Code in effect at time of adoption of the requirement

  - the New Jersey Fire Safety Act as defined within Subchapter 1

  - the New Jersey Uniform Fire Code as defined within Subchapter 3

  - the New Jersey Uniform Construction Code currently in effect

- Who shall determine the applicability of the provisions of Subchapter 4 to existing buildings identified or classified as historic buildings?

  - the local historical commission

  - the local construction code enforcing agency

  - the State of New Jersey Division of Historical Structures (NJDHS)

  - the fire official

- With regard to Subchapter 4, a variation previously granted to a provision of an existing code shall remain in effect subject to which of the following?

  - Finding that equivalent life safety was provided

  - The variation was granted through a formal process or procedure

  - The variation was granted in writing

  - All of the above – A, B & C

  - None of the above – previously issued variations have no standing with regard to Subchapter 4.

- Who must a comprehensive facility fire protection plan be submitted to for approval?

  - The local construction code enforcing agency

  - The Division of Fire Safety

  - The Division of Codes and Standards

  - The fire official

- Which of the following are required to be submitted in a comprehensive facility fire protection plan?

  - All – C, D & E

  - Only D & E

  - Which building the plan is being submitted for

  - A timetable for compliance

  - A written application for a variance for any deviations from Subchapter 4

- The approving agency shall approve or disapprove a comprehensive facility fire protection plan in how many days?

  - 60

  - 30

  - 120

  - 90

- Which of the following criteria requires the installation of a fire suppression system within an A-2 occupancy?

  - All – C, D & E

  - Only – D & E

  - The A-2 occupies a space of 4,000 square feet.

  - A-2 located on a floor with the level of exit discharge more than five feet above adjacent grade

  - The A-2 has an occupant load of 325

- Which of the following criteria requires the installation of a fire suppression system within an I-1 occupancy?

  - I-1 Use 2 stories high

  - I-1 Use with less than 20 staff

  - I-1 Use greater than 2 stories high

  - I-1 Use excluding staff

- Which of the following criteria requires the installation of a fire suppression system within an I-2 occupancy?

  - I-2 occupancy within a three story building of Type 2A construction.

  - I-2 occupancy serving a first floor day nursery having direct access to approved exterior exits

  - I-2 occupancy located on the first floor of a building of Type 2B construction.

  - I-2 occupancy located on the second floor of a building of Type 2B construction.

- Which of the following criteria requires the installation of a fire suppressions system?

  - All – C, D & E

  - Only C & D

  - Tempering tanks over 500 gallons

  - Dip tank with 11 square feet of liquid surface

  - Hardening tank with 27 square feet of liquid surface

- When is a fire suppression system required in a windowless story?

  - below the sixth story

  - below the seventh story

  - below the eight story

  - below the tenth story

  - any place called a basement

- A story is still considered windowless when openings are provided on only one side and the opposite wall of such story is more than how many feet from such openings?

  - 25

  - 50

  - 75

  - 100

- What is the minimum fire resistance separating mixed uses  rating In building containing mixed uses (not including Group H) where one or more requires automatic fire suppression, suppression will not be required throughout the entire building provided the uses are separated by fire resistive construction having a minimum of what hour rating?

  - 2

  - .5

  - 1 1/2

  - 1

- What is the height, in stories, when wet standpipes are required for all buildings having floors for human occupancy?

  - six

  - seven

  - eight

  - ten

- What must standpipes be capable of accepting, by fire department apparatus, to the topmost standpipe outlet in buildings without an automatic fire suppression system?

  - 250 gpm @ 75 psi

  - 500 gpm @ 65 psi

  - 500 gpm @ 75 psi

  - 250 gpm @ 65 psi

- Which of the following is correct concerning alarms and Use Group I?

  - All – C, D & E

  - Only – C & E

  - Automatic fire alarms are required in all buildings of Use Group I

  - Alarms systems in buildings of Use Group I need not be supervised

  - In Buildings of Use Group I, smoke detectors located within five feet to a kitchen or bathroom shall be ionization type only.

- Who must give their approval to substitute heat detectors for smoke detectors in location where frequent nuisance alarms may occur?

  - The fire official

  - The construction official

  - The fire chief

  - The fire protection subcode official

- What is the minimum number of occupants needed in a rooming house for the requirement of an approved smoke detection system located in all interior common areas?

  - six

  - eight

  - ten

  - twelve

- All buildings of Use Group R-1 shall have at least one visual alarm type smoke detector, for the hearing impaired, for each group of how many units or fraction thereof?

  - 25

  - 50

  - 60

  - 75

- Automatic fire alarms, required by Subchapter 4, shall be supervised by which of the following?

  - All – B, C, D & E

  - Local alarm only

  - As required by the fire official

  - No monitoring required

  - None of these

- Manual fire alarms are required in which of the following?

  - All – C, D & E

  - Only – C & D

  - In all buildings required to have an automatic fire alarm system except hotels and multiple dwellings with an occupant load less than 25 and less than 10 dwelling units

  - In all building three stories in height having an occupant load of 25 or more

  - In all buildings of Use Group E with an occupant load of 12 or more

- Every story utilized for human occupancy having an occupant load of 501 to 1,000 shave have a minimum of how many exits?

  - Two

  - Three

  - Four

  - Five

- How many exits are required from a mezzanine with a travel distance of 85 feet

  - One open and one enclosed

  - Three

  - Two

  - One

- Which of the following shall be used as guidance when constructing and installing a new fire escape?

  - Uniform Construction Code Formal Technical Opinion No. FTO-3

  - Uniform Construction Code Formal Technical Opinion No. FTO-13

  - N.J.A.C. 5:71-3.7(g) Report No. FSL-250

  - N.J.A.C. 5:72-3.5(a)1

- What is the maximum sill height for approved windows in sleeping rooms of bed and breakfast homestays?

  - 24

  - 36

  - 44

  - 48

- How many exits are required in an open parking structure where vehicle are mechanically parked?

  - One open and one enclosed

  - One

  - Two

  - Three

- Which of the following require only one means of egress?

  - Use Groups R-1 and R-2 from floors that are not more than 18 feet above exterior grade

  - In community residences for developmentally disabled with a maximum occupant load of 16, including staff

  - Use Group R-3

  - All – A, B & C

  - Only – B & C

- How many exits are required for a storage room with a travel distance of 75 feet and an occupant load of 10?

  - One

  - Two, if its storing combustibles

  - Two regardless of contents

  - Three, if it is within Use Group M

- When computing an occupant load, what is the number of occupants, per egress unit, for Use Group B with a fire suppression system using stairways?

  - 60

  - 100

  - 150

  - 90

- When computing an occupant load, what is the number of occupants, per egress unit, for Use Group I-2 without a fire suppression system using a ramp?

  - 45

  - 30

  - 22

  - 100

- When computing an occupant load, how are fractions of a unit of egress width more than 12 inches counted?

  - As a full unit

  - 25% of a unit

  - 50% of a unit

  - 50 occupants

- What is the unit of egress width?

  - 22 inches

  - 12 inches

  - 50 occupants

  - 100 occupants

- What is the occupant load factor in square feet for a dance floor in an Assembly Use?

  - 15 net

  - 5 net

  - 100 gross

  - 7 net

- What is the occupant load factor for fixed seating without dividing arms?

  - One person per 24 inches

  - One person per 18 inches

  - One person per 3 net

  - 5 persons for every booth

- What is the occupant load factor in square feet for the basement of Use Group M?

  - 50 net

  - 100 gross

  - 30 gross

  - 60 gross

- What amount of egress capacity, regardless the total number of exits, is required for the main entrance in all buildings of Use Group A with an occupant load of 100 or more?

  - Two-thirds

  - One-half

  - 100 percent

  - 72 inches

- What is the maximum length of a dead end corridor?

  - 35 feet

  - 40 feet

  - 45 feet

  - 60 feet

- What is the maximum length of a dead end corridor in buildings equipped throughout with an automatic fire alarm system?

  - 35 feet

  - 40 feet

  - 45 feet

  - 50 feet

- What is the maximum length of a dead end corridor in buildings equipped throughout with an automatic fire suppression system?

  - 35 feet

  - 70 feet

  - 50 feet

  - 40 feet

- Which of the following is required to provide means of egress lighting and also illuminate the exit discharge?

  - All – C, D & E

  - Only – D & E

  - Use Group B

  - Use Group F

  - Use Group S

- To what level may the intensity be reduced, at floor level, for means of egress lighting within Use Group A-1 during periods of projection?

  - 0.2 foot candle

  - 1.0 foot candle

  - 0.5 foot candle

  - 1.5 foot candle

- In case of primary power loss, spaces required to have more than one exit shall assure the illumination of means of egress lighting for what time span?

  - Not less than 1 ½ hours

  - Not less that 2 hours

  - Not less than ¾ hours

  - Not less than 1 hour

- When must means of egress door swing in the direction of egress?

  - In all Use Groups with the exception of R

  - When the door serves an area with an occupant load greater than 50

  - When the door serves an area with an occupant load greater than 100

  - When the door is part of a fire wall or fire barrier

- In which of the following is panic hardware required?

  - Buildings, rooms or spaces with an occupant load greater than 100

  - Exit doors equipped with latching devices

  - Building or portions thereof Use Group A

  - All – A, B & C must be present

  - Only – B & C need to be present

- A guard is required for every open portion of a stair, landing or balcony when each is how many inches above the floor or grade below?

  - 24 inches

  - 48 inches

  - 30 inches

  - 66 inches

- What percentage of interior trim may be present as an aggregate of the wall and ceiling area before it must be considered interior finish?

  - Up to 10 %

  - Up to 25%

  - A maximum of 1/28 inch in thickness

  - 100% - if flame spread rating exceeds 450

- What is the corresponding flame spread rating for Class II as determined by ASTM E84?

  - 76-200

  - 26-75

  - 0-25

  - smoke development rating of 250

- What is the interior finish requirement for an exit access enclosure for Use Group E?

  - No minimum

  - Class III

  - Class I

  - Class II

- What is the interior finish requirement for an exit enclosure for Use Group M?

  - No minimum

  - Class III

  - Class I

  - Class II

- What is the interior finish requirement for a room or space for Use Group B?

  - No minimum

  - Class III

  - Class I

  - Class II

- What fire resistance rating of enclosures is required for interior stairways and other vertical openings connecting four to six floor levels?

  - Not less than two hours

  - Not less than one hour

  - Not less than 30 minutes

  - Fire resistance rating not required

- What fire resistance rating of enclosures is required for interior stairways and other vertical openings not exceeding three stories in Use Group B with a floor area not exceeding 3,000 square feet per floor?

  - Not less than two hours

  - Not less than one hour

  - Not less than 30 minutes

  - Fire resistance rating not required

- What fire resistance rating of enclosures is required for interior stairways and other vertical openings not exceeding three stories in Use Group R-1?

  - Not less than two hours

  - Not less than one hour

  - Not less than 30 minutes

  - Fire resistance rating not required

- An informational sign designating the floor level shall be posted at each floor landing in all interior stairways for buildings of what height?

  - More than one story

  - More than two stories

  - More than three stories

  - More than six stories

- What size wire glass panels may be used in smoke barriers?

  - Not exceeding 100 square inches

  - Not exceeding 572 square inches

  - Not exceeding 1,296 square inches

  - Not exceeding 1,500 square inches

- Smoke barriers shall have what fire resistance rating?

  - Not less than one-half hour

  - Not less than 20 minutes

  - Not less than one-hour

  - No fire resistance rating required – only smoke tight

- Use Group I-2 occupancies shall be divided by smoke barriers into which of the following?

  - All – C, D, & E apply

  - Only D & E apply

  - Each smoke compartment shall have a maximum 30 occupants with sleeping arrangements

  - Each smoke compartment shall not exceed 22,500 square feet

  - Each smoke compartment shall not exceed 150 feet in length and width

- Boiler/furnace equipment rooms within day nurseries shall have assemblies of what fire resistance rating?

  - Smoke partition

  - Not less than two hours

  - Not less than one and one-half hours

  - Not less than one hour

- Which of the following require carbon monoxide detectors?

  - All – A, B & C

  - Only - C & E

  - Use Group I-1

  - Use Group E

  - Use Group R-1, R-2, R-3, R-4