- Who shall be responsible for the safety of all persons in, or occupying, such premises with respect to the adequacy of means of egress?

  - fire official

  - owner

  - tenant

  - fire chief

- Who shall check egress facilities before such building is occupied to determine compliance with egress requirements?

  - owner

  - fire official

  - operator or person responsible

  - inspector

- Where shall the posting of the occupant load shall be located?

  - at the main entrance to the building

  - within the managers office

  - within the maintenance office

  - near the main point of egress from the room or space

- Which of the following is correct concerning occupant load signage design?

  - It match the décor of surroundings so as not to create a nuisance

  - It be a minimum  ¾” red letters on a white background

  - It be of contrasting colors

  - It be a bright and reflective isosceles triangle

- What must first be obtained before a means of egress is altered affecting its capacity?

  - a permit from the construction official

  - a variance from the zoning board

  - a permit issued in accordance with subchapter 2 of the Uniform Fire Code

  - approval of the businesses insurance provider

- Who provides approval for occupancy when a structures or portions of structures undergoing repair, renovation, alteration or reconstruction?

  - the fire official

  - the construction official

  - the businesses insurance provider

  - the fire chief

- Who shall receive an approved copy of an Egress Plan as required by section 404.2?

  - the fire chief

  - the police chief

  - the office of emergency management

  - the fire official

- When must a means of egress be illuminated?

  - at all times

  - only at night

  - at all times the building space served by the means of egress is occupied

  - Only at night when the building space served by the means of egress is occupied.

- When is the exit discharge required to be illuminated?

  - when the exit door is opened

  - only at night

  - at all times the building space served by the means of egress is occupied

  - Only at night when the building space served by the means of egress is occupied.

- Where is horizontal-sliding or vertical security grilles permitted?

  - at  loading docks only

  - at the main exit

  - where approved by the fire official

  - never permitted

- How are horizontal-sliding grills or vertical security grills openable?

  - only by remote control

  - only from the exterior

  - from the outside without the use of a key or special knowledge or effort during periods that the space is occupied

  - from the inside without the use of a key or special knowledge or effort during periods that the space is occupied

- When may security grills be closed?

  - during business hours if the gate is staffed and operated by a security guard

  - ½ hour of business closing to prevent additional customers from entering

  - must remain secured in the full-open position during the period of occupancy by the general public

  - other exits remain open

- How shall a means of egress doors be operated?

  - readily openable from the egress side without the use of a key or special knowledge or effort

  - by panic hardware

  - by a non-confusing method without the use of a key

  - automated whenever possible and non-latching

- Where is a bolt lock (manually operated flush bolt) permitted?

  - main entrance door

  - where the door hardware is compliant with the uniform construction code

  - rest room

  - rear egress door for security purposes only

- Where is a manually operated surface bolt permitted?

  - the rear egress door if needed for security purposes

  - any means of egress door

  - only egress doors within retail spaces to suppress shoplifting

  - doors not required for egress in individual dwelling units or sleeping units

- As a general rule, what best applies to interior stairway means of egress doors?

  - They must be of a distinguishable color to aid in identification

  - They shall be openable from both sides without the use of a key or special knowledge or effort

  - They shall be openable from the egress side without use of key or special knowledge or effort

  - They must connect every floor level

- Interior stairway means of egress doors, in buildings greater than four stories, are   permitted to be locked from the side opposite the egress side if what is provided?

  - they are openable from the egress side

  - they capable of being unlocked simultaneously without unlatching upon a signal from the fire command center – if present

  - they are sent a signal by emergency personnel from a single location inside the main entrance to the building

  - Answers A, B & C

  - Only A

- Locks or fasteners are allowed on means of egress doors in which of the following?

  - where the door hardware is in compliance with the Uniform Construction Code

  - in mental institutions where properly trained personnel are continuously on duty and approved provisions made to remove occupants safely in case of emergency

  - in problem security areas where approved special-purpose door alarms or locking devises are installed

  - Answers A, B & C

  - Only A

- What must be checked concerning hardware and special performance features for means of egress doors?

  - They must be inspected for proper operation and operates as required by code

  - They needs no further inspection after installation because of its U.L.©  listing

  - They needs no further inspection after construction office issues a C of O

  - They must only be inspected if door or door assembly is damaged

- Panic hardware is required in which of the following?

  - assembly or educational occupancies

  - assembly or educational occupancies with an occupant load of greater than 100

  - assembly or educational occupancies with an occupant load of greater than 100 with latching devices on means of egress doors

  - Answers A, B & C

- Which of the following requires panic hardware?

  - Automobile dealership showroom with glass doors and an occupant load of 57

  - A church’s, grade level, multipurpose room with an occupant load of 124 with side-swing latching doors

  - A Business basement level, with side swinging non-latching doors an occupant load of 152

  - A department store with automatically opening glass doors and an occupant load of 278

- Which of the following requires panic hardware?

  - An office building’s sixth story cafeteria with side swing latching doors and an occupant load of 187

  - An office building’s sixth story training room with glass side swing non latch doors and an occupant load of 187

  - Both A & B

  - Neither A nor B

- Which of the following best describes how to hold open a means of egress door, which also serves as a fire door?

  - It may be held open by use door stops wedges and similar devices

  - It may not be held-open by any device for any reason

  - It may be held open by only a fusible link

  - It may be held open by an automatic-closing device actuated by automatic fire detectors

- When are exit signs required to be illuminated?

  - only when an occupant load of 100 or greater is present in the room or space

  - only at night

  - at all times the structure is occupied

  - only in the rooms in which people gather

- What are supplemental directional exit signs used for?

  - increasing the intensity or brightness of the exit signs

  - indicating the direction and way of egress

  - providing a back-up system to be used when primary exit signs fail

  - they are used only on the exterior to light the exit discharge

- Floor identification signs shall be provided at each floor landing in all interior stairways more than how many stories above grade?

  - two

  - three

  - four

  - five

- What shall happen to all doors, passageways and stairways that are neither a means of egress nor provide access to a means of egress and are so arranged to be mistaken for a means of egress?

  - They shall be properly identified as to their purpose

  - They shall be brought into compliance as a means of egress through the retrofit provision of subchapter four of Uniform Fire Code

  - They shall be brought into compliance as a means of egress through the rehab provisions of subchapter six of the  Uniform Construction Code

  - They shall be left Status Quo in accordance with the Grandfather provision clauses

- When is exit signage allowed to be obscured?

  - when it interferes with the décor of the room

  - when it detracts from theatrical or musical performance

  - when approved by the fire official

  - never; nothing is to distract attention from the exit sign

- What is the minimum aisle width in a department store with an occupant load of 178?

  - 36”

  - 42”

  - 44”

  - 50”

- What is the minimum aisle width in a dentist office with an occupant load of 6?

  - 36”

  - 42”

  - 44”

  - 50”

- What is the minimum aisle width for the cafeteria of the local high school with an occupant load of 278?

  - In accordance with the International Building Code©

  - In accordance with the Rehab provisions of subchapter six of the Uniform Construction Code of the State of New Jersey

  - 44”

  - In accordance with the construction code in effect at the time of first occupancy

- Exits and exit enclosures can be used for which of the following?

  - Housing non combustible vending machines

  - Shall not be used for any purpose other than a means of egress

  - A single student with chair/desk combination measuring not greater than 24”may be allowed within the exit enclosure so long as the chair/desk is readily movable

  - File storage

- Who or what determines the exit capacity?

  - The architect of record

  - The owner

  - The construction code in effect at the time of first occupancy or N.J.A.C. 5:70-4 et seq.

  - the fire official

- When is combustible materials allowed to be stored within a means of egress, elevator hoistway or the bottom of a stairway?

  - When approved by the building owner’s insurance carrier

  - When approved by the fire official

  - When in noncombustible storage containers

  - When it is enclosed and protected as required by the construction code in effect at the time of first occupancy.

- When is emergency escape and rescue openings allowed to have bars grills or similar devices in place over the openings?

  - When the minimum net clear opening size complies with the code that was in effect at time of construction

  - When such devises shall be releasable or removed from the inside without the use of a key, tool or force greater than that which is required for normal operation of the window

  - Neither A no B – bars & grills are not allowed

  - Both A & B are required